3GPP TSG-RAN WG4 Meeting #115                                        R4-2507250

Malta, MT, 19th – 23rd May, 2025

Source:	Samsung

Title:              General discussion on XR RRM requirement

Agenda item:			7.30.3

Document for:	Discussion

1. Introduction

In last meeting, RAN4 had some extensively discussions on XR. In this meeting, we will give some further discussions for some open issues captured in the agreed WF especially semi-static schemes [1].

2 Discussion

2.1 Formulas for measurement delay extension

The following agreements were achieved in last meeting

<Way forward>: - Option 1: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - Max(200 ms × CSSFinter, 8 × Max(MGRP , SMTC period) × CSSFinter + Nskip × Max(MGRP , SMTC period)), - where Nskip is the number of skipped MG occasions in the measurement period. - Use this formula as example and follow similar approach for the other cases. - Option 2: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - Max(200 ms, ( 8 + Nskip) Max(MGRP , SMTC period)) CSSFinter - where Nskip is the number of skipped MG occasions in the measurement period. - Use this formula as example and follow similar approach for the other cases. - Option 3: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - Max(200 ms, ( 8 + Nskip) Max(MGRP , SMTC period)) CSSFinter - where Nskip is the number of skipped MG occasions in the measurement period. - Use this formula as example and follow similar approach for the other cases. - FFS: CSSFinter

From our understanding, in the NR-U scenario, the UE is not expected the regular reception of RSs due to the uncertainty of the outcome LBT procedure. From this, the RRM measurement and reporting will be impacted by the missed measurement opportunities of RSs due to LBT failure. While, to maintain the same accuracy, the UE still needs to obtain the same/sufficient number of measurement samples when performing measurement. Accordingly, the measurement period is extended based on those added samples/measurement opportunities. Follow this logic, seems option 2 is more reasonable, since it is +Nskip rather than Nskip ×, i.e., scaling fator. But more details: e.g., 1) The maximum number of skipped MG occasions in the measurement period; 2) CSSFinter should be discussed further

Proposal 1: Take the formula: Max(200 ms, Ceil(( 8 + Nskip ) * Kgap)  Max(MGRP , SMTC period))  CSSFinter as starting point to define the measurement delay extension due to measurement skipping for inter-frequency measurements with gap

- where Nskip is the number of  skipped MG occasions in the measurement period.

- Use this formula as example and follow similar approach for the other cases.

- FFS: CSSFinter

2.2 semi-static schemes

From our understanding, DCI solution is enough, no need to consider extra solutions including RRC and/or MAC-CE.

- RRC is not needed. Periodic pattern is configured based on long-term XR traffic status, while in practice, the XR traffic is likely to vary over time, and the MG occasion skipping is somewhat like “one-shot” pattern, and considering the unpredictable, quasi-periodic arrival and jitter characteristics of the XR data, limited and stable pre-defined MG occasion skipping pattern can not suit all the PDB. In this sense, RRC pattern-based become meaningless. And RRC also has an additional relatively long signalling processing time compared with DCI indication. And there is also ambiguity on the exact time for applicability.

- There is no restriction that a gNB cannot be updating the RRC frequently, theoretically, if an RRC re-configuration is in every PDSCH, there is no difference from DCI.

- No advantage is foreseen for MAC-CE. Since DCI has been agreed so there is a dynamic signalling based solution. Such redundant introduction of MAC-CE bitmap for skipping indication will increase unnecessary spec workload and UE complexity.

- Some companies have concerns that although 1 bit dynamic indication is very suitable for the XR data from NW side, it is hard to determine the measurement period/delay requirement from UE side. However, from our understanding, the same issue was happned in NR-U scenario, the same principle can be considered to define the requirements, that is issue 3-2, accordingly, it is more appropriate to put more efforts on solving such issue.

- Some company argue that an UE optional capability can introduce. While, there are two directions, if UE optional capability is considered, UE side benefits can be obtained, but for NW-UE direction, if RRC is introduced, from gNB implementation perspective, it will be further complicated actually

Proposal 2: Do not consider semi-static solutions

3 Conclusion

In this contribution, we provided our viewpoints on XR requirement related issues. The following observations and proposals are obtained:

Proposal 1: Take the formula: Max(200 ms, Ceil(( 8 + Nskip ) * Kgap)  Max(MGRP , SMTC period))  CSSFinter as starting point to define the measurement delay extension due to measurement skipping for inter-frequency measurements with gap

- where Nskip is the number of skipped MG occasions in the measurement period.

- Use this formula as example and follow similar approach for the other cases.

- FFS: CSSFinter

Proposal 2: Do not consider semi-static solutions

6 References

[1] R4-2504911, “WF on RRM requirements for NR_XR_Ph3”, Nokia