3GPP TSG-RAN WG4 Meeting #115                                    R4-2507251

Malta, MT, 19th – 23rd May, 2025

Source:	Samsung

Title:              Discussion on NES common signal adaptation

Agenda item:			7.27.3.2

Document for:	Discussion

1. Introduction

In last meeting, RAN4 had some discussions on NES common signal adaptation. In this meeting, we will give some further discussions for some open issues captured in the agreed WF [1].

2 RAN1 agreements update (SSB related) and analysis

#120b From Wednesday session Agreement For adaptation of SSB in time-domain, for DCI 2_9-based SSB burst periodicity adaptation for an SCell, - The DCI is scrambled a new RNTI, - Same search space and DCI size as that of cell DTX/DRX DCI if gNB configures both Agreement For DCI 2_9-based SSB burst periodicity adaptation for an SCell, - the starting location of the information block for SSB burst periodicity indication for a SCell within the DCI format 2_9 is configured using a new RRC parameter, - the length of the information block is given by ceil(log2(1+X)), where UE is configured with X additional SSB burst periodicities for the SCell. Agreement For adaptation of SSB in time-domain, UE can be configured with X (<=Xmax) additional SSB burst periodicities for an SCell. - Xmax=2 From Friday session Working Assumption When a UE receives in slot m on the active DL BWP of a first serving cell a PDCCH providing DCI format 2_9 that indicates a change in SSB burst periodicity of the SSB transmission on a second serving cell, the UE assumes SSB is transmitted on the second serving cell according to the indicated SSB burst periodicity from the beginning of the first slot containing the first [actually] transmitted SSB within the first [possible] SSB burst according to the indicated SSB burst periodicity that is no earlier than the slot m+d of the first serving cell where d is a number of slots for the SCS of the active DL BWP of the first serving cell [in Table 11.5-1 of TS 38.213]. - FFS: how to determine the first [possible] SSB burst.

3 RAN2 agreements update (SSB related) and analysis

Agreements on common channel adaptation - Not support MAC CE based signalling to indicate SSB adaptation in addition to DCI agreed in RAN1. - For SSB adaptation, multiple additional SMTC configurations are configured to UE via RRC, together with the mapping between SSB burst periodicity and SMTC configuration. - Upon reception of DCI format 2_9 for SSB burst periodicity switching, UE selects one SMTC configuration accordingly.

Observation 1：Only DCI based signalling, i.e., DCI format 2_9 scrambled with new RNTI is agreed to indicate SSB burst periodicity adaptation

Proposal 1: RAN4 not to define RRM requirements for SSB adaptation for NW triggered MAC-CE based signalling

4 Discussion

4.1 Time location discussion

Regarding time domain location (SFN offset and half frame index) of the SSB bursts, based on the relationship between SSB occasions with larger periodicity and shorter periodicity, the following cases may occur:

Fig.1 Time domain location of the SSB bursts for SSB occasions with different periodicities

Case 1: Periodicity: LargeShort. HF index is same before and after SSB adaptation. SFN offset is same before and after SSB adaptation.

Case 2: Periodicity: ShortLarge. HF index is same before and after SSB adaptation. SFN offset is same or different before and after adaptation.

Different from OD-SSB has concrete agreements that the time domain location of OD-SSB can be explicitly provided by RRC, it is still kept open in RAN1 that

- Introduce new RRC signalling for offset to SSB along with adapted SSB periodicity or

  - Main benefits: UE does not need to perform blind detection (timing-wise) with such explicit indication

- It is up to UE implementation to blind detect or

- Leverage SMTC configuration.

Proposal 2: For SSB adaptation, RAN4 to define requirements considering the following cases

- HF index is same before and after SSB adaptation and SFN offset is same before and after SSB adaptation

- HF index is same before and after SSB adaptation and SFN offset is same or different before and after adaptation

  - Wait for RAN1 conclusions on whether additional RRC signalling for offset of SSB for additional SSB periodicity is needed

4.2 DCI-based SSB burst periodicity adaptation for SCell

Applicable scenarios analysis—RAN1

In RAN1, different companies have shown different views on the applicable scenario for DCI-based indication, prevailing opinions are captured as below

- No additional restriction on scenarios (Scenario 2/2A/3A/3B) （some companies supported: Scenario 2/2A/3B）

  - The DCI 2_9 can be sent on a UE’s PCell, cross-carrier scheduling is considered

  - Inefficient for NW to align all the served UE in the same scenario, and different UEs receiving the DCI 2_9 may not be in the same scenario

- Applicable only for activated SCell (Scenario 3B)

  - Key arguments:

TS 38.321 5.9 Activation/Deactivation of SCells 1> if the SCell is deactivated: 2> not transmit SRS on the SCell; 2> not report CSI for the SCell; 2> not transmit on UL-SCH on the SCell; 2> not transmit on RACH on the SCell; 2> not monitor the PDCCH on the SCell; 2> not monitor the PDCCH for the SCell; 2> not transmit PUCCH on the SCell.

Adaptive SMTC analysis—RAN2

Based on the agreements achieved in RAN2, we have following observations:

- (RAN2 preference) Keep SMTC based L3 RRM framework, introduce additional SMTC configuration according to SSB adaptation for L3 RRM measurement on SCell with SSB adaptation

- NW may use DCI 2_9 to switch the SSB burst periodicity for SSB adaptation for the SCell, and the DCI 2_9 can be repurposed for switching SMTC configurations

  - Mapping relationship between additional SSB burst periodicity and additional SMTC configuration(s) via RRC is considered

  - Upon receiving the DCI 2_9 for switching the SSB periodicity to one of X additional SSB burst periodicities (Xmax=2) for the SCell, the SMTC configuration for L3 RRM measurement on SCell is updated, accordingly

Based on above observations, we have the following proposals:

Proposal 3: From RRM requirement perspective, suggest to define activated SCell L3 measurement requirements with priority

- Upon receiving DCI format 2_9, UE to perform L3 measurement based on additional SMTC configuration according to SSB adaptation

Proposal 4: If RAN4 to define L3 measurement requirements on deactivated SCell with SSB adaptation for DCI 2_9 based SSB adaptation indication

- The same mechanism from OD-SSB case#2 (same frequency but with different periodicities) can be applied by adaptive SMTC

  - FFS: Additional UE processing/preparation time for reception of SSB after adaptation

4.3 L1 requirements after transition – DRX

As per RAN1 agreement guidance, at most 2 additional SSB periodicities for adaptation are defined. Considering the typical SSB periodicity 20 ms, the additional ones can be a denser value (e.g., 5 ms) and a sparser value (e.g., 160 ms), separately. Most likely, NW to adapt the SSB periodicity to a denser value for the purpose of low-latency SCell operation, e.g., fast SCell activation. When SSB adaptation is activated, the DCI 2_9 is sent, then if the UE is able to receive and monitor the DCI 2_9 indication for the deactivated SCell, it can understand the SSB burst periodicity adaptation indication field in the information block, and the dense periodicity SSB can be used to perform measurement, subsequently.

In this sense, especially for the SSB adaptation L1 measurement during SCell activation case, from our understanding, once the SSB adaptation activation has been triggered, one of the most important tasks for the UE is to finish the L1 measurement and to complete Tx beam selection as fast as possible (i.e., with minimum delay), from this, when DRX cycle <=320 ms is configured, if we to consider L1-RSRP measurement under DRX mode, the measurement period will be always limited by DRX cycle, even Tadapted SSB<< TDRX, we cannot see the benefits of SSB adaptation with dense periodicity. To align with the agreements obtained in OD-SSB, i.e., below

#114 Issue 4-2: L1 measurement after SCell activation Agreement: - RAN4 to support OD-SSB L1 measurement during and after SCell activation. - When non-DRX is configured, the current requirement can be reused with the clarification of TSSB in TL1-RSRP_Measurement_Period_SSB is the SSB periodicity of OD-SSB - RAN4 to support OD-SSB L1 measurement during SCell activation. - The requirement when DRX is configured. - Based on OD-SSB periodicity

We have the following updated proposals:

Proposal 5: If RAN4 to support SSB adaptation L1 measurement during SCell activation for DCI 2_9 based SSB adaptation indication

- The requirement when DRX is configured.

  - Based on the additional SSB periodicity after adaptation

While, for the scenario of after SCell activation, from our understanding, in such scenario, it is possible for NW to adapt the SSB periodicity to a typical/sparser value for the purpose of RRM measurement or NES gains acquisition. In this sense, when DRX cycle <=320 ms is configured, the principle of L1-RSRP measurement under DRX mode for serving cell should be kept. To align with the agreements obtained in OD-SSB, we have the following proposals

Proposal 6: RAN4 to support SSB adaptation L1 measurement after SCell activation for DCI 2_9 based SSB adaptation indication

- The requirement when DRX is configured.

  - Following the legacy requirements based on DRX

4.4 SSB adaptation – L3 Requirements impacts

Whether to define L3 measurement requirements at and after transition

From our understanding, it is necessary to define the L3 measurement requirements at and after transition. First, the discussions of measurement period/behavior at transition may occur at L3 measurement based on more SSB samples. Applicability: SSB adaptation happens during one measurement period for multi-sample measurement

- Solution: Measurement period is the maximum {Measurement Period of the first mode before transition, Measurement Period of the second mode after transition}. Thereinto, the first mode and the second mode corresponding to SSB/SMTC periodicity before and after adaptation, respectively

  - Benefits: Long measurement period to cover sufficient samples. The measurement can be performed with reliable measurement accuracy at transition.

Proposal 7: L3 requirements at transition:

- When SMTC periodicity changes due to SSB periodicity adapting during one cell identification/ measurement period, the cell identification and measurement period requirements apply based on the longer period before or after the transition.

Proposal 8: Subsequent to transition, cell identification and measurement period requirements are based on the SMTC periodicity according to SSB adaptation

5 Conclusion

In this contribution, we provided our viewpoints on RRM requirements for NES common signal adaptation. The following observations and proposals are obtained:

Observation 1：Only DCI based signalling, i.e., DCI format 2_9 scrambled with new RNTI is agreed to indicate SSB burst periodicity adaptation

Proposal 1: RAN4 not to define RRM requirements for SSB adaptation for NW triggered MAC-CE based signalling

Proposal 2: For SSB adaptation, RAN4 to define requirements considering the following cases

- HF index is same before and after SSB adaptation and SFN offset is same before and after SSB adaptation

- HF index is same before and after SSB adaptation and SFN offset is same or different before and after adaptation

  - Wait for RAN1 conclusions on whether additional RRC signalling for offset of SSB for additional SSB periodicity is needed

Proposal 3: From RRM requirement perspective, suggest to define activated SCell L3 measurement requirements with priority

- Upon receiving DCI format 2_9, UE to perform L3 measurement based on additional SMTC configuration according to SSB adaptation

Proposal 4: If RAN4 to define L3 measurement requirements on deactivated SCell with SSB adaptation for DCI 2_9 based SSB adaptation indication

- The same mechanism from OD-SSB case#2 (same frequency but with different periodicities) can be applied by adaptive SMTC

  - FFS: Additional UE processing/preparation time for reception of SSB after adaptation

Proposal 5: If RAN4 to support SSB adaptation L1 measurement during SCell activation for DCI 2_9 based SSB adaptation indication

- The requirement when DRX is configured.

  - Based on the additional SSB periodicity after adaptation

Proposal 6: RAN4 to support SSB adaptation L1 measurement after SCell activation for DCI 2_9 based SSB adaptation indication

- The requirement when DRX is configured.

  - Following the legacy requirements based on DRX

Proposal 7: L3 requirements at transition:

- When SMTC periodicity changes due to SSB periodicity adapting during one cell identification/ measurement period, the cell identification and measurement period requirements apply based on the longer period before or after the transition.

Proposal 8: Subsequent to transition, cell identification and measurement period requirements are based on the SMTC periodicity according to SSB adaptation

6 References

[1] R4-2504907 “WF on RRM requirements for Netw_Energy_NR_enh_Part2”, Huawei