3GPP TSG-RAN WG4 Meeting #115                                                                     R4-2507260

Malta, MT, 19th – 23rd May 2025

Source:	NTT DOCOMO INC., SoftBank Corp., Rakuten Mobile, Inc.

Title:	(NR_newRAT-Core) Discussion on EESS protection from n260

Agenda item:	4.2.1

Document for:	Discussion

- Introduction

In Japan, the allocation of new mmW spectrum is being considered. 37 - 43.5GHz is one of the candidates for new spectrum allocation in Japan [1]. Considerations for changing Japanese regulation for them are already in progress, but the requirements for EESS protection from n260 had not yet been discussed in RAN4. Based on the status of this, the current regulatory changes are limited to frequencies from 40.5 to 43.5GHz [2]. Therefore, it is necessary to complete the introduction of it for the next regulatory changes in Japan. This paper proposes the introduction of EESS protection requirements for n260 and encourages companies to provide their views on the A-MPR value at the next meeting.

- Background

In Japan, the allocation of new mmW spectrum is being considered. 37 - 43.5GHz filled in pale red in Figure 2-1 below is one of the candidates for new spectrum allocation in Japan. These ranges can be covered by combining band n259 and band n260 specified in 3gpp.

Figure 2-1: Frequency ranges relationship among new spectrum in Japan, n259, n260, and EESS system

Considerations for changing Japanese regulation for them are already in progress, but some of the requirements had not yet been discussed in RAN4. Based on the status of this, the current regulatory changes are limited to frequencies from 40.5 to 43.5GHz. Therefore, it is necessary to complete the introduction of the missing RAN4 requirements for the next regulatory changes in Japan.

Based on WRC-19 RESOLUTION 243 [3], the protection from unwanted emissions of IMT stations operating in the frequency band 37.0 - 40.5 GHz for the Earth exploration satellite service (EESS) (passive) in the frequency band 36-37 GHz is needed. For that, the following unwanted emissions of IMT stations apply as specified in Table 2-1 below.

Table 2-1: Unwanted emission limits for EESS protection by WRC-19

Observation 1: The requirements for protection from n260 to EESS system have not been introduced in RAN4 spec.

Observation 2: The allocation of new spectrum (37 - 43.5 GHz) was considered in Japan, but the ongoing regulatory changes is limited to frequencies where RAN4 discussions have been completed.

Proposal 1: In preparation for the next regulatory changes in Japan, so EESS protection requirements for n260 is introduced.

- Discussion

  - The requirements for EESS protection from n260

The UE requirements for EESS protection from PC3 n259 have already been specified as shown below. Based on unwanted emission limits by WRC-19 limits, the allowed emission level from n259 is -13dBm/MHz and 7dBm/GHz within the frequency range 36-37 GHz. It has been concluded that A-MPR is unnecessary for this requirement and has been captured in spurious emission requirements for general requirement (shown in Table 6.5.3-2 below) and UE co-existence requirement (shown in Table 6.5.3.1-1 below). On the other hand, since n260 is adjacent to EESS, it will be more difficult to meet the EESS protection requirement than n259. Therefore, RAN4 needs to discuss whether A-MPR is necessary for EESS protection from n260.

Table 6.5.3-2 in TS 38.101-2: Spurious emissions limits (General spurious emission)

| Frequency Range | Maximum Level | Measurement bandwidth |
| --- | --- | --- |
| 30 MHz f < 1000 MHz | -36 dBm | 100 kHz |
| 1 GHz f < 12.75 GHz | -30 dBm | 1 MHz |
| 12.75 GHz ≤ f ≤ 2nd harmonic of the upper frequency edge of the UL operating band in GHz | -13 dBm | 1 MHz |

Table 6.5.3.1-1 in TS 38.101-2: Requirements (Spurious emission for UE co-existence)

| NR Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band/frequency range | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| n259 | NR Band 257 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band 261 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band 262 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band n263 | FDL_low | - | FDL_high | TBD | 100 |  |
|  | Frequency range | 36000 | - | 37000 | 7 | 1000 |  |
|  | Frequency range | 57000 | - | 66000 | 2 | 100 |  |

Observation 3: The requirement for EESS protection from n259 PC3 UE has already specified without A-MPR.

Proposal 2: RAN4 will discuss whether A-MPR is necessary for EESS protection from n260 PC3/PC1 UE. If A-MPR is needed, the introduction of a new NS will be required. Companies are encouraged to provide their views on the A-MPR value at the next meeting.

  - Other aspects

As mentioned above, the rapid preparations for the next regulatory change in Japan are needed and the workload is almost just about discussing whether A-MPR is necessary, it is preferable to deal with this topic in the maintenance AI. If it is necessary to deal with this topic in new WI, it will need to be proposed at the RAN Plenary in June.

Proposal 3: If permitted, these activities will be conducted under maintenance AI. The workload is just about discussing how much A-MPR values are necessary for EESS protection.

This requirement will be introduced at least from Rel-19. Earlier releases can be discussed. It is possible to first have a technical discussion on the related requirements and then discuss which release the UE can support from.

Proposal 4: It will be introduced at least from Rel-19. Earlier releases can be discussed.

- Conclusion

Here we summarize our proposals:

Observation 1: The requirements for protection from n260 to EESS system have not been introduced in RAN4 spec.

Observation 2: The allocation of new spectrum (37 - 43.5 GHz) was considered in Japan, but the ongoing regulatory changes is limited to frequencies where RAN4 discussions have been completed.

Proposal 1: In preparation for the next regulatory changes in Japan, so EESS protection requirements for n260 is introduced.

Observation 3: The requirement for EESS protection from n259 PC3 UE has already specified without A-MPR.

Proposal 2: RAN4 will discuss whether A-MPR is necessary for EESS protection from n260 PC3/PC1 UE. If A-MPR is needed, the introduction of a new NS will be required. Companies are encouraged to provide their views on the A-MPR value at the next meeting.

Proposal 3: If permitted, these activities will be conducted under maintenance AI. The workload is just about discussing how much A-MPR values are necessary for EESS protection.

Proposal 4: It will be introduced at least from Rel-19. Earlier releases can be discussed.

References

- https://www.tele.soumu.go.jp/e/adm/freq/search/actionplan/index.htm, Ministry of Internal Affairs and Communications

- https://www.soumu.go.jp/menu_news/s-news/01kiban14_02000689.html, Ministry of Internal Affairs and Communications (Japanese language only)

- https://www.itu.int/pub/R-ACT-WRC.14-2019, “Final Acts WRC-19,” ITU-R

Annex: Draft CR to introduce EESS protection for n260

<START OF THE CHANGE 1>

### 6.2.3	UE maximum output power with additional requirements

#### 6.2.3.1	General

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band (the IE field freqBandIndicatorNR) and an associated value of additionalSpectrumEmission in the relevant RRC information elements.

To meet these additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in clause 6.2.1. Unless stated otherwise, an A-MPR of 0 dB shall be used.

Table 6.2.3.1-1 specifies the additional requirements with their associated network signalling values and the allowed A-MPR and applicable operating band(s) for each NS value. The mapping of NR frequency band numbers and values of the additionalSpectrumEmission to network signalling labels is specified in Table 6.2.3.1-2. Unless otherwise stated, the allowed total back off is maximum of A-MPR and MPR specified in clause 6.2.2.

Table 6.2.3.1-1: Additional maximum power reduction (A-MPR)

| Network Signalling label | Requirements (clause) | NR Band | Channel bandwidth (MHz) | Resources Blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_200 |  |  |  |  | N/A |
| NS_201 (NOTE 1) | 6.5.3.2.2 | n258 |  |  | 6.2.3.2 |
| NS_202 | 6.5.3.2.3 | n257, n258 | 50, 100, 200, 400 | Table 5.3.2-1 | 6.2.3.3 |
| NS_203 | 6.5.3.2.4 | n258 | 50, 100, 200, 400 | Table 5.3.2-1 | 6.2.3.4 |
| NS_205 | 6.5.3.2.6 | n257, n258 | 50,100, 200, 400 | Table 5.3.2-1 | 6.2.3.6 |
| NS_20X | 6.5.3.2.7 | n260 | 50,100, 200, 400 | Table 5.3.2-1 | 6.2.3.7 |
| NOTE 1: NS_201 is obsolete, the associated additional spurious emission requirements are not applicable. |  |  |  |  |  |

Table 6.2.3.1-2: Mapping of Network Signalling labe

| NR Band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n257 | NS_200 | NS_202 | NS_205 |  |  |  |  |  |
| n258 | NS_200 | NS_201 (NOTE 2) | NS_202 | NS_203 | NS_205 |  |  |  |
| n259 | NS_200 |  |  |  |  |  |  |  |
| n260 | NS_200 | NS_20X |  |  |  |  |  |  |
| n261 | NS_200 |  |  |  |  |  |  |  |
| n263 | NS_200 | NS_204 |  |  |  |  |  |  |
| NOTE 1: additionalSpectrumEmission corresponds to an information element of the same name defined in sub-clause 6.3.2 of TS 38.331 [13]. NOTE 2: NS_201 is obsolete, the associated additional spurious emission requirements are not applicable. |  |  |  |  |  |  |  |  |

<END OF THE CHANGE 1>

<START OF THE CHANGE 2>

#### 6.2.3.7	A-MPR for NS_20X

##### 6.2.3.7.1	A-MPR for NS_20X for power class 1

For power class 1, A-MPR for NS_20X shall be [TBD] dB.

##### 6.2.3.7.2	[Reserved]

##### 6.2.3.7.3	A-MPR for NS_20X for power class 3

For power class 3, A-MPR for NS_20X shall be [TBD] dB.

##### 6.2.3.7.4	A-MPR for NS_205 for power class 4

For power class 4, A-MPR for NS_20X specified in clause 6.2.3.7.3 applies.

##### 6.2.3.7.5	[Reserved]

##### 6.2.3.7.6	[Reserved]

##### 6.2.3.7.7	[Reserved]

<END OF THE CHANGE 2>

<START OF THE CHANGE 3>

#### 6.2A.3.1	General

Additional emission requirements can be signalled by the network with network signalling value indicated by the field additionalSpectrumEmission. To meet these additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in clause 6.2A.1. Unless stated otherwise, an A-MPR of 0 dB shall be used. Unless otherwise stated, the allowed total back off is maximum of A-MPR and MPR specified in clause 6.2A.2.

For intra-band contiguous aggregation with the UE configured for transmissions on two serving cells, the maximum output power reduction specified in Table 6.2A.3.1-1 is allowed for all serving cells of the applicable uplink contiguous CA configurations.

Table 6.2A.3.1-1 specifies the additional requirements and allowed A-MPR with corresponding network signalling label and operating band. The mapping between network signalling labels and the additionalSpectrumEmission IE defined in TS 38.331 [13] is specified in Table 6.2A.3.1-2. Unless otherwise stated, the allowed total back off is maximum of A-MPR and MPR specified in clause 6.2A.2.

Table 6.2A.3.1-1: Additional maximum power reduction (A-MPR)

| Network Signalling value | Requirements (clause) | NR Band | Channel bandwidth (MHz) | Resources Blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| CA_NS_200 |  |  |  |  | N/A |
| CA_NS_201 | 6.5A.3.2.2 | n258 |  |  | 6.2A.3.2 |
| CA_NS_202 | 6.5A.3.2.3 | n257, n258 |  |  | 6.2A.3.3 |
| CA_NS_203 | 6.5A.3.2.4 | n258 |  |  | 6.2A.3.4 |
| CA_NS_205 | 6.5A.3.2.6 | n257, n258 |  |  | 6.2A.3.6 |
| CA_NS_20X | 6.5A.3.2.7 | n260 |  |  | 6.2A.3.7 |
| NOTE: CA_NS_201 is obsolete, the associated additional spurious emission requirements are not applicable. |  |  |  |  |  |

Table 6.2A.3.1-2: Value of additionalSpectrumEmission

| NR Band | Value of additionalSpectrumEmission / NS number |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n257 | CA_NS_200 | CA_NS_202 | CA_NS_205 |  |  |  |  |  |
| n258 | CA_NS_200 | CA_NS_201 | CA_NS_202 | CA_NS_203 | CA_NS_205 |  |  |  |
| n259 | CA_NS_200 |  |  |  |  |  |  |  |
| n260 | CA_NS_200 | CA_NS_20X |  |  |  |  |  |  |
| n261 | CA_NS_200 |  |  |  |  |  |  |  |
| NOTE 1: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331 [13]. NOTE 2: CA_NS_201 is obsolete, the associated additional spurious emission requirements are not applicable. |  |  |  |  |  |  |  |  |

<END OF THE CHANGE 3>

<START OF THE CHANGE 4>

#### 6.2A.3.7	A-MPR for CA_NS_20X

##### 6.2A.3.7.1	A-MPR for CA_NS_20X for power class 1

For intra-band contiguous UL CA with single contiguous RB allocation, A-MPR for CA_NS_20X shall be [TBD] dB.

For intra-band contiguous UL CA with non-contiguous RB allocation or non-contiguous UL CA, A-MPR for CA_NS_20X shall be [TBD] dB.

##### 6.2A.3.7.2	[Reserved]

##### 6.2A.3.7.3	A-MPR for CA_NS_20X for power class 3

For intra-band contiguous UL CA with single contiguous RB allocation, A-MPR for CA_NS_20X shall be [TBD] dB.

For intra-band contiguous UL CA with non-contiguous RB allocation or non-contiguous UL CA, A-MPR for CA_NS_20X shall be [TBD] dB.

##### 6.2A.3.7.4	A-MPR for CA_NS_20X for power class 4

For UL CA, A-MPR for CA_NS_20X specified in sub-clause 6.2A.3.7.3 applies.

##### 6.2A.3.7.5	[Reserved]

##### 6.2A.3.7.6	[Reserved]

<END OF THE CHANGE 4>

<START OF THE CHANGE 5>

##### 6.5.3.2.7	Additional spurious emission requirements for NS_20X

When "NS_20X" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.2.7-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3-1 from the edge of the channel bandwidth.

Table 6.5.3.2.7-1: Additional requirements (NS_20X)

| Frequency band (GHz) | Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 36.0 f 37.0 | -13 | 1 MHz |
|  | +7 | 1000 MHz |

<END OF THE CHANGE 5>