3GPP TSG-RAN WG4 Meeting #115						R4-2507264

Malta, MT, 19th – 23rd May, 2025

Source:	Nokia

Title:	Discussion on methods for reducing the number of UE Rx RF chains

Agenda item:	7.5.3

Document for:	Discussion

Introduction

Agreements at previous meetings

The latest WF for NR FR1 DL fragmented carriers study [1] presents some open issues as listed below:

#### Issue 2-3: Triggering condition to enable ”One Rx RF chain”

RAN4 agree there are two aspects regarding the triggering condition enable/disable the one Rx RF chain mode:

- Option 1: Rely on existing channel quality reporting and procedures. No new indication from UE is needed

- Option 2: The UE need the measurement to determine and indicate to the network whether it can support non-contiguous CCs with one RX RF chain or not, assuming the associated degradation is no greater than allowed degradation that is specified

- Note: detailed conditions to enable/disable can be further discussed and decided separately.

#### Issue 2-1: How to indicate UE capability for supporting ”one Rx RF chain” for DL fragmented carriers

- Consider the following alternatives

  - Alternative 1: use the legacy band combination reporting scheme + Rx sharing capability indication

    - Alternative 1 has no modification of CA configurations in RAN4

  - Alternative 1a: use the legacy band combination reporting scheme + Rx sharing capability and other related information including the frequency separation between two carriers.

    - Alternative 1a has no modification of CA configurations in RAN4

  - Alternative 2: use the new notation for CA configurations with single Rx RF Chain

  - Alternative 2a: use the new bandwidth class conditioned on the frequency separation between two carriers

#### Issue 2-2: Discussion on semi-statically switch hardware resources procedure and fallback considerations

- Further discussions on

  - CA fallback with/without UE architecture switching

    - Consider the fall back

      - from high order CA to lower order CA

      - from CA to single carrier mode

    - Decided by network

  - UE architecture switching:

    - Switching between 2 Rx RF chains and 1 Rx RF chain on CCs of intra-band NC CA

    - FFS on whether it is decided by UE itself or decided by network

      - FFS on whether the additional indication from UE side is needed

#### Issue 2-4: Discussion on applicability for PCell and SCell

A more detailed proposal based on the tentative agreement in online discussion:

- To address the potential impacts on the DL performance for PCell, RAN4 agrees to further discuss the following aspects:

  - To limit the use of this feature to CA configurations where the impact is limited

  - In the CA configurations where there is limited impact, how to further mitigate the impact by means including restricting the PCC and SCC frequency location, or restricting UE locations (e.g., the feature is not enabled when the UE is located at cell edge where wanted signal level is close to receiver sensitivity)

And from RAN4#114 [2] presents a bit more details of the open issues as found below:

How to indicate UE capability for supporting ”one Rx RF chain” for DL fragmented carriers

Option 1: Legacy band combination reporting scheme + Rx sharing capability indication

- FFS on whether the capability is specified as ”per-band per-BC” or “per CC per band per BC”.

Other options are not precluded

Discussion on semi-statically switch hardware resources procedure and fallback considerations

Triggering condition to enable ”One RF Rx chain”

FFS:

- FFS on whether the UE indication solution is needed or not, with understanding that enabling one Rx chain is under the control of network.

- Triggering condition for UE working in one Rx RF chain is needed

- Triggering condition for UE working in legacy Rx RF chains is needed

- The switching should be not be dynamic and be under network control with UE indication of condition.

Discussion on applicability for PCell and SCell

Discussion on the number of Rx and DL MIMO layer configuration for the UE supporting ”one Rx RF chain” mode

Next, we discuss some of these open aspects.

Discussion

In this document we further discuss the methods for reducing the number of UE Rx chains used in relation to the operation of fragmented carriers.

General on fragmented carriers

The issue of UE indicating support of operating a certain carrier combination as fragmented carriers has been discussed extensively. As we have mentioned earlier, we find it necessary to clearly distinguish the radio configuration of the non-contiguous intra-band carrier aggregation between the modes of operation related to using the partially shared RF architecture and the fully shared RF chain at the UE side.

In [2], we have suggested that the currently undefined NR CA bandwidth class (BCS) ‘F’ should be introduced for indicating a CA configuration which is applicable to fragmented carrier operation. Such indication would clearly indicate to a UE capable of supporting fragmented carrier operation that fragmented carrier operation can be applied. Additionally, it enables the network to explicitly configure the UE that this type of CA configuration can be applied.

Using an explicit CA configuration indication allows separating when fragmented carrier operation is applicable from other types of CA configurations.

In general, we assume that if the UE can change its HW configuration between using 1Rx chain or 2Rx chains between a given band combo while still fulfilling the UE minimum requirements, and without causing negative impact on the scheduling, this is already allowed within the current requirements. This would be possible if UE has spare Rx chain to perform such HW reconfiguration.

Figure 1 Illustration where UE changes its HW configuration between 1 Rx chain and 2Rx chain usage for receiving 2 CCs

To facilitate the continued discussions, RAN4 would like to reach a common understanding on a couple of aspects.

Assuming we have the wanted signals (non-contiguous CCs) and one or more unwanted signal(s) (CCs) in between. It is assumed that the unwanted signal occurs due to another operator operating the CC(s) in between the wanted signals. In this scenario the wanted and unwanted signals originate from collocated transmitters.

From deployment point of view, there is a power restriction in place regulating the maximum transmit power of CCs operated by one operator (wanted signal). It is assumed that the maximum transmit power difference between the CCs carrying the wanted signal cannot exceed 6dB between CCs of the same operator.

The maximum transmit power difference between wanted CCs cannot exceed 6dB.

However, there is no transmit power requirements related to the maximum allowed transmit power difference between the wanted CCs and unwanted CC’s. Hence there is no transmit power restrictions between the CC’s of the operators when considering fragmented carrier deployment. The power balancing is done by mutual agreements and alignments between operators. Even if some efforts are put into balancing power between operators, the effective received power difference (between wanted and unwanted signals) at the UE may differ significantly.

Additionally, the basic physical factors, such as the distance from the UE to the gNB or sectorization of the different operators, is effectively determining the field strength at any given location of the UE. As a result, it is not practically possible to limit or make any assumptions concerning the maximum transmit power difference between CCs operated by different operators and thereby the assumed received power difference at UE side. Hence, RAN4 must assume that the power difference between wanted and unwanted signals may vary significantly.

Figure 2 Illustration of maximum allowed power difference between wanted CCs and unwanted interferer

Discuss and agree if there is a maximum allowed transmit power difference between wanted CCs used in Fragmented CA and unwanted interferer (i.e. will there be any maximum transmit power assumption between CCs operated by the different operators).

We assume, that in order facilitate proper operations of CA as a fragmented aggregation, with an unknown/unwanted interferer in between the wanted, the UE must be made aware that a certain CA configuration can be operated with either single Rx chain or separate Rx chains.

General view on SCell control for fragmented CA

If a change in HW configuration between UE using 1 Rx chain or 2 Rx chains for reception in a fragmented carrier aggregation deployment does impact the UE performance, system performance or the network, there is a need to ensure that whatever ‘mode’ the UE is operating in, this is aligned between the UE and the network. Hence, it needs to be well aligned between UE and network whether the UE is applying 1Rx chain or 2 Rx chain.

Single Rx chain operation shall be controlled by network if using single Rx chain for reception in a Fragmented carrier CA has negative consequences compared to using separate Rx.

In many scenarios, if the UE can change between using 1Rx or 2Rx chains without this having negative performance impact, such UE autonomous switching could be feasible. However, if the UE change between using 1Rx or 2Rx in a Fragmented CA setup, does have (negative) system impact, it needs to be clear how and when such HW reconfiguration is allowed and can be performed. In particular, if any impacts are on the PCell, they have to be well aligned with the network.

One such example discussed in last meetings was the control of the SCell. Hence, what shall happen if the UE has PCell and SCell configured in a Fragmented CA setup and the interference from the unwanted signal suddenly increases beyond what the UE can manage?

Figure 3 Illustration of changing from receiving CC1 and CC2 with single Rx chain to receiving only CC1 with single Rx chain. In this case CC2 cannot be received anymore after the change.

If the UE does not have a spare Rx chain to use for change from single Rx chain to separate Rx chain operation, would the SCell (e.g. CC2 in figure 3) remain ‘active’ on the UE side or not? Would the UE just ‘drop’ the SCell? Would some UEs drop the PCell? Or something else? Having unspecified and thereby also different UE behaviours among the UEs in the field is of course not a good solution and will surely lead to a feature which is not very attractive. Additionally, unless the network is aware of the UE having additional Rx chain capacity, this cannot always be used. Hence, UE behaviour and operation in the discussed scenario cannot always be left up to UE implementation. However, how to address the issue will also depend on the issue related to triggering conditions for enabling and disabling single or separate Rx chain operation.

Currently SCell operation is under network control. Hence, the network configures and de-configures the SCell. The network additionally activates and deactivates the SCell. This has been the principle since Rel-10, and this principle should not be broken. In fact, SCell control cannot be changed without involvement of at least RAN2.

Figure 4 Simple illustration of network SCell control

We suggest that RAN4 does not discuss solutions which breaks the current network control of SCell operations.

RAN4 does not discuss solutions which breaks the current network control of SCell operations.

If RAN4 explores solutions which operates differently than the current network controlled SCell operations, RAN4 must involve other working groups and at least RAN2 during the discussions. Such UE (guided) autonomous SCell operations could be for example (as discussed) include UE fallback rules etc.

Any changes to existing SCell operation must include at least RAN2 (during the WI phase).

General RRM aspects

Discussion on applicability for PCell and SCell

Based on the principle that any changes related to PCell and/or SCell in Fragmented CA operations need to be well coordinated between UE and network, several RRM related issues needs to be discussed.

Open issues:

Issue 1: Can the PCell be part of a Fragmented Carrier configuration, or can a Fragmented CA configuration only include SCells?

Currently we do not see it is possible to give a clear answer. Such a restriction may limit the use of the feature. Additionally, if RAN4 defines proper solutions enabling to protect the PCell, there may not be a reason not allowing the PCell being part of a fragmented CA configuration.

RAN4 continue assuming PCell can be part of a fragmented CA configuration.

Hence, a fragmented carrier configuration can consist of PCell + SCell and SCell + SCell. It will in the end depend on outcome of the discussions and the operator deployments.

A fragmented carrier configuration can consist of PCell + SCell and SCell + SCell.

In last meeting it was discussed whether a performance degradation would be acceptable if the fragmented carrier configuration only contains SCell’s. In general, RAN4 can of course make such agreement, but it must be the operator who then has the final word regarding whether UE is then allowed to apply single Rx chain or not.

The operator must have the final word regarding whether UE is then allowed to apply single Rx chain or not, if using single Rx chain degrades the system performance.

Triggering condition to enable ”One Rx RF chain”

When the network configures the UE with fragmented CA configuration, the basic approach is to avoid a design relying on any try-and-error method. Hence, the design should avoid being based on network trying to configure the UE with CA which fails.

Addressing some of the open questions RAN4 should get a common understanding of ‘Triggering condition’. In our view such a discussion is needed to clarify the following aspects:

- Is triggering using 1Rx or 2Rx chains network controlled, and:

  - using existing signaling and measurements?

  - using new signaling and measurements?

- Can triggering using 1Rx or 2Rx chains be decided by the UE with network “pre-configuration/grant”?

- Can triggering using 1Rx or 2Rx chains be UE autonomous, with information status shared by the UE?

  - Can triggering using 1Rx or 2Rx chains be UE autonomous without network control and unknown to the network?

Hence, triggering a switch between using separate Rx chain or single Rx chain for reception of multiple CCs in a fragmented deployment can be performed using different approaches:

- Controlled by network based on UE assistance information.

- Controlled by UE based on network assistance/configuration and with/without network knowledge.

- Controlled by UE autonomously without network assistance and with/without network knowledge.

In addition, switching time between Rx chains should be considered from the beginning.

RAN4 should distinguish the discussion related to triggering conditions into two aspects:

- Triggering conditions (for example UE autonomous, threshold based etc.)

- Assistance information related to performing such triggering (for example measurements, thresholds etc.)

One proposal set forward concerning assistance information, is to request the UE to perform in-gap measurements with reference to the CA configuration, so that any connection feasibility (using single Rx chain), based on the in-gap interferer, can be evaluated in advance. One important point with this, is to avoid any radio link failure (on PCell) once the fragmented CA is configured.

Issue 2: Enabling network-controlled actions (such as a fragmented CA configuration) requires UE assistance. Which measurements and/or UE feedback would provide suitable UE assistance to the network when needed?

Multiple options have already been discussed. However, the basic assumptions still seem unclear. We see that ‘assistance information’, opens for multiple issues:

- Which UE assistance information would be needed on the network side?

- What information would be enough on the network side?

- How could the UE potentially provide such information to the network?

- Would the assistance information be in the form of measurement reporting or something else?

Different companies have diverse views, but at least RAN4 would need to discuss and agree how to configure and control fragmented CA in an orderly manner which is not based on a ‘try-and-error’ approach or other non-robust solutions.

RAN4 need to discuss and agree which (if any) measurements and/or UE feedback would provide suitable network assistance when needed, to enable the network configuring fragmented CA in a robust way.

To exemplify some of the possible assistance information that could be provided by the UE in order to minimize the effect of in-gap interference, three types of measurements could be used for evaluation and control of fragmented carrier pairs.

- The average power of the in-gap interference.

- Peak power of the in-gap interference, so that the ADC does not saturate.

- CQI measurement on the fragmented carrier.

However, CQI measurement are likely not able to precisely identify the cause of signal deterioration, which may result from factors like image interference or out-of-gap interference due to relaxed bandwidth of combined carriers. Hence, CQI measurements may not alone be enough to determine whether use of fragmented CA is feasible – especially before actually having fragmented CA configured.

Hence, there will likely be benefits in knowing up front (before entering into fragmented CA configuration and operation) whether use of single Rx for reception will cause possible negative impact on network, UE and system level.

Here we see that in-gap measurements could provide important measurement assistance information to the network. Based on such measurements the network would be aware whether the UE can be configured with fragmented CA.

Discussion on semi-statically switch hardware resources procedure and fallback considerations

As mentioned, developing a framework for fragmented CA which is not functioning in a robust manner does not seem attractive and can cause very negative impact on the network side and UE side in addition to the user experience.

As discussed, in many situations the HW reconfiguration between using 1Rx chain or 2Rx chain for CA can be done on the UE side without network involvement. This can be done provided there are no negative impact on the system performance and UE fulfilling the existing minimum requirements.

For those cases where there is a network impact there is a need to ensure UE and network alignment. For example, for the UE configured with fragmented CA, if the UE has exhausted the number of Rx chains but cannot anymore receive the CCs involved in the fragmented CA configuration, a HW reconfiguration from 1Rx chain reception to 2Rx chain reception cannot be done without network impact. Additionally, if a performance degradation is allowed (by RAN4) there is a need for operator to do a final decision whether to deploy the feature at the known cost in potential performance degradation.

For these examples where there is negative network impact, any action related to changing between using single or separate Rx chain operations would need to be aligned with the network. For example, either any action is network initiated or at least based on clearly defined rules.

As also discussed in section 2.1 (leading to proposal 1), one aspect to be discussed specifically for fragmented CA would be the assumptions related to the nature of the unwanted signal located in between the wanted signal from the two wanted CCs. This is also related to our discussion above, and which restrictions (if any) that can be assumed in power difference between the wanted and unwanted signals. When discussing potential UE measurement assistance, RAN4 would need to discuss if there are any assumptions concerning how much the fluctuations between the power differences. Additionally, RAN4 would need to discuss what the impact could be from any such fluctuations.

Discuss the assumption concerning the power fluctuations on the unwanted signals.

We assume that for the wanted signal the signals will always be within 6dB. We also assume that within the unwanted signals (if from same operator) the signals will be within 6dB.

Discuss the assumption concerning power difference fluctuations between the wanted and unwanted signals.

The assumption here will need to account the outcome of the proposal 1 in addition to outcome of proposal 8.

From a measurement assistance point of view the outcome of this discussion and proposals 1, 8 and 9 would give insights to the type of any UE assistance information to the network.

Contribution to TR

Based on the progress of the discussions we would propose to capture parts of the open issues and proposals in this paper – including any related agreements - in the TR. In the appendix we have provided a TP for the TR related to the following issues:

- SCell Control for fragmented CA

- Triggering conditions to enable/disable ”One Rx RF chain”

- Side Conditions for fragmented CA

Capture the TP in Appendix, including possible related agreement, in the TR.

Conclusion

In this contribution, we discuss the study item on fragmented carriers. The following Observations and Proposals were made:

General on fragmented carriers:

- Using an explicit CA configuration indication allows separating when fragmented carrier operation is applicable from other types of CA configurations.

The maximum transmit power difference between wanted CCs cannot exceed 6dB.

- Discuss and agree if there is a maximum allowed transmit power difference between wanted CCs used in Fragmented CA and unwanted interferer (i.e. will there be any maximum transmit power assumption between CCs operated by the different operators).

General view on SCell control for fragmented CA:

Single Rx chain operation shall be controlled by network if using single Rx chain for reception in a Fragmented carrier CA has negative consequences compared to using separate Rx.

RAN4 does not discuss solutions which breaks the current network control of SCell operations.

Any changes to existing SCell operation must include at least RAN2 (during the WI phase).

Discussion on applicability for PCell and SCell:

RAN4 continue assuming PCell can be part of a fragmented CA configuration.

A fragmented carrier configuration can consist of PCell + SCell and SCell + SCell.

The operator must have the final word regarding whether UE is then allowed to apply single Rx chain or not, if using single Rx chain degrades the system performance.

Triggering condition to enable ”One Rx RF chain”:

RAN4 need to discuss and agree which (if any) measurements and/or UE feedback would provide suitable network assistance when needed, to enable the network configuring fragmented CA in a robust way.

Discussion on semi-statically switch hardware resources procedure and fallback considerations:

Discuss the assumption concerning the power fluctuations on the unwanted signals.

Discuss the assumption concerning power difference fluctuations between the wanted and unwanted signals.

From a measurement assistance point of view the outcome of this discussion and proposals 1, 8 and 9 would give insights to the type of any UE assistance information to the network.

Contribution to TR:

Capture the TP in Appendix, including possible related agreement, in the TR.

We have provided TP for the TR in the appendix and in [6].

- References

- R4-2505109, WF on NR FR1 DL fragmented carriers study, RAN4#114bis

- R4-2502873, WF on NR FR1 DL fragmented carriers study, RAN4#114

- R4-2504129, General aspects of Fragmented CA, Nokia

- R4-2504128, Discussion on RF requirements for Fragmented CA, Nokia

- R4-2507375, Discussion on RF requirements for Fragmented CA, Nokia

- R4-2507376, TPs to TR 38.755 with aspects of Fragmented CA, Nokia

- Appendix

## 8.x	RRM aspects of fragmented CA

8.x.1	SCell control for fragmented CA

If a change in HW configuration between UE using 1 Rx chain or 2 Rx chains for reception in a fragmented carrier aggregation deployment does impact the UE performance, system performance or the network, there is a need to ensure that whatever ‘mode’ the UE is operating in, this is aligned between the UE and the network. Hence, it needs to be well aligned between UE and network whether the UE is applying 1Rx chain or 2 Rx chain.

In many scenarios, if the UE can change between using 1Rx or 2Rx chains without this having negative performance impact, such UE autonomous switching could be feasible. However, if the UE change between using 1Rx or 2Rx in a Fragmented CA setup, does have (negative) system impact, it needs to be clear how and when such HW reconfiguration is allowed and can be performed. In particular, if any impacts are on the PCell, they have be well aligned with the network.

If RAN4 explores solutions which operates differently than the current network controlled SCell operations, RAN4 must involve other working groups and at least RAN2 during the discussions.

A fragmented carrier configuration can consist of PCell + SCell and SCell + SCell. It will in the end depend on outcome of the discussions and the operator deployments.

8.x.2	Triggering conditions to enable/disable ”One Rx RF chain”

RAN4 would need a common understanding of ‘Triggering condition’. Such a discussion is needed to clarify the following aspects:

- Is triggering using 1Rx or 2Rx chains network controlled, and:

  - using existing signaling and measurements?

  - using new signaling and measurements?

- Can triggering using 1Rx or 2Rx chains be decided by the UE with network “pre-configuration/grant”?

- Can triggering using 1Rx or 2Rx chains be UE autonomous, with information status shared by the UE?

  - Can triggering using 1Rx or 2Rx chains be UE autonomous without network control and unknown to the network?

Hence, triggering a switch between using separate Rx chain or single Rx chain for reception of multiple CCs in a fragmented deployment can be performed using different approaches:

- Controlled by network based on UE assistance information.

- Controlled by UE based on network assistance/configuration and with/without network knowledge.

- Controlled by UE autonomously without network assistance and with/without network knowledge.

In addition, switching time between Rx chains should be considered from the beginning.

RAN4 should distinguish the discussion related to triggering conditions into two aspects:

- Triggering conditions (for example UE autonomous, threshold based etc.)

- Assistance information related to performing such triggering (for example measurements, thresholds etc.)

Enabling a network-controlled solution requires UE assistance. RAN4 would need to consider which measurements and/or UE feedback would provide suitable UE assistance to the network. RAN4 also needs to consider when such information needs to be provided.

Multiple options have been discussed. RAN4 needs to discuss at least the following aspects:

- Which UE assistance information would be needed on the network side?

- What information would be enough on the network side?

- How could the UE potentially provide such information to the network?

- Would the assistance information be in the form of measurement reporting or something else?

Possible assistance information that could be provided by the UE in order to minimize the effect of in-gap interference, at least three types of measurements could be used for evaluation and control of fragmented carrier pairs.

- The average power of the in-gap interference.

- Peak power of the in-gap interference, so that the ADC does not saturate.

- CQI measurement on the fragmented carrier.

8.x.3	Side Conditions for fragmented CA

One aspect to be discussed specifically for fragmented CA would be the assumptions related to the nature of the unwanted signal located in between the wanted signal from the two wanted CCs. This also relates to which restrictions (if any) can be assumed concerning the power difference between the wanted and unwanted signals. When discussing potential UE measurement assistance, RAN4 would need to discuss if there are any assumptions concerning how much the fluctuations between the power differences. Additionally, RAN4 would need to discuss what the impact could be from any such fluctuations.