3GPP TSG-RAN WG4 Meeting #115	R4-2507279

St Julian’s, Malta, 19th - 23th May, 2025

Source:	KDDI, Murata, Samsung

Title:	TP to TR 38.719-02-01 for adding impact of harmonic mixing to CA_n18-n77 with UL n77

Agenda item:			6.3.3

Document for:	Approval

- Introduction

The impacts of uplink (UL) and downlink (DL) harmonic mixing associated with REFSENS, delta Tib, delta Rib, and other factors for CA_n18-n77 have been analyzed and incorporated into TR 38.717-02-01-h00, specifically in Section 6.41 of 3GPP TR 38.717-02-01 V17.0.0.

During the standardization phase for Release 17, studies on REFSENS for operator-specific band combinations were permitted. Band n18 is exclusively allocated in Japan (for KDDI), and for CA_n18-n77, the band n77 is constrained to specific frequency ranges: 3520-3560 MHz, 3700-3800 MHz, and 4000-4100 sMHz. Consequently, the fourth-order harmonic mixing of n18 DL falling within 3440-3500 MHz of n77 was deemed insignificant. Thus, throughout Release 17 and early stage of Release 18, it was widely considered unnecessary to evaluate the impact of fourth-order harmonic mixing of n18 into n77 UL for CA_n18-n77.

However, in the later stages of Release 18, RAN4 decided to assess intermodulation distortion (IMD) and mixing spurious distortion (MSD) across the entire global band of n77. As a result, notes such as Note 8 [2] in Table 7.3A.5-1 were introduced, specifying that certain test points could be omitted for Japan.

This Technical Proposal (TP) addresses the impact of fourth-order harmonic mixing of n18 into n77 UL for CA_n18-n77, considering the full global band range of n77 (3300-4200 MHz).

In the latest specification 3GPP TS 38.101-1-i90 (2025-03), the influence of fourth-order harmonic mixing of n18 into n77 UL for CA_n18-n77 is not explicitly included due to historical considerations. In contrast, the harmonic mixing effects for CA_n5-n77 and CA_n26-n77 are documented in Table 7.3A.4-4 and summarized in Table 1 below.

Table 1 (abstracted from Table 7.3A.4-4): Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC3 aggressor NR UL band for DL NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n77 | n5 | 10 | 15 | 25 | 5 | 5.7 | NOTE 8 | UL1/DL4 |
| n77 | n5 | 10 | 15 | 25 | 20 | 0.8 | NOTE 8 | UL1/DL4 |
| n77 | n26 | 10 | 15 | 25 | 5 | 5.4 | NOTE 8 | UL1/DL4 |

We propose reusing the CA_n26-n77 setup for cases where DL BW = 5 MHz, since band n18 falls within band n26, while 815-824 MHz lies outside the range of band n5. For DL BW = 15 MHz, the MSD value has been calculated accordingly. Furthermore, REFSENS differences between band 26 and band n18 are not considered, following Nokia’s comment on R4-2412850.

- References

[1] 3GPP TR 38.717-02-01 V17.0.0 (2022-06)

[2] 3GPP TS 38-101-1-i90(2025-03)

[3] R4-2412850, TP for TR 38719-02-01 CA_n18A-n41A

Text Proposal

<Start of Text Proposal>

5.x	CA_n18-n77

5.x.1	Common for 1 band UL and 2 bands UL CA

5.x.1.1	Operating bands for CA

Table 5.x.1.1-1:  CA band combination of band nX+nY

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n18 | 815 MHz – 830 MHz | 860 MHz – 875 MHz | FDD |
| n77 | 3300 MHz – 4200 MHz | 3300 MHz – 4200 MHz | TDD |

5.x.1.2	Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination of band nX+nY

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n18-n77 | - | n18 | 5, 10, 15 | 0 |
|  | CA_n18-n77 | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |

5.x.1.3	UE co-existence studies for 1 band UL

Unaffected parts are omitted.

Table 6.41.1.3-2 in [2] summarizes frequency ranges where harmonics mixing occur for CA_n18-n7.

Table 6.41.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n18 | 815 | 830 | 860 | 875 | 1720 | 1750 | 2580 | 2625 | 3440 | 3500 | 4300 | 4375 |
| n77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 | 16500 | 21000 |

Based on above table, 4th harmonic mixing of n18 DL falls into n77 UL from 3440 to 3500MHz, within the range of global band 3300-4200MHz.

5.x.1.4	∆TIB,c and ∆RIB,c values

TIB,c and RIB,c values have already been defined so this section is omitted.

5.x.1.5	REFSENS requirements

Reference sensitivity exceptions due to harmonic mixing from UL n77 for DL CA_n18-n77 is shown in Table 5.x.1.5.

Table 5.x.1.5 (for Table 7.3A.4-4): Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC3 aggressor NR UL band for DL NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n77 | n18 | 10 | 15 | 25 | 5 | 5.4 | NOTE 8 | UL1/DL4X |
| n77 | n18 | 10 | 15 | 25 | 15 | 2.5 | NOTE 8 | UL1/DL4X |
| NOTE X: For a UE which supports this band combination only when the Band n77 frequency range restriction of 3520 – 4100 MHz applies, the harmonic mixing is out of scope and MSD(s) can be skipped. |  |  |  |  |  |  |  |  |

5.x.1.6	OOB blocking exception requirements

OOB exception for this CA combination has already been defined in Rel-17, so this section is omitted.

.

<End of Text Proposal>