3GPP TSG-RAN WG4 Meeting #114bis					 R4-2507281

19th May, 2025 – 23rd May, 2025

Source: 	Qualcomm Incorporated

Title: 	Views on 8Rx UE demodulation and CSI requirements with inter-cell and intra-cell inter-user interference

Agenda item:	 7.17.3

Document for:	Discussion

# 1.Introduction

- Define the following UE performance requirements for 8Rx CPE/FWA/vehicle/industrial devices:

  - PDSCH demodulation and CQI reporting requirements with inter-cell interference with MMSE-IRC receiver

    - Reuse the Rel-17 interference model for 2/4Rx UE as a baseline

  - PDSCH demodulation requirements with intra-cell inter-user interference with MMSE-IRC receiver

    - Reuse the existing interference model for 2/4Rx UE as a baseline

In Rel-18 topic NR_ENDC_ RF_FR1_enh2 WI, 8Rx performance requirements for SU-MIMO (single user) were discussed. In Rel-19, 8Rx performance requirements with inter-cell, and intra-cell inter-user interference to be defined for below objectives [1].

### In [2], RAN4 agreed to most of the performance issues. In this paper, we share simulation results and our proposals to a few general aspects.

# 2. PDSCH Demodulation Requirements inter-cell interference (ICI)

## 2.1 Simulation results

### Initial simulation results for inter-cell interference FDD and TDD scenarios are respectively shared in Table 2.1, and Table 2.2 with following test parameters.

Simulation settings:

- FDD, 15 kHz SCS, 10 MHz BW or TDD, 30 kHz SCS, 40 MHz BW

- TDA30-10, ULA Low

- Rel-17 interference settings with rank1 or rank2

- Interference power levels are 7.58 dB for 1 interferer, 7.77 dB and 2.28 dB for 2 interferers

- Antenna configuration: Rank1: 2 x 8; Rank2: 2 x 8; Rank4: 4 x 8

- SNR values 70% of maximum throughput

Table 2.1 FDD simulation results

| Setting | Channel | MCS | Baseline receiver SNR (dB) | Simplified receiver SNR (dB) | Gap = Simplified – Baseline (dB) |
| --- | --- | --- | --- | --- | --- |
| Serving cell Rank 2 + 2 interfering cells | TDLA 30-10 | 17 | 8.7 | 11.1 | 2.4 |
| Rank 4 serving cell + 1 interfering cell | TDLA 30-10 | 13 | 8.3 | 12.3 | 4.3 |

Table 2.2 TDD simulation results

| Serving cell | Channel | MCS | Baseline receiver SNR (dB) | Simplified receiver SNR (dB) | Gap = Simplified – Baseline (dB) |
| --- | --- | --- | --- | --- | --- |
| Serving cell Rank 2 + 2 interfering cells | TDLA 30-10 | 17 | 9.2 | 11.2 | 2 |
| Rank 4 serving cell + 1 interfering cell | TDLA 30-10 | 13 | 8.7 | 12.9 | 4.2 |

# 3. PDSCH Demodulation Requirements intra-cell inter-user interference

## 3.2 Simulation results for Rank2 MU-MIMO

Initial simulation results for Rank2 for both target UE and co-scheduled UE are shared in Table 3.1.

Table 3.1 Intra-cell and Inter-user interference simulation results

| Setting | Channel | Duplex | MCS | Baseline receiver SNR (dB) | Simplified receiver SNR (dB) | Gap = Simplified – Baseline (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| Rank 2 Target UE + Rank2 co-scheduled UE | TDLA30-10 | FDD | 17 | 11.2 | 14.8 | 3.6 |
| Rank 2 Target UE + Rank2 co-scheduled UE | TDLA 30-10 | TDD | 17 | 11.3 | 14.8 | 3.5 |

# 4. CQI requirements

In [2] Rank1 and Rank2 from serving cell along with Rel-17 interference settings were agreed for initial simulations. Table 4.1 has simulation results.

Table 4.1 Gamma values for FDD CQI scenarios

|  | Rank2 serving cell |  |  |
| --- | --- | --- | --- |
| SINR (dB) | Baseline Gamma (A) | Simplified Gamma (B) | Gap (A - B) |
| -3.0 | 3.5 | 2.7 | 0.8 |
| -2.0 | 2.7 | 2.5 | 0.3 |
| -1.0 | 3.0 | 2.4 | 0.6 |
| 0.0 | 3.2 | 3.1 | 0.1 |

Since SINR -1 dB is appropriate for cell-edge cases and the difference in gamma between the two receivers is more than 0.1, we propose the following:

Proposal 1: RAN4 shall have independent CQI gamma requirements for baseline and simplified receivers.

Proposal 2: RAN4 to use SINR -1 dB for both baseline and simplified receivers.

Proposal 3: RAN4 to use gamma as 2 for CQI requirements.

# 5. Applicability rules

## 5.1 Skip 2Rx/4Rx tests

In [2], the following issue on applicability rules was still open.

1.1.4 Test applicability to skip 2/4Rx requirements

- Option 4: 
Introduce test applicability under inter-cell interference and under intra-cell inter user interference to skip a 2/4Rx requirement if at least the following applies for each considered 8Rx test:
- UE support both 8Rx and 2/4Rx
- The UE passes an 8Rx requirement with same rank and interference configuration as the 2/4Rx requirement.

We propose the following applicability rules to keep limited overall number of test cases to be passed by UE. We support Option 4 of 1.1.4 in [2] where 2/4Rx test cases can be skipped only if the rank settings are the same as that of 8Rx.

Proposal 4: RAN4 to introduce test applicability for inter-cell interference and intra-cell inter user interference to skip 2/4Rx requirements if the UE passes 8Rx requirement with the same rank and interference configuration as 2/4Rx requirement.

## 5.2 MMSE-IRC receiver capability definition

Issue 1-1: Definition of MMSE-IRC receiver capability for 8Rx UE

- Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 and 38.306 section 5.2 to the following:
- Baseline 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions [with support of] up to 8 layers with joint 8Rx MIMO detector.
- Simplified 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions [with support of] only up to 4 layers with two joint 4Rx MIMO detectors.

For the open issue below [2], we support the proposal below including the phrase “with support of”.

Proposal 5: RAN4 to define non-signaling UE capability for 8Rx UE as:

Baseline 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector.
Simplified 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

# 6. Release independence

Since interference requirements are being discussed in Rel-19 feature, release independence shall be considered from Rel-18.

Proposal 6: Rel-19 8Rx performance requirements shall be independent from Rel-18.

# 7. Conclusions

Following is the summary of the proposals:

Proposal 1: RAN4 shall have independent CQI gamma requirements for baseline and simplified receivers.

Proposal 2: RAN4 to use SINR -1 dB for both baseline and simplified receivers.

Proposal 3: RAN4 to use gamma as 2 for CQI requirements.

Proposal 4: RAN4 to introduce test applicability for inter-cell interference and intra-cell inter user interference to skip 2/4Rx requirements if the UE passes 8Rx requirement with the same rank and interference configuration as 2/4Rx requirement.

Proposal 5: RAN4 to define non-signaling UE capability for 8Rx UE as:

Baseline 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector.
Simplified 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

Proposal 6: Rel-19 8Rx performance requirements shall be independent from Rel-18.

# References

[1] RP-241297, “Revised WID on NR demodulation performance: Phase 5”, 3GPP TSG RAN meeting, #104, June 17– June 20, 2024, Shanghai, China.

[2] R4-2504694, “Way Forward for [114bis][324] NR demodulation performance: Phase 5”, 3GPP TSG RAN meeting, #114bis, Apr 7– Apr 11, 2025, Wuhan, China.