3GPP TSG-RAN WG4 Meeting #115	R4-2507296

# Malta, MT, 19th – 23rd May, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on Perf part of Rx beam factor optimization of R19 RRM enhancements

Agenda Item:	7.18.5

Document for:	Discussion

- Introduction

From last RAN4 meeting, the discussion on the performance part around the multi-Rx based Rx beam sweeping factor optimization was triggered and still no progress was achieved. In this paper, we provide our views on the performance part of the Rx beam factor optimization.

- Discussion

During previous meetings around the core part discussion, in which scenarios the BSF is applicable, the following scenarios are approved as the first priority:

  - Scenario 1: SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra

  - Scenario 2: SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra

  - Scenario 3: SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter

  - Scenario 4: SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter,  TSSB_time_index_inter and TSSB_measurement_period_inter

  - Scenario 5: Handover

  - Scenario 7: RRC Re-establishment/RRC Connection Release with Redirection

So it is necessary to verify all these 6 scenarios, the corresponding test cases should be defined.

Proposal 1: It is necessary to verify all the approved scenarios in which the BSF reduction is applicable:

- Scenario 1: SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra

- Scenario 2: SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra

- Scenario 3: SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter

- Scenario 4: SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter  and TSSB_measurement_period_inter

- Scenario 5: Handover

- Scenario 7: RRC Re-establishment/RRC Connection Release with Redirection

Regarding the DRX and non-DRX, it is approved that the BSF reduction covers both DRX and non-DRX, i.e. the BSF reduction is independent of DRX configuration. So both DRX and non-DRX configurations should be verified.

Proposal 2: Both DRX and non-DRX cases should be verified.

- Conclusion

In this contribution, we have the following proposals on the perf part of Rx BSF reduction:

Proposal 1: It is necessary to verify all the approved scenarios in which the BSF reduction is applicable:

- Scenario 1: SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra

- Scenario 2: SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra

- Scenario 3: SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter

- Scenario 4: SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter  and TSSB_measurement_period_inter

- Scenario 5: Handover

- Scenario 7: RRC Re-establishment/RRC Connection Release with Redirection

Proposal 2: Both DRX and non-DRX cases should be verified.

- Reference

- R4-2504900 “WF on RRM requirements for NR_RRM_Ph5_Part2”, CATT