3GPP TSG-RAN4 Meeting # 115	R4-2507304

# Malta, MT, 19th – 23rd May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev | – | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on measurement gap skipping in intra-frequency measurement with gaps for R19 XR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_XR_Ph3-Core |  |  |  |  |  | Date: |  |  | 2025-05-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The requirements of the intra-frequency measurement with gap with measurement skipping has been determined. The relevants requirements need to be captured into 38.133. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Capture all relevant requirements of the intra-frequency measurement with gap with measurement skipping. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements of the intra-frequency measurement with gap with measurement skipping is absent. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.6.2, 9.2.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change #1>

9.2.6	Intra-frequency measurements with measurement gaps

9.2.6.1	Void

9.2.6.2	Intra-frequency cell identification

When a measurement gap is provided or an activated Pre-MG is provided without any pre-MG status changed during the measurement period, the UE shall be able to identify a new detectable intra-frequency cell within Tidentify_intra_without_index if UE is not indicated to report SSB based RRM measurement result with the associated SSB index (reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured), or the UE has been indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled). Otherwise UE shall be able to identify a new detectable intra-frequency cell within Tidentify_intra_with_index. The UE shall be able to identify a new detectable intra-frequency SS block of an already detected cell within Tidentify_intra_without_index. It is assumed that deriveSSB-IndexFromCell is always enabled for FR1 TDD and FR2 with SCS smaller or equal to 480 kHz.

Tidentify_intra_without_index = TPSS/SSS_sync_intra + TSSB_measurement_period_intra ms

Tidentify_intra_with_index = TPSS/SSS_sync_ntra + TSSB_measurement_period_intra + TSSB_time_index_intra ms

Where:

TPSS/SSS_sync_intra: it is the time period used in PSS/SSS detection given in tables 9.2.6.2-1, 9.2.6.2-2 or 9.2.6.2-9.

-	For UE supporting power class 6 with highSpeedMeasFlagFR2-r17 configured, if SMTC ≤ 40ms, TPSS/SSS_sync_intra is given in table 9.2.6.2-9; otherwise, TPSS/SSS_sync_intra is given in table 9.2.6.2-2.

-    For UE capable of [measurement gap skipping indication] and the [measurement gap skipping enable] is configured by NW, if there is at least one non-available SMTC occasion within a measurement gap occasion cancelled due to the UE XR operation during TPSS/SSS_sync_intra,TPSS/SSS_sync_intra is given in table 9.2.6.2-x1, 9.2.6.2-x2.

TSSB_time_index_intra: it is the time period used to acquire the index of the SSB being measured given in table 9.2.6.2-3 or 9.2.6.2-10 (for FR2-2).

-     For UE capable of [measurement gap skipping indication] and the [measurement gap skipping enable] is configured by NW, if there is at least one non-available SMTC occasion within a measurement gap occasion cancelled due to the UE XR operation during TSSB_time_index_intra, TSSB_time_index_intra is given in table 9.2.6.2-y1, 9.2.6.2-y2.

TSSB_measurement_period_intra: equal to a measurement period of SSB based measurement given in table 9.2.6.3-1 or 9.2.6.3-2.

-	For UE supporting power class 6 with highSpeedMeasFlagFR2-r17 configured, if SMTC ≤ 40ms, TSSB_measurement_period_intra is given in table 9.2.6.3-4; otherwise, T SSB_measurement_period_intra is given in table 9.2.6.3-2.

-	For power class 6 UE supporting measEnhCAInterFreqFR2-r18 when highSpeedMeasFlagFR2-r17 is configured, the TSSB_measurement_period_intra given in table 9.2.6.3-4 (if SMTC ≤ 40ms) and table 9.2.6.3-2 (if SMTC > 40ms) shall apply for SCC.

-    For UE capable of [measurement gap skipping indication] and the [measurement gap skipping enable] is configured by NW, if there is at least one non-available SMTC occasion within a measurement gap occasion cancelled due to the UE XR operation during TSSB_measurement_period_intra, TSSB_measurement_period_intra is given in table 9.2.6.3-z1, 9.2.6.3-z2.

CSSFintra: it is a carrier specific scaling factor and is determined according to CSSFwithin_gap,i in clause 9.1.5.2 for measurement conducted within measurement gaps.

Kgap is the scaling factor for a SSB frequency layer to be measured within an associated measurement gap pattern. Kgap = 1 when the UE is not configured with or the UE does not support concurrent GAPs or MUSIM gaps. Otherwise, Kgap = Ntotal / Navailable, where Navailable and Ntotal are calculated as follows:

For a window W of duration max(SMTC period, xRP_max), where xRP_max is the maximum xRP across all configured per-UE GAPs, periodic MUSIM gaps and per-FR GAPs within the same FR as the SSB frequency layer, and starting from the beginning of any SMTC occasion:

-	Ntotal is the total number of SMTC occasions that are covered by instances of the associated measurement gap within the window W, including those overlapped with other GAP and MUSIM gap occasions within the window, and

Navailable is the number of SMTC occasions that are covered by instances of the non-dropped associated measurement gap within the window W after accounting for GAP and MUSIM gap collisions by applying the collision rules for GAP and MUSIM gap in clauses 9.1.8.3, 9.1.10.4, 9.1.10.5, 9.1.12.3, and 9.1.13.3, respectively.

-	xRP = MGRP when configured GAP is activated Pre-MG or MG, and xRP = VIRP when configured GAP is NCSG, also xRP = MGRP for periodic MUSIM gap.

When concurrent GAPs are configured, requirements in this clause do not apply if Navailable =0.

When UE supports [MUSIM-GapConfig-17] and the configured aperiodic MUSIM gap collides with the measurement gap associated with the target frequency layer, where MUSIM gap collision rule in clause 9.1.10.4 is applied, longer cell identification period for the target intra-frequency is expected.

Mpss/sss_sync_with_gaps : For a UE supporting FR2-1 power class 1 or 5, Mpss/sss_sync with_gaps=40. For a UE supporting FR2-1 power class 2, Mpss/sss_sync with_gaps =24.  For a UE supporting FR2-1 power class 3, Mpss/sss_sync with_gaps =24. For a UE supporting FR2-1 power class 4, Mpss/sss_sync with_gaps =24. For a UE supporting FR2-1 power class 6, Mpss/sss_sync with_gaps =24. For a UE supporting FR2-2 power class 1, Mpss/sss_sync with_gaps = 60. For a UE supporting FR2-2 power class 2, Mpss/sss_sync with_gaps = 36. For a UE supporting FR2-2 power class 3, Mpss/sss_sync with_gaps = 36.

Mmeas_period_ with_gaps: For a UE supporting FR2-1 power class 1 or 5, Mmeas_period_ with_gaps =40. For a UE supporting FR2-1 power class 2, Mmeas_period_ with_gaps =24. For a UE supporting FR2-1 power class 3, Mmeas_period_ with_gaps =24. For a UE supporting FR2-1 power class 4, Mmeas_period with_gaps =24. For a UE supporting FR2-1 power class 6, Mmeas_period with_gaps =24. For a UE supporting FR2-2 power class 1, Mmeas_period_ with_gaps = 60. For a UE supporting FR2-2 power class 2, Mmeas_period_ with_gaps = 36. For a UE supporting FR2-2 power class 3, Mmeas_period_ with_gaps = 36.

-	MSSB_index_intra: For a UE supporting FR2-2 power class 1, MSSB_index_intra = 72. For a UE supporting FR2-2 power class 2, MSSB_index_intra = 48. For a UE supporting FR2 power class 3, MSSB_index_intra = 48.

If the higher layer signaling in TS 38.331 [2] of smtc2 is present and smtc1 is fully overlapping with measurement gaps and smtc2 is partially overlapping with measurement gaps, requirements are not specified for Tidentify_intra_without_index or Tidentify_intra_with_index.

If MCG DRX is in use, cell identification requirements for intra-frequency measurement in MCG specified in table 9.2.6.2-1, table 9.2.6.2-2, and table 9.2.6.2-3 shall depend on the MCG DRX cycle. If SCG DRX is in use, cell identification requirements for intra-frequency measurement in SCG specified in table 9.2.6.2-1, table 9.2.6.2-2, and table 9.2.6.2-3 shall depend on the SCG DRX cycle. Otherwise, the requirements for when DRX is not in use shall apply.

Table 9.2.6.2-1: Time period for PSS/SSS detection (FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600 ms, 5 x Kgap x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320 ms | max(600 ms, ceil(M2Note 1x 5 x Kgap) x max(MGRP, SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | Ceil( 5 x Kgap ) x max(MGRP, DRX cycle) x CSSFintra |
| NOTE 1: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1. NOTE 2: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. NOTE 3: For a UE supporting concurrent GAPs, if multiple concurrent GAPs are configured, the MGRP is the periodicity of the activated Pre-MG or the MG pattern associated to the intra-frequency layer. NOTE 4: When highSpeedMeasCA-Scell-r17 is configured, the requirements apply to UE on measurements of secondary component carrier with active SCell. |  |

Table 9.2.6.2-2: Time period for PSS/SSS detection (FR2)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600 ms, Mpss/sss_sync_with_gaps x KFR x Kgap x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320 ms | max(600 ms, ceil(1.5x Mpss/sss_sync_with_gaps x KFR x Kgap) x max(MGRP, SMTC period, DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | Ceil(Mpss/sss_sync_with_gaps x KFR x Kgap ) x max(MGRP, DRX cycle) x CSSFintra |
| NOTE 1: For a UE supporting concurrent GAPs, if multiple concurrent GAPs are configured, the MGRP is the periodicity of the activated Pre-MG or the MG pattern associated to the intra-frequency layer. NOTE 2: KFR is a scaling factor depending on the frequency range and the SSB SCS. For FR2-1, KFR = 1. For FR2-2: KFR = 1 if the SCS of the SSB of the cell being detected is 120 kHz, KFR = 2 if the SCS of the SSB of the cell being detected is 480 kHz, and KFR = 3 if the SCS of the SSB of the cell being detected is 960 kHz. |  |

Table 9.2.6.2-x1: Time period for PSS/SSS detection (FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | [max(600 ms, (5+Nskip) x max(MGRP, SMTC period)) x CSSFintra] |
| DRX cycle≤ 320 ms | [max(600 ms, ceil(M2Note 1x (5+Nskip)) x max(MGRP, SMTC period,DRX cycle)) x CSSFintra] |
| DRX cycle>320 ms | [Ceil( 5+Nskip) x max(MGRP, DRX cycle) x CSSFintra] |
| NOTE 1: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1. NOTE 2: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. NOTE 3: For a UE supporting concurrent GAPs, if multiple concurrent GAPs are configured, the MGRP is the periodicity of the activated Pre-MG or the MG pattern associated to the intra-frequency layer. NOTE 4: When highSpeedMeasCA-Scell-r17 is configured, the requirements apply to UE on measurements of secondary component carrier with active SCell. NOTE 5: Nskip is the number of skipped gap occasions overlapping with SMTC in the measurement period. |  |

Editor note: The formulas depend on the suspending issue discussion.

Table 9.2.6.2-x2: Time period for PSS/SSS detection (FR2)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | [max(600 ms, (Mpss/sss_sync_with_gaps+Nskip) x KFR x max(MGRP, SMTC period)) x CSSFintra] |
| DRX cycle≤ 320 ms | [max(600 ms, ceil(1.5x (Mpss/sss_sync_with_gaps+Nskip) x KFR) x max(MGRP, SMTC period, DRX cycle)) x CSSFintra] |
| DRX cycle>320 ms | [Ceil(Mpss/sss_sync_with_gaps x KFR) x max(MGRP, DRX cycle) x CSSFintra] |
| NOTE 1: For a UE supporting concurrent GAPs, if multiple concurrent GAPs are configured, the MGRP is the periodicity of the activated Pre-MG or the MG pattern associated to the intra-frequency layer. NOTE 2: KFR is a scaling factor depending on the frequency range and the SSB SCS. For FR2-1, KFR = 1. For FR2-2: KFR = 1 if the SCS of the SSB of the cell being detected is 120 kHz, KFR = 2 if the SCS of the SSB of the cell being detected is 480 kHz, and KFR = 3 if the SCS of the SSB of the cell being detected is 960 kHz. NOTE 3: Nskip is the number of skipped gap occasions overlapping with SMTC in the measurement period. |  |

Editor note: The formulas depend on the suspending issue discussion.

Table 9.2.6.2-3: Time period for time index detection (FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(120 ms, ceil(3 x Kgap ) x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320 ms | max(120 ms, ceil(M2Note 1x 3 x Kgap) x max(MGRP, SMTC period,DRX cycle) x CSSFintra) |
| DRX cycle>320 ms | Ceil(3 x Kgap )x max(MGRP, DRX cycle) x CSSFintra |
| NOTE 1: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1. NOTE 2: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. NOTE 3: For a UE supporting concurrent GAPs, if multiple concurrent GAPs are configured, the MGRP is the periodicity of the activated Pre-MG or the MG pattern associated to the intra-frequency layer. NOTE 4: When highSpeedMeasCA-Scell-r17 is configured, the requirements apply to UE on measurements of secondary component carrier with active SCell. |  |

Table 9.2.6.2-7: Void

Table 9.2.6.2-8: Void

Table 9.2.6.2-8: Void

Table 9.2.6.2-9: Time period for PSS/SSS detection when highSpeedMeasFlagFR2-r17 is configured, (FR2) when SMTC period ≤ 40ms

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600 ms, M1Note 2 x Kgap x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 80 ms | max(600 ms, ceil(M1Note2 x Kgap) x max(MGRP, SMTC period, DRX cycle)) x CSSFintra |
| 80 ms< DRX cycle≤ 320 ms | max(600 ms, ceil(Mpss/sss_sync_with_gaps x Kgap) x max(MGRP, SMTC period, DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | Ceil( Mpss/sss_sync_with_gaps x Kgap ) x max(MGRP, DRX cycle) x CSSFintra |
| NOTE 1: For a UE supporting concurrent GAPs, if multiple concurrent GAPs are configured, the MGRP is the periodicity of the activated Pre-MG or the MG pattern associated to the intra-frequency layer. NOTE 2: For UE supporting power class 6, M1 = 6 if highSpeedMeasFlagFR2-r17 = set1 or M1 = 18 if highSpeedMeasFlagFR2-r17 = set2 NOTE 3: Void |  |

Table 9.2.6.2-10: Time period for time index detection (FR2-2)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(200 ms, ceil(MSSB_index_intra x Kgap x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320 ms | max(200 ms, ceil(1.5 x MSSB_index_intra x Kgap) x max(MGRP, SMTC period, DRX cycle) x CSSFintra) |
| DRX cycle>320 ms | Ceil(MSSB_index_intra x Kgap)x DRX cycle x CSSFintra |

Table 9.2.6.2-11: Void

Table 9.2.6.1-12: Time period for time index detection for a UE operating on a target cell with 12 PRB SSB (FR1) (FR1)

| DRX cycle | TSSB_time_index_intra_less_than_5Mhz |
| --- | --- |
| No DRX | max(120 ms, 7 x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320 ms | max(120ms, ceil(M2 x 7) x max(MGRP, SMTC period,DRX cycle) x CSSFintra) |
| DRX cycle>320 ms | 7 x max(MGRP, DRX cycle) x CSSFintra |
| NOTE 1: Void NOTE 2: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1. NOTE 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16. |  |

Table 9.2.6.2-y1: Time period for time index detection (FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | [max(120 ms, ceil((3+Nskip)) x max(MGRP, SMTC period)) x CSSFintra] |
| DRX cycle≤ 320 ms | [max(120 ms, ceil(M2Note 1x (3+Nskip)) x max(MGRP, SMTC period,DRX cycle) x CSSFintra)] |
| DRX cycle>320 ms | [Ceil(3+Nskip) x max(MGRP, DRX cycle) x CSSFintra] |
| NOTE 1: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1. NOTE 2: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. NOTE 3: For a UE supporting concurrent GAPs, if multiple concurrent GAPs are configured, the MGRP is the periodicity of the activated Pre-MG or the MG pattern associated to the intra-frequency layer. NOTE 4: When highSpeedMeasCA-Scell-r17 is configured, the requirements apply to UE on measurements of secondary component carrier with active SCell. NOTE 5: Nskip is the number of skipped gap occasions overlapping with SMTC in the measurement period. |  |

Editor note: The formulas depend on the suspending issue discussion.

Table 9.2.6.2-y2: Time period for time index detection (FR2-2)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | [max(200 ms, ceil((MSSB_index_intra+Nskip) x max(MGRP, SMTC period)) x CSSFintra] |
| DRX cycle≤ 320 ms | [max(200 ms, ceil(1.5 x (MSSB_index_intra+Nskip)) x max(MGRP, SMTC period, DRX cycle) x CSSFintra)] |
| DRX cycle>320 ms | [Ceil((MSSB_index_intra+Nskip))x DRX cycle x CSSFintra] |
| NOTE 1: Nskip is the number of skipped gap occasions overlapping with SMTC in the measurement period. |  |

Editor note: The formulas depend on the suspending issue discussion.

9.2.6.3	Intra-frequency Measurement Period

The requirements in this clause apply when a measurement gap is provided or when an activated Pre-MG is provided without any pre-MG status changed during the measurement period.

The measurement period for FR1 intra-frequency measurements with gaps is as shown in table 9.2.6.3-1.

The measurement period for FR2 intra-frequency measurements with gaps is as shown in table 9.2.6.3-2.

When highSpeedMeasFlag-r16 is configured, T SSB_measurement_period_intra is specified in table 9.2.6.3-3.

For UE supporting power class 6 with highSpeedMeasFlagFR2-r17 configured, if SMTC ≤ 40ms, TSSB_measurement_period_intra is given in table 9.2.6.3-4; otherwise, TSSB_measurement_period_intra is given in table 9.2.6.3-2.

For power class 6 UE supporting measEnhCAInterFreqFR2-r18 when highSpeedMeasFlagFR2-r17 is configured, the TSSB_measurement_period_intra is given in table 9.2.6.3-4 (if SMTC ≤ 40ms) and table 9.2.6.3-2 (if SMTC > 40ms) shall apply for SCC.

If MCG DRX is in use, measurement period requirements for intra-frequency measurement in MCG specified in table 9.2.6.3-1 and table 9.2.6.3-2, shall depend on the MCG DRX cycle. If SCG DRX is in use, measurement period requirements for intra-frequency measurement in SCG specified in table 9.2.6.3-1and table 9.2.6.3-2, shall depend on the SCG DRX cycle. Otherwise, the requirements for when DRX is not in use shall apply.

For either an FR1 or FR2 serving cell, longer measurement period would be expected during the period Tidentify_CGI when the UE is requested to decode an NR CGI.

Table 9.2.6.3-1: Measurement period for intra-frequency measurements with gaps (FR1)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(200 ms, ceil(5 x Kgap )x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320 ms | max(200 ms, ceil(1.5x 5 x Kgap) x max(MGRP, SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | Ceil(5 x Kgap ) x max(MGRP, DRX cycle) x CSSFintra |
| NOTE 1: For a UE supporting concurrent GAPs, if multiple concurrent GAPs are configured, the MGRP is the periodicity of the activated Pre-MG or the MG pattern associated to the intra-frequency layer. |  |

Table 9.2.6.3-2: Measurement period for intra-frequency measurements with gaps (FR2)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(400 ms, ceil(Mmeas_period with_gaps x Kgap ) x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320 ms | max(400 ms, ceil(1.5 x Mmeas_period with_gaps x Kgap) x max(MGRP, SMTC period, DRX cycle)) Note 1 x CSSFintra |
| DRX cycle>320 ms | Ceil( Mmeas_period with_gaps x Kgap ) x max(MGRP, DRX cycle) x CSSFintra |
| NOTE 1: For a UE supporting concurrent GAPs, if multiple concurrent GAPs are configured, the MGRP is the periodicity of the activated Pre-MG or the MG pattern associated to the intra-frequency layer. |  |

Table 9.2.6.3-3: Measurement period When highSpeedMeasFlag-r16 is configured (FR1)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(200 ms, ceil( 5 x Kgap ) x max(MGRP, SMTC period)) Note 1 x CSSFintra |
| DRX cycle≤ 160 ms | max(200 ms, ceil(M2Note 2 x 5 x Kgap) x max(MGRP, SMTC period,DRX cycle)) x CSSFintra |
| 160 ms < DRX cycle≤ 320 ms | max(200 ms, ceil(M2Note 2 x 4 x Kgap) x max(MGRP, DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | Ceil(Y Note 3 x Kgap ) x max(MGRP, DRX cycle) x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1 NOTE 3: Y=3 when SMTC <= 40 ms, Y=5 when SMTC > 40 ms NOTE 4: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. NOTE 5: For a UE supporting concurrent GAPs, if multiple concurrent GAPs are configured, the MGRP is the periodicity of the activated Pre-MG or the MG pattern associated to the intra-frequency layer. NOTE 6: When highSpeedMeasCA-Scell-r17 is configured, the requirements also apply to UE on measurements of secondary component carrier with active SCell. |  |

Table 9.2.6.3-4: Measurement period for intra-frequency measurements with gaps when highSpeedMeasFlagFR2-r17 is configured (FR2-1) when SMTC period≤40ms

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(400 ms, ceil(M1Note 2 x Kgap ) x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 80 ms | max(400 ms, ceil(M1Note 2 x Kgap) x max(MGRP, SMTC period, DRX cycle)) Note 1 x CSSFintra |
| 80 ms< DRX cycle≤ 320 ms | max(400 ms, ceil(Mmeas_period with_gaps x Kgap) x max(MGRP, SMTC period, DRX cycle)) Note 1 x CSSFintra |
| DRX cycle>320 ms | Ceil( Mmeas_period with_gaps x Kgap ) x max(MGRP, DRX cycle) x CSSFintra |
| NOTE 1: For a UE supporting concurrent GAPs, if multiple concurrent GAPs are configured, the MGRP is the periodicity of the activated Pre-MG or the MG pattern associated to the intra-frequency layer. NOTE 2: For UE supporting power class 6, M1 = 6 if highSpeedMeasFlagFR2-r17 = set1 or M1 = 18 if highSpeedMeasFlagFR2-r17 = set2 NOTE 3: Void |  |

Table 9.2.6.3-z1: Measurement period for intra-frequency measurements with gaps (FR1)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | [max(200 ms, ceil(5+Nskip) x max(MGRP, SMTC period)) x CSSFintra] |
| DRX cycle≤ 320 ms | [max(200 ms, ceil(1.5x (5+Nskip)) x max(MGRP, SMTC period,DRX cycle)) x CSSFintra] |
| DRX cycle>320 ms | [Ceil(5+Nskip) x max(MGRP, DRX cycle) x CSSFintra] |
| NOTE 1: For a UE supporting concurrent GAPs, if multiple concurrent GAPs are configured, the MGRP is the periodicity of the activated Pre-MG or the MG pattern associated to the intra-frequency layer. NOTE 2: Nskip is the number of skipped gap occasions overlapping with SMTC in the measurement period. |  |

Editor note: The formulas depend on the suspending issue discussion.

Table 9.2.6.3-z2: Measurement period for intra-frequency measurements with gaps (FR2)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | [max(400 ms, ceil((Mmeas_period with_gaps+Nskip)) x max(MGRP, SMTC period)) x CSSFintra] |
| DRX cycle≤ 320 ms | [max(400 ms, ceil(1.5 x (Mmeas_period with_gaps+Nskip)) x max(MGRP, SMTC period, DRX cycle)) Note 1 x CSSFintra] |
| DRX cycle>320 ms | [Ceil( (Mmeas_period with_gaps+Nskip)) x max(MGRP, DRX cycle) x CSSFintra] |
| NOTE 1: For a UE supporting concurrent GAPs, if multiple concurrent GAPs are configured, the MGRP is the periodicity of the activated Pre-MG or the MG pattern associated to the intra-frequency layer. NOTE 2: Nskip is the number of skipped gap occasions overlapping with SMTC in the measurement period. |  |

Editor note: The formulas depend on the suspending issue discussion.

<End of Change #1>