3GPP TSG RAN WG4 Meeting #115	R4-2507337

Malta, 19 May 2025 – 23 May 2025

Agenda Item:	7.12.6

Source: 	Ericsson

Title:	Discussions on Rel-19 ATG CA RRM Performance Enhancement

Document for:	Discussion

# Introduction

In the last meetings, we discussed and agreed on several general issues regarding enhancing the ATG CA performance. The approved WF of RAN4#114-bis can be found in [1]. This contribution discusses the remaining issues related to the ATG CA enhancement performance part.

# Discussion

RAN4 made significant progress in defining the core RRM requirements at the last meeting; however, the CRs have not yet been endorsed. In this contribution, we discuss our initial views on the performance requirements based on the agreements regarding core requirements and the remaining core requirements' open issues.

## Test Methods and scaling factor for UE with antenna array

Issue 2-2-4: Test method and scaling factor for UE with antenna array FFS: - The scaling factor related with antenna array will not be applied in test requirements - The test parameters, test requirements for UE with antenna array on both PCC and SCC - Proposals - Proposal 1: (CATT) - RAN4 need to discuss whether to reuse the legacy test method and scaling factor for all types of UE. - Proposal 2: (ZTE) - Reuse the test method defined in R18 ATG for all antenna array cases including intra-band CA and inter-band CA.

In the last meeting, it was discussed that the scaling factor related to the antenna array will not be applied in the test requirements. As per our understanding, in Rel-18, the scaling factor was not considered for the ATG test, as no OTA test was included. Therefore, RAN4 should not consider the scaling factor for the UE with the antenna array following the Rel-18 test method for UE antenna type.

RAN4 should not consider scaling factor for UE with antenna array following Rel-18 test method.

It is assumed that the ATG UE with antenna array on both PCC and SCC supports simultaneous reception, and only DL co-located intra-band and inter-band is considered, where no additional restriction is required. Hence, the NR-CA baseline test parameters and requirements can be reused. Also, in case 5, two separate Rx chains are considered for PCC and SCC. ATG test parameters for UE with antenna array defined for PCC can be reused for SCC without any foreseen impact.

Test parameters, test requirements for UE with antenna array on both PCC and SCC can reuse the NR-CA baseline test parameters and requirements.

## Test Setups

In the last meeting, we agreed on the following test setups.

Issue 2-2-3: Test setups Agreement: - Following test setups agreed in R18 can be reused: - TDD pattern: The legacy TDD pattern are used for PCell, FFS for SCell - Channel model: AWGN with residual doppler channel model - Number of neighbour cells: one neighbour cell in the test if needed - GNSS and UE mobility setup: UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system. - BS location: The specific gNB reference location is emulated by test system - RMC: the legacy NR RMCs defined in section A.3.1 of TS 38.133 are reused for ATG test cases. - OCNG: - Generic OCNG pattern for all unused REs defined in A.3.2.1.1 in TS 38.133. - Generic OCNG pattern for unused REs in the same bandwidth as CORESET defined in A.3.2.1.3 of TS 38.133. - Generic OCNG pattern for all unused REs outside SSB slot(s) defined in A.3.2.1.4 in TS 38.133

As there is no uplink in ATG CA, SCell can use all the TDD slots except the GP slots. Hence, the baseline pattern defined in clause A.3.1.4 of 38.133 cannot be reused in this case.

The baseline pattern defined in clause A.3.1.4 of 38.133 cannot be reused for ATG CA, SCell.

A new TDD pattern should be specified that captures the TDD pattern required for ATG SCell. TDDConf.1.3 in Table 1 and TDDConf.2.4 in Table 2 proposed below should be discussed and captured for 15kHz and 30kHz, respectively. RAN4 should discuss and add TDDConf.1.3 in Table 1 and TDDConf.2.4 in Table 2.

Table 1: TDD UL/DL configuration for SCS=15 kHz

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Reference channel |  | TDDConf.1.1 | TDDConf.1.2 | TDDConf.1.3 |
| referenceSubcarrierSpacing | kHz | 15 | 15 | 15 |
| TDD UL/DL pattern 1 Note 2 |  | ‘DSUU’ S=’10DL:2GP:2UL’ | ‘DSUU’ S=’ 6DL: 2GP: 6UL’ | ‘D’ |
| dl-UL-TransmissionPeriodicity | ms | 4 | 4 | 1 |
| nrofDownlinkSlots |  | 1 | 1 | 1 |
| nrofDownlinkSymbols |  | 10 | 6 | 0 (full slot DL) |
| nrofUplinkSlot |  | 2 | 2 | 0 |
| nrofUplinkSymbols |  | 2 | 6 | 0 |
| TDD UL/DL pattern 2 Note 2 |  | ‘D’ | ‘D’ | ‘D’ |
| dl-UL-TransmissionPeriodicity | ms | 1 | 1 | 1 |
| nrofDownlinkSlots |  | 1 | 1 | 1 |
| nrofDownlinkSymbols |  | 0 | 0 | 0 |
| nrofUplinkSlot |  | 0 | 0 | 0 |
| nrofUplinkSymbols |  | 0 | 0 | 0 |
| NOTE 1: As specified in TS 38.213 [3] and TS 38.331 [2]. NOTE 2: For information. |  |  |  |  |

Table 2: TDD UL/DL configuration for SCS=30 kHz

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | TDDConf.2.1 | TDDConf.2.2 | TDDConf.2.3 | TDDConf.2.4 |
| referenceSubcarrierSpacing | kHz | 30 | 30 | 30 | 30 |
| TDD UL/DL pattern 1 Note 2 |  | ‘3D1S4U’ S=’6DL:4GP:4UL’ | ‘1D1S2U’ S=’11DL: 1GP:2UL’ | ‘3D1S4U’ S=’4DL:4GP:6UL’ | ‘DD’ |
| dl-UL-TransmissionPeriodicity | ms | 4 | 2 | 4 | 1 |
| nrofDownlinkSlots |  | 3 | 1 | 3 | 2 |
| nrofDownlinkSymbols |  | 6 | 11 | 4 | 0 |
| nrofUplinkSlot |  | 4 | 2 | 4 | 0 |
| nrofUplinkSymbols |  | 4 | 2 | 6 | 0 |
| TDD UL/DL pattern 2 Note 2 |  | ‘DD’ | Not configured | ‘DD’ | ‘DD’ |
| dl-UL-TransmissionPeriodicity | ms | 1 | Not configured | 1 | 1 |
| nrofDownlinkSlots |  | 2 | Not configured | 2 | 2 |
| nrofDownlinkSymbols |  | 0 | Not configured | 0 | 0 |
| nrofUplinkSlot |  | 0 | Not configured | 0 | 0 |
| nrofUplinkSymbols |  | 0 | Not configured | 0 | 0 |
| NOTE 1: As specified in TS 38.213 [3] and TS 38.331 [2]. NOTE 2: For information |  |  |  |  |  |

RAN4 should discuss and add TDDConf.1.3 in Table 1 and TDDConf.2.4 in Table 2.

## Test Cases

In the previous meeting, a proposal for test cases was merged into a table in issues 2-2-5 [1]. We agreed to discuss which test cases should be considered. Below, one can find our comments on the scope of each test case.

Table 3: Test case Lists for ATG CA in Rel-19

| Test category | Test cases | Company comments |
| --- | --- | --- |
| Interruption | Interruptions during measurements on deactivated NR SCC in FR1 | Extend NR test cases for ATG-specific interruption behavior based on A.6.5.2.1 |
|  | Interruption requirement at SRS antenna port switching with 1 SRS symbol in a slot | Extend NR test cases for ATG-specific interruption behavior based on A.6.5.2.3 |
|  | Interruption requirement at SRS antenna port switching with more than 1 SRS symbol in a slot | Extend NR test cases for ATG-specific interruption behavior based on A.6.5.2.4 |
| SCell Activation and deactivation | SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle | Modify Test case for SCell known based on A.6.5.3.1 |
|  | SCell Activation and deactivation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle | Modify Test case for SCell known based on A.6.5.3.2 |
|  | SCell Activation and deactivation of unknown SCell in FR1 in non-DRX | Modify Test case for SCell unknown based on A.6.5.3.3 |
|  | Direct SCell activation at SCell addition of known SCell in FR1 | Modify Test case for Direct SCell known based on A.6.5.3.4 |
|  | Direct SCell activation at handover with known SCell in FR1 | Modify Test case for Direct SCell known based on A.6.5.3.5 |
|  | Direct SCell Activation at RRC Resume | The motivation to introduce this new test case is not clear. No test case required. |
|  | Fast SCell Activation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle | Modify Test case for Fast SCell known based on A.6.5.3.10 |
|  | Fast SCell Activation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle | Modify Test case for Fast SCell known based on A.6.5.3.11 |
|  | SCell Activation of unknown SCell with valid L3 measurement results in FR1 in non-DRX for 160 ms SCell measurement cycle | Modify Test case based on A.6.5.3.14 |
|  | TRS based SCell Activation of SSB-less SCell in FR1 inter-band CA in non-DRX | Modify Test case based on A.6.5.3.15 |
|  | Inter-band SSB-less SCell Activation based on A-TRS | Modify Test case based on A.6.5.3.16 |
| Beam Failure Recovery | Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode for ATG | Modify test case based on A.6.5.5 |
| Active BWP Switch | Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs | Modify test case based on A.6.6.3 |
|  | Simultaneous RRC-based Active BWP Switch on multiple CCs | Modify test case based on A.6.6.3 |
|  | non-simultaneous timer-based BWP switch delay on multiple CCs | No test case required. |
| SCell dormancy switch | SCell dormancy switch | Motivation for this test case is not clear. Further discussion needed. |
| Deactivated SCell measurement | Event triggered reporting on SCC with deactivated SCell test under non-DRX | No test case required. |
| Pre-configured measurement gap | Test case for Pre-configured measurement gap requirement | New test case required for Pre-MG activation/deactivation delay |
| Others | Intra-frequency measurement (with scheduling restriction verified as well) | FFS for Case 4 UEs and different subcarrier spacing. |
|  | Inter-frequency measurement (with scheduling restriction verified as well) | FFS for Case 4 UEs and different subcarrier spacing. |
|  | L1-RSRP measurement | L1-RSRP measurements for beam reporting for ATG PCell are already defined in A.19.5.3. Since the SCell and PCell are co-located, we do not see any need to introduce this for SCell. |
|  | CSI-RS based intra and inter frequency Measurements for event-triggered reporting tests without gap under non-DRX | New test case required based on A.6.6.10 and A.6.6.11 for L3 measurement |
|  | The interruption of SCell addition/release | Motivation for this test case is not clear. Further discussion needed. |
|  | The interruption of SCell activation/deactivation | Motivation for this test case is not clear. Further discussion needed. |

Furthermore, based on the agreements on the open issues we may need to introduce new test cases in future meetings. RAN4 should discuss and finalize the test cases based on Table 3 and decide on the work split during this meeting.

RAN4 should discuss and agree on the test cases based on Table 3.

# Summary and Conclusion

In this contribution, we have analyzed RAN4 RRM aspects for ATG UE DL CA and made the following observations.

Observation 1	The baseline pattern defined in clause A.3.1.4 of 38.133 cannot be reused for ATG CA, SCell.

Based on the discussion in the previous sections, we propose the following:

Proposal 1	RAN4 should not consider scaling factor for UE with antenna array following Rel-18 test method.

Proposal 2	Test parameters, test requirements for UE with antenna array on both PCC and SCC can reuse the NR-CA baseline test parameters and requirements.

Proposal 3	RAN4 should discuss and add TDDConf.1.3 in Table 1 and TDDConf.2.4 in Table 2.

Proposal 4	RAN4 should discuss and agree on the test cases based on Table 3.

# References

- R4-2504898, WF on RRM requirements for NR_ATG_enh, CMCC

- RP-240804, New WID on Enhancements for Air-to-ground network for NR.

- R4-2502591, WF on RRM requirements for NR_ATG_enh, CMCC

- R4-2420237, WF on RRM requirements for NR_ATG_enh, CMCC

- R4-2420338, WF on NR ATG UE RF, CMCC