3GPP TSG-RAN WG4 Meeting #115	R4- 2507346

# Malta, MT, 19th – 23rd May, 2025

Agenda Item:	7.32.3

Source:	MediaTek Inc.

Title:	Discussion on RF requirement impact for IoT NTN phase 3

Document for:	Approval

- Introduction

In RAN#104 plenary meeting, the WID of “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3” was approved in [1]. One of the main objectives is to enable multiplexing of multiple UEs via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH and to update RF requirements accordingly if needed.

In this contribution, our views on the RF requirements impact are provided for discussion.

- Discussion

2.1		RF requirement impact for IoT-NTN phase 3

Some of the main Rel-19 WID [1] objectives for IoT NTN are shown below for reference.

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2] - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3] Note: Strive to minimise UE impact. Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required - Support of Capacity enhancements for uplink - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2, RAN4] - Multi-tone support for 15 kHz SCS should also be considered - Specify necessary signalling, if needed - Update RF requirements accordingly, if needed Note: Impact of impairment shall be taken into account

2.2		Previous Meeting Agreements

In RAN4#114bis meeting, the approved WF [2] provided the guidance for following works. The partial content is listed below for reference.

RF core requirements Issue 2-2: Whether NW would need to use CFO grouping or resynchronization schemes to avoid frequency inaccuracy between a pair of OCC UEs and whether this will result in UE RF specification impacts. - Agreement: - FFS CFO group requirements for UE and gNB. - Other methods to ensure the CFO grouping performance are not precluded if time allowed.

In RAN1#120bis meeting, the approved LS [3] provided the information for the following works. The content is listed below for reference.

Title: LS on requirements for the phase continuity and power consistency for OCC with PUSCH in NR NTN Ph3 1. Overall description RAN1 assumes that the UE is required to maintain phase continuity and power consistency for the duration of one OCC group with PUSCH. - FFS: under which conditions the above applies, e.g., under the same conditions that phase continuity applies for DMRS bundling. RAN1 agreed the following: 2. Actions: RAN1 respectfully ask RAN4 whether the same phase continuity requirements as for DMRS bundling can be applied for OCC length 2 and/or OCC length 4 under the same conditions as for DMRS bundling, or if new requirements are needed.

2.3		Phase continuity and power consistency for OCC

It is expected that the UE is required to maintain phase continuity and power consistency for the duration of one OCC group with PUSCH.

Observation 1: It is expected that the UE is required to maintain phase continuity and power consistency for the duration of one OCC group with PUSCH.

Proposal 1: Consider whether the same phase continuity requirements as for NR-NTN DMRS bundling can be also applied or leveraged as requirements for IoT-NTN OCC operation.

3 Conclusion

In this paper, our views on RF requirements impact for NPUSCH with OCC feature are provided as follows:

Observation 1: It is expected that the UE is required to maintain phase continuity and power consistency for the duration of one OCC group with PUSCH.

Proposal 1: Consider whether the same phase continuity requirements as for NR-NTN DMRS bundling can be also applied or leveraged as requirements for IoT-NTN OCC operation.

4 Reference

- RP-241624, “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”, MediaTek Inc.

- R4-2504717, “Way Forward for [114bis][315] IoT_NTN_Ph38”, MediaTek Inc.

- R1-2503071, “LS on requirements for the phase continuity and power consistency for OCC with PUSCH in NR NTN Ph3”, MediaTek Inc.