3GPP TSG-RAN WG4 Meeting #115	R4-2507353

# Malta, MT, 19th – 23rd May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | Draft | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR38.133 L1-SINR measurements for SSB periodicity adaptation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG4 |  |  |  |  |  |  |  |  |  |
| Work item code: | Netw_Energy_NR_enh-Core |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Rel-19 work item on network enery saving enhancements specifies the objective of SSB periodicity adaptation. RRM requirements for L1-SINR measurements need to be specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of requirements for L1-SINR measurements when SSB periodicity adaptation is supported by UE and configured by network. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing requirements for L1-SINR measurements for SSB periodicity adaptation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.8.1, 9.8.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR …CR … |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | None. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--- Start of change 1 ---

## 9.8	L1-SINR measurements for Reporting

### 9.8.1	Introduction

When configured by the network, the UE shall be able to perform L1-SINR measurements with the measurement resources configured as the selection of:

-	CSI-RS based CMR and no dedicated IMR configured;

-	SSB based CMR and dedicated IMR configured;

-	CSI-RS based CMR and dedicated IMR configured.

The measurements shall be performed for a serving cell, including PCell, PSCell, or SCell, on the resources configured for L1-SINR measurements within the active BWP. For UE supporting bwpOperationMeasWithoutInterrupt-r18, the UE shall additionally be able to perform L1-SINR measurements  for a serving cell, including PCell, PSCell, or SCell, on the resources configured for L1-SINR outside the active BWP, provided that the SSB is within the configured UE-specific CBW. For UE supporting ncd-SSB-BWP-Wor-r18, the SSB and SMTC in this clause applies for both CD-SSB and NCD-SSB if it is not explicitly specified otherwise. If SSB in the active DL BWP of serving cell i is NCD-SSB, for serving cell i the requirements in clause 9.8 apply provided that serving cell i is PCell or activated PSCell.

For UE supporting [SSB periodicity adaptation], when SSB based CMR and dedicated IMR is configured, SSB in the active DL BWP of the of serving cell i is NCD-SSB, and the requirements in clause 9.8 apply provided that serving cell i is activated SCell. UE shall perform L1-SINR measurements for the activated SCell according to the SSB periodicity indicated in the latest common group DCI 2_9 command according to TS 38.214 [26].

The UE shall be able to measure all CSI-RS resources and/or SSB resources and/or CSI-IM resources of the nzp-CSI-RS-ResourceSet and/or csi-SSB-ResourceSet and/or CSI-IM-ResourceSet within the CSI-ResourceConfig settings for L1-SINR for the active BWP,  and measure interference on corresponding NZP CSI-RS or CSI-IM resources if configured, provided that the number of resources does not exceed the UE capability indicated by beamManagementSSB-CSI-RS. If the UE supports bwpOperationMeasWithoutInterrupt-r18, UE shall be able to measure the SSB resources of csi-SSB-ResourceSet within the CSI-ResourceConfig configured outside active BWP, provided that the SSB is within the configured UE-specific CBW.

The UE shall report the measurement quantity (reportQuantity) and send periodic, semi-persistent or aperiodic reports, according to the reportConfigType according to the CSI reporting configuration(s) (CSI-ReportConfig) for the active BWP.

The measurement reporting delay can be longer for the measurement reporting requirements in this clause when IDC autonomous denial is configured.

--- End of change 1 ---

--- Start of change 2 ---

### 9.8.7	Minimum requirement at transitions

For UE supporting ncd-SSB-BWP-Wor-r18, when the L1-SINR measurement transitions from measurements performed on CD-SSB to measurements performed on NCD-SSB or vice versa due to BWP switching during one measurement period, the UE shall use a L1-SINR measurement period that is the maximum of the L1-SINR measurement period corresponding to the first SSB type and the second SSB type after the BWP switching.

Subsequent to the above-mentioned measurement periods, the L1-SINR measurement period requirements are based on the SSB type after the BWP switch.

For UE supporting [SSB periodicity adaptation], when SSB based CMR and dedicated IMR is configured, when the L1-SINR measurement transitions from measurements performed on NCD-SSB with a first SSB periodicity to measurements performed on NCD-SSB with a second periodicity, after receiving a group DCI 2_9 command indicating a change of SSB periodicity for the activated SCell according to clause x.x.x.x in TS 38.214 [26], during one measurement period, the UE shall use a L1-SINR measurement period that is the maximum of the L1-SINR measurement periods corresponding to the first SSB periodicity before SSB periodicity adaptation and the second SSB periodicity after SSB periodicity adaptation.

Subsequent to the above-mentioned measurement period, the L1-SINR measurement period requirements are based on the SSB periodicity indicated in the SSB periodicity adaptation command.

--- End of change 2 ---