3GPP TSG-RAN WG4 Meeting #115	R4-2507386

Malta, Malta, May 19th – May 23rd, 2025

Source:	Nokia

Title:	Other RRM requirements for MIMO ph5

Agenda item:	7.24.4.2

Document for:	Discussion

Introduction

In this document, we present Nokia’s views regarding the core requirements that RAN4 needs to discuss in relation to the WID RP-242394, in particular about asymmetric DL sTRP/UL mTRP deployment scenarios (Objective 5) and the aspects related to CJT calibration reporting (Objective 3).

Discussion on two TAs

It was agreed in RAN4#114 meeting to further discuss the following  Error! Reference source not found.:

Issue 2-1: For Scenario#2, how to update the RRM requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP? Agreement: - Wait for RAN1 reply to the LS on whether the existing agreement for a single downlink timing reference is extended to Scenario#2. Issue 2-2: For Scenario#1, how to update gradual timing requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP? Way forward: - Discuss whether gradual timing adjustment applies to both TAGs or not. - Discuss whether gradual timing adjustment applies to DL/UL TRP or also to UL TRP. Issue 2-3: For Scenario#1, whether other updates are needed for 2TA enhancement for asymmetric DL sTRP/ UL mTRP? Way forward: - Discuss whether RACH transmission requirements need to be updated. Issue 2-4: For Scenario#1 in FR2, whether to update the RRM requirements for 2TA enhancement for FR2? Agreement: - RAN4 to define timing requirements for FR1 as well as FR2.

As per the RAN1 LS reply is as follows:

Question: Based on RAN1 LS (R1-2410870), RAN4 discussed timing requirement for two TA enhancement for asymmetric DL sTRP/ UL mTRP. RAN4 would like to check with RAN1 whether RAN1 agreements in RAN1#118bis “One downlink reference timing is supported and applied to both TAGs” are applied for the scenario that UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP(s). Answer: The RAN1 agreements in RAN1#118bis “One downlink reference timing is supported and applied to both TAGs” are NOT applied for the scenario that UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP(s). And RAN1 made the following new agreement for further clarification: Agreement: - For the scenario that the UE is configured with PL offset in joint/UL TCI state(s), when the UE is configured with rel-19 2 TAs, the UE maintains one single downlink reference timing - For the scenario that the UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP(s), when the UE is configured with rel-19 2 TAs: - The UE maintains two downlink reference timings; - Baseline assumption for this feature is that Rx timing difference between two DL reference timings is no larger than one CP length, while it is subject to optional UE capability that the Rx timing difference between two DL reference timings can be assumed to be larger than CP length. - The reference point for PRACH transmission is indicated as follows: - if “PRACH association indicator” in DCI format 1_0 is 0, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the first TAG. - if “PRACH association indicator” in DCI format 1_0 is 1, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the second TAG - Above applies for the case UE is configured with SSB-MTC-additionalPCI.

Reference timing

When the UE is not configured with a PL offset and the UL-only TRP is transmitting reference signals such as SSB(s) (which is the Scenario#2), the UE can independently monitor and measure the DL reference signals associated with the respective TRPs. The Rel-18 two TAs enhancement for multi-DCI multi-TRP scenario allow UL transmissions from a UE toward two TRPs whose links may be characterized by a large difference in the propagation delays with respect to the CP length.

As with legacy single TA requirements, it is up to the UE to decide which RS, among the RSs transmitted by each TRP, to use as DL reference timing for each TAG.

Similar to the Rel-18 two TA enhancements for m-DCI m-TRP[3][4], when the UE is not configured with a PL offset, the UE shall monitor two DL reference timings, one per TAG or TRP, and apply each TA command to the respective DL reference timing to determine the UL transmission timing toward that TAG. In that case, the system performance would be improved by using the independent reference timing from both TRPs, since the UL timing would be more robust against small timing offset variations caused by UE movement.

Use of the independent reference timing from both TRPs will ensure that the UL timing is robust for UE movement.

With asymmetric DL sTRP/UL mTRP, in case the UE is not configured with PL offset (Scenario#2), the UE shall monitor two DL reference timings, one per TAG or TRP.

MTTD requirements

Another point that was not discussed so far is regarding transmit timing different. In Rel-18, MIMO_evo has defined a capability for operation with MRTD>CP. However, since this feature name is related to “receive” time difference, it would be unclear to reuse the feature as it is defined. Therefore, we propose that a new feature is defined for the operation with larger MTTD.

Rel 18 MIMO_evo has introduced optional UE capability for operation with MRTD > CP.

With asymmetric DL sTRP/UL mTRP, in case the UE is not configured with PL offset (Scenario#2), there can be an optional UE capability to support a case scenario when Rx timing difference between two DL reference timings is larger than CP length.

With asymmetric DL sTRP/UL mTRP, in case the UE is not configured with PL offset (Scenario#2), the maximum receive timing difference requirement is needed. RAN4 to reuse Rel 18 values for Rel 19.

With asymmetric DL sTRP/UL mTRP, in case the UE is not configured with PL offset (Scenario#2), and in case the UE is configured with PL offset (Scenario#1), the maximum transmit timing difference requirement is needed. RAN4 to reuse Rel 18 values for Rel 19.

Autonomous timing adjustments

Additionally, in Scenario#1, when a UE needs to perform autonomous timing adjustments, monitoring a single downlink reference signal from the DL/UL capable TRP and using that for autonomous timing adjustments for the UL-only TRP may result in increased timing errors towards the UL-only TRP.

If the UE does not perform any autonomous timing adjustment towards the UL-only TRP, then the network would be forced to send a greater number of timing advance commands to the UE resulting in larger signaling overheads.

While performing autonomous timing adjustments, the likelihood of timing errors is lower when the UE independently monitors and measures the DL RS from each TRP (Scenario#2) when compared to when the UE uses only one DL reference timing to perform autonomous timing adjustments to two TRPs (Scenario#1).

If the UE doesn’t do autonomous timing adjustments towards the UL-only TRP, then the network will need to send a relatively larger number of timing advance commands than it would when the UE performs autonomous timing adjustment.

In Scenario#2, considering that the UE is expected to monitor two downlink reference timings, UE autonomous timing adjustment applies to both TAGs/TRPs.

With asymmetric DL sTRP/UL mTRP, in case the UE is not configured with PL offset (Scenario#2), autonomous timing adjustment applies to both TAGs or TRPs.

In general, it would be beneficial to define in Scenario#1 conditions to determine if UE autonomous timing adjustment applies to just TRP1 or to both TRP1 and TRP2. For example, that condition can be based on some UE measurements, although other options are not precluded. At least we can consider the case where low mobility is detected at the UE, for example reusing the “Relaxed measurement criterion for low mobility” already defined in clause 5.7.13.1 of TS 38.331. Using that criterion, the threshold SSearchDeltaP-Connected can be used to determine if the UE is satisfying the low mobility condition or not. If the mobility is high, i.e., the RSRP variation measured at the UE is large, autonomous timing adjustment towards the UL-only TRP is disabled, otherwise, if the mobility is low, i.e., the RSRP variation measured at the UE is small, autonomous timing adjustment towards the UL-only TRP is enabled.

With asymmetric DL sTRP/UL mTRP, in case the UE is configured with PL offset (Scenario#1), specify conditions under which UE autonomous timing adjustment applies to just the DL/UL TRP or also to the UL-only TRP.

With asymmetric DL sTRP/UL mTRP, in case the UE is configured with PL offset (Scenario#1), autonomous timing adjustment towards the UL-only TRP is disabled when RSRP variation measured at the UE is large.

PRACH timing

Regarding issue 2-3 < For Scenario#1, whether other updates are needed for 2TA enhancement for asymmetric DL sTRP/ UL mTRP?>  In an asymmetric DL sTRP / UL mTRP scenario the issue has recently been observed in R4-2503935 in RAN4 #114 bis. With an UL-only TRP that does not transmit RSs such as SSBs (Scenario#1), the UE uses a single DL reference timing for both the DL/UL capable TRP (TAG1) and the UL-only TRP (TAG2). Because of that, the UE transmission timing errors toward the UL-only TRP tends to be larger when compared to a scenario, like in Rel-18, where the UE is required to monitor a DL reference timing per TAG. This timing error is then compensated via Timing advance commands (TACs) by the network. On the other hand, when the UE transmits a RACH preamble, this transmission is not time advanced to align with the uplink reception time at the gNB receiver as TACs doesnt apply to PRACH.

For example, during transmission of a preamble as part of the PRACH procedure, as specified in TS 38.133, the reference point for the UE initial transmit timing control requirement is the downlink timing of the reference cell. So, when PRACH is triggered toward an UL-only TRP that does not transmit RSs such as SSBs, as the UE doesn’t have the downlink reference timing from the UL-only TRP, the UE transmits a RACH preamble toward the UL-only TRP based on:

  - the DL reference timing from the DL/UL capable TRP

  - a timing advance offset n-TimingAdvanceOffset/n-TimingAdvanceOffset2 optionally configured by the network.

In this “asymmetric DL sTRP / UL mTRP scenario”, exactly because the UE may not be able to have a DL reference timing based on a RS sent by the UL-only TRP, the error on the transmission timing of the PRACH preamble may be too large, causing a) interference and b) the network not being able to detect such RACH preamble. Because of the guard period (GP) at the end of the RACH preamble (in case of most preamble formats), late arrival seems not a big issue, as the GP after the preamble sequence ensures that the preambles when received by the gNB receiver do not cause interference with other uplink transmissions, but early arrival can be problematic. In R4-2503935, there was a solution that was proposed to have a default timing offset depending on the TAE, more specifically 0.5*TAE ≈ 1.5 µs (TAE is defined in 38.104: maximum TAE shall not exceed 3 µs).  On the other hand, such solution is in our view not enough, because it tackles just the TAE, which relates to the imperfect UE time synchronization, but it does not tackle the difference in propagation delays from the two TRPs. Note that such 1.5 µs means roughly a gap that would allow handling a propagation delay difference without any TAE of about 450 meters, which could be enough in FR2, but not really in FR1.

Similar to the Rel-18 enhancements, the transmission timing offset of PRACH can be controlled by the network with the higher layer parameters n-TimingAdvanceOffset (and n-TimingAdvanceOffset2). More specifically, the network can configure the UE with two different PRACH transmission timings for the two TAGs. On the other hand, the network control of such PRACH transmission timings is quite rough, as n-TimingAdvanceOffset (and n-TimingAdvanceOffset2) can take only limited valuesIn addition, there could also be a timing offset/delta associated with these parameters to improve the granularity of the configurable PRACH transmission timing and be better adjusted for different deployments.

For Scenario#1, for 2TA enhancement for asymmetric DL sTRP/ UL mTRP, there is uplink interference due to a misaligned RACH preamble transmission toward an UL-only TRP which does not transmit any RS.

For an asymmetric DL sTRP/UL mTRP deployment with two TAs, RAN4 to introduce a further timing offset/delta in addition to n-TimingAdvanceOffset/n-TimingAdvanceOffset2 when PRACH is transmitted by the UE.

Discussion on TCI switching requirements

Regarding asymmetric DL sTRP/UL mTRP TCI switching, the following was agreed and discussed in the last RAN4#114 bis meeting Error! Reference source not found.:

Issue 2-5: Whether to define UL TCI state activation requirement based on SRS? Agreement: No RRM impacts on TCI state switching requirement. Issue 2-6: Whether to define UL TCI state activation requirement for Scenario#2? Way forward: discuss the following options in the next meeting for Scenario#2 - Option 1: No RRM impacts on TCI state switching requirement. - Option 2: Legacy SSB based UL TCI state activation requirement can be re-used. Scenario#2: if UE is not configured with PL offset in joint/UL TCI state(s), UE may expect to receive SSB from UL TRP(s).

For this issue, we believe that existing mTRP Rel 18 requirements are the same for Rel 19 UL-only TRP not based on SRS. It is just not clear what the proposals mean exactly, therefore we propose simply that we add clarification on Rel 18 clauses 8.23, 38.133 that the requirements also apply for Rel 19 UEs operating in UL-only TRP.

Capture in specification that Rel 18 TCI switching requirements also apply for Rel 19 UL-only TRP.

Reporting for CJT calibration

Regarding asymmetric UE reporting enhancement for CJT calibration, the following was agreed and discussed in the last RAN4#114 bis meeting Error! Reference source not found.:

Issue 1-1: On the simulation assumption for delay and frequency offset for CJT calibration Agreement: - Update the report resolution to (MD = 256, MFO = 256). Moderator’s note: simulation results are collected in R4-2504955, companies can use the table structure in the excel doc to update their results. Issue 1-2: How to define RRM core requirements for delay and frequency offset reporting based on the simulation results for CJT calibration? Agreement: - Define core requirements for delay and frequency offset reporting for - Measurement period (FFS number of samples) - Scheduling restriction (if identified)

Nokia’s viewpoint regarding the remaining open issues on the topic of UE reporting enhancement for CJT calibration and the simulation results as per the agreements are presented below.

Required accuracy for CJT calibration offsets reporting

In this section we present our views regarding CJT reporting requirements. Parameters used for simulation evaluation can be found in the appendix. In order to make this analysis, we start with system simulation results that show how CJT reporting is influencing the system throughput. Parameters for the SLS simulations can be found in Appendix A.

Figure 1 shows that normalized SE gain due to CFO compensation. In the left plot we can observe the gain of CJT with perfect reporting is small when mean UE SE is considered. However, the gain is significantly larger when the normalized SE gain due to CFO compensation is considered. In that case, the gain increases to around 12% when ell-edge UE SE is considered.

Additionally, we can also observe that the CJT gain is sensitive to the accuracy of the CJT reporting. The SLS spectral efficiency starts degrading significantly when CFO compensation error is above 10Hz, what is equivalent to 4 steps when M_CFO is equal to 256.

SLS spectral efficiency starts degrading significantly when CFO compensation error is above 10Hz, what is equivalent to 4 steps when M_CFO is equal to 256.

Figure 1: Normalized SE gain due to CFO compensation – mean UE SE vs Cell-edge UE SE

Figure 2 shows the that normalized SE gain due to PO compensation. In that figure, the CJT gain would be around 11% when mean UEs are considered. However, normalized SE gain due to CFO compensation increases to around 40% when cell-edge UEs are considered. Therefore, CJT provides better gains in cell edge than in the cell center.

Figure 2: Normalized SE gain due to PO compensation – mean UE SE vs Cell-edge UE SE

Simulations show that CJT provides better gains in cell edge than cell center.

Based on this observation, it could be recommended to use SNR=-3 dB for defining the requirements which emulate the operating conditions for a UE at the cell edge.

Use SNR=-3 dB for defining the requirements.

Core requirements for CJT calibration offsets reporting

Issue 1-2: How to define RRM core requirements for delay and frequency offset reporting based on the simulation results for CJT calibration? Agreement: - Define core requirements for delay and frequency offset reporting for - Measurement period (FFS number of samples) - Scheduling restriction (if identified)

In this section we are going to analyze LLS results and express our view on the measurement period for CJT calibration offset reporting and on the scheduling restrictions. The actual LLS campaign results are provided in the next section.

The usage of the averaging shows improvement in the accuracy for all the reported offsets, but it is not clear whether such high accuracy is required. On the other hand, collecting multiple measurement samples for averaging can introduce latency, and could add more complexity to the computation of CJT calibration reporting.

The SLS results of Figure 1 show that CFO CJT reporting accuracy of 10 Hz is good enough for providing CJT gains at cell edge. When comparing with the accuracy results of Table 2, which shows that 88 Hz (24 level accuracy) is achieved with -3 dB SNR without averaging, we can conclude that some averaging is necessary for CFO.

The results of Figure 2 show that PO CJT reporting accuracy of about 90 degrees is enough for providing good CJT gains at cell edge, and 50 degrees for excellent gain at cel edge. When comparing this with the accuracy results of Table 7, 9 levels can be achieved with -3 dB SNR without averaging. Therefore, we can conclude that 5 samples averaging is needed in this case.

Additionally, for delay offset, our results show accuracy of 1,5% of CP (4 steps) can be achieved at -3 dB SNR without averaging. That is very small in comparison to CP length, and is expected to provide good CJT gains. Even in other companies results, the worst case in RAN4 114 bis was 29 quantization steps, which is 10% of CP, which should be good enough for time offset for CJT compensation.

Use 1 sample for DO offset calibration measurement error.

Use 1 or 3 samples for PO offset calibration measurement error.

Use 5 samples for CFO offset calibration measurement error.

In a realistic scheduling scenario, the symbols not carrying TRS are typically also used for PDSCH/PDCCH, therefore it could be recommended to use data on all the OFDM symbols; except for the resource elements carrying TRS which matches with the general operational conditions of a UE for the test setup.

Additionally, Table 3 shows results comparing CFO accuracy with and without scheduling restrictions. From those results, a minor difference can be observed, only 2 quantization levels for 1 sample cases. Therefore we can conclude that scheduling restrictions are not necessary for CJT reporting.

No scheduling restrictions needed to be applied for CJT reporting.

Another issue that was raised was whether CSI-RS precoding needs to be applied for PO offset compensation. In this approach, the channel phase is precoded in the CSI-RS in order remove the channel effect from the PO estimation. To test whether this is necessary, we ran AWGN against TDLC300 simulations. AWGN is shown as an estimation of an upper bound of what theoretically is achievable with CSI-RS phase precoding. The result of this evaluation can be observed below. From those results, phase precoding could only improve accuracy to a minor extent. The greatest difference was found for the cases with 1 sample and -3 dB SNR, the error is decreased from 9 to 6 quantization levels for the comparison from TDLC with AWGN results which is a minor difference.

RAN4 not to consider CSI-RS precoding for PO reporting for CJT calibration.

Link level simulation results

Carrier frequency offset (CFO) requirements

Table 1. Simulation assumptions for the CFO reporting simulations.

| Parameter | Value |
| --- | --- |
| Waveform | OFDM |
| Carrier Frequency | 3.5GHz |
| SCS | 15kHz, 30kHz |
| Channel Model | TDLC300-10 |
| TRP number | 2 |
| UE antenna configuration | 1x2 |
| UE speed | 3km/h |
| CSI-RS Bandwidth | 48 RB |
| CSI-RS configuration | 4 TRSs per TRP |
| CSI-RS periodicity | 40, 20 slots |
| SNR | -3dB, 0dB, 3dB |
| Frequency offset between TRPs | 100ppb |
| AFO | 200ppb |

Simulation results for carrier frequency offset estimation are provided on Error! Reference source not found..

The simulation results shown below are without considering scheduling restrictions. i.e. Data on all the OFDM symbols; except for the resource elements carrying TRS.

Table 2: Carrier frequency offset estimation error.

| SCS | SNR | Frequency offset (FO) b/w two TRPs | FO estimation error in AFO = 0.2 ppm and MFO = 256 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Max (CDF5%, CDF95%) # levels 256 |  |  |
|  |  |  | # samples |  |  |
|  |  |  | #1 | #3 | #5 |
| 15KHz | -3 | 350 Hz | 24 | 18 | 5 |
| 15KHz | 0 | 350 Hz | 16 | 4 | 4 |
| 15KHz | 3 | 350 Hz | 11 | 2 | 4 |
| 30KHz | -3 | 350 Hz | 42 | 18 | 19 |
| 30KHz | 0 | 350 Hz | 27 | 19 | 19 |
| 30KHz | 3 | 350 Hz | 17 | 2 | 5 |

Table 3: Carrier frequency offset estimation error comparison of performance with scheduling restrictions and without scheduling restrictions.

| FO error max(P5,P95) for SNR=0dB |  | scenario 1: with scheduling restrictions | scenario 3: without scheduling restrictions |
| --- | --- | --- | --- |
|  | 1 sample | 15 | 16 |
|  | 3 samples | 4 | 4 |
|  | 5 samples | 4 | 4 |
|  | 1 sample | 29 | 27 |
|  | 3 samples | 19 | 19 |
|  | 5 samples | 17 | 19 |

Time offset (TO) requirements

Table 4. Simulation assumptions for the TO reporting simulations.

| Parameter | Value |
| --- | --- |
| Waveform | OFDM |
| Carrier Frequency | 3.5GHz |
| SCS | 15kHz, 30kHz |
| Channel Model | TDLC300-10 |
| TRP number | 2 |
| UE antenna configuration | 1x2 |
| UE speed | 3km/h |
| CSI-RS Bandwidth | 48 RB |
| CSI-RS configuration | 4 TRSs per TRP |
| CSI-RS periodicity | 40 slots for 30kHz; 20 slots for 15kHz |
| CSI-RS offset | Slot 10 and 11 for 15kHz , slot 20 and 21 for 30KHz |
| SNR | -3dB, 0dB, 3dB |
| Time offset between TRPs | CP/2 |
| AD | CP |

Simulation results for time offset estimation are provided on Table 5.

The simulation results shown below are without considering scheduling restrictions. i.e. Data on all the OFDM symbols; except for the resource elements carrying TRS.

Table 5: Time offset estimation error.

| SCS | SNR | Time offset (TO) b/w two TRPs | TO estimation error in AD = CP and MD = 256 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Max (CDF5%, CDF95%) # levels 256 |  |  |
|  |  |  | # samples |  |  |
|  |  |  | #1 | #3 | #5 |
| 15KHz | -3 | 0.5CP | 8 | 2 | 1 |
| 15KHz | 0 | 0.5CP | 8 | 2 | 1 |
| 15KHz | 3 | 0.5CP | 5 | 2 | 1 |
| 30KHz | -3 | 0.5CP | 8 | 2 | 1 |
| 30KHz | 0 | 0.5CP | 8 | 2 | 1 |
| 30KHz | 3 | 0.5CP | 8 | 1 | 1 |

Phase offset (PO) requirements

Table 6. Simulation assumptions for the PO reporting simulations.

| Parameter | Value |
| --- | --- |
| Waveform | OFDM |
| Carrier Frequency | 3.5GHz |
| SCS | 30kHz |
| Channel Model | TDLC300-10 |
| TRP number | 2 |
| UE antenna configuration | 1x2 |
| UE speed | 3km/h |
| CSI-RS Bandwidth | 48 RB |
| CSI-RS configuration | Single port CSI-RS (row 2) |
| CSI-RS periodicity | 40 slots |
| SNR | -3dB, 0dB, 3dB |
| Phase offset between TRPs | 180 degrees |

Phase offset estimation error values as degrees from 0 to 180 are shown in Table 7.

Table 7: Phase offset estimation error max(5th, 95th) percentile in steps.

|  |  |  | Estimation and quantization error in steps |  |  |
| --- | --- | --- | --- | --- | --- |
| Quant. Levels | SCS | SNR | No averaging | 3 samples | 5 samples |
| 32 | 30 | -3 | 10 | 5 | 5 |
| 32 | 30 | 0 | 7 | 4 | 4 |
| 32 | 30 | 3 | 5 | 3 | 3 |

Conclusion

The following Observations and Proposals were made:

Observation 1: Use of the independent reference timing from both TRPs will ensure that the UL timing is robust for UE movement.

Proposal 1: With asymmetric DL sTRP/UL mTRP, in case the UE is not configured with PL offset (Scenario#2), the UE shall monitor two DL reference timings, one per TAG or TRP.

Observation 2: Rel 18 MIMO_evo has introduced optional UE capability for operation with MRTD > CP.

Proposal 2: With asymmetric DL sTRP/UL mTRP, in case the UE is not configured with PL offset (Scenario#2), there can be an optional UE capability to support a case scenario when Rx timing difference between two DL reference timings is larger than CP length.

Proposal 3: With asymmetric DL sTRP/UL mTRP, in case the UE is not configured with PL offset (Scenario#2), the maximum receive timing difference requirement is needed. RAN4 to reuse Rel 18 values for Rel 19.

Proposal 4: With asymmetric DL sTRP/UL mTRP, in case the UE is not configured with PL offset (Scenario#2), and in case the UE is configured with PL offset (Scenario#1), the maximum transmit timing difference requirement is needed. RAN4 to reuse Rel 18 values for Rel 19.

Proposal 5: With asymmetric DL sTRP/UL mTRP, in case the UE is not configured with PL offset (Scenario#2), autonomous timing adjustment applies to both TAGs or TRPs.

Proposal 6: With asymmetric DL sTRP/UL mTRP, in case the UE is configured with PL offset (Scenario#1), specify conditions under which UE autonomous timing adjustment applies to just the DL/UL TRP or also to the UL-only TRP.

Proposal 7: With asymmetric DL sTRP/UL mTRP, in case the UE is configured with PL offset (Scenario#1), autonomous timing adjustment towards the UL-only TRP is disabled when RSRP variation measured at the UE is large.

Proposal 8: For an asymmetric DL sTRP/UL mTRP deployment with two TAs, RAN4 to introduce a further timing offset/delta in addition to n-TimingAdvanceOffset/n-TimingAdvanceOffset2 when PRACH is transmitted by the UE.

Proposal 9: Capture in specification that Rel 18 TCI switching requirements also apply for Rel 19 UL-only TRP.

Observation 6: SLS spectral efficiency starts degrading significantly when CFO compensation error is above 10Hz, what is equivalent to 4 steps when M_CFO is equal to 256.

Observation 7: Simulations show that CJT provides better gains in cell edge than cell center.

Proposal 10: Use SNR=-3 dB for defining the requirements.

Proposal 11: Use 1 sample for DO offset calibration measurement error.

Proposal 12: Use 1 or 3 samples for PO offset calibration measurement error.

Proposal 13: Use 5 samples for CFO offset calibration measurement error.

Proposal 14: No scheduling restrictions needed to be applied for CJT reporting.

Proposal 15: RAN4 not to consider CSI-RS precoding for PO reporting for CJT calibration.

- References

- R4-2504903, WF on RRM requirements for NR_MIMO_Ph5_Part2

- R4-2502667, LS on DL timing reference to support two TA enhancement for asymmetric DL sTRP/ UL mTRP, CMCC

- R4-2310178, WF on NR MIMO evolution RRM requirements, Samsung

- R4-2307185, Discussion on timing requirements for UL multi-DCI multi-TRP with two Tas, Nokia, Nokia Shanghai Bell

# Appendix A	SLS assumptions for CJT

| Parameter | Value |
| --- | --- |
| Scenario | UMa |
| Inter-site distances | 200 m |
| Carrier frequencies | 4GHz |
| Channel type | DU |
| SCS/ Number of RBs/ simulation bandwidth | 15kHz/ 52/ 10MHz |
| BS Transmit Power | Macro: 41 dBm |
| BS Height | Macro: 25m |
| BS Antenna Configuration | 16 ports: (M,N,P,Mg,Ng,Mp,Np) = (8,4,2,1,1,2,4) |
| UE Distribution | 20% outdoor (30kmph) , 80% indoor (3kmph) |
| UE Antenna Configuration | 4 Rx: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ |
| Receiver | Non-ideal 4RX MMSE |
| CJT scheduling set size | 9 TRPs |
| CodebookMode | Mode 2 |
| Number of TRPs in CJT CSI reporting (NTRP=N0) | Up to 4 TRPs, gNB configured |
| paramComb-CJT-L | {4,4,4,4} |
| Feedback assumptions | Scheduling delay (from CSI feedback to PDSCH transmission): 4ms CSI-RS period: 20ms SRS (ideal) period: 40ms |
| Traffic model | FTP model 1 with packet size 0.5 Mbytes |
| Modulation | Up to 256 QAM |
| Carrier Frequency Offset (CFO) model | i.i.d and uniformly distributed for all TRPs in (-ΔfCFO,max, ΔfCFO,max) with ΔfCFO,max corresponding to an error of 0.0875 ppm |
| Time offset (TO) model | ΔtTRP=ΔtTAE+Δtprop ΔtTAE i.i.d and uniformly distributed for all TRPs in (0, CP length) |