3GPP TSG-RAN4 Meeting  #115	R4-2507391

St Julian’s, Malta, 19th – 23rd May, 2025

Agenda item:	7.5.2

Source:	Ericsson

Title:	TP to TR38.755 on UE indication to the network

Document for:	Discussion

# 1	Introduction

This contribution provides text proposals to the TR on study of means for a UE to inform the network of appropriate CA configuration [1]:

- New bandwidth class conditioned on the frequency separation between two carriers together with several examples of UE indication to the network to support a single Rx chain for the fragmented carriers. [2].

# 2	TP to TR 38.755

<<Start of Change for TR 38.755>>

# 8	Study of means for a UE to inform the network of appropriate CA configuration it can support

## 8.1.4 Indication of fragmented CA alternative 2a: use the new bandwidth class conditioned on the frequency separation between two carriers

The network only configured supported band combination based on the UE capability and the gNB band support, not the RF requirements in 38.101-1. For instance, a slight relaxation of an RF requirement by a few dB on SCell (if necessary) may be acceptable if this means that a UE would be able to indicate support of a more complex band combination. The reason is that REFSENS degradation in PCell is not acceptable, as the PCell coverage may be lost and lead to radio link failure.

Three example configurations and one practical example use case are presented in the sub-sections below to elaborate on how the UE would indicate to the network to support a fragmented carriers with a single Rx chain.

### 8.1.4.1 Configuration of nXA + nXA with separate RX chains

It is assumed that bands nX and nY share the antenna and have 8 Rx chains available in the frequency range. As shown in Figure 8-1, two carriers in band nX are aggregated, each carrier supports 4 layers (4L). The PCell and SCell are supported by separate Rx chains and no relaxation of PCell REFSENS.

Figure 8-1. Example configuration with PCell (dark grey) and SCell (light grey): (a) nXA + nXA.

### 8.1.4.2 Configuration of nXA”+nXA”+nYA with single RX chains

In case the fragmented carriers in nX are supported by a single RX chain and the number of supported layers is reduced to 2, an additional band nY can be supported with 4 layers without being in a more complex band combination without increasing the total number of Rx chains. The PCell is configured in band nY as shown in Figure 8-2 assuming that the UE cannot support a PCell in band nX without a REFSENS degradation.

This band combination is indicated in the UE capability, but for the network to correctly parse the capability in nX (the number of layers supported) a condition on the maximum frequency separation must be added. This could be done by specifying a new BW class A” only applicable to intra-band NCCA and conditioned on a maximum frequency separation between the lowest and highest carrier edges by 100 MHz (possibly 50 MHz for FDD). A network does not comprehend the new BW class would not configure a band combination containing it, thereby ensuring that this feature does not interfere with the operation of legacy networks.

A limited frequency spacing in band nX will thus enable a single Rx implementation. In this case, a larger number of carriers can be supported with the same 8 Rx chains if the frequency separation of nX is ≤ 50 MHz (based on a single FFT in the FDD case) while maintaining the number of MIMO layers possibly with a slight REFSENS relaxation in SCells. i.e. the spared Rx chains would be used in band nY.

Figure 8-2. Example configuration with PCell (dark grey) and SCell (light grey): nXA” + nXA” + nYA (assuming that the UE cannot support a PCell in band nX without a REFSENS degradation).

### 8.1.4.3 Configuration of nXA”+nXA”+nYA+nZA with single Rx chains and with modified MIMO capability

Furthermore, an additional configuration in Figure 8-3 illustrates that reducing the number of supported layers in one of the bands allows for reducing the number of Rx chains. The Rx chain that are freed-up can then be reallocated to support more CCs. It is important to note that the fragmented carriers implemented with a single Rx chain only support SCells.

Figure 8-3. Example configuration with PCell (dark grey) and SCell (light grey): nXA” + nXA” + nYA + nZA (fragmented carriers implemented with a single Rx chain only support SCells).

### 8.1.4.4 Practical example use case for Canada, n2+n66+n78

A practical example use case for Canada with X = 2, Y = 66, and Z = 78 as depicted in the figure below. Note that although the n2 and n66 bands are ordered as n66 UL, n2 UL, n2 DL, and n66 DL, this order does not affect the overall argument. The layers supported in each band are provided as examples only.

Figure 8-4. Example use case in Canada.

As illustrated by the examples above, all supported band combinations, regardless of the UE implementation, must be explicitly included in the UE capability signaling. The UE cannot inform the NW that it has a spare Rx chain and expects the NW’s action based on this information. Additionally, the use of RF requirements to configure, release, activate, or deactivate carriers is not feasible. However, new RF requirements could enable implementations with fully shared Rx chains, thereby enable more complex band combinations.

To facilitate this, introducing a new BW class capability for the fragmented carriers within a band with restricted frequency separations and MRTD (to ensure collocated scenarios) could indicate the support for such fragmented carriers with a single Rx chain. The capabilities of the corresponding band entry (i.e. feature set) would then be conditioned on a maximum frequency separation of the carriers.

<<End of Change for TR 38.755>>

# 4	References

TR 38.755 V0.2.0

R4-2504039, "Discussion on UE capability indication for single Rx chain," Ericsson.