3GPP TSG-RAN WG4 Meeting #115									                          R4- 2507392

St Julian’s, Malta, 19th – 23rd May 2025

Agenda item:			7.5.3

Source:	Ericsson

Title:	Discussion on signaling aspects of fragmented carriers

Document for:	Approval

# Introduction

Fragmented carriers in the same band requiring multiple Rx chains may limit the UE CA capabilities if part of complex band combinations with many DL CCs combined with DL MIMO in fragmented operator spectrum holdings. A new RAN4 study item was approved in the RAN #104 meeting to investigate solutions to address this.

The objectives of the SI are as follows [1]:

Identify methods for reducing the number of UE Rx chains (e.g. from separate RF chains per CC to shared RF chains) needed for single DL band with frequency span ≤ 100 MHz, containing two non-contiguous CCs within a CA combination for the inter-operator co-located scenario, considering:

-	Which RF requirements could be adjusted for the inter-operator co-located BS scenario, e.g. existing UE RF requirements such as ΔRIBNC, ACS and in-band blocking [RAN4];

- Other requirements are not precluded, if identified

-	The ability to semi-statically switch hardware resources (i.e. Rx chains) [RAN4, RAN2 – See note 2];

-	Up to 6 dB DL received power spectral density imbalance between the two non-contiguous CCs [RAN4];

-	Determine a reasonable level for the power spectral density difference between carriers of co-located adjacent channel operators for study [RAN4]

-	Impacts on DL performance [RAN4];

-	Means for a UE to inform the network of appropriate CA configuration it can support with adjusted RF requirements [RAN4, RAN2 – See note 2].

NOTE 1:	No RAN1 impact is foreseen

NOTE 2:	RAN2 work, if necessary, will be triggered by RAN4 LS

NOTE 3:	This study starts from single DL band. Sharing RF chain is not considered among inter-band DL carriers.

- Whether and how to apply the conclusions to intra-band component of inter-band CA will be decided after the study on 2CC in a single DL band is completed.

The following sections will discuss some signaling-related open issues of fragmented carriers.

# Discussions

## Indication of UE capability for supporting “one Rx RF chain”

- Consider the following alternatives [2]

  - Alternative 1: use the legacy band combination reporting scheme + Rx sharing capability indication

    - Alternative 1 has no modification of CA configurations in RAN4

  - Alternative 1a: use the legacy band combination reporting scheme + Rx sharing capability and other related information including the frequency separation between two carriers.

    - Alternative 1a has no modification of CA configurations in RAN4

  - Alternative 2: use the new notation for CA configurations with single Rx RF Chain

  - Alternative 2a: use the new bandwidth class conditioned on the frequency separation between two carriers

During the last meeting, we presented several examples and one practical use case in Canada (n2 + n66 + n78) to elaborate our proposal of indicating “one Rx chain” using a new bandwidth class conditioned on the frequency separation between two carriers. A more detailed analysis can be found in [3].

It also noted that all band combinations containing fragmented carriers supported by single Rx chain(s) must be indicated by existing BC capabilities. The capabilities could possibly include a new BW class, including UL and DL association. A corresponding TP for TR38.755 was also provided [4].

The necessity of whether relaxation is needed, or to what extent the relaxation would be, needs to be decided, taking into account the risk of proliferation of combinations with these relaxed requirements using a single Rx chain, and afterward, decide how to indicate these particular band combinations. The reason is that for instance, a slight relaxation of an RF requirement by a few dB (if necessary) on SCell may be acceptable if this means that a UE would be able to indicate support of a more complex band combination. The NW only configures supported band combinations based on the UE capability and the gNB band support, not RF requirements in 38.101-1.

We observe and propose:

- All band combinations (BC) containing fragmented carriers supported by single Rx chain(s) must be indicated by existing BC capabilities, possibly including a new BW class, including UL and DL association.

All supported band combinations, regardless of how it is implemented, must be included in the UE capability report. If a UE does not report a certain band combination, the network shall not configure it. The UE cannot dynamically inform the NW that it could have a spare Rx chain and expects the NW’s action based on that information. Use of RF requirements for configuring/releasing/activating/deactivating carriers is not feasible.

However, new RF requirements could enable implementations with fully shared Rx chains and thus more complex band combinations. Therefore, we propose as below:

- Introduce a new BW class capability for the fragmented carriers within a band with restricted frequency separations and MRTD (to ensure collocated scenarios) to indicate the UE support for a single Rx chain.

## Triggering condition to enable “one Rx RF chain”

RAN4 agree there are two aspects regarding the triggering condition enable/disable the one Rx RF chain mode:

- Option 1: Rely on existing channel quality reporting and procedures. No new indication from UE is needed

- Option 2: The UE need the measurement to determine and indicate to the network whether it can support non-contiguous CCs with one RX RF chain or not, assuming the associated degradation is no greater than allowed degradation that is specified

- Note: detailed conditions to enable/disable can be further discussed and decided separately

As we elaborated in [5] that a band combination received by a single Rx chain with significant REFSENS degradation in PCell DL is not considered feasible. In the case of a fragmented carrier in TDD or FDD with PCell operating in another band (as example configurations presented in the previous section), a slight SCell degradation may be acceptable (e.g. band nX in the previous example) if this means that a more complex band combination could be supported. In this case, the ACS and in-band blocking requirements can be maintained.

Having said that, a few dB of slight degradation in SCell cannot be used as a criterion for the NW to configure or release a cell for a supported band combination, and both the ACS and IBB will remain unchanged. The NW would use existing mechanisms for releasing, configuring, and de/activating carriers and we do not see any benefit from an additional UE indication. The number of Rx chains is always up to UE implementation, but slightly modified RF requirements could facilitate the use of shared Rx chains for fragmented carriers. As suggested in the previous section, a new BW class capability for the fragmented carriers with restricted frequency separations and MRTD (ensure collocated scenarios) would work.

- Since a gNB can use the existing mechanisms for releasing, configuring, and de/activating carriers, and both ACS and IBB remain unchanged, neither the UE indication of “a single Rx chain” nor the in-gap interference measurement is needed.

## Fall back issue

#### Issue 2-2: Discussion on semi-statically switch hardware resources procedure and fallback considerations [2]

- Further discussions on

  - CA fallback with/without UE architecture switching

    - Consider the fall back

      - from high order CA to lower order CA

      - from CA to single carrier mode

    - Decided by network

  - UE architecture switching:

    - Switching between 2 Rx RF chains and 1 Rx RF chain on CCs of intra-band NC CA

    - FFS on whether it is decided by UE itself or decided by network

### FFS on whether the additional indication from UE side is needed

As proposed above that, the UE could indicate support for single Rx chain to the network by introducing a new bandwidth class conditioned on the frequency separation of the two carriers and MRTD. The network would continue to use the existing mechanisms for releasing, configuring, activating and deactivating carriers. Unlike other alternatives, this mechanism is backwards compatible since legacy networks would ignore the new bandwidth class. Consequently, the fallback behavior for the fragmented carrier’s operation makes no difference with the existing fallback procedure and should therefore follow the legacy UE band combination fallback procedure.

- The UE fallback behavior for the fragmented carrier operation should follow the existing band combination fallback procedure.

Regarding UE architecture switching, e.g. transitions between 2Rx chains and 1 Rx chain, such changes will directly impact the UE’s reported capability, including its MIMO capability. The configuration of such UE architecture switches should always be controlled/decided by the network.

- The transition between 2Rx and 1Rx chain should always be controlled by the network.

# Proposal

In the previous sections, we made the following observations:

Observation 1	All band combinations (BC) containing fragmented carriers supported by single Rx chain(s) must be indicated by existing BC capabilities, possibly including a new BW class, including UL and DL association.

Based on the discussion in the previous sections, we propose the following:

Proposal 1	Introduce a new BW class capability for the fragmented carriers within a band with restricted frequency separations and MRTD (to ensure collocated scenarios) to indicate the UE support for a single Rx chain.

Proposal 2	Since a gNB can use the existing mechanisms for releasing, configuring, and de/activating carriers, and both ACS and IBB remain unchanged, neither the UE indication of “a single Rx chain” nor the in-gap interference measurement is needed.

Proposal 3	The UE fallback behavior for the fragmented carrier operation should follow the existing band combination fallback procedure.

Proposal 4	The transition between 2Rx and 1Rx chain should always be controlled by the network.

# References

- RP-241669, Study on NR FR1 DL Fragmented Carriers

- R4-2505109, WF on NR FR1 DL fragmented carriers study, MediaTek, Apple, Qualcomm.

- R4-2504036, Discussion on UE capability indication for single Rx chain, Ericsson.

- R4-25xxx, TP to TR 38.755 on UE indication to the network, Ericsson.

- R4-2504037, Discussion on the DL PCell REFSENS degradation of fragmented carriers, Ericsson.