3GPP TSG-RAN WG4 Meeting #115									                          R4- 2507396

St Julian’s, Malta, 19th – 23rd May 2025

Agenda item:			7.12.2.4

Source:	Ericsson

Title:	Discussion on power declaration on ATG UE CA and MIMO in the feature list

Document for:	Approval

# Introduction

In 3GPP release 18, RAN4 has completed the standardization work for the air-to-ground (ATG) network WI, and the main work focused on the FR1 single carrier scenario. To enhance the throughput to meet growing ATG capacity demand and fully utilize operators’ spectrum, the RAN plenary meeting #103 has approved a new WI on enhancements for the air-to-ground network for NR [1].

# ATG UE feature list discussion

In Rel-18, ATG UE maximum output power was agreed to be declaration based. The corresponding feature set was captured in 38.822 as below [2]:

| Features | Index | Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 | Parent IE in TS 38.331 | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 35. NR_ATG | 35-3 | Rated max output power | Indicate the support of rated maximum output power at maximum modulation order and full PRB configurations |  | maxOutputPowerATG-r18 | BandNR | No | FR1 only | Value range from 23dBm to 40dBm with 1dB as granularity | Mandatory with capability signaling for UE supports NR communication via ATG |

In Rel-19 ATG Enh WI, CA and MIMO was included in the objective and the requirements were defined accordingly. For MIMO, it was agreed to reuse the rated maximum output power reported by ATG UE in Rel-18 [3].

Agreement:

- Reuse the rated maximum output power reported by the ATG UE capability maxOutputPowerATG-r18, and consider the existing NR UL-MIMO requirement with some adaption in requirement description specific to ATG UE.

- Detailed modification of NR UL MIMO requirement with ATG capability antennaArrayType is based on conclusion of issue 4-2-1

Agreement:

- Reuse the rated maximum output power reported by the ATG UE capability maxOutputPowerATG-r18, and consider the existing NR UL-MIMO requirement with some adaption in requirement description specific to ATG UE.

- Detailed modification of NR UL MIMO requirement with ATG capability antennaArrayType is based on conclusion of issue 4-2-1

For ATG UE CA, it was agreed to reuse the capability in Rel-18.

Issue 3-1-1: whether maximum output power is declared per single carrier or per band(aggregated across all CCs) for inter-band CA and intra-band contiguous CA

Agreement:

- Reuse the existing Rel-18 capability for MOP for ATG UE in Rel-19

Issue 3-1-1: whether maximum output power is declared per single carrier or per band(aggregated across all CCs) for inter-band CA and intra-band contiguous CA

Agreement:

- Reuse the existing Rel-18 capability for MOP for ATG UE in Rel-19

However, the capability in Rel-18 did not explicitly mention whether the power is per band or per band combination. This needs to be clarified either as a new entry or update clarification in the feature list.

- Update the Rated maximum output power in the feature list with: For ATG UE single carrier (per band) For ATG UE CA (per band combination and optionally per band per band combination)

# Conclusion

In the previous sections, we made the following observations:

Based on the discussion in the previous sections, we propose the following:

Proposal 1	Update the Rated maximum output power with note: For ATG UE single carrier (per band) For ATG UE CA (per band combination)

# References

- RP-240804, New WID on Enhancements for Air-to-ground network for NR.

- TS 38.822 v18.0.0

- R4-2410687, WF on Rel-19 ATG UE requirements, CMCC.

- R4-2406594, WF on Rel-19 ATG UE requirements, CMCC.