3GPP TSG-RAN WG4 Meeting # 115	R4-2507425

# Malta, MT, 19th – 23rd May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR | xxxx | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on TS 38.101-3 to add missing CA operating bands for three bands |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_SUL_R19-Core |  |  |  |  |  | Date: |  |  | 2025-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | (NR_CADC_R19_3BDL_xBUL) The following CA operating bands are missing and should be added. - CA_n25-n71-n260 - CA_n66-n71-n260 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the missing inter-band CA operating bands for three bands. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The above mentioned CA operating bands will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2A.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... TS 38.521-3 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

### 5.2A.1	Inter-band CA between FR1 and FR2

NR carrier aggregation is designed to operate in the operating bands defined in Table 5.2A.1-1 and Table 5.2A.1-2. The band combinations include at least one FR1 operating band and one FR2 operating band.

Operating bands for CA including Band n90 are defined by the corresponding operating bands for CA including Band n41 with Band n90 replacing Band n41. For brevity the said operating bands for CA including Band n90 are not listed in the tables below but are covered by this specification.

If the mandatory simultaneous Rx/Tx capability applies for a lower order band combination, when the applicable lower order band combination is a band pair in a higher order band combination, the mandatory simultaneous Rx/Tx capability also applies for the band pairin the higher order band combination.

Table 5.2A.1-1: Band combinations for inter-band CA between FR1 and FR2 (two bands)

| NR CA Band |  | NR Band |
| --- | --- | --- |
| CA_n1-n2571 |  | n1, n257 |
| CA_n1-n2581 |  | n1, n258 |
| CA_n2-n2571 |  | n2, n257 |
| CA_n2-n2581 |  | n2, n258 |
| CA_n2-n2601 |  | n2, n260 |
| CA_n2-n2611 |  | n2, n261 |
| CA_n3-n2571 |  | n3, n257 |
| CA_n3-n2581 |  | n3, n258 |
| CA_n5-n2571 |  | n5, n257 |
| CA_n5-n2581 |  | n5, n258 |
| CA_n5-n2601 |  | n5, n260 |
| CA_n5-n2611 |  | n5, n261 |
| CA_n7-n2571 |  | n7, n257 |
| CA_n7-n2581 |  | n7, n258 |
| CA_n7-n2601 |  | n7, n260 |
| CA_n7-n2611 |  | n7, n261 |
| CA_n8-n2571 |  | n8, n257 |
| CA_n8-n2581 |  | n8, n258 |
| CA_n12-n2571 |  | n12, n257 |
| CA_n12-n2581 |  | n12, n258 |
| CA_n12-n2601 |  | n12, n260 |
| CA_n12-n2611 |  | n12, n261 |
| CA_n14-n2601 |  | n14, n260 |
| CA_n18-n2571 |  | n18, n257 |
| CA_n25-n2571 |  | n25, n257 |
| CA_n25-n2581 |  | n25, n258 |
| CA_n25-n2601 |  | n25, n260 |
| CA_n25-n2611 |  | n25, n261 |
| CA_n26-n2581 |  | n26, n258 |
| CA_n28-n2571 |  | n28, n257 |
| CA_n28-n2581 |  | n28, n258 |
| CA_n30-n2571 |  | n30, n257 |
| CA_n30-n2581 |  | n30, n258 |
| CA_n30-n2601 |  | n30, n260 |
| CA_n30-n2611 |  | n30, n261 |
| CA_n34-n2581 |  | n34, n258 |
| CA_n38-n2571 |  | n38, n257 |
| CA_n38-n2581 |  | n38, n258 |
| CA_n39-n2581 |  | n39, n258 |
| CA_n40-n2571 |  | n40, n257 |
| CA_n40-n2581 |  | n40, n258 |
| CA_n41-n2571 |  | n41, n257 |
| CA_n41-n2581 |  | n41, n258 |
| CA_n41-n2601 |  | n41, n260 |
| CA_n41-n2611 |  | n41, n261 |
| CA_n48-n2581 |  | n48, n258 |
| CA_n48-n2601 |  | n48, n260 |
| CA_n48-n2611 |  | n48, n261 |
| CA_n48-n2631 |  | n48, n263 |
| CA_n66-n2571 |  | n66, n257 |
| CA_n66-n2581 |  | n66, n258 |
| CA_n66-n260 |  | n66, n260 |
| CA_n66-n261 |  | n66, n261 |
| CA_n71-n2571 |  | n71, n257 |
| CA_n71-n2581 |  | n71, n258 |
| CA_n71-n2601 |  | n71, n260 |
| CA_n71-n2611 |  | n71, n261 |
| CA_n77-n2571 |  | n77, n257 |
| CA_n77-n2581 |  | n77, n258 |
| CA_n77-n2591 |  | n77, n259 |
| CA_n77-n2601 |  | n77, n260 |
| CA_n77-n2611 |  | n77, n261 |
| CA_n78-n2571 |  | n78, n257 |
| CA_n78-n2581 |  | n78, n258 |
| CA_n78-n2591 |  | n78, n259 |
| CA_n79-n2571 |  | n79, n257 |
| CA_n79-n2581 |  | n79, n258 |
| CA_n79-n2591 |  | n79, n259 |
| CA_n105-n2571 | n105, n257 |  |
| CA_n105-n2581 | n105, n258 |  |
| NOTE 1: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability. |  |  |

Table 5.2A.1-2: Band combinations for inter-band CA between FR1 and FR2 (three bands)

| NR CA Band | NR Band |
| --- | --- |
| CA_n1-n3-n257 | n1, n3, n257 |
| CA_n1-n3-n258 | n1, n3, n258 |
| CA_n1-n8-n257 | n1, n8, n257 |
| CA_n1-n18-n257 | n1, n18, n257 |
| CA_n1-n28-n2571 | n1, n28, n257 |
| CA_n1-n28-n258 | n1, n28, n258 |
| CA_n1-n40-n258 | n1, n40, n258 |
| CA_n1-n41-n2571 | n1, n41, n257 |
| CA_n1-n77-n2571 | n1, n77, n257 |
| CA_n1-n78-n2571 | n1, n78, n257 |
| CA_n1-n78-n258 | n1, n78, n258 |
| CA_n1-n79-n2571 | n1, n79, n257 |
| CA_n1-n105-n257 | n1, n105, n257 |
| CA_n1-n105-n258 | n1, n105, n258 |
| CA_n2-n5-n260 | n2, n5, n260 |
| CA_n2-n5-n261 | n2, n5, n261 |
| CA_n2-n12-n260 | n2, n12, n260 |
| CA_n2-n14-n260 | n2, n14, n260 |
| CA_n2-n30-n260 | n2, n30, n260 |
| CA_n2-n48-n260 | n2, n48, n260 |
| CA_n2-n48-n261 | n2, n48, n261 |
| CA_n2-n66-n260 | n2, n66, n260 |
| CA_n2-n66-n261 | n2, n66, n261 |
| CA_n2-n77-n260 | n2, n77, n260 |
| CA_n2-n77-n261 | n2, n77, n261 |
| CA_n3-n7-n257 | n3, n7, n257 |
| CA_n3-n7-n258 | n3, n7, n258 |
| CA_n3-n8-n257 | n3, n8, n257 |
| CA_n3-n18-n257 | n3, n18, n257 |
| CA_n3-n28-n2571 | n3, n28, n257 |
| CA_n3-n28-n258 | n3, n28, n258 |
| CA_n3-n41-n257 | n3, n41, n257 |
| CA_n3-n77-n2571 | n3, n77, n257 |
| CA_n3-n78-n2571 | n3, n78, n257 |
| CA_n3-n78-n258 | n3, n78, n258 |
| CA_n3-n79-n2571 | n3, n79, n257 |
| CA_n3-n79-n258 | n3, n79, n258 |
| CA_n3-n105-n257 | n3, n105, n257 |
| CA_n3-n105-n258 | n3, n105, n258 |
| CA_n5-n30-n260 | n5, n30, n260 |
| CA_n5-n48-n260 | n5, n48, n260 |
| CA_n5-n48-n261 | n5, n48, n261 |
| CA_n5-n66-n260 | n5, n66, n260 |
| CA_n5-n66-n261 | n5, n66, n261 |
| CA_n5-n77-n260 | n5, n77, n260 |
| CA_n5-n77-n261 | n5, n77, n261 |
| CA_n7-n25-n257 | n7, n25, n257 |
| CA_n7-n25-n260 | n7, n25, n260 |
| CA_n7-n26-n258 | n7, n26, n258 |
| CA_n7-n66-n257 | n7, n66, n257 |
| CA_n7-n66-n260 | n7, n66, n260 |
| CA_n7-n71-n257 | n7, n71, n257 |
| CA_n7-n71-n260 | n7, n71, n260 |
| CA_n7-n78-n258 | n7, n78, n258 |
| CA_n7-n105-n257 | n7, n105, n257 |
| CA_n7-n105-n258 | n7, n105, n258 |
| CA_n8-n77-n257 | n8, n77, n257 |
| CA_n8-n78-n2571 | n8, n78, n257 |
| CA_n12-n30-n260 | n12, n30, n260 |
| CA_n12-n66-n260 | n12, n66, n260 |
| CA_n12-n77-n260 | n12, n77, n260 |
| CA_n14-n30-n260 | n14, n30, n260 |
| CA_n14-n66-n260 | n14, n66, n260 |
| CA_n14-n77-n260 | n14, n77, n260 |
| CA_n18-n28-n257 | n18, n28, n257 |
| CA_n18-n41-n257 | n18, n41, n257 |
| CA_n18-n77-n257 | n18, n77, n257 |
| CA_n18-n78-n257 | n18, n78, n257 |
| CA_n25-n41-n257 | n25, n41, n257 |
| CA_n25-n41-n258 | n25, n41, n258 |
| CA_n25-n41-n260 | n25, n41, n260 |
| CA_n25-n66-n257 | n25, n66, n257 |
| CA_n25-n66-n260 | n25, n66, n260 |
| CA_n25-n71-n257 | n25, n71, n257 |
| CA_n25-n71-n260 | n25, n71, n260 |
| CA_n25-n77-n257 | n25, n77, n257 |
| CA_n25-n77-n260 | n25, n77, n260 |
| CA_n26-n78-n258 | n26, n78, n258 |
| CA_n28-n39-n258 | n28, n39, n258 |
| CA_n28-n40-n258 | n28, n40, n258 |
| CA_n28-n41-n257 | n28, n41, n257 |
| CA_n28-n41-n258 | n28, n41, n258 |
| CA_n28-n77-n2571 | n28, n77, n257 |
| CA_n28-n78-n2571 | n28, n78, n257 |
| CA_n28-n78-n258 | n28, n78, n258 |
| CA_n28-n79-n2571 | n28, n79, n257 |
| CA_n30-n66-n260 | n30, n66, n260 |
| CA_n30-n77-n260 | n30, n77, n260 |
| CA_n39-n40-n258 | n39, n40, n258 |
| CA_n39-n41-n258 | n39, n41, n258 |
| CA_n40-n41-n258 | n40, n41, n258 |
| CA_n40-n77-n257 | n40, n77, n257 |
| CA_n40-n78-n257 | n40, n78, n257 |
| CA_n40-n78-n258 | n40, n78, n258 |
| CA_n40-n79-n258 | n40, n79, n258 |
| CA_n41-n66-n257 | n41, n66, n257 |
| CA_n41-n66-n260 | n41, n66, n260 |
| CA_n41-n71-n257 | n41, n71, n257 |
| CA_n41-n71-n260 | n41, n71, n260 |
| CA_n41-n77-n257 | n41, n77, n257 |
| CA_n41-n78-n257 | n41, n78, n257 |
| CA_n41-n79-n257 | n41, n79, n257 |
| CA_n41-n79-n258 | n41, n79, n258 |
| CA_n48-n66-n260 | n48, n66, n260 |
| CA_n48-n66-n261 | n48, n66, n261 |
| CA_n48-n77-n260 | n48, n77, n260 |
| CA_n48-n77-n261 | n48, n77, n261 |
| CA_n66-n71-n257 | n66, n71, n257 |
| CA_n66-n71-n260 | n66, n71, n260 |
| CA_n66-n77-n257 | n66, n77, n257 |
| CA_n66-n77-n260 | n66, n77, n260 |
| CA_n66-n77-n261 | n66, n77, n261 |
| CA_n71-n77-n257 | n71, n77, n257 |
| CA_n71-n77-n260 | n71, n77, n260 |
| CA_n77-n79-n257 | n77, n79, n257 |
| CA_n77-n79-n258 | n77, n79, n258 |
| CA_n77-n79-n259 | n77, n79, n259 |
| CA_n77-n257-n2591 | n77, n257, n259 |
| CA_n78-n79-n257 | n78, n79, n257 |
| CA_n78-n79-n259 | n78, n79, n259 |
| CA_n78-n105-n257 | n78, n105, n257 |
| CA_n78-n105-n258 | n78, n105, n258 |
| CA_n78-n257-n2591 | n78, n257, n259 |
| CA_n79-n257-n2591 | n79, n257, n259 |
| NOTE 1: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability. |  |

==============================================================

<< End of changes >>