3GPP TSG-RAN WG4 Meeting #115	R4-2507427

St Julian’s, Malta, 19th – 23rd May, 2025

Source:	BT, Ericsson

Title:	Temporal and Frequency Coherence for MIMO channels in band n78

Agenda item:	7.16.2

Document for:	Discussion

Introduction

The Study on Spatial Channel Modelling for Demodulation Performance Requirements was approved at RAN#104 [1], and RAN approved way forward agreements [2-5].

In this contribution, we present observations on the temporal and frequency-domain characteristics of measured MIMO channels from typical deployments, highlighting the spatial variability of coherence metrics across different locations. This contribution aims to support the group in identifying important properties of typical propagation environments to be reflected as closely as reasonably possible in a spatial channel model. In this contribution, we extend the analysis of measurements in n78, which is an important band for many commercial networks.  Other observations of fundamental properties of MIMO channels in sample typical deployments are provided in [6, 7, 8], focusing on 2100 and 2600 MHz bands.

Field measurements from typical deployments

Figure 1: Measurement Scenario.

Measurement setup and scope

Uplink sounding reference signals (SRS) were used for the evaluation of respective MIMO channel between a typical commercial MIMO base station and a UE.  Measurements were conducted using a 40MHz channel in n78 band (3680 – 3720 MHz) in four locations A to D, as shown in Figure 1. Reported samples were collected in static positions during office hours, with some movement of people and vehicles in the coverage sector, yet not in the immediate vicinity of the measurement setup. The output of the measurements was reconstructed into channel gains per transmitter and receiver port pair (the H matrices), including all system gains and losses.

Locations A and C can have a dominant line-of-sight component (LoS) with several reflections from surrounding buildings. Locations B and D can be characterised as non-line-of-sight (NLoS) due to obstructions from surrounding buildings. However, the LoS or NLoS determination is indicative. The determination is based on an analysis of power delay profiles from previous test campaigns utilising the exact locations. A typical PDP sample for the downlink direction for Location A is provided in Figure 3 of [7].

In this contribution, we assess the temporal and frequency stability of the channel.

Time Correlation Analysis

The time correlation across SRS transmissions is computed over the full duration of the dataset by evaluating the similarity between channel snapshots separated by forward time lags of up to 2 seconds.

The general time correlation function R(Δt)  is given by the normalized inner product between channel vectors separated by a time lag Δt,  averaged over all valid time indices: 

RΔt =E{h(t)h*(t + Δt)}E{h(t)h*(t )}E{h(t + Δt)h*(t + Δt)},

where h(t) is the vectorized channel at time t, and Δt=Δ TSRS with TSRS=20 ms and Δ∈{0,1,2,…,100}. The expectation E[⋅] is approximated by a sample average across all antenna pairs and frequency realizations.

At each time lag Δt, the correlation is calculated between the channel at time t and t+Δt. The results from all valid time indices t for which both channel snapshots (t and Δt) are available are then averaged to obtain the average time correlation function RΔt . The coherence time Tc is defined as the minimum time lag (in seconds) at which the average magnitude of the time correlation function drops below a specified threshold γ:

Tc=minΔt  RΔt|≤γ}.

Figure 2: Average Time Correlation Across Four Locations Highlighting Time Lag (s).

|  | - Coherence Time, Tc [s] | - Doppler Spread, fD≈12Tc , [Hz] |
| --- | --- | --- |
| - Location A | - 1.44 | - 0.3472 |
| - Location B | - 0.1 | - 5 |
| - Location C | - 0.12 | - 4.166 |
| - Location D | - 0.1 | - 5 |

Table 1: Temporal Channel Metrics: Coherence Time and Doppler Spread (γ=0.5).

- Figure 2 shows the average time correlation as a function of time lag for four different measurement locations (A–D).

- Location A displays the most temporally stable channel, with correlation remaining above 0.5 for lags approaching 2 seconds, indicating low Doppler spread and minimal channel variation over time. In contrast, Locations B and D exhibit steep correlation decay within the first 0.1 seconds, consistent with higher Doppler spread and more dynamic propagation environments, potentially due to complex scattering. Location C shows slightly slower decay than B and D but still indicates a fast-varying channel.

- These results emphasise the spatial variability of temporal correlation and highlight the need for location-specific modelling when evaluating time coherence characteristics.

- Observation 1:  Measured channels exhibit a temporal correlation and Doppler spread, variable with UE locations.

Frequency Correlation Analysis

To evaluate the frequency selectivity of the wireless channel, we compute the frequency correlation function (FCF) across subcarriers and extract the corresponding coherence bandwidth. The normalized frequency correlation function is calculated as:

Rf(Δf)=E[hf⋅hf+Δf*]Ehf2⋅Ehf+Δf2

where h(f) is the channel frequency response at subband f. The expectation E[⋅] is approximated by a sample average across all antenna pairs and time realizations, and Δf is the frequency lag in subbands. The coherence bandwidth fc is then defined as the frequency lag corresponding to Δf  at which the average magnitude of the FCF falls below a certain threshold γ:

fc=minΔf  |RfΔf|≤γ}.

Figure 3:  Average Frequency Correlation Across Four Locations Highlighting Frequency Lag.

|  | - Coherence bandwidth, fc [Hz] | - Delay Spread, TD≈12 πfc , [ns] |
| --- | --- | --- |
| - Location A | - 7.92 MHz | - 20.1 |
| - Location B | - 720 KHz | - 221 |
| - Location C | - 2.16 MHz | - 73.7 |
| - Location D | - 4.32 MHz | - 36.8 |

Table 2: Channel Metrics: Coherence Bandwidth and Delay Spread (γ=0.5, subcarrier spacing= 30 KHz, 1 Subband = 2 PRBs).

Figure 3 represents the frequency correlation functions computed across four measurement locations (A–D), plotted as a function of frequency lag in subbands. The results highlight distinct frequency selectivity characteristics, with location A exhibiting the slowest decorrelation across frequency, indicative of a flat (frequency-nonselective) channel. In contrast, locations B and C display a faster decorrelation, likely due to multipath delay spread or scattering richness. Location D exhibits intermediate behavior with moderate correlation decay.

Table 2 summarizes the estimated coherence bandwidths and corresponding delay spreads at a threshold of 0.5. The coherence bandwidth is determined as the minimum positive frequency lag at which the frequency correlation magnitude falls below the threshold. The delay spread is then estimated using the relation TD≈12πfc.  These results underscore the spatial variability in frequency-domain characteristics.

Observation 2: Coherence bandwidth and delay spread values vary across locations, yet all experience a strong frequency correlation at small frequency lag intervals.

SCM candidates to demonstrate reasonable coherence properties for modelled channels to ensure representativeness of MIMO feature tests.

Conclusion

Observations and proposals for the SCM are summarised below.

- Observation 1: Measured channels exhibit a temporal correlation and Doppler spread, variable with UE locations.

Observation 2: Coherence bandwidth and delay spread values vary across locations, yet all experience a strong frequency correlation at small frequency lag intervals.

- SCM candidates to demonstrate reasonable coherence properties for modelled channels to ensure representativeness of MIMO feature tests.

- Acknowledgements

The measurements and data processing for this contribution have been conducted by Ericsson Research UK and BT Research. BT team has been supported in part by UKRI Future Leaders Fellowship (MR/Y020065/1).

References

- RP-241610, “3GPP Work Item Description: Study on spatial channel model for demodulation performance requirements”, Nokia, BT Plc, AT&T, Bell Mobility, Bouygues Telecom, China Telecom, CMCC, Deutsche Telekom, Ericsson, Intel Corporation, KDDI, Keysight, KT Corp., MediaTek, NTT Docomo, Orange, Qualcomm, Rohde & Schwarz, Samsung, SK Telecom, Spark NZ, Telecom Italia, Telefonica, Telenor, Telia Company, Telstra, T-Mobile USA, Verizon, Vodafone, ZTE Corporation

- R4-2413606, Way Forward for [112][327] FS_NR_demod_SCM, RAN4#112, Aug 2024.

- R4-2416591, Way Forward for [112b][328] FS_NR_demod_SCM, RAN4#112b, Oct 2024.

- R4-2419782, Way Forward for [113][320] NR_SCM, RAN4#113, Nov 2024.

- R4-2504756, Way Forward for [114bis][320] NR_SCM, RAN4#114b, April 2024.

- R4-2411557, “Scenarios and requirements for spatial channel modelling”, BT, RAN4#112, Aug 2024.

- R4-2415673, “Further observations from deployment and MU-MIMO use cases for SCM evaluation”, BT, Orange, RAN4#112b, Oct 2024.

- R4-2419338, “Further observations on the properties of MIMO channels in typical deployments”, BT, Ericsson, RAN4#113, Nov 2024.

- R4-2418043, “Discussion on Spatial Channel Model for Demodulation Performance Requirements”, Nokia, RAN4#113, Nov 2024.