3GPP TSG-RAN WG4 Meeting #115	R4-2507432

Malta, Malta, May 19th – May 23rd, 2025

Source:	Nokia

Title:	On RAN4 Impacts for RRM measurement prediction in AIML Mobility

Agenda item:	7.23.3.1

Document for:	Discussion

Introduction

The discussions on AI/ML Mobility continued in RAN4#114bis in the scope of Rel-19 SI ‎[1] and ‎‎[2]. Agreements achieved during this meeting are summarized in the WF ‎[3].

Some of the issues related to AI/ML Mobility RRM measurement prediction use case require further discussion. In this paper, we provide some additional views on issues related to RRM requirements, as listed below:

- Performance metrics

- Ground truth definition for RSRP accuracy for FR1

- Impacts on beam/cell level measurements

- Impacts from observation and prediction windows

- Inter-frequency prediction scenario

General aspects, Measurement event prediction and Testability related aspects are discussed in our accompanying papers ‎[4], ‎[5] and ‎[6].

Discussion

Performance Metrics

The following scenarios have been prioritized as high priority in RAN2:

| Scenario | Prioritization | Evaluation scenario combination | Objective | Methodology |
| --- | --- | --- | --- | --- |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | Measurement reduction | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | Measurement reduction | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | Mobility optimization | Intra-cell |

Relative Accuracy

In the RAN4#114 meeting, the following agreement was made on relative RSRP accuracy.

Issue 2-1-1a: Relative RSRP Prediction Accuracy Agreement: - Predicted relative RSRP accuracy = (reported predicted RSRP of cell 1/beam1 – reported RSRP of cell 2/beam2) – (ground truth of RSRP of cell 1/beam1 – ground truth of RSRP of cell 2/beam2) - FFS: Beam 1 and Beam 2 can be from same cell or different cells. - FFS: whether ground truth can be reported or ideal - FFS: whether the reported RSRP of cell2/beam2 can be predicted or measured or a combination of both predicted and measured samples.

Following agreement was reached during RAN4#114bis meeting:

Agreement: - For intra-frequency L3-RSRP, - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 and cell 2 are on the same frequency, or - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of beam 1 – reported predicted RSRP of beam 2) – (ground truth of RSRP of beam 1 – ground truth of RSRP of beam 2), where beam 1 and beam 2 are from the same cell. - FFS if this definition will be needed. - For inter-frequency L3-RSRP, - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2, if cell 1 and cell 2 are the target cells for AI/ML-based L3-RSRP inference. - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2, [if cell 1 is the target cell for AI/ML-based L3-RSRP inference based on the measurements on cell 2]. - Depending on the agreement on ground truth definition, we may revisit relative RSRP accuracy definition. - FFS: whether the reported RSRP of cell2 can be predicted or measured or a combination of both predicted and measured samples. - FFS, if there is any impact on this definition for different scenarios/different frequency ranges.

According to the legacy definition of relative accuracy as specified in 3GPP TS 38.133, the following applies for SS-RSRP in both intra-frequency and inter-frequency scenarios (for FR1):

- Intra-frequency (FR1):“The relative accuracy of SS-RSRP is defined as the SS-RSRP measured from one cell compared to the SS-RSRP measured from another cell on the same frequency, or between any two SS-RSRP levels measured on the same cell (in FR1).”

- Inter-frequency (FR1): “The relative accuracy of SS-RSRP in the inter-frequency case is defined as the RSRP measured from one cell on a frequency in FR1 compared to the RSRP measured from another cell on a different frequency in FR1.”

TS 38.133 supports relative intra-frequency measurement accuracy evaluations in both inter-cell and intra-cell measurement contexts. For intra-cell cases, this includes comparisons between beams of the same cell, and for inter-cell cases, between cells on same carrier frequency.

Given the legacy definition encompasses both intra-cell and inter-cell comparisons, it is expected that AI/ML-based measurement prediction methods should similarly support intra-cell comparisons, e.g., among beams of the same cell as well as inter-cell comparisons. This expectation is aligned with the study in Scenario 6, which considers intra-frequency, intra-cell, spatial-domain prediction.

For intra-frequency L3-RSRP relative accuracy definition, RAN4 to cover comparison between two RSRP levels from the same cell or different cells.

The current proposed accuracy definition for intra-frequency predicted relative L3-RSRP is limited to scenarios where both values being compared are predicted. However, in practical scenarios such as event-triggered measurement reporting a UE may predict the RSRP of one cell while comparing it against a measured value from another cell.

When defining the relative accuracy for intra-frequency predicted L3-RSRP, RAN4 to consider both of the following cases: Comparison between two predicted values, and comparison between one predicted value and one measured value.

RAN4 consider the following definition of accuracy of relative in intra-frequency; Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP level 1 – reported RSRP level 2) – (ground truth of RSRP level 1 – ground truth of RSRP of level 2), where level 1 and level 2 may belong to the same or different cells on the same frequency. [Note: Reported RSRP can be either measured or predicted]

Regarding inter-frequency predicted relative L3-RSRP accuracy, the current definition allows comparison between a predicted value and a measured value only when the predicted value is inferred based on the measured value. This constraint excludes scenarios where the two compared values are independent, i.e., one value is predicted based on a different input and not inferred from the other. An illustrative scenario may include the followings:

- The UE is served by Cell#3 on frequency F1.

- The UE measures target Cell#2 on frequency F1.

- The UE predicts target Cell#1 on a different frequency F2 using measurements from Cell#3.

Here, Cell#1 is not predicted based on Cell#2, yet a comparison between the predicted value of Cell#1 and the measured value of Cell#2 may be needed for evaluation or reporting logic.

Current draft definition of inter-frequency predicted relative L3-RSRP accuracy allows comparison between a predicted value and a measured value only when the predicted value is inferred based on the measured value.

RAN4 to extend the definition of inter-frequency predicted relative L3-RSRP accuracy to include comparisons between a predicted value and a measured value, regardless of whether one is used as the input for the other.

For inter-frequency scenarios, RAN4 consider the following definition relative accuracy; Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported  RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2. [Note: Reported RSRP can be either measured or predicted]

Regarding measurement prediction especially in the domain of temporal L3 RSRP prediction as presented in  Figure 1, the applied filtering coefficient necessitates that the UE may or may not combine and reports both predicted and measured samples as one value. These combined samples must then be compared with reports from other cells or beams to ensure accurate performance assessment. Given this process, it is crucial that the requirements for predicted relative accuracy are clearly defined and aligned with the established definitions and requirements for legacy relative accuracy specified in [TS 38.133]. This alignment ensures consistency and reliability in performance metrics, facilitating seamless integration and comparison across different network elements and technologies. Additionally, maintaining this alignment helps in optimizing network performance and enhances the overall user experience by ensuring that the predicted values are as close to the actual measured values as possible.

Figure 1: Examples of temporal domain predictions.

It is expected Case-B temporal domain prediction with applied L3 filtering to include both combination of measured and predicted values and comply with accuracy performance requirements similar to those of the legacy relative and absolute accuracy performance defined for measurement.

For temporal domain Case-B prediction, RAN4 may consider accuracy performance requirements similar to those of the legacy relative and absolute accuracy performance defined for measurement.

Ground truth definition for RSRP accuracy for FR1

In the RAN4#114 meeting, the following agreements were made for FR1 Ground truth definition for RSRP accuracy:

Issue 2-1-2: Ground Truth Definition for RSRP accuracy Agreement: - The way the ground truth is extracted in the testing may differentiate between FR1 and FR2.

Issue 2-1-2: Ground Truth Definition for RSRP accuracy - Agreement: - Alt 1: Ground truth for FR1 will be based on the transmitted or reception power - Alt 2: Ground truth for FR1 will be based on the reported measurement value under certain conditions - Further discuss the advantages and disadvantages for Alt 1 and Alt 2 considering the aspects such as: - The information at the TE side, on the timing UE performs and/or finishes the measurement and/or prediction - Impact of L3 filtering - Number of samples for L1 filtering - Timing window, if it exists, where UE can’t measure the ground truth in Alt2 - channel condition - Avoid UE cheat in the test - S(I)NR variations in the test at different time instances (for Alt2) - Other aspects are not precluded - Companies are encouraged to provide analysis on all or some of the aspects in the list

Following agreement was reached in RAN4#114bis meeting:

Issue 2-1-5: Ground truth definition for RSRP Accuracy for FR1 Agreement: During the SI, RAN4 to provide analysis on potential issue and possible solution to the two Alternatives. If RAN4 cannot make decision during the SI phase, RAN4 will discuss in the WI phase. - At least capture the definition of the two Alternatives in the TR.

Following is the RAN4#114 agreement for AIML Beam management usecase:

Agreement: - Use CDL-based channel model as starting point

Alternative 1, the ground truth for FR1 can be derived by TE based on the transmitted or received power:

- This is aligned with the legacy requirement for L3 RSRP accuracy. However, the legacy test is based on AWGN channel model. The AWGN channel is a deterministic channel, where signal power will be constant for both spatial and time domain.

- Based on the RAN4#114 agreement for Beam management which indicates the fading channel i.e. CDL channel model to be considered for FR2 as the starting point, hence CDL may also be considered as starting point for FR1 and FR2 AIML Mobility usecase.

- Derivation of ground truth based on the transmitted or received power is not suitable for the fading channel since it is challenging to identify the exact time period of the measured and the predicted samples and this may lead to inaccuracies of ground truth derivation.

- The L1 filtering method, filtering co-efficient and the number of samples for filtering are not known by the TE since it is UE proprietary implementation, hence it will impact the derivation of the ground truth at TE.

- The advantage with this alternative is that there will be no issue of UE cheating since the ground truth is derived at TE.

Alternative 1 where the FR1 ground truth can be derived by TE based on the transmitted or received power (legacy approach) is not suitable for AIML Mobility due to the fading channel and due to the L1 filtering which is UE implementation specific and not known to the TE.

Alternative 2, the ground truth for FR1 is derived based on the measurements reported by UE based on certain conditions:

- With this approach, there is no complexity of considering the channel variations and the parameters that are UE implementation specific (eg: L1 filtering related information) since these would be considered at UE while reporting the measurements.

- If the timing window is configured, some UEs may not be able to measure and predict at the same time (depending upon the UE capabilities). However, this can be handled during testing as described below:

  - For temporal domain case A, measurement is always performed (i.e. there is no measurement reduction), hence UE can always report the measured values that will serve as ground truth.

  - For temporal domain case B, few measurements are skipped in the observation window. Hence the continuous measurement values will not be available at UE to report to TE. However, there are mechanisms to overcome this issue during testing where UE can perform legacy measurements and AIML predictions parallelly and report to TE depending upon the UE capability. Another method is to perform the test with iteration 1 configured for legacy method of measurement reporting (serves as ground truth) and next iteration of test can be configured for AIML predictions under same channel conditions in both the iterations.

- S(I)NR variations in the test at different time instances may pose the challenge for this alternative. However, this can be handled in multiple ways. For example: One option is to maintain average SINR value during the test however measure at higher SINR level for more accurate ground truth and the SINR may be lower during the prediction with reference to the average SINR.

- Another important aspect is to ensure that UE cheating does not occur with this alternative. This can be handled via various test procedures. Test procedures to identify UE cheating have been described in our accompanying paper on the testability aspects [1].

Alternative 2, where the FR1 ground truth is derived based on the measurements reported by UE based on certain conditions, is more suitable for AIML Mobility since the channel variations and L1 filtering are already considered by UE while performing measurements that serve as ground truth.

Alternative 2, where the FR1 ground truth is derived based on the measurements reported by UE based on certain conditions, is the preferred option for ground truth definition for RSRP accuracy even though there are inaccuracies in the UE measurements, since it is more feasible option for the dynamic channel conditions.

Impacts on beam/cell level measurements

In addition, as outlined in below from RAN4#112bis, further investigation is needed on the performance metrics used to evaluate the effectiveness of AI/ML measurement predictions, as discussed in the previous meeting.

Issue 2-1-1: Performance Metrics/KPIs for Measurement Prediction use case Agreement: - RAN4 to first study the impacts on requirements for the following case: - L3 cell level RSRP (Point C) prediction - FFS L1 beam-level (Point A1) - FFS L3 beam-level (Point E) - Other cases can be considered based on RAN4 further discussion and RAN2 further progress.

Accordingly, RAN4 has agreed to focus on studying L3 cell-level RSRP (Point C) for evaluating measurement prediction. Additionally, two other options—L1 beam-level (Point A1) and L3 beam-level (Point E)—remain open for further discussion. For L1 measurements, as outlined in sub-use cases 1 and 3, the UE can either derive L3 cell-level measurements from predicted L1 measurements or directly predict cell-level measurements based on actual L1 measurements. Since L3 cell-level measurements and predictions are expected to be reported by the UE or used by the network for RRM and handover decisions, it is logical to consider these as the primary evaluation point for performance metrics. As L3 beam-level measurements could further optimize mobility, they can be examined after the study of cell-level measurements or undergo additional evaluation by RAN2.

Additionally, considering both L3 beam-level and L3 cell-level measurements, RAN2#127 and RAN2#127bis have reached agreements prioritizing L3 cell-level measurements for simulation and performance evaluation.

RAN2#127 FS_NR_AIML_Mob - Release 19 Agreements - Companies can consider to do L3 filtered beam level results for any of this cases. L3 filtered beam level prediction cases are lower priority. - Case 1: To predict L1 filtered beam level results, then generate L3 filtered results based on the predicted L1 beam results. - Case 2: To directly predict L3 filtered beam level results based on the L3 beam level measurement results. - Case 3: To directly predict L3 filtered beam level results based on the L1 beam level measurement results. - If companies do L3 filtered beam level prediction simulations, they should focus on FR2-to-FR2 intra-frequency temporal domain prediction case A

RAN2#127bis FS_NR_AIML_Mob - Release 19 Agreements - To keep two filtering options on the table and up to company to report. - One fixed sampling period of FR2 is introduced for L1/L3 filtering option 1 to replace existing one i.e., 20ms. The detail value is FFS. - In the definition of 3 RRM sub-cases, all cell level measurement result(s) refers to L3 filtered cell level measurement

Following agreement was reached in RAN4#114bis meeting:

Agreement: - For Rel. 19 study: - RAN4 to not consider L1 beam-level (Point A1) measurement prediction.

Based on RAN2 agreements reached during RAN2#127bis meeting, RAN4 may need to consider L3 beam-level measurements as well.

RAN4 to focus first on L3 cell level RSRP (Point C) prediction and after that L3 beam-level prediction (Point E).

Impacts from observation and prediction windows

Following agreement was reached in RAN4#114 meeting:

Issue 2-1-9: Impacts from observation and prediction windows Agreement: - RAN4 to consider the impact of UE speed, MRRT and OW and PW lengths on prediction accuracy requirements for temporal predictions. - RAN4 to also consider number of measurements performed during OW and its impacts on the prediction accuracy.

For temporal domain prediction case A, no measurement reduction is considered. Depending on the subcase, either L1 measurements (sub-cases 1 and 3) or L3 measurement (sub-case 2) are considered as input to the ML model. The estimation of the number of L1 measurement samples within the OW should account for multiple factors such as L1 sampling period and filter parameters (which are UE proprietary implementation). Thus, it is not straightforward to evaluate the number of L1 measurements performed during the OW and therefore assess its impact on the prediction accuracy. Furthermore, it is possible to know the number of L3 measurement samples in a given OW (using measurement periodicity), however the number of L3 measurements input to the AI/ML model may still be considered as UE proprietary and should not be considered by RAN4.

RAN4 to not consider number of L1/L3 measurements performed during OW and its impacts on the prediction accuracy

For temporal domain prediction case B, the UE is not required to measure all the time and may use instead predicted measurements. The degree of measurement reduction can be selected through the Measurement reduction rate in temporal domain (MRRT) parameter.

For the testing, the MRRT can be set to [50, 60, 66, 75, 80] % and UE speed can be set to [30, 60, 90, 120] km/h, resulting in a total number of 20 combinations. The number of test configurations can further increase with different OW and PW values as well.

The number of possible test cases can increase considerably for a set of selected OW, PW and MRRT with multiple UE speed cases.

For temporal domain prediction case B, RAN4 to agree on pre-selected reference testing scenarios relating to OW, PW and MRRT as well as UE speed.

However, for a given MRRT value, different possible ways can be employed to achieve the targeted measurement reduction level. As shown in the example below, 50% MRRT can be reached with two different skipping patterns: either completely replacing consecutive measurement slots with predictions in the PW (example 1) or alternating measured and skipped slots within the OW (example 2).

Figure 2: Example of MRRT 50% scenario 2 with different skipping patterns

As discussed in RAN2, for a given MRRT, the skipping pattern applies to L1 samples and depend on the UE proprietary L1 filter implementation. For this reason, it is under discussion if the UE needs to be able to indicate the measurement skipping patterns it supports to the NW. Based on this, the NW can eventually configure the MRRT through the explicit indication of the selected skipping pattern (from the set indicated by the UE).

Therefore, for RAN4, two options can be foreseen for the testing of the measurement reduction feature:

- Option 1: The skipping pattern information is UE proprietary and is not shared with the NW. The TE will then only indicate the MRRT to the DUT without precise knowledge of what is skipped or not.

- Option 2: The skipping pattern information is UE proprietary, but the UE informs the NW about the skipping patterns it supports. In this case, the TE should select the skipping pattern related to the MRRT to be tested and indicate it to the DUT as part of the test configuration.

Note that with option 1, the UE can possibly cheat, and the test framework is not robust. In this case, the TE may only set up a requirement on the prediction accuracy and verify if the test passes or not.

If skipping pattern information is not shared by the UE, RAN4 to only consider MRRT, OW and PW to UE as test configuration and set up a requirement on prediction accuracy.

If skipping pattern information is shared by the UE, TE should select the supported skipping pattern related to the MRRT to test. The test configuration should then include skipping pattern, OW and PW.

Inter-frequency prediction scenario

Following agreement was reached during RAN4#114bis meeting:

Agreement: - RAN4 to study the factors potentially impacting RSRP prediction accuracy for inter-frequency prediction scenario considering at least the following aspects: - the side condition of frequency prediction (e.g., EPRE difference) - the impact of correlation coefficient between measured and predicted frequency layers - impact of cluster approach, e.g., - When measurement from single cell in one carrier frequency is used by the UE as an input to predict the RRM measurement for the intra-FR and co-located cell in another carrier frequency. - When measurement from a group of cells in one carrier frequency is used by the UE as an input to predict the RRM measurement for the intra-FR and co-located cell in another carrier frequency.

One of the high-priority study items for AI/ML in NR inter-cell mobility is inter-cell inter-frequency measurement prediction. Inter-frequency measurements require configuring measurement gaps, during which the UE must temporarily stop communicating with the serving cell to measure other frequencies. This process can lead to service interruptions and increased power consumption. AI/ML techniques can help reduce the number of inter-frequency measurements a UE needs to perform, thereby minimizing the need for continuous measurement and enhancing UE energy efficiency.

By utilizing measurement data from two frequency layers, an ML model can be trained to understand the correlation between radio measurements across these layers. Specifically, the model can either predict first-layer measurements based on second-layer measurements or determine if a UE connected to a macro layer can connect to a micro layer. This may involve a clustering approach, where measurements from multiple cells serve as inputs for prediction.

Impact of Correlation Coefficient

Correlation coefficient between different frequency layers needs some clarification in RAN4. It has been used in RAN2 simulations without much clarity on the deployment aspects. Currently, in RAN2 signalling aspects haven been discussed.

For inter-frequency prediction scenario, RAN2 has no sufficient clarity on the definition of correlation coefficient; therefore, RAN4 to wait for the progress in RAN2.

Impact of Cluster Approach

In our view, AIML model performance can be enhanced when measurements from a group of cells in one carrier frequency are used as an input for AIML model. The rationale behind this is diverse inputs can help to enhance AIML model performance.

Feeding the input from various sources might enhance AIML model performance.

For inter-frequency prediction scenario, in a cluster approach, RRM measurement prediction performance can be enhanced when measurement from a group of cells in one carrier frequency is used by the UE as an input. Therefore, RAN4 should consider the impact of cluster approach.

Conclusion

In this paper we provide our further view on the RRM Measurement prediction use case and RRM requirements to be studied, we particularly addressed the following topics:

- Performance metrics

- Ground truth definition for RSRP accuracy for FR1

- Impacts on beam/cell level measurements

- Impacts from observation and prediction windows

- Inter-frequency prediction scenario

The following Observations and Proposals were made:

Observation 1: TS 38.133 supports relative intra-frequency measurement accuracy evaluations in both inter-cell and intra-cell measurement contexts. For intra-cell cases, this includes comparisons between beams of the same cell, and for inter-cell cases, between cells on same carrier frequency.

Proposal 1: For intra-frequency L3-RSRP relative accuracy definition, RAN4 to cover comparison between two RSRP levels from the same cell or different cells.

Proposal 2: When defining the relative accuracy for intra-frequency predicted L3-RSRP, RAN4 to consider both of the following cases: Comparison between two predicted values, and comparison between one predicted value and one measured value.

Proposal 3: RAN4 consider the following definition of accuracy of relative in intra-frequency; Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP level 1 – reported RSRP level 2) – (ground truth of RSRP level 1 – ground truth of RSRP of level 2), where level 1 and level 2 may belong to the same or different cells on the same frequency. [Note: Reported RSRP can be either measured or predicted]

Observation 2: Current draft definition of inter-frequency predicted relative L3-RSRP accuracy allows comparison between a predicted value and a measured value only when the predicted value is inferred based on the measured value.

Proposal 4: RAN4 to extend the definition of inter-frequency predicted relative L3-RSRP accuracy to include comparisons between a predicted value and a measured value, regardless of whether one is used as the input for the other.

Proposal 5: For inter-frequency scenarios, RAN4 consider the following definition relative accuracy; Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported  RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2. [Note: Reported RSRP can be either measured or predicted]

Observation 3: It is expected Case-B temporal domain prediction with applied L3 filtering to include both combination of measured and predicted values and comply with accuracy performance requirements similar to those of the legacy relative and absolute accuracy performance defined for measurement.

Proposal 6: For temporal domain Case-B prediction, RAN4 may consider accuracy performance requirements similar to those of the legacy relative and absolute accuracy performance defined for measurement.

Observation 4: Alternative 1 where the FR1 ground truth can be derived by TE based on the transmitted or received power (legacy approach) is not suitable for AIML Mobility due to the fading channel and due to the L1 filtering which is UE implementation specific and not known to the TE.

Observation 5: Alternative 2, where the FR1 ground truth is derived based on the measurements reported by UE based on certain conditions, is more suitable for AIML Mobility since the channel variations and L1 filtering are already considered by UE while performing measurements that serve as ground truth.

Proposal 7: Alternative 2, where the FR1 ground truth is derived based on the measurements reported by UE based on certain conditions, is the preferred option for ground truth definition for RSRP accuracy even though there are inaccuracies in the UE measurements, since it is more feasible option for the dynamic channel conditions.

Proposal 8: RAN4 to focus first on L3 cell level RSRP (Point C) prediction and after that L3 beam-level prediction (Point E).

Proposal 9: RAN4 to not consider number of L1/L3 measurements performed during OW and its impacts on the prediction accuracy

Observation 6: The number of possible test cases can increase considerably for a set of selected OW, PW and MRRT with multiple UE speed cases.

Proposal 10: For temporal domain prediction case B, RAN4 to agree on pre-selected reference testing scenarios relating to OW, PW and MRRT as well as UE speed.

Proposal 11: If skipping pattern information is not shared by the UE, RAN4 to only consider MRRT, OW and PW to UE as test configuration and set up a requirement on prediction accuracy.

Proposal 12: If skipping pattern information is shared by the UE, TE should select the supported skipping pattern related to the MRRT to test. The test configuration should then include skipping pattern, OW and PW.

Proposal 13: For inter-frequency prediction scenario, RAN2 has no sufficient clarity on the definition of correlation coefficient; therefore, RAN4 to wait for the progress in RAN2.

Observation 7: Feeding the input from various sources might enhance AIML model performance.

Proposal 14: For inter-frequency prediction scenario, in a cluster approach, RRM measurement prediction performance can be enhanced when measurement from a group of cells in one carrier frequency is used by the UE as an input. Therefore, RAN4 should consider the impact of cluster approach.

- References

- RP-242393, “Revised SID on AIML for mobility in NR”, OPPO, RAN#105 meeting, 9 – 12 September 2024, Melbourne, Australia.

- 3GPP TR 38.744, Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR; (Rel. 19)

- R4-2504901, WF on AI/ML Mobility, Nokia, RAN4#114bis, Wuhan, China, 7th – 11 April, 2025.

- R4- 2507534, “On General aspects in AIML Mobility”, Nokia, RAN4#115 meeting, 19-23 May 2025, Malta, Malta.

- R4-2507433, “On RAN4 Impacts for Measurement Event Prediction in AIML Mobility”, Nokia, RAN4#115 meeting, 19-23 May 2025, Malta, Malta.

- R4-2507535, “Testability and Interoperability Issues for AIML Mobility”, Nokia, RAN4#115 meeting, 19-23 May 2025, Malta, Malta.