3GPP TSG-RAN WG4 Meeting #115	R4-2507433

Malta, Malta, May 19th – May 23rd, 2025

Source:	Nokia

Title:	On RAN4 Impacts for Measurement Event Prediction in AIML Mobility

Agenda item:	7.23.3.2

Document for:	Discussion

Introduction

The discussions on AI/ML Mobility continued in RAN4#114bis within the scope of Rel-19 SI ‎[1]‎ and ‎[2], with a dedicated agenda item assigned to measurement Event prediction in AI/ML Mobility.

In this paper, we provide our views on identified measurement event prediction related issues which need further discussions.

Discussion

Performance Metric

Following agreement was reached in RAN4#114bis meeting:

Agreement: - Potential RRM requirement impact for event prediction use case - for indirect case: - RAN4 to study the requirement for the predicted event triggered reporting - RAN4 to study the requirement for [delay and accuracy] - RAN4 to discuss the metric for the event prediction - Additionally, if the report includes the predicted RSRPs [corresponding to the predicted event occurrence time], - Candidate options: - Option 1: RAN4 will define requirement for the predicted RSRPs - Option 2: RAN4 will not define requirement for the predicted RSRPs

In indirect measurement event prediction for intra-frequency scenarios—such as temporal domain case A, temporal domain case B, or the frequency domain—, as described in Figure 1, a Radio Resource Management (RRM) measurement prediction model first predicts the measurement result(s), potentially utilizing AI/ML models. Subsequently, the predicted results, along with optionally included actual historical measurement data from the same cell, are used to determine whether a measurement event will occur at a specific future time instance. The post-processing phase operates without further involvement of an AI/ML model.

Figure 1: Indirect measurement event prediction.

RAN2 agreed on 4 models of indirect measurement event prediction: indirect measurement event prediction with time domain case A, indirect measurement event prediction with time domain case B, frequency-domain indirect measurement event prediction, and spatial-domain indirect measurement event prediction. However, based on RAN2 progress the main RAN2 focus is on:

- Indirect measurement event prediction with time domain case A in FR2 with Objective of Predicting A3 events to enhance HO execution via earlier HO Prep upon A3 event prediction, i.e., HO performance enhancement.

- Indirect measurement event prediction with time domain case B; Predict A3 events in the presence of skipped measurements, i.e., overhead reduction while maintaining similar HO performance.

Figure 3 provides an example of time domain case-A RRM measurement prediction.

Figure 3: Time domain case A RRM measurement prediction

In indirect measurement event prediction with time domain case A RRM measurement prediction, an A3 measurement event may be predicted before it actually occurs and the prediction potentially used in optimizing mobility decisions. Two options for handover execution decision have been documented in TR 38.744 by RAN2:

Option 1: Relying on legacy measurement event

Option 2: Relying on predicted measurement event

In both options, the decision for an early handover preparation may be done based on the predicted measurement event.

In time domain case B RRM measurement prediction, the number of required downlink radio measurements is reduced by the UE skipping specified measurements periods and predicting the skipped measurements from the past measurements using a ML model. Two examples for skipping patterns are shown in Figure 4.

Figure 4: Examples of time domain case B measurement prediction

It has been agreed in RAN2 to study the measurement event prediction also with time domain case B RRM measurement prediction in FR1. In this case, the PW is typically short, and predictions are not made for every measurement period, so any gains that can be expected from the future predictions is low. On the other hand, all mobility decisions are made based on measurement-event-triggered reporting based on actual measurements and predictions, which may set higher requirements for the accuracy of the predictions. Comparing the deployment and use cases, case A and case B impose different requirements and performance metrics, necessitating further RAN4 evaluation.

- The following scenarios have been considered in RAN2 for indirect measurement event prediction using temporal domain predictions: 1) Temporal domain Case A: The UE predicts a probable future measurement event, which may be used in mobility optimization decisions, but there is no measurement reduction. 2) Temporal domain Case B: The UE is predicting measurements for measurement reduction, which should not have significant impact on the mobility performance.

- For indirect measurement event prediction, the choice between temporal domain Case A and Case B may impose different performance requirements, necessitating further RAN4 evaluation.

RAN4 to study the requirements and performance metrics for indirect measurement event prediction temporal domain Case A and Case B.

Requirements and Performance metrics for indirect measurement event prediction with temporal domain

Based on TS 38.133, the UE shall send one Event A3-triggered measurement report within the defined measurement reporting delay. The UE shall not send event-triggered measurement reports if the reporting criteria are not met. The rate of correct events observed during repeated tests shall be at least 90%.

However, in RAN2, F1 score with the following details has been agreed as the evaluation metric for AI/ML measurement event prediction [TR 38.744].

The performance metric F1 score is defined as following: F1 score = 2*Precision*Recall/(Precision + Recall) Where: Precision = n3/(n1+n3) Recall =n3/(n2+n3) For indirect prediction, the counter n1, n2 and n3 in the formula are defined as following: Counter n3(true event prediction): it increases by 1 when a ground-truth event occurs around a predicted event with ETD, whose range is [0, maximum ETD] or vice versa Counter n1(false event detection): it increases by 1 when no ground-truth event occurs around a predicted event with ETD, whose range is [0, maximum ETD] Counter n2(missed event detection): it increases by 1 when no event is predicted around a ground-truth event with ETD, whose range is [0, maximum ETD] The ETD i.e. timing difference between ground-truth event and predicted event is illustrated in Figure 5.3.1-1: Figure 5.3.1-1: illustration of ETD As illustrated in Figure 5.3.1-1, only if the ETD between a predicted event and a ground-truth event e.g. ground-truth event 2 is less than or equal to maximum ETD, the ETD can still be tolerated. Otherwise, both false event and missed event are detected.

Though an accepted RAN2 metric, testing and evaluating the F1 score can be challenging due to several practical considerations.

First, the measurement event prediction inference may be either periodic or event-based:

- Periodic measurement event prediction: Whenever configured with an A3, the A3 prediction is inferred after each measurement sampling period.

- Event-based measurement event prediction: The A3 measurement event prediction is inferred when specified prediction condition is met. For example, when an A3 entry condition is fulfilled, then predict if the condition will hold for TTT.

The choice between periodic and event-based measurement event prediction significantly impacts the clarity of initial points and intervals for counter updates. Specifically, it is often unclear how the counters are initially triggered and how frequently they are updated. This ambiguity can complicate the accurate assessment of the F1 score For instance, in periodic measurement event prediction, if initiated well before the A3 entry conditions are met (i.e., when the serving cell quality is significantly higher than the target cell), the counter values and the resulting F1 score can differ significantly from those in event-based measurement event prediction, where inference begins upon A3 entry fulfillment.

Using performance metric F1 score, the choice between periodic and event-based measurement event prediction significantly impacts the clarity of initial points and intervals for counter updates in F1 score, and final F1 score results.

Additionally, if there is more than one target cell candidate, it should be clarified if the prediction accuracy counters are to be evaluated for each target candidate cell individually, i.e., each target candidate cell is its own data sample in the evaluation, or if predicting the right target candidate cell is included in the metrics calculation. In the latter case, RAN2 should discuss how the metrics are calculated.

If there is more than one target cell candidate, it should be clarified if the prediction accuracy counters are to be evaluated for each target candidate cell individually, or if predicting the right target candidate cell is included in the F1 score calculation.

In practice, verifying whether an AI/ML-based UE satisfies a required F1 score for A3 measurement event prediction is challenging due to the significant testing overhead. Evaluating F1 scores reliably requires collecting large volumes of labeled data across diverse scenarios to capture true positives, false positives, and false negatives. This demands repeated field or lab trials under controlled conditions, making the process time-consuming and resource intensive.

Testing the UE with F1 score as the metric for A3 measurement event prediction is challenging due to significant testing overhead.

RAN4 to study a simpler metric for A3 measurement event prediction.

For a measurement event prediction with time domain case A which is used in mobility optimization predicting A3 events ahead of the actual ground-truth trigger is often desirable. An early prediction allows the network to preemptively prepare target cells, reduce HO latency, and improve user experience during mobility. However, the standard F1 evaluation metric, even with a symmetric Event Time Deviation (ETD) window, does not differentiate between early and late predictions. It treats a prediction occurring before the ground-truth event the same as one occurring after—despite their vastly different operational impact. In reality, a late A3 prediction may lead to HO failure or interruption, whereas an early one enhances robustness. Therefore, for n3 counter (true event prediction) evaluation, minimum event prediction distance should be considered, i.e., how much before the ground truth measurement event the prediction should be made to increase the n3.

For a measurement event prediction with time domain case A which is used in mobility optimization predicting A3 events ahead of the actual ground-truth trigger is often desirable.

For the time domain case A measurement event prediction, RAN4 to consider prediction of A3 event early enough a head of ground truth (actual event) as performance metric for true event prediction.

For the time domain case B measurement event prediction evaluation, as the gain is in measurement reduction, it is expected the UE to predict the event within a short prediction window. So, it seems possible to define a same performance metric for both temporal domain case A and B, with difference performance values and consider case specific requirements.

For the time domain case B measurement event prediction, RAN4 may consider a similar performance metric definition as that of temporal domain case A.

Measurement Reporting

Regarding, measurement report (e.g, predicted RSRPs) as in all indirect cases the event prediction is based on the predicted measurement.

In indirect event prediction scenarios, providing only the binary event outcome (e.g., A3 triggered or not) may limit the network's ability to make flexible and context-aware decisions. By also reporting the predicted measurement values (such as RSRP or SINR for both serving and neighbor cells or predicted delta values) the network gains richer insight into the underlying radio conditions that led to the event inference. This enables several operational advantages for example estimation of the urgency of the mobility (e.g., HO) decisions/actions, prioritization among several received event notifications from different users, enabling network to adds up cell specific thresholds or dynamic offsets (e.g., for load balancing) transparent from the UE, and enhance the robustness  as predicted measurement trends can be cross-checked against actual values to diagnose false positives or missed events.

Reporting predicted measurements alongside the predicted event provides greater operational flexibility, transparency, and reusability for the network.

Based on TS 38.133, for event-triggered measurement reporting, the measurement reporting delay is defined as the time between the occurrence of an event that triggers a measurement report and the point when the UE begins transmitting the measurement report over the air interface. The reported RSRP measurements in event-triggered measurement reports must comply with the requirements specified in 38.133, specifically in Clauses 10.1.2.1 (RSRP for FR1) and 10.1.3.1 (RSRP for FR2).

Similar accuracy definition and requirements have been considered for Even-based measurement reporting and periodic measurement in TS 38.133.

For event-based temporal domain measurement reporting based-event prediction, a similar accuracy requirement as that of the measurement prediction can be considered and there seems no further definition of accuracy requirements is needed.

For event-based measurement reporting based on indirect event prediction, RAN4 to consider a similar accuracy definition as measurement prediction.

Conclusion

In this paper we provide our view on the Rel-19 AI/ML mobility measurement event prediction scenario to be studied in RAN4 working group.

The following Observations and Proposals were made:

Observation 1: The following scenarios have been considered in RAN2 for indirect measurement event prediction using temporal domain predictions: 1) Temporal domain Case A: The UE predicts a probable future measurement event, which may be used in mobility optimization decisions, but there is no measurement reduction. 2) Temporal domain Case B: The UE is predicting measurements for measurement reduction, which should not have significant impact on the mobility performance.

Observation 2: For indirect measurement event prediction, the choice between temporal domain Case A and Case B may impose different performance requirements, necessitating further RAN4 evaluation.

Proposal 1: RAN4 to study the requirements and performance metrics for indirect measurement event prediction temporal domain Case A and Case B.

Observation 3: Using performance metric F1 score, the choice between periodic and event-based measurement event prediction significantly impacts the clarity of initial points and intervals for counter updates in F1 score, and final F1 score results.

Observation 4: If there is more than one target cell candidate, it should be clarified if the prediction accuracy counters are to be evaluated for each target candidate cell individually, or if predicting the right target candidate cell is included in the F1 score calculation.

Observation 5: Testing the UE with F1 score as the metric for A3 measurement event prediction is challenging due to significant testing overhead.

Proposal 2: RAN4 to study a simpler metric for A3 measurement event prediction.

Observation 6: For a measurement event prediction with time domain case A which is used in mobility optimization predicting A3 events ahead of the actual ground-truth trigger is often desirable.

Proposal 3: For the time domain case A measurement event prediction, RAN4 to consider prediction of A3 event early enough a head of ground truth (actual event) as performance metric for true event prediction.

Proposal 4: For the time domain case B measurement event prediction, RAN4 may consider a similar performance metric definition as that of temporal domain case A.

Observation 7: Reporting predicted measurements alongside the predicted event provides greater operational flexibility, transparency, and reusability for the network.

Observation 8: Similar accuracy definition and requirements have been considered for Even-based measurement reporting and periodic measurement in TS 38.133.

Proposal 5: For event-based measurement reporting based on indirect event prediction, RAN4 to consider a similar accuracy definition as measurement prediction.

- References

- RP-242393, “Revised SID on AIML for mobility in NR”, OPPO, RAN#105 meeting, 9 – 12 September 2024, Melbourne, Australia.

- 3GPP TR 38.744, Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR; (Rel. 19)