# 3GPP TSG-RAN WG4 Meeting #115	R4-2507444

# Malta, MT, 19th – 23rd May, 2025

Source: 	Huawei, HiSilicon

Title: 		Introduction of OTA requirements for network-nodes specifications for 3MHz channel bandwidth

Agenda Item:	5.7.1

Document for:	Discussion

# Introduction

In this contribution, we provide motivation to introduce OTA requirements for network-nodes specifications for 3 MHz channel bandwidth.

WID objectives (RP-234027) are provided below for reference:

# 4 Objective ### 4.1 Objective of SI or Core part WI or Testing part WI The following objectives shall be included for dedicated FDD spectrum in FR1: - Identify and specify necessary changes to NR physical layer with minimum specification impact to operate in spectrum allocations from approximately 3 MHz up to below 5 MHz [RAN1]: - Restrict to subcarrier spacing of 15kHz and the use of normal cyclic prefix. - For SSB: - Reuse PSS/SSS specification without puncturing. - PBCH based on current design - Identify and specify necessary minimum changes to PDCCH, CSI-RS/TRS, PUCCH, and PRACH for functional support based on existing design, without optimization. - Specify necessary RAN4 requirements to support deploying NR in spectrum allocations from approximately 3 MHz up to below 5 MHz [RAN4], including in bands n100, n106, n26, n28 and n85: - Specify system parameters (including channel and sync rasters) for the associated dedicated spectrum. - Minimize impact on RF requirements: - Reuse 5 MHz channel bandwidth at least for FRMCS use case (assuming co-located NR and GSM-R with same operator). - Specify the required RF requirements for optional 3 MHz channel bandwidth in bands n100, n106, n26, n28 and n85. - Specify RRM requirements while minimizing specification impact to support operation in dedicated spectrum allocations from approximately 3 MHz up to below 5 MHz. - Specify requirements for UE speeds up to 500km/h for Band n100. - Standardize corresponding signalling and UE capabilities [RAN2, RAN3]. This WI considers single-carrier operation, excluding RedCap. ### 4.2 Objective of Performance part WI NOTE: Leave empty if the WI proposal does not contain a RAN performance part. Specify necessary UE/BS performance requirements for NR operation in dedicated FDD FR1 spectrum allocations from approximately 3MHz up to below 5MHz, corresponding to the core requirements: - Specify necessary RRM performance requirements (RAN4) - Specify necessary UE demodulation performance and CSI reporting requirements (RAN4) - Specify necessary BS demodulation performance requirements (RAN4) - Specify necessary BS conformance tests (RAN4)

# Discussion

This topic is motivated by similar discussion in Rel-19 WI on 7MHz channel bandwidth introduction, where both conducted and OTA requirements were agreed to be introduced.

System parameters and conducted requirements were introduced for BS RF in Rel-18. Based on similar exercise already done for Rel-19 7MHz WI it was noticed, that the workload required to introduce OTA requirements to applicable network-nodes specifications (having system parameters, and conducted requirements already done) is low.

In general, it would be advised that for any future work item, RAN4 handles both conducted and OTA requirements as package, as those are based on the same very same system parameters.

Furthermore, we would like to avoid situation, there a separate WI would be needed in case a specific request is received to introduce 3MHz for operating band with AAS BS architecture (i.e., requiring OTA requirements).

When it comes to potential argument on low frequency bands not requiring AAS BS architecture, it shall be noted that during Rel-19 we have already discussed possibility of restricting AAS BS specification to higher frequency bands. It was concluded among BS vendors that such restriction is not agreeable, as AAS products are expected to be developed during 5G-Advanced phase for below 1 GHz bands.

# Conclusions

Based on the discussion above, the following proposals were formulated:

Proposal 1: Introduce complementary OTA requirements for 3 MHz channel bandwidth in related network-nodes specifications (under Rel-18 WI code).

Proposal 2: Arrange work-split to cover at least the following BS RF specifications:

- TS 38.104

- TS 38.141-2

- TS 37.105

- TS 37.145-2

# References

[1] 	R4-2504588, OTA requirements consideration for network-nodes specifications for 7MHz channel introduction, RAN4#114bis