3GPP TSG-RAN WG4 Meeting #115							         	       	R4-2507473

# Malta, MT, 19th – 23rd May, 2025

Agenda Item:	7.1.2.1.1 General aspects (incl. duty cycle solutions, MSD rules) [NR_ENDC_RF_Ph4-Core]

Title: 	Simplification proposal for HPUE 2UL 2CC MSD

Source: 	Skyworks Solutions, Inc.

Document for:	Approval

# 1	Background

In RAN#114bis, together with other contributors, we discussed in [2,3] the different MSD cases for HPUE inter-band combinations for 2UL IMDs and made a proposal for simplification of the existing Rel-17/18/19 cases with DeltaMSD LUTs and mapping. A way forward [3] captured the progress in the meeting, but mostly for 1UL HPUE MSD. In this contribution, we re-iterate our analysis for all 2UL HPUE cases (and sub-cases). In addition, a simplified LUT and mapping Table is proposed for 2UL 2CC with 2/3DL HPUE MSD.

# Discussion

## 2.1 Detailed description of the 2UL 2CC cases and assumptions

To analyze the different cases, it is essential to get the details of how the interference level increases and how it compares with the PA linearity (e.g., is the linearity staying the same or increasing with the interference level and is it the same for each UL band or not?).

Table 1 provides the details of the HPUE cases when NR-U is ignored with each PA in each band and the associated 2UL power:

- The first two columns provide a case number and which release introduced such combinations.

- The next two columns provide the inter-band 2UL power class and the associated total number of Tx.

- Then, parameters are provided per each UL band that are essential for the analysis of the IMD increases, as follows:

  - The power class per band

  - The number of Tx per band

  - The 1UL max power per band

  - The 2UL power per band (sum is limited to inter-band power class)

  - Associated PA back-off per band in 2UL mode.

- Finally, the last two columns provide the potential maximum power with both bands at their maximum and whether this opens for the increased power option. Note that increased power above PC1.5 may need further discussion, but this is not in the scope of this contribution.

Table 1: Detailed description of the different inter-band HPUE 2UL combinations without NR-U

| case | Rel | ULCA PC | #Tx | FDD/TDD Types (R19 only) | /Bd PC |  | /Bd #Tx |  | /Bd 1UL Pwr |  | /Bd 2UL power |  | /Bd 2UL BO |  | Max Tot Pwr | Inc Pwr option |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Bd1 | Bd2 | Bd1 | Bd2 | Bd1 | Bd2 | Bd1 | Bd2 | Bd1 | Bd2 |  |  |
| 1 | R15 | PC3 | 2 | F+T, T+T, F+F | PC3 | PC3 | 1 | 1 | 23 | 23 | 20 | 20 | -3 | -3 | 26 | N(+3dB) |
| 2 | R17 | PC2 | 2 |  | PC3 | PC3 | 1 | 1 | 23 | 23 | 23 | 23 | 0 | 0 | 26 | N |
| 3 | R17 | PC2 | 2 | F+T, T+T, F+F | PC3 | PC2 | 1 | 1 | 23 | 26 | 23 | 23 | 0 | -3 | 27.8 | Y(+1.8dB) |
| 4 | R18 | PC2 | 3 |  | PC3 | PC2 | 1 | 2 | 23 | 2x23 | 23 | 2x20 | 0 | -3 | 27.8 | Y(+1.8dB) |
| 5 | R18 | PC2 | 2 | F+T, T+T, F+F | PC2 | PC2 | 1 | 1 | 26 | 26 | 23 | 23 | -3 | -3 | 29 | N(+3dB) |
| 6 | R19 | PC2 | 3 |  | PC2 | PC2 | 1 | 2 | 26 | 2x23 | 23 | 2x20 | -3 | -3 | 29 | N(+3dB) |
| 7 | R19 | PC1.5 | 2 |  | PC2 | PC2 | 1 | 1 | 26 | 26 | 26 | 26 | 0 | 0 | 29 | N |
| 8 | R19 | PC1.5 | 3 |  | PC2 | PC2 | 1 | 2 | 26 | 2x23 | 26 | 2x23 | 0 | 0 | 29 | N |
| 9 | R18 | PC1.5 | 3 | F+T, T+T | PC3 | PC1.5 | 1 | 2 | 23 | 2x26 | 23 | 2x24.7 | 0 | -1.3 | 30 | Y(+1dB) |
| 10 | R19 | PC1.5 | 3 |  | PC2 | PC1.5 | 1 | 2 | 26 | 2x26 | 26 | 2x23 | 0 | -3 | 30.8 | Y(+1.8dB) |

Observation: One first observation is that the cases in yellow (including the baseline PC3 case) would allow the implementation of the next power class of inter-band HPUE provided no other UE limitation applies:

- PC3 case 1 implementation can support PC2 case 2.

- PC2 case 5 can support PC1.5 case 7.

- PC2 case 6 can support PC1.5 case 8.

- There is one case where one of the PA back-off is different from 0 or 3dB.

- This may be a later option for simplification to merge case 5/6 with 7/8 respectively.

## 2.2 High-level assumptions to derive IMD interference level

To derive the IMD increase, after the details are known, one must make some assumptions in term of the dominating path for the IMD. Based on experience and existing analysis, we propose to use the assumptions in Table 3 for MSD to own band or to third band depending on the FDD/TDD types of the two bands and the number of Tx per band:

- T+T means 2UL is TDD+TDD => only the third band may be affected by MSD.

- F+T means 2UL is FDD+TDD => only the FDD band can be affected by MSD for two bands

- F+F means 2UL is FDD+FDD => both bands may be affected by MSD for two bands

- #DL denotes the number of DL bands in the inter-band combination.

Assumption on dominating IMD: The following table is used as assumptions for IMD increase calculations

Table 3: Assumptions on dominating IMD generator versus 2/3 band DL, TDD/FDD and 2/3Tx

| #DLB | 2UL | Assumptions on dominating IMD | 3Tx assumption |
| --- | --- | --- | --- |
| 2 | F+T | Only FDD band is victim => dominated by FDD PA | If 2Tx on TDD => Only one TDD PA dominates coupling to FDD PA |
|  |  |  | If 2Tx on FDD => Only one FDD PA dominates IMD |
|  | F+F | Dominated by FDD PA associated with victim DL band | if 2Tx on victim FDD => Only one PA dominates IMD |
|  |  |  | if 1Tx on victim FDD => Only one of 2Tx PA dominates coupling to FDD PA |
| 3 | F/T+F/T | Dominated by victim band LNA | Only one LNA IMD is dominated by both UL |

Beyond these assumptions on the dominating IMD generator, there are other aspects where assumptions and possibly short cuts are needed:

- For the PA and the LNA, even if the power levels are the same at the two PA outputs, all IMDs will be asymmetric in level because:

  - A PA sees only reverse IMD, thus its own signal and a small, coupled signal from the other PA

  - The LNA sees each PA with a different coupling loss.

- Asymmetric IMD is ignored as there is no way to set a generic imbalance and, even if that were the case, one would then have to look at which side IMD is the interfering one. Even with a thorough analysis, there is currentlty no chance that the assumptions and levels would be the same as the one used for the PC3 analysis. In most cases, this issue is ignored regardless.

- When there is asymmetry in the level in each PA, this is considered in the IMD increase calculation by retaining the worst-case side.

- For a thorough analysis beyond the IMD order, one would have to account for the IMD index, especially when there is asymmetry in the two UL band levels.

- We also propose to ignore the IMD index since the asymmetry is not known or generic. Thus, we assume that the IMD increase is only proportional to the IMD order. Only the asymmetry of the power per band is considered and the worst-case IMD retained.

Assumption on IMD asymmetry and indexes: Since there is no way to adopt a generic approach in terms of UL band level asymmetry, we propose to ignore asymmetric levels and IMD indexes in the IMD increase calculations. However, when the level in each UL band is asymmetric, the worst-case IMD side is considered.

Finally, there could be as many architectures and antenna sharing schemes as one would like. The assumption here is the most common and, in most cases, the worst-case: the two UL bands share the main path antenna and thus the main antenna PA (1Tx or 2Tx) associated with the victim band dominates the IMD generation. Also, the levels at the victim LNA input are larger in the main path.

Assumption on architecture: The main path of each UL band shares the same antenna and IMD interference dominates in the main DL path and is generated by the main PA associated with the victim band.

## 2.3 IMD increase calculations for 2B DL/2B UL

Using the assumptions described in the previous section, we have calculated the IMD increase for all nine HPUE cases and sub-cases by comparing step by step between the different cases. In this case, the IMD generator is the PA associated with the victim FDD band that shares its antenna.

These calculations are summarized in Table 4:

- The first seven columns provide the case description:

  - Column 1: Case number and related release

  - Column 2: Inter-band power class and number of Tx

  - Column 3-7: Band1 or band 2 parameters: Power class per band / Number of Tx per band/Output power per band in 2UL mode / Back-off per band in 2UL mode

- The eighth column shows the IMD increase versus the PC3 baseline in dB and in proportion of the IMD order “Nxorder” in bold. It then shows the increase versus another case which is often useful to check how IMD evolves iteratively.

  - In cases 3, 4, 6, and 8, some sub-cases are needed to account on which band (power class, number of Tx) dominated the IMD when the two bands are of different types.

- Finally, the last four columns provide the maximum applicable ∆MSD per IMD order.

Table 4: ∆MSD calculations summary for 2B DL/2B UL

| case /Rel | ULCA PC /#Tx | Bd | /Bd PC | /Bd #Tx | /Bd 2UL power | /Bd 2UL BO | 2DL2UL interference level inrease vs Ref / vs case(x) [dB] | ∆MSD max [dB] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | 2 | 3 | 4 | 5 |
| 1 | PC3 | Bd1 | PC3 | 1Tx | 20 | -3 | Ref | 0 | 0 | 0 | 0 |
| R15 | 2Tx | Bd2 | PC3 | 1Tx | 20 | -3 |  |  |  |  |  |
| 2 | PC2 | Bd1 | PC3 | 1Tx | 23 | 0 | +3xorder / =(1)+3xorder | 6 | 9 | 12 | 15 |
| R17 | 2Tx | Bd2 | PC3 | 1Tx | 23 | 0 |  |  |  |  |  |
| 3 | PC2 | Bd1 | PC3 | 1Tx | 23 | 0 | 3a: if PC3 is FDD victim band: +3xorder / =(2) | 6 | 9 | 12 | 15 |
| R17 | 2Tx | Bd2 | PC2 | 1Tx | 23 | -3 | 3b: if PC2 is FDD victim band: +3 / =(1)+3 | 3 | 3 | 3 | 3 |
| 4 | PC2 | Bd1 | PC3 | 1Tx | 23 | 0 | 4a: if PC3 is FDD victim band: +3x(order-1)=+3xorder-3 / =(2)-3 in Bd2 | 3 | 6 | 9 | 12 |
| R18 | 3Tx | Bd2 | PC2 | 2Tx | 2x20 | -3 | 4b: if PC2 is FDD victim band: +3x(order-1)=+3xorder-3 / =(1)+3 in Bd1 | 3 | 6 | 9 | 12 |
| 5 | PC2 | Bd1 | PC2 | 1TX | 23 | -3 | +3 / =(1)+3 => since this could be PC1.5: | 3 | 3 | 3 | 3 |
| R19 | 2Tx | Bd2 | PC2 | 1Tx | 23 | -3 |  |  |  |  |  |
| 6 | PC2 | Bd1 | PC2 | 1TX | 23 | -3 | 6a: if 1Tx is FDD victim band: +3-3x(order-2)=-3xorder+9 / =(5)-3 in Bd2 | 3 | 0 | -3 | -6 |
| R19 | 3Tx | Bd2 | PC2 | 2Tx | 2x20 | -3 | 6b: if 2Tx is FDD victim band: +3x(order-1)=+3xorder-3 / =(1)+3 Bd1 | 3 | 6 | 9 | 12 |
| 7 | PC1.5 | Bd1 | PC2 | 1TX | 26 | 0 | +3xorder+3 / =(2)+3dB | 9 | 12 | 15 | 18 |
| R19 | 2Tx | Bd2 | PC2 | 1Tx | 26 | 0 |  |  |  |  |  |
| 8 | PC1.5 | Bd1 | PC2 | 1TX | 26 | 0 | 8a: if 1Tx is FDD victim band: +3x(order-1)+3=3xorder / =(4)+3dB | 6 | 9 | 12 | 15 |
| R19 | 3Tx | Bd2 | PC2 | 2Tx | 2x23 | 0 | 8b: if 2Tx is FDD victim band: +3*(order+order-1)=6xorder-3 / =(2)+3*(order-1) | 9 | 15 | 21 | 27 |
| 9 | PC1.5 | Bd1 | PC3 | 1Tx | 23 | 0 | +3*order+1.7*(order-1)=+4.7xorder-1.7/ =(2)+1.7*(order-1) | 7.7 | 12.4 | 17.1 | 21.8 |
| R18 | 3Tx | Bd2 | PC1.5 | 2Tx | 2x24.7 | -1.3 |  | =>9 | =>12 | =>18 | =>21 |
| 10 | PC1.5 | Bd1 | PC2 | 1TX | 26 | 0 | +3*(order-1)+3=3xorder / =(4)+3dB | 6 | 9 | 12 | 15 |
| R19 | 3Tx | Bd2 | PC1.5 | 2Tx | 2x23 | -3 |  |  |  |  |  |

Observation:

- The worst-case IMD increase versus PC3 is 27dB for IMD5 of PC1.5=PC2+PC2 2Tx when 2Tx is the victim band. This is high, but one needs to consider that PC3 IMD5MSD levels are low, thus 27dB is highly unlikely.

- There are cases where the MSD would decrease (red highlight) for the PC2=PC2+PC2 2Tx case. This is because the levels are asymmetric and the PA generating the IMD has higher capability, thus higher-order IMDs would decrease. Since this is difficult to account for and the same implementation is capable of PC1.5, we propose to clamp the negative values at zero.

- Where yellow highlight appears, the UL level on one band is not increased by a 3dB step, resulting in IMD increases that are not a multiple of 3dB. Here, we suggest rounding to the closest 3dB increment.

- Overall, for any IMD increases, the maximum DeltaMSD is from 0 to 27dB in 3dB increments. The higher values are for higher IMD orders where the PC3 MSD is small, thus the maximum DeltaMSD is not reached.

- It should therefore be noted that higher order IMDs (orders 6,7..?) could be of concern for the inter-band highest power classes.

## 2.3 IMD increase calculations for 3B DL/2B UL

Using the assumptions for the third DL band MSD being dominated by the victim LNA IMD, we have calculated the IMD increase for all nine HPUE cases by comparing step by step between the different cases. These calculations are summarized in Table 5, which is ordered in the same fashion as Table 3.

Table 5: ∆MSD calculations summary for 3B DL/2B UL

| case /Rel | ULCA PC /#Tx | Bd | /Bd PC | /Bd #Tx | /Bd 2UL power | /Bd 2UL BO | 2DL2UL interference level inrease vs Ref / vs case(x) [dB] | ∆MSD max [dB] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | 2 | 3 | 4 | 5 |
| 1 | PC3 | Bd1 | PC3 | 1Tx | 20 | -3 | Ref | 0 | 0 | 0 | 0 |
| R15 | 2Tx | Bd2 | PC3 | 1Tx | 20 | -3 |  |  |  |  |  |
| 2 | PC2 | Bd1 | PC3 | 1Tx | 23 | 0 | +3xorder / =(1)+3xorder | 6 | 9 | 12 | 15 |
| R17 | 2Tx | Bd2 | PC3 | 1Tx | 23 | 0 |  |  |  |  |  |
| 3 | PC2 | Bd1 | PC3 | 1Tx | 23 | 0 | +3xorder / =(1)+3xorder | 6 | 9 | 12 | 15 |
| R17 | 2Tx | Bd2 | PC2 | 1Tx | 23 | -3 |  |  |  |  |  |
| 4 | PC2 | Bd1 | PC3 | 1Tx | 23 | 0 | +3x(order-1)=+3xorder-3 / =(1)+3 Bd1 | 3 | 6 | 9 | 12 |
| R18 | 3Tx | Bd2 | PC2 | 2Tx | 2x20 | -3 |  |  |  |  |  |
| 5 | PC2 | Bd1 | PC2 | 1TX | 23 | -3 | +3xorder / =(1)+3xorder | 6 | 9 | 12 | 15 |
| R19 | 2Tx | Bd2 | PC2 | 1Tx | 23 | -3 |  |  |  |  |  |
| 6 | PC2 | Bd1 | PC2 | 1TX | 23 | -3 | +3x(order-1)=+3xorder-3 / =(1)+3 Bd1 | 3 | 6 | 9 | 12 |
| R19 | 3Tx | Bd2 | PC2 | 2Tx | 2x20 | -3 |  |  |  |  |  |
| 7 | PC1.5 | Bd1 | PC2 | 1TX | 26 | 0 | +6xorder / =(2)+3xorder | 12 | 18 | 24 | 30 |
| R19 | 2Tx | Bd2 | PC2 | 1Tx | 26 | 0 |  |  |  |  |  |
| 8 | PC1.5 | Bd1 | PC2 | 1TX | 26 | 0 | +3x(order)+3*(order-1)=6xorder-3 / =(2)+3 Bd1 | 9 | 15 | 21 | 27 |
| R19 | 3Tx | Bd2 | PC2 | 2Tx | 2x23 | 0 |  |  |  |  |  |
| 9 | PC1.5 | Bd1 | PC3 | 1Tx | 23 | 0 | +3*order+1.7*(order-1)=+4.7xorder-1.7/ =(2)+1.7*(order-1) | 7.7 | 12.4 | 17.1 | 21.8 |
| R18 | 3Tx | Bd2 | PC1.5 | 2Tx | 2x24.7 | -1.3 |  | =>9 | =>12 | =>18 | =>21 |
| 10 | PC1.5 | Bd1 | PC2 | 1TX | 26 | 0 | +3x(order)+3*(order-1)=6xorder-3 / =(2)+3 Bd1 | 9 | 15 | 21 | 27 |
| R19 | 3Tx | Bd2 | PC1.5 | 2Tx | 2x23 | -3 |  |  |  |  |  |

Observation:

- The worst-case IMD increase versus PC3 is 30dB for IMD5 of PC1.5=PC2+PC2. This is because the LNA power capability does not increase.

- Where yellow highlights appear, the UL level on one band is not increased by a 3dB step resulting in IMD increases that are not a multiple of 3dB. Here, we suggest rounding to the closest 3dB increment.

- Overall, for any IMD increase, the maximum ∆MSD is from 0 to 30dB in 3dB increments. The higher values are for higher IMD orders where the PC3 MSD is small, thus the maximum ∆MSD is not reached.

- It should therefore be noted that higher order IMDs could be of concern for the inter-band highest power classes similarly to the two band DL case.

## 2.4 Summary of the number of LUTs needed for all ten cases and four IMD orders.

Since the IMD increase equation and maximum ∆MSD is applicable to multiple cases, we propose re-arranging the cases and assigning them to the related maximum ∆MSD values in 3dB increments. This is shown in Table 6.

Table 6: ∆MSD curve assignment per IMD order and 2DL/3DL 2UL cases.

| #DL band MSD | #Tx | Inte-band PC | per-band PC | Victim band PC | case | ∆MSDmax [dB] /IMD order |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 0 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 |
| 2DL | 2Tx | PC2 | PC3+PC3/PC3+PC2 | PC3 | 2/3a |  |  | 2 | 3 | 4 | 5 |  |  |  |  |  |
|  |  |  | PC3+PC2/PC2+PC2 | PC2 | 3b/5 |  | 2345 |  |  |  |  |  |  |  |  |  |
|  |  | PC1.5 | PC2+PC2 | PC2 | 7 |  |  |  | 2 | 3 | 4 | 5 |  |  |  |  |
|  | 3Tx | PC2 | PC3+PC2 2Tx/PC2+PC2 2Tx | PC3 1Tx/PC2 2Tx | 4/6b |  | 2 | 3 | 4 | 5 |  |  |  |  |  |  |
|  |  |  | PC2+PC2 2Tx | PC2 1Tx | 6a | 345 | 2 |  |  |  |  |  |  |  |  |  |
|  |  | PC1.5 | PC2+PC2 2Tx/PC2+PC1.5 2Tx | PC2 1Tx | 8a/10 |  |  | 2 | 3 | 4 | 5 |  |  |  |  |  |
|  |  |  |  | PC2 2Tx | 8b |  |  |  | 2 |  | 3 |  | 4 |  | 5 |  |
|  |  |  | PC3+PC1.5 2Tx | N/A | 9 |  |  |  | 2 | 3 |  | 4 | 5 |  |  |  |
| 3DL | 2Tx | PC2 |  |  | 2/3/5 |  |  | 2 | 3 | 4 | 5 |  |  |  |  |  |
|  |  | PC1.5 |  |  | 7 |  |  |  |  | 2 |  | 3 |  | 4 |  | 5 |
|  | 3Tx | PC2 |  |  | 4/6 |  | 2 | 3 | 4 | 5 |  |  |  |  |  |  |
|  |  | PC1.5 |  | PC2 band | 8/10 |  |  |  | 2 |  | 3 |  | 4 |  | 5 |  |
|  |  |  |  | PC3 band | 9 |  |  |  | 2 | 3 |  | 4 | 5 |  |  |  |

Table 6 description:

- Column 1: MSD to own DL band out of 2DL/2UL combination or third band in a 3DL / 2UL combination.

- Column 2: Inter-band power class, Column 3: Number of Tx, Column 4: Per band power class (when needed), Column 5: Power class of victim band.

- Column 6: Corresponding cases in previous Tables 4 and 5.

- Finally, to which maximum ∆MSD curve each IMD order applies per cases.

Observations:

- Multiple cases fall on the same ∆MSD curve, especially for 3DL and some are common between 2DL and 3DL.

- Although it looks like a high number of cases to be mapped, all the curves have a 3dB scaling to the others and are like the 1UL curves with 3, 6 and 9dB ∆MSDmax.

## 2.5 Simplified LUT mapping by merging cases

In RAN4#114b meeting, we have proposed in [2] a unified LUT approach for 1UL and 2UL, for this meeting we have split 1UL and 2UL to facilitate the discussion but also because the fine input granularity in PC3 MSD is only necessary for 1UL cases but essential for the large MSDmax 2UL cases.

Based on merging 2DL and 3DL cases and retaining only 2DL exception resulting in more than 6dB MSD improvement compared to the proposed PC2 and PC1.5 MSD, we propose the following LUT and MSD tables for 2UL HPUE MSD simplification.

Proposal on simplified 2UL HPUE MSD mapping:

- 2UL 2CC with 2DL and 3DL MSD type mapping to the “MSDmax” LUT is specified in the companion Table 2 below with the following exceptions:

- 2DL/2UL PC2 cases where the victim band achieves PC2 with 1Tx have IMD2 to IMD5 MSDs mapped on the 1UL 3dB LUT.

- 2DL/2UL PC1.5 cases where the victim band achieves PC2 with 1Tx have IMD2 to IMD5 MSDs mapped on the PC2 mapping in the Table 2 below.

Table 2: 2UL 2CC with 2/3DL MSD mapping versus IMD order and power class

| MSD type |  | MSDmax (dB) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 6 | 9 | 12 | 15 | 18 | 24 | 30 |
| 2UL 2CC | IMD2 | PC2 |  | PC1.5 |  |  |  |  |
|  | IMD3 |  | PC2 |  |  | PC1.5 |  |  |
|  | IMD4 |  |  | PC2 |  |  | PC1.5 |  |
|  | IMD5 |  |  |  | PC2 |  |  | PC1.5 |

- The 2UL 2CC MSD are calculated on the following Table 3 LUT based on their associated MSDmax in Table 2.

Table 3: MSD look-up tables (LUT) for 2UL versus PC3 MSD and MSDmax mapping

| PC3 MSD (dB) | MSDmax (dB) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 3 | 6 | 9 | 12 | 15 | 18 | 24 | 30 |
| 0.1≤ - ≤0.3 | 0.7 | 1.1 | 2.5 | 2.8 | 4.3 | 6.3 | 11.0 | 16.5 |
| 0.4≤ - ≤0.6 |  |  |  | 4.2 | 6.1 | 8.4 | 13.6 | 19.3 |
| 0.7≤ - ≤0.9 |  |  |  | 5.1 | 7.2 | 9.7 | 15.1 | 21.0 |
| 1.0≤ - ≤1.9 | 1.2 | 2.1 | 4.1 | 7.1 | 9.6 | 12.3 | 18.1 | 24.0 |
| 2.0≤ - ≤2.9 | 1.6 | 2.7 | 5.1 | 8.2 | 10.9 | 13.7 | 19.6 | 25.5 |
| 3.0≤ - ≤3.9 | 1.8 | 3.3 | 5.9 | 8.9 | 11.7 | 14.6 | 20.5 | 26.5 |
| 4.0≤ - ≤4.9 | 2.0 | 3.8 | 6.5 | 9.5 | 12.4 | 15.3 | 21.3 | 27.3 |
| 5.0≤ - ≤6.9 | 2.3 | 4.5 | 7.3 | 10.4 | 13.4 | 16.3 | 22.3 | 28.3 |
| 7.0≤ - ≤9.0 | 2.5 | 5.0 | 7.9 | 11.0 | 14.0 | 16.9 | 22.9 | 28.9 |
| ≥10 | 3 | 6 | 9 | 12 | 15 | 18 | 24 | 30 |

Observations:

- Although the PC1.5 MSDmax values for IMD4 and IMD5 seem very large, since PC3 MSD values are rather small (<6dB) the large values are attained. This aspect also justifies increased granularity for the small PC3 MSD.

- We recognize that some of the analysis provided in this contribution would require more space for detailed explanations. We hope to provide more explanations online, offline, or during ad-hoc in the May meeting.

# Conclusion

In this contribution, we continued to analyze the delta MSD for the 2UL IMDs for HPUE inter-band combinations. We based our work on the following assumptions.

Assumption on dominating IMD: The following table is used as assumptions for IMD increase calculations

Table 3: Assumptions on dominating IMD generator versus 2/3 band DL, TDD/FDD and 2/3Tx

| #DLB | 2UL | Assumptions on dominating IMD | 3Tx assumption |
| --- | --- | --- | --- |
| 2 | F+T | Only FDD band is victim => dominated by FDD PA | If 2Tx on TDD => Only one TDD PA dominates coupling to FDD PA |
|  |  |  | If 2Tx on FDD => Only one FDD PA dominates IMD |
|  | F+F | Dominated by FDD PA associated with victim DL band | if 2Tx on victim FDD => Only one PA dominates IMD |
|  |  |  | if 1Tx on victim FDD => Only one of 2Tx PA dominates coupling to FDD PA |
| 3 | F/T+F/T | Dominated by victim band LNA | Only one LNA IMD is dominated by both UL |

Assumption on IMD asymmetry and indexes: Since there is no way to adopt a generic approach in terms of UL band level asymmetry, we propose to ignore asymmetric levels and IMD indexes in the IMD increase calculations. However, when the level in each UL band is asymmetric, the worst-case IMD side is considered.

Assumption on architecture: The main path of each UL band shares the same antenna and IMD interference dominates in the main DL path and is generated by the main PA associated with the victim band.

Based on those assumptions, the different ∆MSD curves were calculated, mapped, and simplified in the following proposals per maximum ∆MSD.

Proposal on simplified 2UL HPUE MSD mapping:

- 2UL 2CC with 2DL and 3DL MSD type mapping to the “MSDmax” LUT is specified in the companion Table 2 below with the following exceptions:

- 2DL/2UL PC2 cases where the victim band achieves PC2 with 1Tx have IMD2 to IMD5 MSDs mapped on the 1UL 3dB LUT.

- 2DL/2UL PC1.5 cases where the victim band achieves PC2 with 1Tx have IMD2 to IMD5 MSDs mapped on the PC2 mapping in the Table 2 below.

Table 2: 2UL 2CC with 2/3DL MSD mapping versus IMD order and power class

| MSD type |  | MSDmax (dB) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 6 | 9 | 12 | 15 | 18 | 24 | 30 |
| 2UL 2CC | IMD2 | PC2 |  | PC1.5 |  |  |  |  |
|  | IMD3 |  | PC2 |  |  | PC1.5 |  |  |
|  | IMD4 |  |  | PC2 |  |  | PC1.5 |  |
|  | IMD5 |  |  |  | PC2 |  |  | PC1.5 |

- The 2UL 2CC MSD are calculated on the following Table 3 LUT based on their associated MSDmax in Table 2.

Table 3: MSD look-up tables (LUT) for 2UL versus PC3 MSD and MSDmax mapping

| PC3 MSD (dB) | MSDmax (dB) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 3 | 6 | 9 | 12 | 15 | 18 | 24 | 30 |
| 0.1≤ - ≤0.3 | 0.7 | 1.1 | 2.5 | 2.8 | 4.3 | 6.3 | 11.0 | 16.5 |
| 0.4≤ - ≤0.6 |  |  |  | 4.2 | 6.1 | 8.4 | 13.6 | 19.3 |
| 0.7≤ - ≤0.9 |  |  |  | 5.1 | 7.2 | 9.7 | 15.1 | 21.0 |
| 1.0≤ - ≤1.9 | 1.2 | 2.1 | 4.1 | 7.1 | 9.6 | 12.3 | 18.1 | 24.0 |
| 2.0≤ - ≤2.9 | 1.6 | 2.7 | 5.1 | 8.2 | 10.9 | 13.7 | 19.6 | 25.5 |
| 3.0≤ - ≤3.9 | 1.8 | 3.3 | 5.9 | 8.9 | 11.7 | 14.6 | 20.5 | 26.5 |
| 4.0≤ - ≤4.9 | 2.0 | 3.8 | 6.5 | 9.5 | 12.4 | 15.3 | 21.3 | 27.3 |
| 5.0≤ - ≤6.9 | 2.3 | 4.5 | 7.3 | 10.4 | 13.4 | 16.3 | 22.3 | 28.3 |
| 7.0≤ - ≤9.0 | 2.5 | 5.0 | 7.9 | 11.0 | 14.0 | 16.9 | 22.9 | 28.9 |
| ≥10 | 3 | 6 | 9 | 12 | 15 | 18 | 24 | 30 |

Observations:

- Although the PC1.5 MSDmax values for IMD4 and IMD5 seem very large, since PC3 MSD values are rather small (<6dB) the large values are attained. This aspect also justifies increased granularity for the small PC3 MSD.

- We recognize that some of the analysis provided in this contribution would require more space for detailed explanations. We hope to provide more explanations online, offline, or during ad-hoc in the May meeting.

# References

[1] R4-2503169 Simplification of HPUE 2UL DeltaMSD, Skyworks Solutions Inc., RAN4#114b

[2] R4-2503239 Unified HPUE 1UL2UL MSD simplification proposals for Rel-19, Skyworks Solutions Inc., RAN4#114b

[3] R4-2505143 WF on MSD simplification for HPUE, Skyworks, Qualcomm, Murata, Nokia, Samsung, Huawei, MediaTek, Vivo, ZTE, RAN4#114b