3GPP TSG-RAN WG4 Meeting #115                                      R4-2507476

St Julian’s, Malta, May 19th – May 23rd, 2025

Source: 	Huawei, HiSilicon

Title: 	TP for TR 38.755 on triggering condition

Agenda Item:	7.5.2

Document for:	Discussion

Introduction

At RAN4#114bis meeting, the agreement about triggering condition was captured in the WF.

- Issue 2-3: Triggering condition to enable ”One Rx RF chain”

RAN4 agree there are two aspects regarding the triggering condition enable/disable the one Rx RF chain mode:

- Option 1: Rely on existing channel quality reporting and procedures. No new indication from UE is needed

- Option 2: The UE need the measurement to determine and indicate to the network whether it can support non-contiguous CCs with one RX RF chain or not, assuming the associated degradation is no greater than allowed degradation that is specified

- Note: detailed conditions to enable/disable can be further discussed and decided separately

This contribution proposes the text proposal in the TR 38.755 to capture the discussion on triggering condition.

Text Proposal

<<Start of Change 1 for TR 38.755>>

- 8.3	Triggering condition to enable “One Rx RF chain”

Based on the study of RF performance of “One Rx RF chain”, performance degradations are observed if there is a strong self-interference or a strong in-gap interferer. It’s discussed whether some particular triggering condition to enable “One Rx RF chain” is needed to minimize the impact from self-interference or in-gap interferer.

- 8.3.1 Triggering condition proposal for option 1: Rely on existing channel quality reporting and procedures

One possible option of the triggering condition is to rely on existing channel quality reporting and procedures. Below Figure 8.3.1-1 shows the existing channel quality reporting procedures related to CA. Before the SCell is activated, L3-measurement is available, including measurement quantities of RSRP/RSRQ/SINR. Measurement gap or interruption is specified depending on the measurement type, i.e. inter-frequency/intra-band CA/inter-band CA. L3-measurement is filtered and could reflect relatively long-term channel quality. After the SCell is activated, CQI measurement and report is available. The L3-measurement is also available but no interruption is allowed.

Figure 8.3.1-1	Existing channel quality reporting and procedures

Some companies suggested to fully reuse the existing channel quality reporting procedure without any change. With this approach, the CA with “One Rx RF chain” is handled in the same way of all the other CA configurations. One of the reasons is that the observed RF performance degradation of “One Rx RF chain” is comparable with many CA configurations in existing specification. It’s not necessary to distinguish the REFSENS MSD caused by “One Rx RF chain” and caused by other factors. Another benefit of this approach is the easier commercial deployment of this feature since there is no need to upgrade the network.

Some companies suggested to refine the procedures and monitor the impact of “One Rx RF chain” to get a better decision on CA configuration and UE RF architecture. In current channel quality reporting procedure, the L3-measurement or CQI measurement on SCell is based on separate RF chain architecture. The measurement result can’t reflect the degradation of RF performance due to strong self-interference or strong in-gap interference. To improve the CA configuration decision, it’s considered to allow the UE to additionally perform the measurement based on “One Rx RF chain”. The measurement report could include the measurement results of both RF architectures, or the difference between two RF architectures. To get the measurement result based on “One Rx RF chain”, the interruptions due to hardware switching need to be considered.

- 8.3.2 Triggering condition proposal for option 2: UE measurement to determine and indicate to the network

For the purpose of monitoring the impact of “One Rx RF chain”, some companies suggested to introduce some new type of UE measurements.

One option is to measure and report the absolute power of the in-gap interference or the relative power between in-gap interference and wanted carrier. However, with the same in-gap interference, the performance loss might vary largely due to UE’s bandwidth, MCS and the UE’s receiving performance. The network is not likely to make better decision barely according to the information of in-gap interference.

Considering the UE has the complete information of the channel status and its own receiving performance, another option is that the UE makes the decision of whether the “One Rx RF chain” could work well and indicate to the network. The UE could evaluate how good it could behave with the “One Rx RF chain” based on the measurements it needs, such as the self-interference, the strength and location of the in-gap interference, etc. Then the UE indicate the evaluation result to the network, either a simple indication of “yes” or “no”, or in some finer granularity. The exact meaning of the indication could be further clarified so that the UE and the gNB have a common understanding. Some of the UE’s measurements needs to be done with the “One Rx RF chain” mode. In this case the interruptions due to hardware switching need to be considered.

- References

- RP-241669, “New SID: Study on NR FR1 DL Fragmented Carriers”, MediaTek Inc, RAN#104 Meeting, June 2024

- R4-2505109, “WF on NR FR1 DL fragmented carriers study”, Moderator (Mediatek Inc.), Apple, Qualcomm, RAN4#114bis Meeting, April 2025