3GPP TSG-RAN WG4 Meeting #115                                                               R4-2507481

Malta, MT, 19th – 23rd May, 2025

Source:	Samsung

Title:	Remaining issues for SBFD general aspects

Agenda item:			7.25.2

Document for:	Approval

# 1. Introduction

Based on Rel-18 study item on evolution of NR duplex operation, the support of subband non-overlapping full duplex has been studied, which is targeted to provide enhanced UL coverage, reduced latency, improved system capacity, and improved configuration flexibility for NR TDD operation [2].

In RAN#102, the work item on evolution of NR duplex operation (SBFD) has been approved, with WID further revised in the follow-up RAN plenary [1]. According to the objectives in WID, RAN1 is tasked to specify the mechanisms to support SBFD, including semi-static indication of time/frequency location, random access in SBFD symbols, and other transmission, reception and measurement behavior and procedures for SBFD aware UE. Furthermore, the enhancement for CLI handing, including gNB-to-gNB and UE-to-UE CLI handling, will also be specified in RAN1. Accordingly, from RAN4 perspective, it is tasked to specify BS RF, necessary RRM and demodulation requirements.

In this contribution, we would like to provide our further analysis and viewpoints on the general aspects including RAN4 aspects for SBFD system parameters, PUSCH with resource muting for CLI handling, requirement applicability and BS RF specification structure for SBFD-capable BS.

# 2. RAN4 aspects for SBFD system parameters

### 2.1 Declaration and flexibility for SBFD operation

Based on last meeting’s agreement, it is confirmed that the restriction (i.e., the SBFD capable BS product shall not operate any declared SBFD configuration (DL/UL subband) other than those that have been tested) shall be applied to FR1 WA BS and FR1 MR BS with TRP or total conducted power larger than a pre-defined threshold, while not applied to FR1 LA SBFD capable BSs.

<Agreement in RAN4#114> - The SBFD capable BS product shall not operate any declared SBFD configuration (DL/UL subband) other than those that have been tested. - This restriction shall not be applied to FR1 LA BS - FFS FR1 MR BS <Agreement in RAN4#114bis> - For FR1 LA SBFD capable BSs, which are not expected to require analog filter(s), for each declared channel bandwidth and each declared SBFD pattern (either DUD or DU/UD), the configurations with the narrowest and widest UL/DL subband sizes out of all declared configurations shall be tested. - For FR1 MR SBFD capable BSs, a power level threshold could be established: Above this threshold, all configurations shall be tested; Below this threshold, analog filters are not expected to be used, and the configurations with the narrowest and widest UL/DL subband sizes out of all the declared configurations for each channel bandwidth and each SBFD pattern (either DUD or DU/UD) shall be tested. - Use X dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs related to testing. - FFS the value of X, e.g., X = 33dBm as proposed by one company as candidate value, and study the value for different number of TX chains.

The remaining issues are (1) the restriction is applied to FR2 BS or not, and (2) the pre-defined threshold for TRP or conductive power for FR1 MR BS.

For FR2 BS, based on the analysis in the study item phase, for considering the input power at LNA for FR2 BS could be much less than FR1 WA BS, because of the better spatial isolation and lower BS output power, there is no necessity to have subband filtering to improve the linearity of LNA. Therefore, the restriction of “The SBFD capable BS product shall not operate any declared SBFD configuration (DL/UL subband) other than those that have been tested.” shall not be applicable to FR2 SBFD-capable BS. Therefore, the following proposal is provided:

Proposal 1: For FR2 BS, a which are not expected to require analog filter(s), for each declared channel bandwidth and each declared SBFD pattern (either DUD or DU/UD), the configurations with the narrowest and widest UL/DL subband sizes out of all declared configurations shall be tested.

For FR1 MR SBFD capable BSs, it is expected that a power level e.g., [33dBm] either as TRP or total conducted power threshold could be established. Above this threshold, all configurations shall be tested; Below this threshold, analog filters are not expected to be used, and the configurations with the narrowest and widest UL/DL subband sizes out of all the declared configurations for each channel bandwidth and each SBFD pattern (either DUD or DU/UD) shall be tested. Based on our analysis captured in TR 38.858, two analog filtering designs are provided to mitigate the self-interference and co-site inter-sector interference, for which 15dBc and 11.8dBc are considered. For FR1 MR BS, based on our analysis, we confirm the feasibility to have a 38dBm TX power while no analog filter is needed.

Proposal 2: For FR1 MR BS, use X = 38 dBm (either as TRP (for BS Type 1-O) or total conducted power (for BS Type 1-H)) as the power level threshold, to differentiate all declared configurations shall be tested or not.

- Note: As agreed, the power level threshold is established for FR1 MR BS: Above this threshold, all configurations shall be tested; Below this threshold, analog filters are not expected to be used, and the configurations with the narrowest and widest UL/DL subband sizes out of all the declared configurations for each channel bandwidth and each SBFD pattern (either DUD or DU/UD) shall be tested

### 2.2 DL/UL subband configurations support for SBFD

As requested in WID, to support SBFD operation at gNB side within a TDD carrier, the mechanism for semi-static indication of frequency location of SBFD subbands to UE shall be specified. Based on the agreement given in RAN4#113, the option 2b is adopted to specify DL/UL subband configurations support for SBFD, with the following restrictions on SBFD configuration provided [11]:

Agreement: - For Rel-19, option 2b is adopted, with the following restrictions on SBFD configuration: - For DUD case, the UL subband should be placed in the middle of channel bandwidth, i.e., symmetric DUD - DL subband 1 and DL subband 2 shall have the same size or different sizes by only 1RB - For a specific channel BW supported for SBFD, maximum [2 or 3] specific UL subband sizes are allowed to be declared and shall be selected from the channel bandwidth sizes from Table 5.3.2-1 in TS38.104. - FFS different sizes for specific UL subband for DUD and DU/UD. - No further restriction is needed on the range of the ratio of the UL subband size relative to the channel bandwidth. - This restriction shall be applied to FR1 WA BS only, as the starting point - FFS the restriction is also applied to FR1 MR BS - For a specific channel BW and a specific UL subband size supported, only one DL subband size is allowed to be declared and tested for DU/UD case, and only one DL subband size is allowed to be declared and tested for DUD case - This restriction shall be applied to FR1 WA BS only, as the starting point - FFS the restriction is also applied to FR1 MR BS

And the option 2b is given in RAN4#112:

Agreement: - To specify DL/UL subband configuration in BS RF requirements, the following 3 options for DL/UL subband configuration are identified: - Option-2b: - UL subband configuration shall be selected from the existing values of BS transmission bandwidth configuration, as provided in Table 5.3.2-1 for FR1 and Table 5.3.2-2 for FR2-1. - DL subband size(s) and guardband size(s) depends on BS implementation - Example (FR1 DU or UD in 100MHz and 30kHz SCS, 80-20MHz case): As long as BS transmission configuration for SBFD shall be 273 (NRB,SBFD for 100MHz and 30kHz SCS) and the value of NRB,SBFD,UL is selected from Table 5.3.2-1 (according to the 30kHz SCS), from BS RF requirement perspective, no limitation on the values NRB,SBFD,DL and NRB,SBFD,GB.

Here we would like to further discuss the remaining issues on the restrictions on SBFD configuration.

Restriction-1: For DUD case, the UL subband should be placed in the middle of channel bandwidth

During last meeting, for DUD case, it is agreed that the UL subband should be placed in the middle of channel bandwidth, i.e., symmetric DUD, in which DL subband 1 and DL subband 2 shall have the same size or different sizes by only 1RB. There is no remaining issue for this restriction except how to implement this agreement into the specification.

Restriction-2: Restriction on the number of UL subband sizes supported

Based on the agreement in RAN4#114bis, the following agreement is reached:

Agreement: - For a specific channel BW supported for SBFD, the maximum number of specific UL subband sizes are allowed to be declared - Maximum 3 subband configurations allowed for FR1 WA BS - These 3 suband configurations shall cover all DUD and DU/UD ones supported by this BS. - One UL subband size for DU/UD cases, should be counted as 2 configurations, e.g., UD 20-80MHz and DU 80-20MHz should be counted as 2 configurations. - For FR1 MR SBFD capable BSs, a power level threshold could be established: Above this threshold, maximum 3 subband configurations allowed as FR1 WA BS; Below this threshold, the restriction on the number of UL subband sizes supported is not applicable. - Use X dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs. - FFS the value of X, e.g., X = 33dBm as proposed by one company as candidate value, and study the value for different number of TX chains.

Based on the above proposal for “Declaration and flexibility for SBFD operation”, it is believed that the similar threshold should be adopted.

Proposal 3: For FR1 MR SBFD capable BSs, adopt the same power level threshold as the one to differentiate whether all declared configurations shall be tested or not.

- Note: As agreed, the power level threshold is established for FR1 MR BS: Above this threshold, maximum 3 subband configurations allowed as FR1 WA BS; Below this threshold, the restriction on the number of UL subband sizes supported is not applicable:

Restriction-3: Restriction on the number of DL subband sizes supported

It is agreed that for a specific channel BW and a specific UL subband size supported, only one DL subband size is allowed to be declared and tested for DU/UD case, and only one DL subband size is allowed to be declared and tested for DUD case, for FR1 WA BS. For FR1 MR BS, the following agreement is given in RAN4#114bis:

Agreement: - For a specific channel BW and a specific UL subband size supported, only one DL subband size is allowed to be declared and tested for DU/UD case, and only one DL subband size is allowed to be declared and tested for DUD case - This restriction shall be applied to FR1 WA BS - For FR1 MR BS SBFD-capable BSs, a power level threshold could be established: Above this threshold, this restriction is applicable as FR1 WA BS; Below this threshold, the restriction on the number of DL subband sizes supported is not applicable. - Use X dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs. - FFS the value of X, e.g., X = 33dBm as proposed by one company as candidate value, and study the value for different number of TX chains.

Similar as the above proposal for “Declaration and flexibility for SBFD operation”, it is believed that the similar threshold should be adopted.

Proposal 4: For FR1 MR SBFD capable BSs, adopt the same power level threshold as the one to differentiate whether the restriction on the number of DL subband sizes supported is applied or not.

- Note: As agreed, the power level threshold is established for FR1 MR BS: Above this threshold, this restriction is applicable as FR1 WA BS; Below this threshold, the restriction on the number of DL subband sizes supported is not applicable.

### 2.3 Restriction on DL subband size

In previous meeting, it is FFS the impact from RAN4 perspective for the case that “if the SSB symbol is overlapped with SBFD symbol”. It should be noted that the following RAN1 working assumption is provided:

Working Assumption For SSB symbols configured with SBFD subbands, - Option 1: The SSB symbols configured with SBFD subbands are SBFD symbols. Only DL receptions within DL usable PRBs are allowed for SBFD aware UEs. - Note: The SSB block is assumed to be within DL subband

With the above working assumption, it is expect that the SSB can still be received for both SBFD-aware and legacy UEs, and accordingly there is no need to introduce additional restriction from RAN4 perspective.

Proposal 5: If the SSB symbol is overlapped with SBFD symbol, there is neither restriction required nor RAN4 requirement impact identified.

### 2.4 Update for HPUE duty cycle evaluation

RAN4 has discussed the necessity of the specification update for HPUE duty cycle evaluation, and the following WF is provided in RAN4#114bis:

Way forward: - RAN4 has discussed the two following points: - For non-SBFD-aware UE, the “uplink symbols transmitted” used for uplink duty cycle evaluation are symbols with uplink transmission; - For SBFD-aware UE, the symbols used for uplink duty cycle evaluation are symbols with uplink transmission, including the non-SBFD symbols and SBFD symbols with UL transmission. - FFS RAN4 specification impact by taking into account the above understanding for SBFD-aware UE.

Based on our understanding, the term “uplink symbols” used in TS38.101 is not a precise description, but it is intended to cover all symbols possible for UE transmission, i.e., including 'uplink' or 'flexible' symbols as defined in TS 38.211 even for Rel-15. After introducing SBFD feature, it is possible that SBFD-aware UE is able to perform UL transmission in 'uplink' or 'flexible' or “SBFD” symbols. It is also noted that the current SBFD WID does not contain TS38.101 series as the impacted specification, and we think the following options are possible solution to this identified issue.

Proposal 6: RAN4 consider the following two options to guarantee the common understanding for SBFD-aware UE for HPUE duty cycle evaluation:

- Option 1: RAN4 shall introduce CR from Rel-15 to explain that “uplink symbols” should be interpreted as any symbols possible for UE transmission.

- Option 2: After listing TS 38.101 series as impacted specification in SBFD WID, RAN4 shall introduce CR to explicitly explain that “uplink symbols” should be interpreted as any symbols possible for UE transmission “including SBFD symbols”.

# 3. Conformance testing key issues for SBFD-capable BS

As we elaborated in the work plan for performance part, the discussion on BS RF conformance testing requirement shall begin from this meeting. Here we would like firstly highlight the potential problem to be discussed for conformance testing requirement.

Proposal 7: RAN4 shall begin the BS RF conformance testing discussion by considering the following key general issues:

- Issue-1: Additional manufacturer declaration(s) is needed for SBFD-capable BS.

  - It should be noted that at least the supported SBFD configuration for FR1 WA BS and some FR1 MR BS should be included.

  - FFS other declaration is required

- Issue-2: Additional test configuration(s) particular for SBFD requirement.

- Issue-3: RF channels to be tested for SBFD operation

  - FFS RF channels to be tested for SBFD operation, e.g,, to guarantee SBFD carrier is configured at the left/right-most of the operating band.

- Issue-4: Additional test models are needed for SBFD-capable BS.

  - FFS new test models are required to contain SBFD configurations for UL/DL subband and guardband.

# 4. Conclusion

In this contribution, we would like to provide our analysis and viewpoints on the general aspects including RAN4 aspects for SBFD system parameters, accordingly with the following observations and proposals obtained:

RAN4 aspects for SBFD system parameters

Proposal 1: For FR2 BS, a which are not expected to require analog filter(s), for each declared channel bandwidth and each declared SBFD pattern (either DUD or DU/UD), the configurations with the narrowest and widest UL/DL subband sizes out of all declared configurations shall be tested.

Proposal 2: For FR1 MR BS, use X = 38 dBm (either as TRP (for BS Type 1-O) or total conducted power (for BS Type 1-H)) as the power level threshold, to differentiate all declared configurations shall be tested or not.

- Note: As agreed, the power level threshold is established for FR1 MR BS: Above this threshold, all configurations shall be tested; Below this threshold, analog filters are not expected to be used, and the configurations with the narrowest and widest UL/DL subband sizes out of all the declared configurations for each channel bandwidth and each SBFD pattern (either DUD or DU/UD) shall be tested

Proposal 3: For FR1 MR SBFD capable BSs, adopt the same power level threshold as the one to differentiate whether all declared configurations shall be tested or not.

- Note: As agreed, the power level threshold is established for FR1 MR BS: Above this threshold, maximum 3 subband configurations allowed as FR1 WA BS; Below this threshold, the restriction on the number of UL subband sizes supported is not applicable:

Proposal 4: For FR1 MR SBFD capable BSs, adopt the same power level threshold as the one to differentiate whether the restriction on the number of DL subband sizes supported is applied or not.

- Note: As agreed, the power level threshold is established for FR1 MR BS: Above this threshold, this restriction is applicable as FR1 WA BS; Below this threshold, the restriction on the number of DL subband sizes supported is not applicable.

Proposal 5: If the SSB symbol is overlapped with SBFD symbol, there is neither restriction required nor RAN4 requirement impact identified.

Proposal 6: RAN4 consider the following two options to guarantee the common understanding for SBFD-aware UE for HPUE duty cycle evaluation:

- Option 1: RAN4 shall introduce CR from Rel-15 to explain that “uplink symbols” should be interpreted as any symbols possible for UE transmission.

- Option 2: After listing TS 38.101 series as impacted specification in SBFD WID, RAN4 shall introduce CR to explicitly explain that “uplink symbols” should be interpreted as any symbols possible for UE transmission “including SBFD symbols”.

Conformance testing key issues for SBFD-capable BS

Proposal 7: RAN4 shall begin the BS RF conformance testing discussion by considering the following key general issues:

- Issue-1: Additional manufacturer declaration(s) is needed for SBFD-capable BS.

  - It should be noted that at least the supported SBFD configuration for FR1 WA BS and some FR1 MR BS should be included.

  - FFS other declaration is required

- Issue-2: Additional test configuration(s) particular for SBFD requirement.

- Issue-3: RF channels to be tested for SBFD operation

  - FFS RF channels to be tested for SBFD operation, e.g,, to guarantee SBFD carrier is configured at the left/right-most of the operating band.

- Issue-4: Additional test models are needed for SBFD-capable BS.

  - FFS new test models are required to contain SBFD configurations for UL/DL subband and guardband.

# 5. Reference

[1] RP-240789 (revision from RP-234035), “WID revision: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, RAN#103.

[2] 3GPP TR 38.858 v18.0.0, “Study on Evolution of NR Duplex Operation (Release 18)”.

[3] R4-2406107, “Way Forward for [110bis][313] NR_duplex_evo”, Samsung, RAN4#110bis.

[4] R4-2409763, “Discussion on RAN4 aspects for SBFD system parameters”, Samsung, RAN4#111.

[5] R4-2409958, “Way Forward for [111][313] NR_duplex_evo”, Samsung, RAN4#111.

[6] R4-2411637, “Discussion on SBFD general aspects”, Samsung, RAN4#112.

[7] R4-2413514, “Way Forward for [112][307] NR_duplex_evo_General”, Samsung, RAN4#112.

[8] R4-2416497, “Discussion on RAN4 aspects for SBFD system parameters”, Samsung, RAN4#112bis.

[9] R4-2416584, “Way forward for [112bis][306] NR_duplex_evo_General”, Samsung, RAN4#112bis.

[10] R4-2418097, “Discussion on RAN4 aspects for SBFD system parameters”, Samsung, RAN4#113.

[11] R4-2419907, “Way Forward for [113][306] NR_duplex_evo_General”, Samsung, RAN4#113.

[12] R4-2500006 (R1-2410889), “LS on impact on transmit signal quality/MPR requirement”, Huawei, RAN1#119.

[13] R4-2501967, “Further discussion on general aspects for SBFD system”, Samsung, RAN4#114.

[14] R4-2502310, “Way Forward for [114][306] NR_duplex_evo_General”, Samsung, RAN4#114.

[15] R4-2504218, “Further discussion on general aspects for SBFD system”, Samsung, RAN4#114bis.

[16] R4-2504751, “Way Forward for [114bis][306] NR_duplex_evo_General”, Samsung, RAN4#114bis.