3GPP TSG-RAN WG4 Meeting #115                                                               R4-2507483

Malta, MT, 19th – 23rd May, 2025

Source:	Samsung

Title:	On the new requirements for SBFD operation for FR1 and FR2-1

Agenda item:			7.25.3.1

Document for:	Approval

# 1. Introduction

Based on Rel-18 study item on evolution of NR duplex operation, the support of subband non-overlapping full duplex has been studied, which is targeted to provide enhanced UL coverage, reduced latency, improved system capacity, and improved configuration flexibility for NR TDD operation [2].

In RAN#102, the work item on evolution of NR duplex operation (SBFD) has been approved, with WID further revised in the follow-up RAN plenary [1]. According to the objectives in WID, RAN1 is tasked to specify the mechanisms to support SBFD, including semi-static indication of time/frequency location, random access in SBFD symbols, and other transmission, reception and measurement behavior and procedures for SBFD aware UE. Furthermore, the enhancement for CLI handing, including gNB-to-gNB and UE-to-UE CLI handling, will also be specified in RAN1.

Accordingly, from RAN4 perspective, it is tasked to “Specify BS RF requirements for SBFD operation at gNB [RAN4]”, and this work shall be based on the good outcome from the study, i.e., Section 10.1, “Impact on BS RF requirements” in TR 38.858. Furthermore, the way forward encourages companies to continue contributing to ongoing discussions on (1) the modification of existing requirements for SBFD-capable BS and (2) potentially new requirement for SBFD operation, and address issues that have been raised during meetings [4]. In the RAN4#111 to #114bis, further agreements are achieved on the potentially new requirements for SBFD operation [6] [8] [10] [12] [14] [16]. In this contribution, accordingly, we would like to further provide our analysis and viewpoints on the potentially new BS RF requirements for SBFD operation for FR1 and FR2-1.

# 2. Discussion on in-channel adjacent subband requirements

### 2.1 In-channel adjacent subband leakage requirement

In RAN4#113, RAN4 agree to introduce new requirement for in-channel adjacent subband leakage ratio, which is further confirmed to be an absolute PSD limit and the requirement is renamed as “in-channel adjacent subband leakage requirement” in RAN4#114 and 114bis:

Agreement (in RAN4#114): - New requirement for In-channel adjacent subband leakage ratio: - In-channel adjacent subband leakage ratio requirement is defined with the existing ACLR requirement in absolute PSD limit value as baseline (i.e., Table 6.6.3.2-2 in TS 38.104 for FR1 and Table 9.7.3.3-2 for FR2 in TS 38.104) - FFS the value for absolute PSD limit - FFS the naming of this requirement, by considering the absolute PSD limit used instead of the ratio. Note that the current naming in TS 38.104 is ACLR absolute limit. Agreement (in RAN4#114bis): - The requirement (In-channel adjacent subband leakage ratio) is renamed as “in-channel adjacent subband leakage requirement” (by removing “ratio”). Way forward (in RAN4#114bis): - No need to further discuss the necessity of in-channel adjacent subband leakage ratio, based on agreement from RAN4#114.

Furthermore, based on RAN4#114bis, the following way forwards are agreed on the consideration for assumption and exemplary cases:

Way forward: - FFS whether/how this assumption can be captured in the RAN4 requirement for in-channel adjacent subband leakage. Way forward: - Absolute limit for in-channel adjacent subband leakage: - FFS by considering the following exemplary cases by performing analysis including measurement or simulation: - FR1: - DU/UD: 80MHz (DL) – 20MHz (UL) - DUD: 40MHz (DL) – 20MHz (UL) – 40MHz (DL) - Other cases are not precluded - FR2-1: - DU/UD: 50MHz (DL) – 50MHz (UL); 100MHz (DL) – 100MHz (UL) - DUD: 75MHz (DL) – 50MHz (UL) – 75MHz (DL) - Other cases are not precluded - Note: The study on different configuration does not mean RAN4 will define separate requirements for different configuration. RAN4 still try to use a unified requirements for absolute limit.

Based on the study in Rel-18, we have demonstrated that our FR1 3.5 GHz testbed implementation which utilizes digital pre-distortion (DPD) techniques to improve upon the non-linearity characteristics of the PA can achieve 45 dBc isolation between the SBFD DL and UL subbands.

For FR1 SBFD DU case example given in last meeting, by taking 80MHz (DL) – 20MHz (UL) into account and assuming 50dBm as BS TX power, the power density is around -14.03dBm. However, for 20MHz DL subband, still taking 50dBm as BS TX power, the power density can be as high as -8dBm. The benefit of adopt the absolute limit for the in-channel adjacent subband leakage is to simplify the requirement to cover all cases, from that perspective, we think additional 5dB relaxation on topic of the existing absolute basic limit for ACLR is reasonable to be take into account. Furthermore, n104 is another case to be consider, for which gNB vendors can fulfill the existing ACLR requirement by applying a more relaxed relative ACLR requirement (38dBc), which needs additional 7dB relaxation.

Proposal 1: For in-channel adjacent subband leakage requirement, the absolute basic limit values for FR1 SBFD DU/UD case can be specified by separating the different BS categories and bands:

| BS category / BS class | In-channel adjacent subband leakage absolute basic limit for bands other than n104 | In-channel adjacent subband leakage absolute basic limit for bands n104 |
| --- | --- | --- |
| Category A Wide Area BS | -8 dBm/MHz | -1 dBm/MHz |
| Category B Wide Area BS | -10 dBm/MHz | -3 dBm/MHz |
| Medium Range BS | -20 dBm/MHz | -13 dBm/MHz |
| Local Area BS | -27 dBm/MHz | -20 dBm/MHz |

As discussed in last meeting, Cumulative Adjacent Channel Leakage power Ratio (CACLR) defined in a sub-block gap or the Inter RF Bandwidth gap can be used as the reference to define the in-channel adjacent subband leakage requirement for DUD case. By considering the UL subband declared to be supported is typically not too small, seems the same absolute limit as proposed in Proposal 1 can be reused for DUD case.

Proposal 2: For in-channel adjacent subband leakage requirement, the absolute basic limit values for FR1 SBFD DUD case can be defined with the same limit given in Proposal 1.

### 3.2 In-channel adjacent subband blocking

In RAN4#113, the following agreement is obtained, i.e., RAN4 agree to introduce new requirement for in-channel adjacent subband blocking requirement, while no need to introduce new requirement for in-channel adjacent subband selectivity, and no further agreement achieved in RAN4#114:

Agreement: - New requirement for in-channel adjacent subband blocking should be defined - FFS the impact of in-channel interference source(s), with and without gNB-to-gNB CLI handling schemes, based on reasonable expected gNB-to-gNB isolation. - FFS in-channel selectivity (ICS) requirement can cover in-channel adjacent subband blocking requirement. - The requirement level shall be driven by feasibility consideration.

And the following further way forward is achieved in RAN4#114bis:

Way forward: - New requirement for in-channel adjacent subband blocking: - The requirement can be derived by considering co-site inter-sector case and/or inter-site case - If co-site inter-sector case is considered, FFS whether/how the assumption of isolation is reflected in specification. - The range of [-35dBm to -27dBm] can be used as the starting point for further study.

By taking into account the analysis in the study item phase, the RF simulation has shown the antennal isolation for co-channel co-site inter-sector gNB-gNB CLI is in the range of 62-93dB, depending on different antenna pair and co/cross-polarization, and 75dB can be regarded as typical value as RAN4 agreement. By taking 50dBm as the assumption for TX BS power, we shall specify -35 dBm as the requirement level for the SBFD in-channel adjacent subband blocking.

Proposal 3: For FR1 WA BS even without gNB-to-gNB CLI handling into account, -35 dBm can be viewed as a reasonable level to define the SBFD in-channel adjacent subband blocking requirement.

# 4. Conclusion

In this contribution, we provided BS RF requirement for SBFD-capable BS, accordingly with the following observations and proposals obtained:

In-channel adjacent subband requirements

Proposal 1: For in-channel adjacent subband leakage requirement, the absolute basic limit values for FR1 SBFD DU/UD case can be specified by separating the different BS categories and bands:

| BS category / BS class | In-channel adjacent subband leakage absolute basic limit for bands other than n104 | In-channel adjacent subband leakage absolute basic limit for bands n104 |
| --- | --- | --- |
| Category A Wide Area BS | -8 dBm/MHz | -1 dBm/MHz |
| Category B Wide Area BS | -10 dBm/MHz | -3 dBm/MHz |
| Medium Range BS | -20 dBm/MHz | -13 dBm/MHz |
| Local Area BS | -27 dBm/MHz | -20 dBm/MHz |

Proposal 2: For in-channel adjacent subband leakage requirement, the absolute basic limit values for FR1 SBFD DUD case can be defined with the same limit given in Proposal 1.

Proposal 3: For FR1 WA BS even without gNB-to-gNB CLI handling into account, -35 dBm can be viewed as a reasonable level to define the SBFD in-channel adjacent subband blocking requirement.

# 5. Reference

[1] RP-240789 (revision from RP-234035), “WID revision: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, RAN#103.

[2] 3GPP TR 38.858 v18.0.0, “Study on Evolution of NR Duplex Operation (Release 18)”.

[3] R4-2405655, “Discussion on BS RF requirements for SBFD-capable BS”, Samsung, RAN4#110bis.

[4] R4-2406107, “Way Forward for [110bis][313] NR_duplex_evo”, Samsung, RAN4#110bis

[5] R4-2409765, “On the potentially new requirements for SBFD operation for FR1 and FR2-1”, Samsung, RAN4#111.

[6] R4-2409958, “Way Forward for [111][313] NR_duplex_evo”, Samsung, RAN4#111.

[7] R4-2411640, “On the potentially new requirements for SBFD operation for FR1 and FR2-1”, Samsung, RAN4#112.

[8] R4-2413514, “Way Forward for [112][307] NR_duplex_evo_General”, Samsung, RAN4#112.

[9] R4-2416498, “On the potentially new requirements for SBFD operation for FR1 and FR2-1”, Samsung, RAN4#112bis.

[10] R4-2416584, “Way forward for [112bis][306] NR_duplex_evo_General”, Samsung, RAN4#112bis.

[11] R4-2418098, “On the potentially new requirements for SBFD operation for FR1 and FR2-1”, Samsung, RAN4#113.

[12] R4-2419907, “Way Forward for [113][306] NR_duplex_evo_General”, Samsung, RAN4#113.

[13] R4-2501968, “On the potentially new requirements for SBFD operation for FR1 and FR2-1”, Samsung, RAN4#114.

[14] R4-2502310, “Way Forward for [114][306] NR_duplex_evo_General”, Samsung, RAN4#114.

[15] R4-2504219, “On the new requirements for SBFD operation for FR1 and FR2-1”, Samsung, RAN4#114bis.

[16] R4-2504751, “Way Forward for [114bis][306] NR_duplex_evo_General”, Samsung, RAN4#114bis.