3GPP TSG-RAN WG4 Meeting #115                                                               R4-2507485

Malta, MT, 19th – 23rd May, 2025

Source:	Samsung

Title:	On the modification of existing TX requirements for SBFD-capable BS

Agenda item:			7.25.3.2

Document for:	Approval

# 1. Introduction

Based on Rel-18 study item on evolution of NR duplex operation, the support of subband non-overlapping full duplex has been studied, which is targeted to provide enhanced UL coverage, reduced latency, improved system capacity, and improved configuration flexibility for NR TDD operation [2].

In RAN#102, the work item on evolution of NR duplex operation (SBFD) has been approved, with WID further revised in the follow-up RAN plenary [1]. According to the objectives in WID, RAN1 is tasked to specify the mechanisms to support SBFD, including semi-static indication of time/frequency location, random access in SBFD symbols, and other transmission, reception and measurement behavior and procedures for SBFD aware UE. Furthermore, the enhancement for CLI handing, including gNB-to-gNB and UE-to-UE CLI handling, will also be specified in RAN1.

Accordingly, from RAN4 perspective, it is tasked to “Specify BS RF requirements for SBFD operation at gNB [RAN4]”, and this work shall be based on the good outcome from the study, i.e., Section 10.1, “Impact on BS RF requirements” in TR 38.858. Furthermore, the way forward encourages companies to continue contributing to ongoing discussions on (1) the modification of existing requirements for SBFD-capable BS and (2) potentially new requirement for SBFD operation, and address issues that have been raised during meetings [4]. In the RAN4#111, #112 and #112bis, further agreements are achieved on the modification of existing requirements for SBFD-capable BS [6] [8] [10] and [12]. In this contribution, accordingly, we would continue to provide our analysis and viewpoints on the modification of existing TX requirements for FR1 and FR2-1 SBFD-capable BS.

# 2. Deployment scenarios for FR1 SBFD macro BS

The issue of SBFD-SBFD co-existence case has been discussed over several meetings, while still diverged view on how this can be proceeded. In previous RAN4 meeting, it has been confirmed by several operators that SBFD-SBFD co-existence for FR1 macro BS is identified as an important deployment scenario and should be considered. It should be noted that the following way forward is approved:

- Way forward - Encourage companies to provide more inputs on whether SBFD-SBFD co-ex case should be in Rel-19 scope. - If it is in the scope of Rel-19, companies are also encouraged to provide input on whether/how to derive the additional requirements. FFS aspects that may need to be taken into account include: - Expected emissions and blocking levels, which could be estimated by means of considering, e.g.: - Option 1: Potential expected coupling loss and total receiver desensitization (considering self-interference, co-channel interference and adjacent operator interference) considering relevant site deployment scenarios - Option 2: Co-existence simulation and potentially analysis of co-located scenarios - Expected spectrum allocation and sub-band sizes - Expected feasible filtering performances considering the above two aspects

Based on previous discussion, the difficulty of this discussion comes from two dilemma (1)	Whether/how SBFD-SBFD scenario can be confirmed or not, and in which release this scenario should be handled. (2) If the SBFD-SBFD with lower grid-shift percentage is confirmed as a valid scenario, the contradicting views on the necessity of tightened TX emission requirement (derived by the new scenario) and the necessity of tightened RX requirement.

Considering only two meeting cycles left, we propose RAN4 needs make clear decision/recommendation among the possible ways to handle this SBFD-SBFD co-ex case, which can be taken into account in June RAN-Plenary (RAN#108) in 6G SID, 5G-A Rel-20 scope discussion.

Proposal 1: In this RAN4 meeting, RAN4 shall down-select among the following possible ways to handle this SBFD-SBFD adjacent-channel co-ex case, which can be taken as important input in RAN#108 decision making on 6G SID, and 5G-A Rel-20 scope:

- Option 1: Confirm SBFD-SBFD co-ex case in Rel-19 SBFD WI

- Option 2: Include SBFD-SBFD co-ex case as RAN4 objective in potential Rel-20 RAN1-led SBFD WI

- Option 3: Include SBFD-SBFD co-ex case as Rel-20 RAN4-led BS RF WI

- Option 4: Include SBFD-SBFD co-ex case as part of 6G SI co-ex study scope.

# 3. Modification on Existing TX requirement for SBFD

## 3.1 Base Station output power and radiated transmit power

For the requirement of BS output power and radiated transmit power, the following agreements is achieved in the study phase [2]:

Since configuration (e.g. antenna, power configuration etc) between SBFD and non-SBFD symbols/slots might be different, RAN4 reached the following consensus for the BS RF requirement of BS output power for both conducted and OTA output power: - It is allowed to have the different conducted declaration for normal DL symbols/slots and SBFD DL symbols/slots. - It is allowed to have different EIRP/TRP declaration (for level and direction) for normal DL symbols/slots and SBFD DL symbols/slots. - Accuracy requirement for TRP/EIRP and conducted power shall be the same for normal DL symbols/slots and SBFD DL symbols/slots.

And the following agreement is achieved in RAN4#111 [6]:

Issue 2-1-1: PSD scaling for normal and SBFD slots/symbols - Agreement: - PSD scaling for normal and SBFD slots/symbols: - No need to introduce the restriction on PSD scaling for normal and SBFD slots/symbols - Vendors can declare different TX power values for normal and SBFD slots/symbols

Based on the above agreements achieved in study item and RAN4#111, we expect the manufacturer declaration-based method will be used for base station output power, i.e., radiated transmit power and OTA base station output power, and further discussion could be expected on the specification implementation by allowing different declarations of output power for normal DL symbols and SBFD DL symbols. Based on our accompanying contribution on general aspects for SBFD BS RF requirement, RAN4 take the new chapter for SBFD-capable BS in TS38.104.

Taking the conducted requirement 6.2B “Base station output power for SBFD” as example, we see the following text proposal can be given for the corresponding requirement for SBFD operation. Different from last meeting’s proposal, we have updated the text proposal by removing the BS type 1-C part, according to the agreement from RAN4#113, i.e., “For SBFD, BS type 1-C is precluded.”

Proposal 2: The following text proposal is endorsed for BS output power requirement (taking the conducted requirement as example):

=================== Start of Text Proposal ===================

## 12.2.2 Base station output power for SBFD

### 12.2.2.1	General

The BS conducted output power requirement for SBFD operation is specified over SBFD symbols, in which the rated carrier output power for SBFD could be different from the rated carrier output powers for normal symbols as specified in clause 6.2.1.

For SBFD-capable BS, the rated carrier output power for SBFD of the BS type 1-H shall be specified in table 12.2.2.1-1.

Table 12.2.2.1-1: BS type 1-H rated output power limits for BS classes

| BS class | Prated,c,sys | Prated,c,TABC |
| --- | --- | --- |
| Wide Area BS | (Note) | (Note) |
| Medium Range BS | ≤ 38 dBm +10log(NTXU,counted) | ≤ 38 dBm |
| Local Area BS | ≤ 24 dBm +10log(NTXU,counted) | ≤ 24 dBm |
| NOTE: There is no upper limit for the Prated,c,sys or Prated,c,TABC of the Wide Area Base Station. |  |  |

### 12.2.2.2	Minimum requirement for BS type 1-H

The same requirement as clause 6.2.3 is applicable to BS output power requirement for SBFD operation.

=================== End of Text Proposal ===================

## 3.2 Output power dynamics

In the Rel-18 study item, the following conclusion has been reached for output power dynamics:

#### 10.1.2.2 Output power dynamics Regarding the output power dynamic requirement, which mainly consists of RE power control dynamic range requirement and total dynamic range requirement, RAN4 reached the following consensus: - To reuse the existing RE power control dynamic range requirement for SBFD BS; - The total dynamic range requirement is applicable for SBFD-capable BS during normal DL symbols/slots, it is agreed to define the output power dynamic range requirement for SBFD as the ratio of the declared rated output power with all DL RBs active for SBFD (maximum) and the same single RB power as non-SBFD (minimum).

Based on the agreement from RAN4#113, the requirement of total dynamic range can be defined as [10], by considering the different definition for DU/UD and DUD case respectively:

Agreement: - The downlink (DL) total power dynamic range for each NR carrier shall be larger than or equal to the level calculated as 10log(NRB,SBFD,DL) if one SBFD DL subband is configured, and as 10log(NRB,SBFD,DL,1 + NRB,SBFD,DL,2) if two SBFD DL subbands are configured.

Accordingly, we further propose to setting up the new sub-clauses 6.3.3B (Total power dynamic range for SBFD) and 9.4.3B (OTA total power dynamic range for SBFD) by defining the output power dynamic range requirement for SBFD as the ratio of the declared rated output power with all DL RBs active for SBFD (maximum) and the same single RB power as non-SBFD (minimum).

Proposal 3: The following text proposal is endorsed for BS output power dynamics requirement (taking the conducted requirement as example):

=================== Start of Text Proposal ===================

### 12.2.3	Total power dynamic range for SBFD

#### 12.2.3.1	General

The BS total power dynamic range for SBFD is the difference between the maximum and the minimum transmit power of an SBFD symbol for a specified reference condition.

For BS type 1-H this requirement shall apply at each TAB connector supporting transmission in the operating band.

NOTE 1:	The upper limit of the total power dynamic range for SBFD is the OFDM symbol power for a BS when transmitting on all RBs in transmission bandwidth configuration for SBFD DL subband(s) at maximum output power. The lower limit of the total power dynamic range for SBFD is the average power for single RB transmission. The OFDM symbol shall carry PDSCH and not contain RS or SSB.

NOTE 2:	The requirement does not apply to operation with shared spectrum channel access.

#### 12.2.3.2	Minimum requirement for BS type 1-H

The downlink (DL) total power dynamic range for each NR carrier shall be larger than or equal to the level calculated as 10log(NRB,SBFD,DL) if one SBFD DL subband is configured, and as 10log(NRB,SBFD,DL,1 + NRB,SBFD,DL,2) if two SBFD DL subbands are configured.

=================== End of Text Proposal ===================

## 3.3 Transmit OFF power

For the requirement of transmit ON/OFF power, the following agreement is provided in the study item:

Regarding the transmitter ON/OFF power requirement, RAN4 mainly focus on the ON-OFF time mask and concluded that transmit ON/OFF power requirement is not applicable within SBFD time slot.

For the study item conclusion that “transmit ON/OFF power requirement is not applicable within SBFD time slot”, we expect there are two options for specification implementation: (1) adding Notes in both clause 6.4.1.1 and 9.5.2.1; (2) adding new sub-clauses 6.4.1B and 9.5.2B for SBFD operation. Among these two options, “adding notes in both clause 6.4.1.1 and 9.5.2.1” is preferred by us for its simplicity. To implement agreements, the following Note can be adopted to clarify OFF power requirement shall not be applied for SBFD symbols

Proposal 4: The following text proposal is endorsed for transmit OFF power (taking the conducted requirement as example):

=================== Start of Text Proposal ===================

### 12.2.4	Transmit ON/OFF power

### 12.2.4.1	Transmitter OFF power

#### 12.2.4.1.1	General

For SBFD operation, transmitter OFF power requirements shall not be applied in SBFD symbols.

=================== End of Text Proposal ===================

## 3.4 Unwanted emissions

For the ACLR requirement of unwanted emissions, the following agreements is achieved in the study phase [2]:

- For ACLR requirement, it shall be defined outside of the whole carrier instead of sub-band for SBFD DL symbols/slots and ACLR requirement is still defined as the ratio of sum of TX power within the whole carrier to the adjacent carrier.

By separating the discussion on ACLR requirement by (1) coexisting with legacy TDD system in adjacent channel and (2) coexisting with new SBFD system in adjacent channel, the following agreement and way forward is achieved in RAN4#112 and #113:

## <from RAN4#112> Agreement: - When coexisting with legacy TDD system in adjacent channel, RAN4 shall apply the legacy ACLR and OBUE requirement for SBFD-capable BS. WF: - FFS on the followings: - For FR1 macro deployments, when coexisting with new SBFD system in adjacent channel, RAN4 to define additional ACLR and/or OBUE requirement to ensure feasible coexistence with SBFD-capable gNB. The assumption for the victim BS DESENS and CL of BS2BS is FFS. ## <from RAN4#113> Agreement: - FFS whether/how to introduce the additional ACLR and/or OBUE requirements for FR1 WA BS.

Especially for the scenario in which SBFD BS coexisting with new SBFD BS in the adjacent channel, RAN4 has not yet achieved further progress in RAN4#112bis and #113.

If the SBFD-SBFD coexistence scenario is not considered in Rel-19, the current work item should conclude that the same ACLR/OBUE requirements shall be applicable to the SBFD-capable BS which is targeted for SBFD-TDD adjacent-channel co-existence case. If the SBFD-SBFD coexistence scenario is not considered in Rel-19, based on the technical analysis in [R4-2413283], it is proposed that the new OBUE requirement can be defined with tightened emission mask. Similarly, the ACLR requirement when coexisting with new SBFD BS in the adjacent channel is another option. Introducing either of new tightened requirements can be regarded as the requirement of implementing TX filtering before band filter.

Proposal 5: In this RAN4 meeting, by following the selected option given in Proposal 1, RAN4 shall:

- If the SBFD-SBFD coexistence scenario is not considered in Rel-19,

RAN4 conclude that the same ACLR/OBUE requirements shall be applicable to the SBFD-capable BS which is targeted for SBFD-TDD adjacent-channel co-existence case.

- If the SBFD-SBFD coexistence scenario is not considered in Rel-19,

RAN4 perform ACIR simulation campaign before Aug. meeting, to finish the tightened TX requirement by Aug meeting.

If the method of new OBUE requirement is considered, the BS RF Bandwidth edge should be based on the declared SBFD DL subband and the frequency offset of measurement shall be determined by considering the practical co-existence scenario.

Proposal 6: If the method of new OBUE requirement is considered, the BS RF Bandwidth edge should be based on the declared SBFD DL subband and the frequency offset of measurement shall be determined by considering the practical co-existence scenario.

# 4. Conclusion

In this contribution, we provided the viewpoints on the modification of existing TX requirements for FR1 and FR2-1 SBFD-capable BS, accordingly with the following observations and proposals obtained:

Proposal 1: In this RAN4 meeting, RAN4 shall down-select among the following possible ways to handle this SBFD-SBFD adjacent-channel co-ex case, which can be taken as important input in RAN#108 decision making on 6G SID, and 5G-A Rel-20 scope:

- Option 1: Confirm SBFD-SBFD co-ex case in Rel-19 SBFD WI

- Option 2: Include SBFD-SBFD co-ex case as RAN4 objective in potential Rel-20 RAN1-led SBFD WI

- Option 3: Include SBFD-SBFD co-ex case as Rel-20 RAN4-led BS RF WI

- Option 4: Include SBFD-SBFD co-ex case as part of 6G SI co-ex study scope.

Proposal 2: The following text proposal is endorsed for BS output power requirement (taking the conducted requirement as example):

=================== Start of Text Proposal ===================

## 12.2.2 Base station output power for SBFD

### 12.2.2.1	General

The BS conducted output power requirement for SBFD operation is specified over SBFD symbols, in which the rated carrier output power for SBFD could be different from the rated carrier output powers for normal symbols as specified in clause 6.2.1.

For SBFD-capable BS, the rated carrier output power for SBFD of the BS type 1-H shall be specified in table 12.2.2.1-1.

Table 12.2.2.1-1: BS type 1-H rated output power limits for BS classes

| BS class | Prated,c,sys | Prated,c,TABC |
| --- | --- | --- |
| Wide Area BS | (Note) | (Note) |
| Medium Range BS | ≤ 38 dBm +10log(NTXU,counted) | ≤ 38 dBm |
| Local Area BS | ≤ 24 dBm +10log(NTXU,counted) | ≤ 24 dBm |
| NOTE: There is no upper limit for the Prated,c,sys or Prated,c,TABC of the Wide Area Base Station. |  |  |

### 12.2.2.2	Minimum requirement for BS type 1-H

The same requirement as clause 6.2.3 is applicable to BS output power requirement for SBFD operation.

=================== End of Text Proposal ===================

Proposal 3: The following text proposal is endorsed for BS output power dynamics requirement (taking the conducted requirement as example):

=================== Start of Text Proposal ===================

### 12.2.3	Total power dynamic range for SBFD

#### 12.2.3.1	General

The BS total power dynamic range for SBFD is the difference between the maximum and the minimum transmit power of an SBFD symbol for a specified reference condition.

For BS type 1-H this requirement shall apply at each TAB connector supporting transmission in the operating band.

NOTE 1:	The upper limit of the total power dynamic range for SBFD is the OFDM symbol power for a BS when transmitting on all RBs in transmission bandwidth configuration for SBFD DL subband(s) at maximum output power. The lower limit of the total power dynamic range for SBFD is the average power for single RB transmission. The OFDM symbol shall carry PDSCH and not contain RS or SSB.

NOTE 2:	The requirement does not apply to operation with shared spectrum channel access.

#### 12.2.3.2	Minimum requirement for BS type 1-H

The downlink (DL) total power dynamic range for each NR carrier shall be larger than or equal to the level calculated as 10log(NRB,SBFD,DL) if one SBFD DL subband is configured, and as 10log(NRB,SBFD,DL,1 + NRB,SBFD,DL,2) if two SBFD DL subbands are configured.

=================== End of Text Proposal ===================

Proposal 4: The following text proposal is endorsed for transmit OFF power (taking the conducted requirement as example):

=================== Start of Text Proposal ===================

### 12.2.4	Transmit ON/OFF power

### 12.2.4.1	Transmitter OFF power

#### 12.2.4.1.1	General

For SBFD operation, transmitter OFF power requirements shall not be applied in SBFD symbols.

=================== End of Text Proposal ===================

Proposal 5: In this RAN4 meeting, by following the selected option given in Proposal 1, RAN4 shall:

- If the SBFD-SBFD coexistence scenario is not considered in Rel-19,

RAN4 conclude that the same ACLR/OBUE requirements shall be applicable to the SBFD-capable BS which is targeted for SBFD-TDD adjacent-channel co-existence case.

- If the SBFD-SBFD coexistence scenario is not considered in Rel-19,

RAN4 perform ACIR simulation campaign before Aug. meeting, to finish the tightened TX requirement by Aug meeting.

Proposal 6: If the method of new OBUE requirement is considered, the BS RF Bandwidth edge should be based on the declared SBFD DL subband and the frequency offset of measurement shall be determined by considering the practical co-existence scenario.

# 5. Reference

[1] RP-240789 (revision from RP-234035), “WID revision: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, RAN#103.

[2] 3GPP TR 38.858 v18.0.0, “Study on Evolution of NR Duplex Operation (Release 18)”.

[3] R4-2405655, “Discussion on BS RF requirements for SBFD-capable BS”, Samsung, RAN4#110bis.

[4] R4-2406107, “Way Forward for [110bis][313] NR_duplex_evo”, Samsung, RAN4#110bis.

[5] R4-2409764, “On the modification of existing requirements for FR1 and FR2-1 SBFD-capable BS”, Samsung, RAN4#111.

[6] R4-2409958, “Way Forward for [111][313] NR_duplex_evo”, Samsung, RAN4#111.

[7] R4-2411638, “On the modification of existing TX requirements for SBFD-capable BS”, Samsung, RAN4#112.

[8] R4-2413515, “Way Forward for [112][308] NR_duplex_evo_BSRF”, Huawei, RAN4#112.

[9] R4-2416499, “On the modification of existing TX requirements for SBFD-capable BS”, Samsung, RAN4#112bis.

[10] R4-2416585, “Way Forward for [112bis][307] NR_duplex_evo_BSRF”, Huawei, RAN4#112bis.

[11] R4-2418099, “On the modification of existing TX requirements for SBFD-capable BS”, Samsung, RAN4#113.

[12] R4-2419908, “Way Forward for [113][307] NR_duplex_evo_BSRF”, Huawei, RAN4#113.

[13] R4-2501969, “On the modification of existing TX requirements for SBFD-capable BS”, Samsung, RAN4#114.

[14] R4-2502380, “Way Forward for [114][307] NR_duplex_evo_BSRF”, Huawei, RAN4#114.

[15] R4-2504220, “On the modification of existing TX requirements for SBFD-capable BS”, Samsung, RAN4#114bis.

[16] R4-2504752, “Way Forward for [114bis][307] NR_duplex_evo_BSRF”, Huawei, RAN4#114bis.