3GPP TSG-RAN WG4 Meeting # 115						R4-2507491

St Julian’s, Malta, 19th – 25th May 2025

Source:		Ericsson

Title:			On system parameter of WUR

Agenda item:		7.28.3.1

Document for:		Approval

# Introduction

In this paper, we present our overview on system parameter for WUR.

# Discussion

There is no agreement on transmission bandwidth system parameter in WF[2] and issue in WF[1] is discussed.

Issue 1-2-2: Whether Channel bandwidth clause for LP-WUS/WUR is needed (5.3M Channel Bandwidth for LP-WUS)?

Agreement:

- [Transmission bandwidth configuration] for LP-WUS/WUR signal is needed to specify.

For transmission bandwidth, RAN1 related agreement as follows:

Agreement

Update agreement in last meeting as below:

From RAN1 perspective, support X PRBs for LP-WUS and LP-SS with SCS 30kHz (blanked guard RBs are not included) for a channel bandwidth equal or larger than 5MHz

- X = 11

- FFS if other number of PRBs needed, for LP-SS and LP-WUS with a channel bandwidth equal or less than 5MHz

FFS: Whether the above is applicable to FR2

Agreement

From RAN1 perspective, support X=11 PRBs for LP-WUS and LP-SS with SCS 15kHz (blanked guard RBs are not included) for a channel bandwidth equal or larger than 5MHz.

A table with different bandwidth is suitable and example text could be :

The maximum transmission bandwidth configuration NRB for WUR and subcarrier spacing is specified in Table 5.3.2x-1.

Table 5.3.2x-1: Maximum transmission bandwidth configuration NRB_WUR for WUR

| SCS (kHz) | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB_WUR | NRB_WUR | NRB_WUR | NRB_WUR | NRB_WUR | NRB_WUR | NRB_WUR | NRB_WUR | NRB_WUR | NRB_WUR | NRB_WUR | NRB_WUR | NRB_WUR | NRB_WUR | NRB_WUR | NRB_WUR |
| 15 | N/A | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 |
| 30 | N/A | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 |
| 60 | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |

Maximum Transmission bandwidth should be defined in table above.

# Conclusions

In this contribution, we present our overview on general view of WUR with below proposal:

Proposal-1: Maximum Transmission bandwidth should be defined in table above.

# References

- R4-2503028, WF on UE RF requirements for LP-WUS, Vivo

- R4-2505234, WF for [114bis][131] NR_LPWUS_UERF, Vivo