# 3GPP TSG-RAN WG4 Meeting # 115						R4-2507493

# St Julian’s, Malta, 19th – 25th May 2025

Source:		Ericsson

Title:			Rx requirements of IBB, OBB, intermodulation, spurious emissions and others

Agenda item:		7.28.3.3

Document for:		Approval

# Introduction

In this paper, we present our view on the WUR RF requirement of IBB, OBB, intermodulation, spurious emissions and others .

# Discussion

In WF[1], the open issue relating to OBB and IBB is quoted below:

Issue 3-1-1: IBB, OBB requirements

Agreements:

- Same interference level as MR. further discuss and decide next meeting on relaxed wanted signal.

Figure 1:WUR selectivity and blocking requirement overview

When discussing the RF requirement for WUR, the relation to the main receiver should be discussed. Our view is that as WUR will first receive the wake up signal and then wake up the main receiver, it should at least tolerate the same level jammer/interference as the main receiver. In this way, the wake up receiver can wake up the main receiver in the presence of the same interference. Otherwise, the main receiver will not waken up even it can tolerate more interferer than WUR, which also means there is no need to design a good main receiver in RF perspective.

The WUR should tolerate the same absolute level RF interferer of IBB and OBB as main receiver.

Figure 1 illustrated the overall receiver requirement adding the signal of both WUR and MR. It can be observed that there is bandwidth difference between WUR and MR. As the frequency offset of the interferer is set in relation of the MR channel BW and as such, if the WUR signal could be placed at the near edge of the MR channel where is adjacent to the interferer, it could be possible to test the WUR with one MR bandwidth only. This is because that when placing the WUS signal at the edge of the channel, there would be only the guard band of the MR channel between the WUS signal and interferer and therefore, the test case with minimum MR channel bandwidth could set the worst-case scenario. Therefore, WUR test could be tested with one MR bandwidth which reduce the test cases for WUR.

It is possible to define one test case for WUR IBB and OOB with min MR channel bandwidth.

For the wanted signal level of WUR, the starting point for WUR is the same allowance of relaxation of MR for IBB and OBB. But this could be confirmed when ACS test case is decided. For additional shifted RB, as IBB is the non-adjacent channel selectivity, there is no need to set the additional guard RB and wanted signal can be placed at the edge of the channel.

There is no need to consider the guard or shifted RB for IBB and OOB case.

The wanted signal level of WUR, the starting point for WUR is the same allowance of relaxation of MR for IBB and OBB

For intermodulation response rejection, it is a measure of the capability of the receiver to receive a wanted signal on its assigned channel frequency in the presence of two or more interfering signals which have a specific frequency relationship to the wanted signal as such that the intermodulation product falls within the wanted signal band. Similar with IBB and OOB test case, for the IM test case, it is also possible to select one MR configuration only as the WUR bandwidth does not change and as such, the PSD of the IM product falling into the WUR bandwidth would become lower when testing with larger MR bandwidth (the interference bandwidth is the same with the MR channel BW for the case where for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz, but the power of Pinterferer2 is the same (-46 dBm) for different MR channel bandwidth). Therefore, choosing to test the min MR bandwidth together with WUR would be fine.

For intermodulation response rejection requirement, the WUR requirement should be tested with interferer setting with minimum MR bandwidth.

For maximum input power, the main receiver is tested considering the PAPR of the NR test signal, in this sense, the LP-WUS signal PAPR should be compared with the NR signal 64 QAM and therefore, if there is any power difference, such difference should be considered in the test requirements as the LP-WUS signal are not tested in presence of a NR signal.

PAPR for WUS-only signal:

- OOK M=4 (0,1,0,1): 4.9 dB​

- OOK M=1: 2.7 dB

As the LP-WUS only Manchester encoding so the PAPR for M=4 is 4.9 dB. The NR 64QAM PAPR is 9 dB. There is 4 dB difference if only LP-WUS signal is tested, therefore, the PAPR difference should be considered in maximum input power if only LP-WUS signal is tested.

Additional 4 dB should be considered to compensate the PAPR loss when only LP-WUS signal is tested.

Proposed text starts here:

7.4M	Maximum input level

Maximum input level is defined as the maximum mean power received at the UE antenna port, at which the MDR performance shall meet for the specified reference measurement channel. The MDR performance shall be less than 1% measured with the reference measurement channels as specified in Annexs A.3.2x with parameters specified in Table 7.4M-1.

Table 7.4M-1: Maximum input level

| Rx Parameter | Units | Channel bandwidth (MHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 3, 5, 10, 15, 20 | 25, 30, 35, 40, 45, 50 | 60, 70, 80, 90, 100 |
| Power in WUR ransmission Bandwidth Configuration1 | dBm | -25- 10log10(NRB/NRBLP-WUS) + [4] | -25 + 10log10(BWChannel /20) -- 10log10(NRB/NRBLP-WUS) + [4] | -20 - 10log10(NRB/NRBLP-WUS) + [4] |
| NOTE 1: 10log10(x) is rounded to the nearest 0.5dB value. |  |  |  |  |

# Conclusions

In this contribution, we present our view on WUR RF requirement for IBB, OBB, intermodulation, spurious emissions and others with below proposal:

Proposal-1: The WUR should tolerate the same level RF interferer of IBB and OBB as main receiver.

Observation 1 It is possible to define one test case for WUR IBB and OOB with min MR channel bandwidth.

Observation 2 There is no need to consider the guard or shifted RB for IBB and OOB case.

Proposal-2:The wanted signal level of WUR, the starting point for WUR is the same allowance of relaxation of MR for IBB and OBB

Proposal-2: For intermodulation response rejection requirement, the WUR requirement should be tested with interferer setting with minimum MR bandwidth.

# References

- R4-2505234, WF for [114bis][131] NR_LPWUS_UERF, vivo