# 3GPP TSG-RAN WG4 Meeting # 115						R4-2507494

# St Julian’s, Malta, 19th – 25th May 2025

Source:		Ericsson

Title:			WUR RF requirement of REFSESN, ASC, ASCS

Agenda item:		7.28.3.2

Document for:		Approval

# Introduction

In this paper, we present our view on the WUR RF requirement of REFSENS, ASC, ASCS.

# Discussion

### ASCS

Issue 2-3-1: ASCS requirements for OOK-based receiver

Agreement:

- For Type1 UE requirements, which are derived based on the assumption that demodulation of OOK content of LP-WUS is targeted

  - 1 RB guard for ASCS and 0.5dB de-sens

- For Type2 UE requirements, which are derived based on the assumption that demodulation of OFDM content of LP-WUS is targeted

  - 0 RB guard for ASCS and 0 dB de-sens

- For Type 1 and Type2 UE are not limited to specific LP-WUS waveform.

- FFS how to define test case and requirement applicability for Type 1 and Type 2

  - FFS how type 1 and type 2 capability related to RAN1 UE feature

- A UE can only support either Type 1 or Type 2 but not both for LP-WUR

Issue 2-3-3: Channel bandwidth for ASCS Test case

Agreements:

  - ASCS requirement should be applied to all supported UE CBW. Conformance test case and potential down-selection will be defined by RAN5.

  - For conformance testing, it is expected that not all CBW need to be tested.

Issue 2-3-4: whether high in-band signal (both LP-WUS and NR) case is considered for alternative ASCS requirements

Agreements:

  - FFS For ASCS, whether additional test case that the wanted signal is 14dB higher than LP-WUS REFSENS, i.e., REFSENSLP_WUR + 14 dBm, should be considered or not.

For ASCS test case, as there is a need on the guard RB so the placement of the this guard RB position in relation to the NR signal and LP-WUS signal should be clarified in the requirements. Currently the ASCS is tested with the same PSD for both NR signal and LP-WUS signal and there is no discussion on total number of NR RB. As the power leakage due to the filter folding is related to the interference power, we suggest testing the worst case, where maximum NR RB is configured at both side of the LP-WUS signal.  A note can be added referring to the

[REFSENSLP-WUS] + 0.5 + 10log10((NRB - X* Guard RB- NRB,LP-WUS)/NRB,LP-WUS) (4)

“NOTE 5:  1 Guard RB is placed present immediate adjacent to on one both/[either] side of LP-WUS RB when and non-zero number of interfering NR RB(s) is present on both side of LP-WUS and in this case X =2;  X=1 when non-zero number of interfering NR RB(s) is only present on one side of LP-WUS and 1 Guard RB is placed immediate adjacent to the LP-WUS in between NR RB and LP-WUS RB;

For the issue 2-3-4, the original proposal is to test the ASCS case, similar to the ACS case, where both small signal and high input power cases are tested. As this is to mimic the UE moves to cell centre, WUR should work in this case also. In such a case, the LP-WUS wanted signal PSD is the same with NR signal which in total combined signal reach to the maximum input power level. So small signal and high input power test also dynamic range as SNR of LP-WUS should be high. The ACS test case 2 could cover the dynamic range, so no need to test ASCS high input power (case 2).

ASCS is tested with the maximum NR signal power besides the two side of the LP-WUS signal.

Use a note to specify the guard RB relation to the LP-WUS and NR signal when needed.

ASCS case 2 can be dependent on ACS case 2 and no need to test.

### ACS

Issue 2-4-1: Baseline ACS requirements (same interference) and required guard RB for SET-1 (worse NF) LP-WUR

- Proposals

  - Proposal 1: check the following input from companies and converge the range for set-1 LR.  (Moderator)

|  | vivo | CMCC | ZTE | Huawei | Nokia | E/// | Sony | QC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ACS (dB) | 28 | 22-24 | 24 | 11 (Note1) | 22 | Note 2 | Higher wanted signal (Note3) | Higher wanted signal Note4 |
| Guard RB (as test parameter, not requirements) | 1RB | 1RB | 1RB | Less RB | 2RBs |  |  |  |

Note 1: For LP-WUR type aimed at moderate power consumption, the guard RBs could be derived with better phase noise profile and the interferer level could similar to that of MR.

Note 2: The same set of the inteference signal and wanted signal defined for both two set of the LP-WUS requirments. Further discuss the number of guard RB for two set of requirment.

Note 3: A higher wanted signal can be specified for OOK type LP-WUR ACS requirement to balance the power consumption and interference mitigation capability.

Note 4: LPWUS ACS is specified as (assuming higher wanted signal is higher than refsens by 14 dB). DgMR is the diversity gain used for legacy PDSCH REFSENS derivation
ACSLPWUS=  NFlegacy-NFLPWUS+10logCBWBWLPWUS+ACSlegacy-DgMR

  - Proposal 2: Whether consider a higher the wanted signal for set-1 ACS requirements, e.g., LP REFSENS+ >14dB.  (Moderator)

In the proposal of deriving the LP-WUS ACS, there is diversity gain factor is added. We share our understanding as below.

In RAN4 report#50 (R4-093227), it is agreed that no diversity gain is used in ACS legacy test.

Executive Summary

Maintenance of E-UTRA

UE Requirements

- ACS test currently in the spec does not consider the rx diversity gain.

Considering this decision and the interpretation of ACS can be three facts as below:

- The intput power of legacy wanted signal should be REFSENS + diversity gain

- The relaxation factor is 11 dB instead of the 14 dB

- From #1, the absolute interference signal level will be + diversity gain bigger then WUR test signal as WUR test signal diversity gain = 0

Referring to the formular in R4-030209, where the equation of ACS just account the noise increase due to the jammer presence.

“The conversion formula between the ACS and the signal levels can be written[4] as

a N = N + I/ACS					(1)

ACS = I/{(a-1)N)					(2)

Where 	a is the margin of wanted signal level above reference sensitivity in linear unit,

N is the noise floor in linear unit,

I , the adjacent channel interference signal level expressed in linear unit,

ACS, the adjacent Channel Selectivity expressed in linear unit

If the margin parameter a is big enough, the noise floor N on the right side of the equation (1) can be neglected, in this case, the formula (2) becomes

ACS  I/(a N)						(3)

In logarithmic unit, the exact relation (2) and the approximation relation (3) become respectively

ACS = I – N – A1					(4)

with A1=10log10(a-1), ACS expressed in dB, I and N are expressed in dBm.

and

ACS  I – N – A0					(5)

with A0=10log10(a), ACS expressed in dB, I and N are expressed in dBm.	”

Using the notation of above paper, considering a  diversity gain difference between the wanted signal of legacy receiver and WUR

IntrfrWUR=IntrfrWUR+diversity gain 				(6)

Then with the same formular for interference signal power:

Intrfrlegacy=ACS+N+A1=ACSlegacy+IMACS+Thermalnoise+Nlegacy+10logCBW_legacy+10log101110-1legacy

For WUR ,the same:

IntrfrWUR=ACS+N+A1=ACSWUR+IMACS+Thermalnoise+NWUR+10logCBW_WUR+10log101110-1WUR

Therefore, we have

ACSLPWUS=  NFlegacy-NFLPWUS+10logNRBRBLPWUS+ACSlegacy-Diversity gain

As the diversity gain issue in ACS test has not been discussed before so in above discussion, we present a way to derive the WUR ACS in relation to main receiver. It should be noted that in the above calculation, the relaxation factor of 11 dB is used (so wanted signal is REFSENS + 3dB, no diversity gain). If the WUR is using the 14 dB relaxation factor, it means the noise floor of WUR would be 3 dB higher, and in that, there is no need to consider the 3 dB in equation (6). This cancels the diversity gain if 14 dB relaxation would be used.

The diversity gain consideration should not be used together with 14 dB relaxation.

It should be also noted that for RedCap UE with single antenna, the same relaxation factor of 14 dB is used and no diversity gain factor is considered. This approach make more sense as a UE with better REFSENS should tolerate the same interferer with this better REFSENS. If the UE with better REFSENS are tested without the diversity gain (so the wanted signal is higher to cancel the diversity gain), basically what it means is that the hardware of claiming better REFSENS is the same blocking performance as a normal REFSENS. Well, good to reuse the same hardware platform. In this sense, maybe there is a need to spell out whether the blocking performance would be improved when some Rx branches are added/removed

Use the equation above in ACS requirements.

# Conclusions

In this contribution, we present our view on WUR RF requirement for REFSENS, ASCS and ACS with below proposal and observations:

Proposal-1: ASCS is tested with the maximum NR signal power besides the two side of the LP-WUS signal.

Proposal-2:Use a note to specify the guard RB relation to the LP-WUS and NR signal when needed.

Proposal-3:ASCS case 2 can be dependent on ACS case 2 and no need to test.

Proposal-4:The diversity gain consideration should not be used together with 14 dB relaxation.

Proposal-5: Use the equation above in ACS requirements.

# References

- R4-2420528, WF on UE RF requirements for LP-WUS, vivo

- R4-030209, BS ACS performance, Nortel Networks