# 3GPP TSG-RAN WG4 Meeting # 115						R4-2507499

# St Julian’s, Malta, 19th – 25th May 2025

Source:		Ericsson

Title:			RF Impact for FDD bands for PC2 RedCap

Agenda item:		7.6.2

Document for:		Approval

# Introduction

In this paper, we present our view on PC2 RedCap UE RF for FDD bands added in WID [1].

# Discussion

In WF[1], the remaining issue is the 1 Rx FD-FDD REFSENS.

Issue 1-1-1: REFSENS for 1Rx in FD-FDD

Agreement: Consider the following formats for specifying REFSENS for 1Rx PC2 RedCap in FD-FDD

- Option1: REFSENS (1Tx (26dBm) /2Rx) + ΔR1R : ΔR1R may be specified based on RSD ranges

  - Flat 3dB ΔR 1R for all CBW no larger than 20MHz or use values for FDD bands in table 7.3I.2-1 for bands whose PC2 1Tx/2RX RSD is 0dB.

  - Companies to evaluate REFSENS for other bands.

  - The format in which the REFSENS is shown in the specs is TBD pending evaluations.

- Option2: REFSENS (1Tx (26dBm) /2Rx) + ΔR1R : ΔR1R may be specified based on the following conditions

  - ΔR 1R value may be considered from LTE TS 36.101 Table 7.3.1E-1A for bands and CBW which are already defined in TS 36.101 Table 7.3.1E-1A. The ΔR 1R value is calculated by comparing the REFSESN specified in TS 36.101 Table 7.3.1E-1A with TS 36.101 Table 7.3.1.

  - Companies are encouraged to bring measurement based evaluation for ΔR1R for other bands and CBW which are not covered by LTE TS 36.101 Table 7.3.1E-1A.

  - The format in which the REFSENS is shown in the specs is TBD pending evaluations.

Way forward: It is encouraged for companies to provide analysis for option 1 and 2.

In previous meeitng discussion, some companies use either measured or reverse engineer to derive the 2 Rx to 1 Rx MSD relaxation factors. We propose to use the methodology in [2] to derive the relaxation factor from PC2 2 Rx to PC2 1Rx in this paper.

In this method, the MSD is calculated for single Rx using equation (1)

(1)

and further simplified to (2), where,  and .

(2)

The MSD for this single TX/RX receiver is calculated by comparing to the no de-sensitization (by allowing half dB desens  = 0.5 dB) and de-senstization by the Tx leakage.

For the dual Rx receiver branch, the MSD is calculated using the equation (3)

(3)

Where:

- atx-rx is the duplexer attenuation at RX / Tx-Rx filter isolation

- Lrx is the receive insertion loss RX insertion loss set to a more typical of 3 dB

- Lcpl is the branch coupling loss/ diversity antenna isolation

- Pout-ant at antenna: 22 dBm (MPR = 1 dB for 20 RBs allocation)

- is the margin for the TX noise that must be applied to achieve no desensitization,  = 0.5 dB

- B is the bandwidth

- Fmax is the noise factor

Equation (3) can be simplified to equation (4), where

(4)

As the equation (2) and (4) derive the MSD by comparing to the no-desens from Tx leakage (or use the 0.5 margin for de-sens). We assume 5MHz REFSENS of 2 Rx PC3 has no desens.

Firstly, the MSD for the channel bandwidth other than 5MHz for 2Rx PC3 can be derived using the following equations and illustrated in Table 1.

For legacy FDD band, the Rx de-sentization relaxation can be derived with the formular below for 15kHz SCS:

Rx de-sensitization of CBW of 15kHz SCS = CBW REFSENS of 15kHz SCS– ( 5MHz REFSENS of 15kHz SCS + 10log(N_RB/25)), where N_RB is the maximum RB allocation for the CBW of 15kHz SCS.

the Rx de-sentization relaxation can be derived with the formular below for 30kHz SCS:

Rx de-sensitization of CBW of 30kHz SCS = CBW REFSENS of 30kHz SCS – ( 5MHz REFSENS of 15kHz SCS + 10log(N_RB*2/25)), where N_RB is the maximum RB allocation for the CBW of 30kHz SCS.

Table 1:MSD factor for different BW for 2Rx PC3 (Refer to Table 7.3.2-1a for REFSENS)

| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz |  | 15 MHz |  | 20 MHz |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | REFSENS (dBm) | MSD (dB) | REFSENS (dBm) | MSD (dB) | REFSENS (dBm) | MSD (dB) |
| n1 | 15 | -100.0 | -96.8 | 0.0194 | -95.0 | 0.0031 | -93.8 | -0.0737 |
|  | 30 |  | -97.1 | 0.0670 | -95.1 | 0.0713 | -94.0 | -0.1066 |
| n8 | 15 | -97.0 | -93.8 | 0.0194 | -91.4 | 0.6031 | -85.8 | 4.9263 |
|  | 30 |  | -94.1 | 0.0670 | -91.7 | 0.4713 | -87.2 | 3.6934 |

For band n1 and n8. The MSD is for 2 Rx PC3 and adding the relaxation factor for band n1 and n8 in Table 7.3.2-1c in TS 38.101-1, we have total MSD for PC2 illustrated in Table 2.

Table 2: MSD factor for different BW for 2Rx PC2

| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz | 15 MHz | 20 MHz |
| --- | --- | --- | --- | --- | --- |
|  |  | 2 Rx MSD (dB) | 2Rx MSD (dB) | 2Rx MSD (dB) | 2Rx MSD (dB) |
| n1 | 15 | 0 | 0.0194 | 0.0031 | -0.0737 |
|  | 30 |  | 0.0670 | 0.0713 | -0.1066 |
| n8 | 15 | 0.5 | 0.72 | 1.4 | 7.2 |
|  | 30 |  | 0.7670 | 1.2713 | 5.9934 |

With this MSD in Table 2 and equation (4) assuming the coupling loss Lcpl is between 5 to 15 dB. The ratio Vt/Vn can be derived. The coupling loss is chosen so that 1 Rx MSD is worse than 2 Rx MSD in Table 3 and final relaxation delta from 2 Rx PC2 to 1 Rx PC2 should not exceed 6 dB.

Table 3: MSD for 2Rx and 1Rx for PC2

| Operating Band | SCS kHz | 5 MHz (dBm) |  | 10 MHz |  | 15 MHz |  | 20 MHz |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 2Rx MSD (dB) | 1Rx MSD (dB) | 2Rx MSD (dB) | 1Rx MSD (dB) | 2 Rx MSD (dB) | 1Rx MSD (dB) | MSD (dB) | 1Rx MSD (dB) |
| n1 | 15 | 0 | 0 | 0.0194 | 0.4741 | 0.0031 | 0.4426 | N/A | N/A |
|  | 30 |  |  | 0.0670 | 0.5663 | 0.0713 | 0.5747 | N/A | N/A |
| n8 | 15 | 0.5 | 1.0160 | 0.72 | 1.3463 | 1.4 | 2.3535 | 7.2 | 9.7890 |
|  | 30 | N/A | N/A | 0.7670 | 1.4167 | 1.2713 | 2.1628 | 5.9934 | 8.3848 |

Using this ratio in equation (2), then 1 Rx MSD can be derived. The derived MSD is listed in Table 3.  As the REFSENS of 2 Rx PC3 is the reference,  there is a need to derive the MSD from the 2 Rx PC2 to 1 Rx PC2 without Tx leakage. Obviously, this MSD is 3 dB. Considering this and the Table 3, the final relaxation factor from the PC2 REFSENS of 2 Rx to PC2 RESENS of 1 RX can be derived and this method is illustrated in Figure 1.

Figure 1: MSD from 2 Rx PC2 REFSENS to 1 Rx PC2 REFSENS

Table 4: Relaxation factor from 2Rx PC2 REFSENS to 1 Rx PC2 REFSENS

| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz | 15 MHz | 20 MHz |
| --- | --- | --- | --- | --- | --- |
|  |  | Relaxation delta (dB) |  |  |  |
| n1 | 15 | 3 | 3.5 | 3.43 | 3.5 |
|  | 30 |  | 3.5 | 3.50 | 3.5 |
| n8 | 15 | 3.5 | 3.6 | 4 | 5.6 |
|  | 30 |  | 3.65 | 3.9 | 5.4 |

In Table 4, for band n1, one can observe that the MSD derived in Table 2 has very small value, meaning it may be rounding error. This manifests in MSD of the bandwidth 20MHz which has negative error. We suggest using the flat MSD for band n1 as the Tx leakage to receiver for n1 is not obvious in the specification.

For band n1, the MSD of 3 dB for 5MHz and 3.5 dB for other channel BW could be used from 2 Rx PC2 REFSENS to 1 Rx PC2 REFSENS.

For band n8, the relaxation factor in Table 4 is recommended to consider when averaging the final MSD from other companies.

Consider the MSD relax factor from 2 Rx PC2 REFSENS to 1 Rx PC REFSENS in Table 4 for band n8.

# Conclusions

In this contribution, we present our overview on the PC2 RedCap RF overview with below proposals:

Proposal-1: For band n1, the MSD of 3 dB for 5MHz and 3.5 dB for other channel BW could be used from 2 Rx PC2 REFSENS to 1 Rx PC2 REFSENS.

Proposal-2: Consider the MSD relax factor from 2 Rx PC2 REFSENS to 1 Rx PC REFSENS in Table 4 for band n8.

# References

- R4-2505231, WF on PC2 RedCap UE RF requirements, China Telecom

- R4-081148, REFSENS and allowed MSR, Ericsson