# 3GPP TSG-RAN WG4 Meeting # 115						R4-2507500

# St Julian’s, Malta, 19th – 25th May 2025

Source:		Ericsson

Title:			RedCap UE RF impact on HD-FDD

Agenda item:		7.31.3.1

Document for:		Approval

# Introduction

In this paper, we present our views on the RedCap UE RF remaining issue.

# Discussion

In WF[1], the remaining issue is below:

## Issue 1-1: Output power enhancement for NR NTN PC3

Agreements:

- If defined, additional output power enhancement will be optional for UE to support.

  - FFS on need of new UE capability

- Evaluate potential to further enhance output power for RAN4#115:

  - Amount of output power enhancement [0.5 to 1 dB]

    - Less than 0.5 dB improvement is not specified

  - Initially consider all RB allocations and modulation orders

    - Consider both MPR reduction and MOP increase

      - FFS how to capture MOP increase in the specification

    - Choice between MPR reduction and MOP increase can be different for inner/outer allocations and modulation orders

  - Evaluate both HD-FDD and FD-FDD

    - Consider at least insertion loss impact

    - FFS whether to apply same enhancement to both HD-FDD and FD-FDD

  - Evaluate the option to support both rel-18 and rel-19 output power enhancement schemes simultaneously

    - FFS whether simultaneous support is specified

    - Consider whether PC3 or PC2 ACLR requirement should apply

  - FFS Applicability to PC3 non-RedCap NR NTN

Rel-18 coverage enhancement work item introduced the power boosting feature for PC2 and PC3. It has introduced two capabilities (powerBoosting-pi2BPSK-QPSK-r18 and/or powerBoosting-pi2BPSK-QPSK-Modified-r18). The difference of these two capabilities is that whether the spectrum flatness requirements would be relaxed compared to the legacy spectrum flatness requirements. For UE supporting capability of powerBoosting-pi2BPSK-QPSK-r18, the enhanced inner region (reduced RB ranged within the legacy inner region), no need to relax the spectrum flatness while for UE supporting powerBoosting-pi2BPSK-QPSK-Modified-r18, the spectrum flatness is relaxed and tested against the clause 6.4.2.4.2. No matter which capabilities will be supported by Rel-18 NR UE, either legacy RB inner region or enhanced inner region will be applied and outer RB region is excluded in the power boosting feature.  Also the Rel-18 coverage enhancement is generic feature and it is possible for (e) RedCap UE to support based on capabilities reporting.

For UE supporting Rel-18 power boosting capabilities only inner or enhanced inner region apply and outer region is excluded for power boosting.

Rel-18 (e) RedCap UE can support the Rel-18 power boosting feature.

A Rel-19 NTN (e) RedCap UE can also support the Rel-18 power boosting feature as the feature is generic for NR UE as per agreement in WF [1]. Moreover, as the power increase due to the reduction of the insertion loss of the Tx filter, such power boosting is different with Rel-18 power boosting scope where a transparent scheme would be applied, e.g FDSS, peak cancellation, clipping + filtering etc.  Though power increase by reducing the insertion loss of filter can be rendered using the Rel-18 power boosting capabilities, it can also be used independently with the Rel-18 power boosting feature enabled with FDSS. Therefore, a (e)RedCap UE could increase its output power above the power level when such UE also enabling the Rel-18 power boosting feature simultaneously by reducing the filter insertion loss and at the same time retaining the same PA and baseband (e.g FDSS or peak cancellation still apply) if the unwanted emission requirements still be met. Therefore, a new output power enhancement can be supported simultaneously with rel-18 power boosting.

Additional new output power enhancement can be supported simultaneously with rel-18 power boosting.

For example, for a UE supporting Rel-18 power boosting, the boosted power is 1 dB for PC3 and 0.5 dB for PC2. With insertion loss reduction, the power boosting delta relative to the UE nominal power could be stretched to 1+ X dB for PC3 and 0.5 + Y for PC2 , where X dB for PC3 or Y dB for PC2 is due to the insert loss reduction and it could apply to all modulation scheme which is good property compared to the Rel-18 power boosting which only apply to BPSK/QPSK. On top of this, such power increase (X dB for Pc3 and Y dB for PC2) can apply to all RB allocation without limiting to only inner or enhanced inner region, considering the power boosting mechanism could be different for a (e)RedCap UE. A (e)RedCap UE can support power boosting for outer region with a different power boosting capability on top of Rel-18 power boosting capabilities. As power boosting features are optional features, the below options could be possible for a (e) RedCap UE:

- A (e)RedCap UE operating in HD-FDD mode (indicating the halfDuplexFDD-TypeA-RedCap-r17) supports only Rel-18 power boosting capabilities (powerBoosting-pi2BPSK-QPSK-r18 and/or powerBoosting-pi2BPSK-QPSK-Modified-r18)

- A (e)RedCap UE operating HD-FDD mode and Rel-19 new power boosting enhancement capability supports only the power boosting by boosting the power with X dB for PC3 and Y dB for PC2.

- A (e)RedCap UE operating HD-FDD mode supports both Rel-18 power boosting enhancement capability and Rel-19 new power enhancement X dB for PC3 and Y dB for PC2 due to insertion loss reduction. The total configured output power could be two options below:

  - Either Rel-18 power boosting (1 dB for PC3 and 0.5 dB for PC2) or Rel-19 new power enhancement (X dB for PC3 and Y dB for PC2) can be configured, not simultaneously.

  - Both Rel-18 power boosting and Rel-19 new power enhancement can be configured simultaneously and the total output power boosting could be 1 + X dB for PC3 and 0.5 + Y for PC2.

If the option 3 above would be possible, it should further discuss whether a tightened ALCR compared to PC3 ACLR should apply.

Discuss the (e) RedCap power boosting with the three options listed above.

A (e)RedCap UE can support the power boosting with X dB for PC3 and Y dB for PC2 with insertion loss reduction and apply to both outer region and inner region, also apply to other modulation than BPSK/QPSK if unwanted emission would be met.

Power boosting due to the insertion loss reduction could apply both FD-FDD and HD-FDD.

There is a need to discuss the applied RB region and modulation scheme though. The Rel-18 power boosting feature only applies to the inner RB region where 0 MPR is applied. As a starting point, this region should be assumed when Rel-18 power boosting and Rel-19 new power enhancement would be enabled with legacy power boosting simultaneously.  There is also a possibility to extend the RB region to outer RB allocation as the transparent scheme applied can reduce the MPR at outer region also.

By applying the transparent scheme, Rel-19 power boosting enhancement could be extended to outer region when insertion less reduction is enabled simultaneously.

In principle, if a new power enhancement is to be specified for Rel-19 NTN (e) RedCap, either insertion loss reduction or another transparent scheme could be applied. Whether or not to apply the different schemes also depend on the target of the power boosting. In this sense, we think RAN4 can discuss to set a target power boosting value and further discuss if the requirement impact.  For such purpose, we suggest X = Y=0.5 dB for a starting point for Rel-19 new power enhancement.

Target X=Y=0.5 dB for both PC3 and PC2 for Rel-19 new power enhancement.

From the previous meeting discussion, companies confirmed that it is possible to boost the power with more than 0.5 dB considering the insertion loss reduction. As a matter of fact, the power boosting motivation is motivated by the companies feasibility. The network-controlled power boosting is an alternative to relax the output power tolerance so network still can gain from the better hardware equipped in UE.

It is feasible to boost the power with the at least 0.5 dB due to the insertion loss for HD-FDD RedCap UE.

As there is only one meeting left before finalizing the Rel-19 work item which target to RAN#109 (September 2025), we recommend agreeing to add a UE capability for the power enhancement at least for HD-FDD RedCap UE. This capability is network controlled as similar as the Rel-18 power boosting capability. As the new power boosting scheme originates in NTN Rel-19 but it can be applied to TN RedCap UE also. It will be up to UE to report and associate to either TN or NTN band as an optional feature.

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| [powerBoosting-pi2BPSKQPSK-Rel-19] Indicates whether [(e) RedCap] UE supports power boosting for pi/2 BPSK and QPSK, when applicable as defined in 6.2 of TS 38.101-1 [2]. [ [The (e) Redcap] UE can include this field only if the UE supports halfDuplexFDD-TypeA-RedCap-r17]. | Band | NO | NO | FR1 only |

Use the above capability definition to introduce the power boosting scheme for RedCap UE

As this is network controlled power boosting, a new IE needs to be specified. Together with the new UE capability, an LS to RAN2 should be initiated, the suggested text is below:

Below capability is agreed to be added in Rel-18:

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| [powerBoosting-pi2BPSKQPSK-Rel-19] Indicates whether [(e) RedCap] UE supports power boosting for pi/2 BPSK and QPSK, when applicable as defined in 6.2 of TS 38.101-1 [2]. [ [The (e) Redcap] UE can include this field only if the UE supports halfDuplexFDD-TypeA-RedCap-r17]. | Band | NO | NO | FR1 only |

Additionally, RAN4 agrees to enable/disable the Rel-19 power boosting with RRC signalling and agree to add separate one RRC IE, [powerBoostPi2BPSKQPSK-Rel-19] power boosting.

Send LS to RAN2 for the added UE capability and IE to configure the power boosting.

For the specification impact, it needs to be discussed that if the new power boosting could be extend beyond the inner region where MPR =0. This could be either confirmed by manufacture measurement results or by MPR simulations. We propose the inner region as a starting points for Rel-19 power boosting and companies can further investigate the other RB region for next meeting. For modulation scheme, BPSK and QPSK should be focused.

RB inner region and BPSK/QPSK should be starting point for basic power boosting enhancements.

For other region and other modulation, further study is needed for next meeting.

# Conclusion

In this contribution, we present our view on the remaining issue for  Redcap UE RF:

Observation 1 For UE supporting Rel-18 power boosting capabilities only inner or enhanced inner region apply and outer region is excluded for power boosting.

Observation 2 Rel-18 (e) RedCap UE can support the Rel-18 power boosting feature.

Observation 3 Additional new output power enhancement can be supported simultaneously with rel-18 power boosting.

Observation 4 A (e)RedCap UE can support the power boosting with X dB for PC3 and Y dB for PC2 with insertion loss reduction and apply to both outer region and inner region, also apply to other modulation than BPSK/QPSK if unwanted emission would be met.

Observation 5 By applying the transparent scheme, Rel-19 power boosting enhancement could be extended to outer region when insertion less reduction is enabled simultaneously.

Proposal-1: Discuss the (e) RedCap power boosting with the three options listed above.

Proposal-2: Power boosting due to the insertion loss reduction could apply both FD-FDD and HD-FDD.

Proposal-3: Target X=Y=0.5 dB for both PC3 and PC2 for Rel-19 new power enhancement.

Proposal-4: Use the above capability definition to introduce the power boosting scheme for RedCap UE

Proposal-5: Send LS to RAN2 for the added UE capability and IE to configure the power boosting.

Proposal-6: RB inner region and BPSK/QPSK should be starting point for basic power boosting enhancements.

Proposal-7: For other region and other modulation, further study is needed for next meeting.

# References

- R4-2504708, Way Forward for [114bis][309] NR_NTN_Ph3_General_UE_SAN_RF, Qualcomm Inc

- R4-2313475, RF spec impact for transparent MPR reduction scheme, Ericsson