# 3GPP TSG-RAN WG4 Meeting # 115						R4-2507501

# St Julian’s, Malta, 19th – 25th May 2025

Source:		Ericsson

Title:			Other NTN UE RF impact

Agenda item:		7.31.3.2

Document for:		Approval

# Introduction

In this paper, we present our general overview on NTN UE RF impact on uplink capability objective.

# Discussion

In LS[1], RAN1 ask RAN4 to evaluate the feasibility of reusing the DMRS bundling on OCC2 /OCC4, or a new requirement would be needed.

RAN1 assumes that the UE is required to maintain phase continuity and power consistency for the duration of one OCC group with PUSCH.

- FFS: under which conditions the above applies, e.g. under the same conditions that phase continuity applies for DMRS bundling.

RAN1 agreed the following:

2. Actions:

RAN1 respectfully ask RAN4 whether the same phase continuity requirements as for DMRS bundling can be applied for OCC length 2 and/or OCC length 4 under the same conditions as for DMRS bundling, or if new requirements are needed.

RAN1 question is very specific and RAN4 task according to LS [1] is quite clear. The LLS simulation campion should be started to answer RAN1 LS and at the same time evaluating the RF impact for the OCC2/OCC4 feature.

LLS simulation campion should be started in RAN4 to answer the RAN1 LS and at the same time evaluating the RF impact of OCC2/OOC4.

In the DMRS requirement in 38.101-1, the requirements are quoted below

Table 6.4.2.5-1: Maximum allowable phase difference for DMRS bundling

| UL channel | Modulation order | Phase difference between any slot p-1 and slot p (NOTE 2) | Phase difference between slot 0 and any slot p (NOTE 3) |
| --- | --- | --- | --- |
| PUSCH | Pi/2 BPSK, QPSK | [25] degrees | [30] degrees |
| PUCCH | Pi/2 BPSK, BPSK, QPSK |  |  |
| NOTE 1: The UE capability of the length of maximum duration refers to the maximum time duration during which UE is able to meet the phase continuity requirements, assuming no phase consistency violating events defined in TS 38.214 in between. NOTE 2: This requirement applies for FDD and TDD bands, for supported DMRS bundling configurations ≤ 8 slots. NOTE 3: This requirement applies only for FDD bands, for supported DMRS bundling configurations of 16 slots. |  |  |  |

The requirement are either between two adjacent time slots or between slot 0 and slop p. The requirement is 5 degree difference also. The adjacent time slot requirement apply to the supported DMRS bundling time window <=8 and requirement between 0 and slot p apply to the time window 16 slot for FDD bands. As FDD bands are specified for NTN feature so both 16 time slot and <=8 slot cases should be simulated.

During the DMRS bundling feature in coverage enhancement work item, the phase offset modelling also proposed:

Issue 1-1: Model of phase variation

Candidate options:

•     Phase offset Option 1: for each individual slot k (k=1…n) within the bundle, an independent offset is generated and applied with respect to the slot 0.

•     Phase offset Option 2: for each individual slot k (k=1…n) within the bundle, an independent offset is generated and applied with respect to the slot k-1. (i.e., the offset is allowed to accumulate)

And

- Issue 1-3-2: Phase continuity tolerance

Agreement in GTW:

- Criterion to derive the tolerance:

  - The degradation of performance for case with phase offset over case without phase offset.

  - The performance gain of using joint channel estimation over not using joint channel estimation when phase offset is modeled.

- Run the simulations for the following cases:

- For Option 1 phase offset, consider offset [-X, X].

  - X is in the range of 10 to 40.

  - Option 1 phase offset means that for each individual slot k (k = 1…n) within the bundle, an independent offset is generated and applied with respect to the slot 0.

- For Option 2 phase offset, consider offset [-X, X].

  - X is in the range of 5 to 20.

  - Option 2 phase offset means that for each individual slot k (k = 1…n) within the bundle, an independent offset is generated and applied with respect to the slot k-1. (i.e., the offset is allowed to accumulate)

We recommend to use the uniform distribution so the phase offset value is bounded. The value X is taken from the DMRS bundling requirement in respective option ½ for TDW = 16 ms or TDW <=8 case.

For other LLS simulation assumptions, following RAN 1, we have the table below:

Align the LLS simulation parameters for simulation.

| Parameter | Value |
| --- | --- |
| Channel model | NTN-TDL-C Rural, 30° elevation angle |
| Carrier frequency | 2 GHz |
| Subcarrier spacing | 15 kHz |
| UE speed | 3 km/h |
| PUSCH mapping | 12 OS per slot excluding DMRS |
| HARQ configuration | No HARQ |
| Channel coding | LDPC |
| TBS | - ≈184 bits payload for VoIP (AMR 4.75kbps) - 104 bits payload for low data rate |
| DMRS configuration / bundling | 1+1 DMRS, no DMRS bundling |
| PRBs/MCS/ MCS table | - MCS table: Table 6.1.4.1-1 VoIP - 2 PRBs - Inter-slot OCC: MCS 5 Baseline (no OCC): MCS 5 |
| OCC length | 4 |
| OCC sequence | Hadamard sequences in Table 6.4.1.2.1.2-1 in TS 38.211 |
| Impairments | - TO: Uniform selection from [-0.94 µs, 0.94 µs], where 0.94 µs=29Ts - FO: Uniform selection from [-0.1 ppm, +0.1 ppm], no frequency drift. Notes: UE CFO grouping is applied , the phase should be added with CFO*time +phase offset for interfering UE. |
| Satellite antenna configuration | 1Rx |
| UE antenna configuration | 1Tx |

And based on the option 1 and option 2, we get the following simulation results:

# Conclusions

In this contribution, we present our general overview for NTN UE RF impact for UL capacity objective with below observations and proposals:

Proposal-1:LLS simulation campion should be started in RAN4 to answer the RAN1 LS and at the same time evaluating the RF impact of OCC2/OOC4.

Proposal-2: Align the LLS simulation parameters for simulation.

# References

- R1-2503070, Draft LS on requirements for the phase continuity and power consistency for OCC with PUSCH in NR NTN Ph3, MTK