3GPP TSG-RAN WG4 Meeting#115                               R4-2507507

Malta , MT, 19th – 23th May, 2025

Agenda item:	7.26.3.2

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on RF requirement of Ambient IoT device

Document for:	Approval

Introduction

In the RAN#106 meeting, New WID on Rel-19 Ambient IoT [7] was approved and it’s expected to start the normative work from the Feb, 2025 meeting. In this contribution, we would like to share some further views based on the SI outcome and scope reached during RAN#106 meeting.

- Topic 3-1: General Issue 3-1-1: General Agreement: - Only define radiated RF requirements for device 1 in Rel-19 WI. The details will be further discussed. - Topic 3-2: Tx Issue 3-2-1: Transmit output power Agreement: - Measurement configuration including input CW power and potential other conditions need to be defined. Issue 3-2-2: Modulation quality requirements Agreement: - Further discuss whether and how to define the modulation signal quality requirements Issue 3-2-3: Occupied bandwidth Agreement: - Further discuss whether to define the occupied bandwidth for device 1. Issue 3-2-4: Emission requirements Agreement: - Consider one composite emission requirements covering the emission frequency range of SEM and spurious emission as starting point. - Further discuss whether to define TRP or EIRP requirement for unwanted emission. Issue 3-2-5: Other requirements Agreement: - Do not define output power dynamics and intermodulation requirements for device 1. Issue 3-2-6: Other requirements (frequency error) Agreement: - Do not define frequency error requirements. - Small frequency shift error will be discussed in modulation quality requirements. Issue 3-2-7: Other requirements (Transmit ON/OFF time mask) Agreement: - Do not define transmit ON/OFF time mask for device 1 - Topic 3-3: Rx Issue 3-3-1: Reference sensitivity Agreement: - For R2D reference sensitivity, consider following options: - Option 1: define reference sensitivity using legacy approach, e.g. PREFSENSdBm=−174dBm+10×log10BW+NF+IM+SNR - Option 2: define reference sensitivity based on link budget and/or physical limits of the receiver circuitry. - Other options are not precluded. Issue 3-3-2: Maximum input power Agreement: - Further discuss the maximum input power for A-IOT device for R2D signal. Issue 3-3-3: Interference rejection Agreement:

Discussion

- 2.1. RF requirement for Ambient IoT device

- In the following section, we would like to share some initial understandings on the RF requirements for A-IoT device.

- During the last RAN4 meeting, there were some discussion on the back scattering loss for BPSK and OOK signal for D2R. Based on the analysis, the commercial capability is expected as -2dB for OOK and -6dB BPSK.

- Proposal 1: for the back scattering loss of device, propose to consider the -2dB for OOK and -6dB BPSK.

- Regarding the OTA REFSENS requirement, the following factors should be considered:

- Antenna gain for device, roughly around 2-2.1dB;

- The OOK circuit detection capability which is expected around -32dBm;

- Power consumption of device around 1uW;

- Based on the above considerations, we propose to define the OTA REFSENS requirement as -34dBm;

- Proposal 2: propose to define REFSENS requirement as -34dBm.

- In addition, one important issue is to define the FRC for R2D detection, based on some RAN1 inputs, we proposed to have the following FRC for R2D for further discussion.

- Proposal 3: propose to consider the following R2D FRC as starting point for further discussion.

| - SCS | - 15 | - 15 | - 15 | - 15 |
| --- | --- | --- | --- | --- |
| - PRB | - 1 | - 2 | - 3 | - 4 |
| - TBS | - 20 | - 20 | - 20 | - 20 |
| - CRC | - 6 | - 6 | - 6 | - 6 |
| - M | - 6 | - 12 | - 24 | - 24 |

Conclusions

In this contribution, we shared some further views on the RF requirement for Ambient IoT device and proposals are made as following:

- Proposals 1: for the back scattering loss of device, propose to consider the -2dB for OOK and -6dB BPSK.

- Proposal 2: propose to define REFSENS requirement as -34dBm.

- Proposal 3: propose to consider the following R2D FRC as starting point for further discussion.

| - SCS | - 15 | - 15 | - 15 | - 15 |
| --- | --- | --- | --- | --- |
| - PRB | - 1 | - 2 | - 3 | - 4 |
| - TBS | - 20 | - 20 | - 20 | - 20 |
| - CRC | - 6 | - 6 | - 6 | - 6 |
| - M | - 6 | - 12 | - 24 | - 24 |

References

- RP-240826,Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR (CMCC), Approved.

- R4-2410597, WF on impacts of A-IoT on RF requirements, Approved.

- R4-2414305,WF on RF requirements for A-IoT, Approved.

- R4-2414304, WF on Co-existence study for ambient IoT, Approved

- ISO/IEC 18046-3 Information technology – Radio frequency identification device performance test methods – Part 3: Test methods for tag performance

- R4-2502235,On the RF requirements for Ambient IoT Device, Huawei, Noted.

- R4-2502858, WF on requirements for A-IoT device, CMCC, approved.

- R4-2502859, WF on requirements for A-IoT BS and CW, Huawei, approved.

- R4-2502858, WF on requirements for A-IoT device, CMCC, approved.

- R4-2502859, WF on requirements for A-IoT BS and CW, Huawei, approved.