3GPP TSG-RAN WG4 Meeting#115                                R4-2507514

Malta, MT, 19th – 23th May, 2025

Agenda item:	7.13.4.1

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on conformance testing for U6GHz BS EEIRP mask

Document for:	Approval

Introduction

In the RAN#103 meeting, new WID for Rel-19 BS RF evolution has been approved and it’s expected to start the normative work from this April meeting. In the last RAN4 and RAN#107 meeting, we have basically completed this objective, however we still have MU etc to to be addressed, therefore we would like to share our further analysis on these remaining issues.

Discussion

## 2.1. MU

Based on the EEIRP errors as captured in TR 38.908 clause 6.6 in the following table, the probability distribution curve for error across different antenna configuration and steering range are provided to check the distribution function. Meanwhile the probability distribution curve for error among all 7 bins across different antenna configuration and steering range is provided to check the distribution function.

Table 6.6.-1.Errors per bin between the EEIRP simulation results of test beam and EEIRP simulation results of reference testing beams

| ZTE |  | Bin index | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Ant 4, 12degree | Error for Rect Ref | -0.98 | -0.17 | -0.70 | -0.28 | -0.30 | 0.04 | 0.36 |
|  |  | Error for Ellipse Ref | 0.25 | -0.22 | -0.03 | 0.07 | 0.21 | -0.11 | -0.40 |
|  | Ant 4, 16degree | Error for Rect Ref | -1.51 | -1.38 | -0.78 | 0.64 | -0.42 | -0.20 | 0.54 |
|  |  | Error for Ellipse Ref | -0.09 | -0.97 | 0.38 | -0.03 | 0.20 | -0.05 | -0.54 |
|  | Ant 1, 12degree | Error for Rect Ref | -0.98 | -0.17 | -0.70 | -0.28 | -0.30 | 0.04 | 0.36 |
|  |  | Error for Ellipse Ref | 0.25 | -0.22 | -0.03 | 0.07 | 0.21 | -0.11 | -0.40 |
|  | Ant 1, 16degree | Error for Rect Ref | -1.13 | -0.09 | 0.03 | 0.09 | 0.14 | 0.33 | 0.72 |
|  |  | Error for Ellipse Ref | 0.14 | 0.10 | 0.03 | 0.02 | -0.01 | -0.06 | -0.12 |
| Huawei |  | Bin index | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
|  | Ant 4, 12degree | Error for Rect Ref | -1.25 | -0.32 | -1.15 | -0.81 | -0.46 | -0.26 | 0.36 |
|  |  | Error for Ellipse Ref | 0.28 | 0.07 | 0.01 | 0.07 | 0.17 | 0.10 | -0.40 |
|  | Ant 4, 16degree | Error for Rect Ref | -0.44 | -1.12 | -0.82 | 0.38 | -0.77 | -0.34 | 0.37 |
|  |  | Error for Ellipse Ref | 0.43 | 0.27 | -0.06 | -0.01 | 0.10 | 0.03 | -0.48 |
|  | Ant 1, 12degree | Error for Rect Ref | -1.08 | -0.18 | -0.05 | 0.01 | 0.05 | 0.28 | 0.66 |
|  |  | Error for Ellipse Ref | 0.14 | 0.01 | 0 | 0 | 0.02 | -0.03 | -0.15 |
|  | Ant 1, 16degree | Error for Rect Ref | -1.15 | -0.34 | -0.17 | -0.09 | -0.01 | 0.25 | 0.60 |
|  |  | Error for Ellipse Ref | -0.01 | 0.01 | 0.03 | 0.04 | 0.03 | 0 | -0.18 |
| Ericsson |  | Bin index | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
|  | Ant 4, 16degree | Error for Rect Ref | -1,35 | -0,43 | 1,03 | 0,7 | 0,82 | 0,49 | 1,96 |
|  |  | Error for Ellipse Ref | 0,48 | -0,19 | 0,2 | 0,57 | 0,11 | 0,15 | 1,07 |
|  | Ant 1, 16degree | Error for Rect Ref | -0,66 | -0,311 | 0,04 | 0,05 | 0,09 | 0,23 | 0,63 |
|  |  | Error for Ellipse Ref | 0,41 | -0,3 | 0,1 | 0,17 | 0,06 | 0,02 | 0,06 |
| Nokia |  | Bin index | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
|  | Ant 1, 12degree | Error for Rect Ref | -1.47 | -0.12 | -0.33 | -0.15 | 0.06 | 0.28 | 0.90 |
|  |  | Error for Ellipse Ref | 1.16 | -0.10 | -0.26 | -0.12 | 0.04 | 0.13 | 0.62 |

In the following table, we summarized the std and also its potential distribution function.

|  | Standard deviation [dB] | Distribution |
| --- | --- | --- |
| Ellipse bin1 | 0.3323 |  |
| Ellipse bin2 | 0.3235 |  |
| Ellipse bin3 | 0.1603 |  |
| Ellipse bin4 | 0.1789 |  |
| Ellipse bin5 | 0.0823 |  |
| Ellipse bin6 | 0.0903 |  |
| Ellipse bin7 | 0.5027 |  |
| Rectangular bin1 | 0.3242 |  |
| Rectangular bin2 | 0.4268 |  |
| Rectangular bin3 | 0.6007 |  |
| Rectangular bin4 | 0.4350 |  |
| Rectangular bin5 | 0.4192 |  |
| Rectangular bin6 | 0.2701 |  |
| Rectangular bin7 | 0.4605 |  |
| Rectangular | 0.6537 | Normal distribution |
| Ellipse | 0.2995 | Normal distribution |
| Fullset including rectangular and ellipse | 0.5172 | Normal distribution |

If we use the all the EEIRP errors across the rectangular reference case and ellipse reference case in the above table, then the standard deviation should around 0.51dB. If we use the ellipse coverage as reference case, then standard deviation should be around 0.3dB.

Considering the all different potential coverage, we propose to consider the full set of EEIRP error with rectangular and ellipse to do the statistical analysis to draw the conclusion on standard deviation.

Proposal 1: propose to use the full set of EEIRP error to do the statistical analysis.

Proposal 2: propose to use the 0.5dB as std of EERIP error.

[More simulations would be provided before the RAN4#114bis meeting starts.]

These two errors have been added to the MU budget as follows:

Table 6.6.3-1. IAC MU value derivation for EEIRP measurement, FR1

| IAC MU value derivation for EEIRP measurement, FR1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UID | Uncertainty source | Uncertainty value (dB) |  |  | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui (dB) |  |  |  |  |  |
|  |  |  |  | 6.425<f≤7.075 GHz |  |  |  |  |  |  |  | 6.425<f≤7.075 GHz |  |
| Stage 2: BS measurement |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A1-1 | Positioning misalignment between the BS and the reference antenna |  |  | 0.03 | Rectangular | 1.73 | 1 |  |  |  |  | 0.02 |  |
| A1-2 | Pointing misalignment between the BS and the receiving antenna |  |  | 0.3 | Rectangular | 1.73 | 1 |  |  |  |  | 0.17 |  |
| A1-3 | Quality of quiet zone |  |  | 0.1 | Gaussian | 1.00 | 1 |  |  |  |  | 0.10 |  |
| A1-4a | Polarization mismatch between the BS and the receiving antenna |  |  | 0.01 | Rectangular | 1.73 | 1 |  |  |  |  | 0.01 |  |
| A1-5a | Mutual coupling between the BS and the receiving antenna |  |  | 0 | Rectangular | 1.73 | 1 |  |  |  |  | 0.00 |  |
| A1-6 | Phase curvature |  |  | 0.05 | Gaussian | 1.00 | 1 |  |  |  |  | 0.05 |  |
| C1-1 | Uncertainty of the RF power measurement equipment (e.g. spectrum analyzer, power meter) |  |  | 0.26 | Gaussian | 1.00 | 1 |  |  |  |  | 0.26 |  |
| A1-7 | Impedance mismatch in the receiving chain |  |  | 0.33 | U-shaped | 1.41 | 1 |  |  |  |  | 0.23 |  |
| A1-8 | Random uncertainty |  |  | 0.1 | Rectangular | 1.73 | 1 |  |  |  |  | 0.06 |  |
| A2-21 | Measurement system dynamic range uncertainty |  |  | 0.5 | Rectangular | 1.73 | 1 |  |  |  |  | 0.29 |  |
| *new | [EEIRP sampling error] |  |  | 0.5 | Guassian | 1 | 1 |  |  |  |  | 0.5 |  |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A1-9 | Impedance mismatch between the receiving antenna and the network analyzer |  |  | 0.05 | U-shaped | 1.41 | 1 |  |  |  |  | 0.04 |  |
| A1-10 | Positioning and pointing misalignment between the reference antenna and the receiving antenna |  |  | 0.01 | Rectangular | 1.73 | 1 |  |  |  |  | 0.01 |  |
| A1-11 | Impedance mismatch between the reference antenna and the network analyzer |  |  | 0.05 | U-shaped | 1.41 | 1 |  |  |  |  | 0.04 |  |
| A1-3 | Quality of quiet zone (normal test conditions) |  |  | 0.10 | Gaussian | 1.00 | 1 |  |  |  |  | 0.10 |  |
| A1-4b | Polarization mismatch between the reference antenna and the receiving antenna |  |  | 0.01 | Rectangular | 1.73 | 1 |  |  |  |  | 0.01 |  |
| A1-5b | Mutual coupling between the reference antenna and the receiving antenna |  |  | 0.00 | Rectangular | 1.73 | 1 |  |  |  |  | 0.00 |  |
| A1-6 | Phase curvature |  |  | 0.05 | Gaussian | 1.00 | 1 |  |  |  |  | 0.05 |  |
| C1-3 | Uncertainty of the network analyzer |  |  | 0.20 | Gaussian | 1.00 | 1 |  |  |  |  | 0.20 |  |
| A1-12 | Influence of the reference antenna feed cable |  |  | 0.05 | Rectangular | 1.73 | 1 |  |  |  |  | 0.03 |  |
| A1-13 | Reference antenna feed cable loss measurement uncertainty |  |  | 0.06 | Gaussian | 1.00 | 1 |  |  |  |  | 0.06 |  |
| A1-14 | Influence of the receiving antenna feed cable |  |  | 0.05 | Rectangular | 1.73 | 1 |  |  |  |  | 0.03 |  |
| C1-4 | Uncertainty of the absolute gain of the reference antenna |  |  | 0.43 | Rectangular | 1.73 | 1 |  |  |  |  | 0.25 |  |
| A1-15 | Uncertainty of the absolute gain of the receiving antenna |  |  | 0.00 | Rectangular | 1.73 | 1 |  |  |  |  | 0.00 |  |
| Combined standard uncertainty (1σ) (dB) |  |  |  |  |  |  |  |  |  |  |  |  | 0.79 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) (dB) |  |  |  |  |  |  |  |  |  |  |  |  | 1.55 |

| CATR MU value derivation for EEIRP measurement, FR1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UID | Uncertainty source | Uncertainty value (dB) |  |  | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui (dB) |  |  |  |  |  |
|  |  |  |  | 6.425<f≤7.075 GHz |  |  |  |  |  |  |  | 6.425<f≤7.075 GHz |  |
| Stage 2: BS measurement |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A2-18a | Misalignment and pointing error of BS (for TRP) |  |  | 0.30 | Rectangular | 1.73 | 1 |  |  |  |  | 0.173 |  |
| C1-1 | Uncertainty of the RF power measurement equipment (e.g. spectrum analyzer, power meter) |  |  | 0.26 | Gaussian | 1.00 | 1 |  |  |  |  | 0.260 |  |
| A2-2a | Standing wave between BS and test range antenna |  |  | 0.21 | U-shaped | 1.41 | 1 |  |  |  |  | 0.148 |  |
| A2-3 | RF leakage (SGH connector terminated & test range antenna connector cable terminated) |  |  | 0.00 | Gaussian | 1.00 | 1 |  |  |  |  | 0.001 |  |
| A2-4a | QZ ripple experienced by BS |  |  | 0.09 | Gaussian | 1.00 | 1 |  |  |  |  | 0.093 |  |
| A2-12 | Frequency flatness of test system |  |  | 0.25 | Gaussian | 1.00 | 1 |  |  |  |  | 0.250 |  |
| A2-17 | Measurement system dynamic range uncertainty |  |  | 0.5 | Rectangular | 1.73 | 1 |  |  |  |  | 0.29 |  |
| *new | [EEIRP sampling error] |  |  | 0.5 | Guassian | 1 | 1 |  |  |  |  | 0.5 |  |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |  |  |  |
| C1-3 | Uncertainty of the network analyzer |  |  | 0.20 | Gaussian | 1.00 | 1 |  |  |  |  | 0.20 |  |
| A2-5a | Mismatch of receiver chain between receiving antenna and measurement receiver |  |  | 0.33 | U-shaped | 1.41 | 1 |  |  |  |  | 0.23 |  |
| A2-6 | Insertion loss of receiver chain |  |  | 0.18 | Rectangular | 1.73 | 1 |  |  |  |  | 0.10 |  |
| A2-3 | RF leakage (SGH connector terminated & test range antenna connector cable terminated) |  |  | 0.00 | Gaussian | 1.00 | 1 |  |  |  |  | 0.00 |  |
| A2-7 | Influence of the calibration antenna feed cable |  |  | 0.02 | U-shaped | 1.41 | 1 |  |  |  |  | 0.02 |  |
| C1-4 | Uncertainty of the absolute gain of the reference antenna |  |  | 0.43 | Rectangular | 1.73 | 1 |  |  |  |  | 0.25 |  |
| A2-8 | Misalignment positioning system |  |  | 0.00 | Exp. normal | 2.00 | 1 |  |  |  |  | 0.00 |  |
| A2-18b | Misalignment and pointing error of calibration antenna (for TRP) |  |  | 0.50 | Exp. normal | 2.00 | 1 |  |  |  |  | 0.25 |  |
| A2-9 | Rotary joints |  |  | 0.05 | U-shaped | 1.41 | 1 |  |  |  |  | 0.03 |  |
| A2-2b | Standing wave between calibration antenna and test range antenna |  |  | 0.09 | U-shaped | 1.41 | 1 |  |  |  |  | 0.06 |  |
| A2-4b | QZ ripple experienced by calibration antenna (normal test conditions) |  |  | 0.01 | Gaussian | 1.00 | 1 |  |  |  |  | 0.01 |  |
| A2-11 | Switching uncertainty |  |  | 0.26 | Rectangular | 1.73 | 1 |  |  |  |  | 0.15 |  |
| Combined standard uncertainty (1σ) (dB) |  |  |  |  |  |  |  |  |  |  |  |  | 0.88 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) (dB) |  |  |  |  |  |  |  |  |  |  |  |  | 1.73 |

| PWS MU value derivation for EEIRP measurement, FR1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UID | Uncertainty source | Uncertainty value (dB) |  |  | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui (dB) |  |  |  |  |  |
|  |  |  |  | 6.425<f≤7.075 GHz |  |  |  |  |  |  |  | 6.425<f≤7.075 GHz |  |
| Stage 2: BS measurement |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A7-1a | Misalignment and pointing error of BS |  |  | 0.10 | Rectangular | 1.73 | 1 |  |  |  |  | 0.06 |  |
| C1-1 | Uncertainty of the RF power measurement equipment (e.g. spectrum analyzer, power meter) |  |  | 0.26 | Gaussian | 1.00 | 1 |  |  |  |  | 0.26 |  |
| A7-2a | Longitudinal position uncertainty (i.e. standing wave and imperfect field synthesis) for BS antenna |  |  | 0.20 | Rectangular | 1.73 | 1 |  |  |  |  | 0.12 |  |
| A7-3 | RF leakage (calibration antenna connector terminated) |  |  | 0.09 | Gaussian | 1.00 | 1 |  |  |  |  | 0.09 |  |
| A7-4a | QZ ripple experienced by BS |  |  | 0.77 | Rectangular | 1.73 | 1 |  |  |  |  | 0.44 |  |
| A7-5 | Miscellaneous uncertainty |  |  | 0.00 | Gaussian | 1.00 | 1 |  |  |  |  | 0.00 |  |
| A7-14 | System non-linearity |  |  | 0.22 | Rectangular | 1.73 | 1 |  |  |  |  | 0.13 |  |
| A7-13 | Frequency flatness of test system |  |  | 0.13 | Rectangular | 1.73 | 1 |  |  |  |  | 0.08 |  |
| A2-21 | Measurement system dynamic range uncertainty |  |  | 0.50 | Rectangular | 1.73 | 1 |  |  |  |  | 0.29 |  |
| *new | [EEIRP sampling error] |  |  | 0.5 | Guasian | 1 | 1 |  |  |  |  | 0.5 |  |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |  |  |  |
| C1-3 | Uncertainty of network analyzer |  |  | 0.20 | Gaussian | 1.00 | 1 |  |  |  |  | 0.20 |  |
| A7-6 | Mismatch (i.e. reference antenna, network analyser and reference cable) |  |  | 0.45 | U-shaped | 1.41 | 1 |  |  |  |  | 0.32 |  |
| A7-7 | Insertion loss of receiver chain |  |  | 0.18 | Rectangular | 1.73 | 1 |  |  |  |  | 0.10 |  |
| A7-3 | RF leakage (calibration antenna connector terminated) |  |  | 0.09 | Gaussian | 1.00 | 1 |  |  |  |  | 0.09 |  |
| A7-8 | Influence of the calibration antenna feed cable |  |  | 0.10 | Rectangular | 1.73 | 1 |  |  |  |  | 0.06 |  |
| C1-4 | Uncertainty of the absolute gain of the reference antenna |  |  | 0.43 | Rectangular | 1.73 | 1 |  |  |  |  | 0.25 |  |
| A7-9 | Misalignment of positioning system |  |  | 0.00 | Exp. normal | 2.00 | 1 |  |  |  |  | 0.00 |  |
| A7-1b | Misalignment and pointing error of calibration antenna |  |  | 0.05 | Rectangular | 1.73 | 1 |  |  |  |  | 0.03 |  |
| A7-10 | Rotary joints |  |  | 0.00 | U-shaped | 1.73 | 1 |  |  |  |  | 0.00 |  |
| A7-2b | Longitudinal position uncertainty (i.e. standing wave and imperfect field synthesis) for calibration antenna |  |  | 0.18 | Rectangular | 1.73 | 1 |  |  |  |  | 0.10 |  |
| A7-4b | QZ ripple experienced by calibration antenna |  |  | 0.20 | Rectangular | 1.73 | 1 |  |  |  |  | 0.12 |  |
| A7-11 | Switching uncertainty |  |  | 0.02 | Rectangular | 1.73 | 1 |  |  |  |  | 0.01 |  |
| A7-12 | Field repeatability |  |  | 0.20 | Gaussian | 1.00 | 1 |  |  |  |  | 0.20 |  |
| Combined standard uncertainty (1σ) (dB) |  |  |  |  |  |  |  |  |  |  |  |  | 0.97 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) (dB) |  |  |  |  |  |  |  |  |  |  |  |  | 1.9 |

Table 1. the summarized table for MU value under the different OTA chamber

| OTA test method | MU [dB] |
| --- | --- |
| IAC | 1.55 |
| CATR | 1.73 |
| PWS | 1.9 |

It could be found that MU for PWS is higher than other two chambers due to the increasing uncertainty for A7-4a, A7-14, A7-6, A7-2b, A7-12 compared with the legacy values as captured in TR 37.941. From our understanding, the increasing MU value could force the BS vendors to have further power backoff to meet the corresponding EEIRP mask requirement which is not desirable from the coverage perspective.

Proposal 3: propose to use the MU value as 1.7dB for conformance testing of EEIRP mask.

## 2.2. Angular transformation for different mechanical tilt

During the last RAN4 meeting, there were some updates as following for the consideration of the change/shift of measured EEIRP profile under the different mechanical tilt.

n

Figure 2.2.1. different coordinate system used in RAN4.

For BS type 1-H and BS type 1-O operating in band n104, the Expected EIRP (EEIRP) in the frequency range 6425 – 7075 MHz, the power measured EEIRP shall not exceed the limits specified in table 6.9.5.1-1 for mechanical down-tilt angle equal to zero. [If non-zero mechanical down-tilt is considered, the measured EEIRP profile needs to be shifted as:

θH=−θ+θMT

, where

θH – angle with respect to the horizon defined between +90° and -90°, above the horizon is positive below the horizon is negative. Note the orientation is opposite to 

 is the angle between the projection of the vector in the x/y plane and the radiating vector and is defined between -90° and +90°.

MT is the down-tilt angle representing the mechanical tilt defined between -90° and +90°. ]

Indeed there are two different interpretations at least on this shift from our understanding according to companies’ feedback:

- Just the coordinate shift where to comply with EEIRP mask in the elevation angle under the different mechanical downtilt.

- The directional range shift due to the mechanical downtilt for measured/sampled EIRP under the OTA chamber without any mechanical downtilt to calculate the EEIRP to comply with the mask above the horizon as requested by WRC-23.

- Our understanding should be the 2nd interpretation to allow the OTA testing lab to how to do this kind of directional range shift to collect the measured EI	RP in the certain bin(s) to calculate the EEIRP mask.

- Proposal 4: propose to consider the following interpretation for the angular shift:

- The directional range shift due to the mechanical downtilt for measured/sampled EIRP under the OTA chamber without any mechanical downtilt to calculate the EEIRP to comply with the mask above the horizon as requested by WRC-23.

- Indeed this kind of the angular shift under the different coordinate system has already been defined in TR 38.901 as following. In this contribution, we also reused the same methodology with some angular update considering the coordinate system defined in TS38.141-2 and WRC-23 coordinate system.

This transformation relates the spherical angles (,) in the global coordinate system to spherical angles (,) in the local (antenna-fixed) coordinate system and is defined as follows:

(7.1-18)

(7.1-19)

where  is the mechanical tilt angle around the y-axis as defined in Figure 7.1.4. Note that the equations (7.1-7), (7.1-8) reduce to equations (7.1-18), (7.1-19) if both  and  are zero.

Figure 7.1.4: Definition of angles and unit vectors when the LCS has been rotated an angle  around the y-axis of the GCS

For example (using the follow legacy coordinate system)

- Global coordinate: θ=90°,φ=30°

- Mechanical downtilt: β=15°

If considering the spatial EIRP sampling range above the horizon without any mechanical downtilt within OTA chamber, then it is defined as following:

- θ1 is ranging from 0 to 90 degree

- φ1 is ranging from -180 degree to 180 degree

However using the above mechanical angular transformation equation as defined above, then we could get the following spatial EIRP sampling range above the horizon.

- θ1_m is ranging from θ1_m1 to θ1_m2 degree

- φ1_m is ranging from φ1_m1 degree to φ1_m2 degree

Considering the following relationship between three coordinate system as above

θ1=θ3+90                                                                 (1)

θ2=−θ3                                                                      (2)

θ1=−θ2+90 										                             (3)

φ1=φ2=φ3 										                                 (4)

Then according to the angular transformation as defined in (7.1-18 and 7.1-19) and equation (1),  the measured EIRP profile needs to be shifted as

In addition, according to the equation in (3), then we could get the following transformation between 2nd and 3rd coordinate system

Note 1: for the different mechanical downtilt, the measured spatial EIRP within the whole radiation range would be changed.

Note 2:BS is supposed to be installed in OTA chamber without any mechanical tilting.

Note 3: EEIRP results for different mechanical downtilt is calculated under the assumption of different spatial EIRP sampling. Some spatial interpolation might be needed for spatial EIRP measurement without any mechanical downtilt for spatial EIRP sampling under the different mechanical downtit.

In short, for angular transformation between the coordinate system defined in TS38.141-2 and WRC-23 coordinate system, it could be defined as following:

Proposal 5: propose to consider the following equation for angular shift with the mechanical tilt considered

Conclusions

In this contribution, we shared some further views on the EIRP mask requirement for band n104 and observations / proposals are made as following:

Proposal 1: propose to use the full set of EEIRP error to do the statistical analysis.

Proposal 2: propose to use the 0.5dB as std of EERIP error.

Proposal 3: propose to use the MU value as 1.73dB for conformance testing of EEIRP mask.

- Proposal 4: propose to consider the following interpretation for the angular shift:

- The directional range shift due to the mechanical downtilt for measured/sampled EIRP under the OTA chamber without any mechanical downtilt to calculate the EEIRP to comply with the mask above the horizon as requested by WRC-23.

Proposal 5: propose to consider the following equation for angular shift with the mechanical tilt considered

References

- R4-2419207	TR 38.908 v1.2.0 Protection of fixed satellite service (FSS) UL within 6425 to 7125 MHz	ZTE Corporation, Sanechips

- R4-2419817	Draft CR for TS 38.141-2: Declarations	CATT

- R4-2419818	TP to TR38.908: clause 3 and 5	ZTE Corporation, Sanechips

- R4-2419819	TP to TR38.908: Clause 6.4 spatial sampling grid	ZTE Corporation, Sanechips

- R4-2419820	Draft CR to 38.141-2 –OTA spatial emissions	Huawei, HiSilicon

- R4-2419821	TP to TR 38.908 – section 6.5	Huawei, HiSilicon

- R4-2419822	TP to TR 38.908 – section 6.6	Huawei, HiSilicon

- R4-2419823	Draft CR to TS 38.104 for introduction of expected EIRP mask core requirement	Nokia, ZTE, Ericsson, Huawei, Samsung, Qualcomm, CATT, Spark NZ

- R4-2419824	TP to TR 38.908 clause 6 – Conformance testing	Nokia

- R4-2419825	Draft CR to TS 38.141-2 with Annex N Expected EIRP calculation	Nokia

- R4-2419826 	Discussion on conformance test aspects for EEIRP and TP to TR 38.908	Ericsson

- R4-2419827	Way Forward for [113][302] NR_BS_RF_Part1_E_EIRP	ZTE

- R4-2419911	Draft Big CR to 38.141-2 Introduction of U6GHz BS EEIRP mask	ZTE

- R4-2502309	Way Forward for [114][302] NR_BS_RF_Part1_E_EIRP, approved.

- R4-2504739  Way Forward for [114bis][302] NR_BS_RF_Part1_E_EIRP, ZTE, approved.