3GPP TSG-RAN WG4 Meeting#115                                R4-2507516

Malta, MT, 19th – 23th May, 2025

Agenda item:	7.13.4.2.1

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on the OTA testing scope reduction

Document for:	Approval

Introduction

In the RAN#103 meeting, new WID for Rel-19 BS RF evolution has been approved and it’s expected to start the normative work from this April meeting. For the OTA testing scope reduction, we would like to share the further views based on the latest agreement in [4].

Issue 1-1-1: test scope reduction for Tx requirement - Agreement: - Further discuss how to reduce the test simplification for OBUE limits. - Option 1: 1st frequency offset - Option 2: 3rd frequency offset to check OBUE limits - Further discuss how to reduce the test simplification for spurious limits. - Option 1: 3rd frequency offset - Option 2: For bands with operating band < 100 MHz to use 1st interferer offset and for bands with operating band >= 100MHz to use 3rd interferer offset. Issue 1-1-2: Reduction of test configuration for Tx requirement - Agreement: - Proposal 1: For the reduction of test configurations for TX IM requirement, regardless of the test configuration selected (ATCRx or ANTCRx), the condition of maximum output power – Rated total output power (Prated,t,AC, or Prated,t,TABC) – should be met. Issue 1-2-1: Out of band blocking - Agreement on step size of OOBB requirement for further discussion: - Option 1: 1/3 of the minimum supported bandwidth - Option 2: Option 2: Use 1% of the momentary frequency as the step size for OTA receiver out-of-band blocking testing for frequencies up to 6000 MHz. Use a fixed step size of 60 MHz for frequencies higher than 6000 MHz

Discussion

- 2.1. Tx requirements

For BS RF requirement, there are two most time consuming requirement e.g. transmitter spurious emission requirement and and receiver OOBB requirement because its applicable frequency range is extremely large compared with other requirements.

For the 3rd party OTA verification chamber, it usually take 300 Euro per hour and maximum 10 working hours per day, therefore to ensure the acceptable test coverage with reasonable testing time is extremely important for vendors from both implementation cost and also development Iteration. As pointed out in [2], it has been found that the test time is prohibitive taking up to 3 times the time to perform the conformance testing compared to conducted conformance testing. Excessive OTA testing time has become a constraint for AAS product design, development and certification .

In the contribution [2], the TX IMD texting is identified to comprise of 96 test cases testing output power, ACLR and out of band emissions (all TRP requirements) with 6 interferer offsets are tested for each (12 for NC operation), we propose to focus on the testing scope reduction for OTA Tx IMD requirement as indicated in the following test cases.

Table 6.8.5.1-1 in 38.141-2: Interfering and wanted signals for the OTA transmitter intermodulation requirement

| Parameter | Value |
| --- | --- |
| Wanted signal | NR single or multi-carrier, or multiple intra-band contiguously or non-contiguously aggregated carriers |
| Interfering signal type | NR signal, the minimum BS channel bandwidth (BWChannel) with 15 kHz SCS of the band defined in clause 5.3.5 of TS 38.104 [2] |
| Interfering signal power level | min(46 dBm, Prated,t,TRP) |
| Interfering signal centre frequency offset from the lower (upper) edge of the wanted signal or edge of sub-block inside a gap | , for n=1, 2 and 3 |
| NOTE 1: Interfering signal positions that are partially or completely outside of any downlink operating band of the BS are excluded from the requirement, unless the interfering signal positions fall within the frequency range of adjacent downlink operating bands in the same geographical area. NOTE 2: In Japan, note 1 is not applied in Band n77, n78, n79. NOTE 3: For BS type 1-O supporting dual polarization, the interfering signal power shall be equally divided between supported polarizations at the CLTA interfaces. |  |

During the last RAN4 meeting, there were some initial discussions on the candidate frequency offset for Tx IMD requirement as worst case.

f1

f2

2f1-f2

BW=b

interfering signal

2f2-f1

BW=a

wanted signal

(a+b)/2

2a+b

2b+a

a + b

Figure 1. the generic model using for the analysis of 3rd intermodulation mechanism

f1

f2

2f1-f2

BW=5

2f2-f1

BW=a

wanted signal

10 MHz

2a+5 (MHz)

a+10 (MHz)

10 MHz

Figure 2. the 3rd intermodulation product for 5 MHz interfering signal in the 1st adjacent channel

For ACLR requirement, based on the initial understanding, it seems that1st interference signal freq offset should be the worst case for compliance testing.

For spurious emission requirement, the 3rd interference signal frequency offset seems to be the worst case for compliance testing.

For OBUE requirement, some initial simulation results without considering any analog filtering attenuation are provided as following with wanted signal as 40MHz, SCS 30kHz together with 3 different freq offset for interfering signal (40MHz, 80MHz, 120MHz). It could be easily found that the 3rd interfering signal will create the more IMD interference away from the wanted signal compared with other interfering signal and it is also well known that in-band spurious emission requirement will be more stringent than emissions close to the wanted signal. Based on this observation, we propose to use the 3rd interfering signal as the worst case for the conformance testing.

Figure 3. simulation results for TxM considering both wanted signal and interfering signal with different freq offset

During the last RAN4 meeting, for the spurious emission requirement, there were some proposals discussing how to fit the interference signal for different operating bands due to ΔfOBUE requirement. As shown in the following figure, for operating bands wider than 35MHz (figure 4a with 7x5MHz=35MHz), given the testing signal for both wanted and interfering signal according to the Annex and following figures, the 3rd interfering signal configuration should be the worst case without any impacts from the front-end analog filtering.

Figure 4a. the illustration of conformance testing configuration for single carrier or single band operation [Narrow bands]

Figure 4b. simulation results for TxM considering both wanted signal and interfering signal with different freq offset  [Wide bands]

Figure 5. the illustration of conformance testing configuration for multi-band operation [Narrow bands]

If the 3rd interfering signal is falling higher than FDL_high+ ΔfOBUE or lower than FDL_low-ΔfOBUE, the worst emission will falling into the spurious emission region, then the 2nd and 1st interfering signal should be used as the worst case for the conformance testing. Due to the roll-off impacts of front-end filtering within the ΔfOBUE, it will be difficult to identify which interference signal will be the worst case, therefore we propose to test both 2nd and 1st interfering signal if 3rd interfering signal is falling higher than FDL_high+ ΔfOBUE or lower than FDL_low-ΔfOBUE.

Proposals 1: for the conformance testing of TxM for OBUE requirement, propose to use the 3rd interfering signal as the worst case if 3rd interfering signal is falling within the range of FDL_low to FDL_high of the operating bands.

Proposals 2: for the conformance testing of TxM for OBUE requirement, propose to use the 2nd and 1st interfering signal as the worst case if 3rd interfering signal is falling higher than FDL_high+ ΔfOBUE or lower than FDL_low-ΔfOBUE.

Proposals 3: for the conformance testing of TxM for spurious requirement, propose to use the 3rd interfering signal as the worst case regardless of operating bands.

- 2.2. RX out of band blocking

It is well known that RX out of blocking requirement is another most time consuming requirement from receiver perspective in addition to transmitter spurious emission requirement. We also have similar discussion for FR2 BS OOBB requirement in terms of step size for CW signal. The corresponding technical justification is captured in TR 38.817-02. For FR1 BS OOBB requirement testing, the same principle could be reused to reduce the amount of time used for the time-consuming OTA conformance testing, while continue to ensure that any spurious response within the receive bandwidth will still be verified with sufficient granularity.

Extracted from TR38.817-02 The base station out-of-band blocking requirement is tested with a CW interferer and the wanted uplink signal. A spurious response occurs when the m-th harmonic of the interferer frequency mixes with the n-th harmonic of an internal signal, like a clock or a local oscillator signal, and this mixing product falls within the receive bandwidth of the wanted signal (or near the receive bandwidth, if the receiver selectivity is insufficient to suppress the spurious mixing product). The frequency of the unwanted mixing product equals ±m·fCW±n·finternal, where fCW is the interferer frequency and finternal is the frequency of the internal signal involved in the mixing product. m can assume the values 1, 2, 3, … and n can assume the values 0, 1, 2, …. Spurious responses for which the harmonic number m of the interferer signal equals 4 or more are usually not significant. Hence, the frequency step size should be chosen such that any spurious response due to a maximum third harmonic of the interferer will be captured within the receive bandwidth of the wanted signal. To reduce the amount of time used for the time-consuming OTA conformance testing, while continue to ensure that any spurious response within the receive bandwidth will still be verified with sufficient granularity, the measurement step size for interfering signal at frequency above 6 GHz can be set according to the minimum supported BS channel bandwidth as shown in Table 10.6.2-1 in the NR BS type 2-O OTA out-of-band blocking conformance testing. For interfering signal at frequency below 6 GHz, the traditionally measurement step size of 1 MHz can be kept. Table 10.6.2-1: Measurement step size for out-of-band blocking conformance testing Minimum supported BS Channel bandwidth of wanted signal [MHz]; 50; 100; 200; 400. Measurement step size for interfering signal [MHz]; 15; 30; 60; 60.

As shown in the following figure 6, if we want to ensure the ±3·fCW±n·finternal falling within receiver bandwidth, the step size of CW signal should be less than 1/3 of BW.

Figure 6. the illustration of relationship between step size for CW signal and receiver bandwidth for testing

Proposals 4: propose to reuse the principle design for Rel-15 FR2 BS OOBB CW step size e.g. 1/3 of BW as captured in TR 38.817-02.

Conclusions

In this contribution, we shared some initial views on OTA test scope cases in RAN4 and proposals are made as following:

Proposals 1: for the conformance testing of TxM for OBUE requirement, propose to use the 3rd interfering signal as the worst case if 3rd interfering signal is falling within the range of FDL_low to FDL_high of the operating bands.

Proposals 2: for the conformance testing of TxM for OBUE requirement, propose to use the 2nd and 1st interfering signal as the worst case if 3rd interfering signal is falling higher than FDL_high+ ΔfOBUE or lower than FDL_low-ΔfOBUE.

Proposals 3: for the conformance testing of TxM for spurious requirement, propose to use the 3rd interfering signal as the worst case regardless of operating bands.

Proposals 4: propose to reuse the principle design for Rel-15 FR2 BS OOBB CW step size e.g. 1/3 of BW as captured in TR 38.817-02.

References

- RP-240829,New WID: WI resulting from discussion on BS RF requirement evolution, Approved.

- RP-240430,Views on BS RF topics for Rel-19, Huawei, approved.

- R4-2419827	Way Forward for [113][302] NR_BS_RF_Part1_E_EIRP
[4] R4-2419828	Way Forward for [113][304] NR_BS_RF_Part3_OTA_TRP

- R4-2419243	Test case reduction for TX IM, Huawei

- ] R4-1709451   Conducted transmit intermodulation for NR BS range 1, ZTE

- R4-2504740	Way Forward for [114bis][304] NR_BS_RF_Part3_OTA_TRP, ZTE, approved.

Annex: conformance testing for TxM

Table 4.11.1A-1: Signal to be used to build NR TCs

| Operating Band characteristics |  | FDL_high – FDL_low < 100 MHz | FDL_high – FDL_low ≥ 100 MHz |
| --- | --- | --- | --- |
| TC signal characteristics | BWchannel | 5 MHz (Note 1) | 20 MHz (Note 1) |
|  | Subcarrier spacing | Smallest supported subcarrier spacing |  |
| NOTE 1: If this channel bandwidth is not supported, the narrowest supported channel bandwidth shall be used. |  |  |  |

Table 6.7.1-1: Maximum offset of OBUE outside the downlink operating band

| BS type | Operating band characteristics | ΔfOBUE [MHz] |
| --- | --- | --- |
| OTA AAS BS | FDL_high – FDL_low < 100 MHz | 10 |
|  | 100 MHz ≤ FDL_high – FDL_low ≤ 900 MHz | 40 |

RF channels to be tested for single carrier:

-	M; see clause 4.12.1.

Base Station RF Bandwidth positions to be tested for multi-carrier:

-	MRFBW in single-band RIB, see clause 4.12.1; BRFBW_T'RFBW and B'RFBW_TRFBW in multi-band RIB, see clause 4.12.1.

In addition, for multi-band RIB:

-	For BRFBW_T'RFBW, emission testing above the highest operating band may be omitted.

-	For B'RFBW_TRFBW, emission testing below the lowest operating band may be omitted.

Table 6.8.5.1.1-1: Interfering and wanted signals for the OTA transmitter intermodulation requirement

| Parameter | Value |
| --- | --- |
| Wanted signal type | E-UTRA or NR signal |
| Interfering signal type | E-UTRA signal of channel bandwidth 5 MHz |
| Interfering signal power level applied to the CLTA | min(46 dBm, Prated,t,TRP) |
| Interfering signal centre frequency offset from Base Station RF Bandwidth edge or edge of sub-block inside a gap | ±2.5 MHz ±7.5 MHz ±12.5 MHz |
| NOTE 1: Interfering signal positions that are partially or completely outside of any downlink operating band of the RIB is excluded from the requirement, unless the interfering signal positions fall within the frequency range of adjacent downlink operating bands in the same geographical area. In case that none of the interfering signal positions fall completely within the frequency range of the downlink operating band, TS 37.141 provides further guidance regarding appropriate test requirements. NOTE 2: In certain regions, NOTE 1 is not applied in Band 1, 3, 8, 9, 11, 18, 19, 21, 28, 32 operating within 1 475.9 MHz to 1 495.9 MHz, 34. NOTE 3: For OTA AAS BS with dual polarization, the interfering signal power shall be equally divided between the supported polarizations at the CLTA. |  |