3GPP TSG-RAN WG4 Meeting#115                               R4-2507519

Malta, MT, 19th – 23th May, 2025

Agenda item:	7.25.3.1

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on potentially new requirements for SBFD operation

Document for:	Approval

Introduction

In the RAN#102 meeting, new WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD) has been approved. It’s expected to start the RAN4 meeting from April meeting. Since we have already reached good progress on the SBFD BS RF impacts in SI phase and outcome was captured in TR 38.858. In this contribution, we would like to share some further views on these remaining open issues.

- Sub-topic 3-2: In-channel adjacent subband requirements - Issue 3-2-1: In-channel adjacent subband leakage ratio: General Agreement: - New requirement for In-channel adjacent subband leakage ratio: - In-channel adjacent subband leakage ratio requirement is defined with the existing ACLR requirement in absolute PSD limit value as baseline (i.e., Table 6.6.3.2-2 in TS 38.104 for FR1 and Table 9.7.3.3-2 for FR2 in TS 38.104) - FFS the value for absolute PSD limit - FFS the naming of this requirement, by considering the absolute PSD limit used instead of the ratio. Note that the current naming in TS 38.104 is ACLR absolute limit. - Issue 3-2-2: In-channel adjacent subband leakage ratio: Configuration Way Forward: - Further study the configuration of the in-channel adjacent subband leakage ratio requirement, by taking the following table as the baseline: BS DL subband bandwidth BWSBFD,DL (MHz); BS UL subband bandwidth BWSBFD,UL (UL); Filter on UL subband and corresponding filter bandwidth; In-Channel Adjacent Subband Leakage Ratio limit. BWSBFD,DL as configured by gNB; BWSBFD,UL as configured by gNB; Square (BWSBFD,UL); TBD. - Issue 3-2-4: In-channel adjacent subband blocking requirement Way Forward: - RAN4 continue to study In-channel adjacent subband blocking requirement, by following the agreement achieved in RAN4#113.

Discussion

## 2.1. Potential adjacent sub-band leakage

During the last RAN4 meeting, there were the agreement to define the in-band leakage requirement with some specific absolute limit e.g. CACLR limit as baseline. In the following section, we would like to share some further understanding on it and share some basic ground how these requirement were defined in the past.

Way forward: - Absolute limit for in-channel adjacent subband leakage: - FFS by considering the following exemplary cases by performing analysis including measurement or simulation: - FR1: - DU/UD: 80MHz (DL) – 20MHz (UL) - DUD: 40MHz (DL) – 20MHz (UL) – 40MHz (DL) - Other cases are not precluded - FR2-1: - DU/UD: 50MHz (DL) – 50MHz (UL); 100MHz (DL) – 100MHz (UL) - DUD: 75MHz (DL) – 50MHz (UL) – 75MHz (DL) - Other cases are not precluded - Note: The study on different configuration does not mean RAN4 will define separate requirements for different configuration. RAN4 still try to use a unified requirements for absolute limit.

To use the following example non-contiguous spectrum to clarify its background how this requirement was specified in the past. In order to comply with less stringent requirement between absolute and relative ACLR limit, the minimum Wgap was agreed to make sure the 1st carrier and 2nd carrier to be available to be measured, otherwise it will fail its purpose to verify the corresponding requirement. If Wgap is not too big (e.g. wider than 60MHz for ACLR1 conformance testing in FR1), then the impacts from other carrier leakage would be also negligible from the testing perspective.

Figure 2.1.1. the example CACLR requirement for 40+40MHz case in FR1

Figure 2.1.2. the example CACLR requirement for 100+200MHz case in FR2

Based on the above consideration and typical DUD (40,20,40) or DU(80,20) configuration in FR1, the existing CACLR requirement could be also applied for DUD case and DU case, however for FR2 with DUD as (100,100, 200MHz) case, it seems that CACLR requirement for 200MHz is challenging to be met due to the narrow UL sub-band size.

Figure 2.1.3. the example CACLR requirement for 40+40MHz case in FR1

Figure 2.1.4. the example CACLR requirement for 100+100+200MHz case in FR2

Based on the above consideration and typical DUD (40,20,40) or DU(80,20) configuration in FR1, the existing CACLR requirement could be also applied for DUD case and DU case, however for FR2 with DUD as (100,100, 200MHz) case, it seems that CACLR requirement for 200MHz is challenging to be met due to the narrow UL sub-band size.

| SBFD configuration |  | WA Cat B | MR | LA |
| --- | --- | --- | --- | --- |
|  | Baseline | -15dBm/MHz | -25dBm/MHz | -32dBm/MHz |
| DUD | 40+20+40MHz | -7.76dBm/MHz | -18.54dBm/MHz | -32.5dBm/MHz |
| DU | 80+20MHz | -13.5dBm/MHz | -22.2dBm/MHz | -36.2dBm/MHz |
| DU | 50+50MHz | -15.09dBm/MHz | -23.8dBm/MHz | -37.8dBm/MHz |
| DU | 60+40MHz | -15.02dBm/MHz | -23.82dBm/MHz | -37.82dBm/MHz |
| DU | 70+30MHz | -14.12dBm/MHz | -22.90dBm/MHz | -36.90dBm/MHz |
| DU | 90+10MHz | -10.81dBm/MHz | -19.16dBm/MHz | -33.16dBm/MHz |

- Proposal 1: for FR1 local SBFD BS, no relaxation compared with absolute ACLR limit (DU configuration) or absolute CACLR limit (DUD) are needed;

- Proposal 2: for FR1 WA and MR SBFD BS, up to 7.4dB relaxation could be allowed.

## 2.2. Potential adjacent receiver requirements

During the last RAN4 meeting, there were some discussions on the necessity of in-channel adjacent sub-band blocking, it was agreed to define this requirement, however the exact value for adjacent sub-band leakage requirement is not confirmed yet. From our understanding, the DL sub-band signal from the aggressor BSs within the single carrier is orthogonal to the UL sub-band signal reception. In other words, there are no such kind of selectivity needed at all. However based on some comments received online, this DL sub-band interference might cause some nonlinearity to LNA, other RF component similar as in-band blocking signal. Based on our initial simulation results as shown in the following figures, IoT level increase due to BS2BS CLI problem in the DL sub-band region if SBFD BS is capable of receiving the whole carrier instead of UL sub-band only and without considering the BS CLI coordination within the carrier is larger than legacy 25dBc (e.g. 16dBc Iot+9dB for 16QAM demod) for FR1 ICS requirement and also 14dBc (e.g. 5dBc Iot+9dB for 16QAM demod) for FR2 ICS requirement except for FR2 WA scenario; However if so, the BS2BS CLI power level is still expected to lower than in-band blocking power level according to the following evaluation results. The absolute power for in-band blocking level could be provided later on.

FR1:

Without any CLI coordination

With some CLI coordination

Observation 1: BS co-channel coordination .e.g. beam nulling or beam coordination could alleviate the in-band blocking level.

Observation 2:

|  | Without CLI | With CLI coordination | In-band blocking |
| --- | --- | --- | --- |
| FR1 WA SBFD BS | -51.28dBm | -63.8dBm | -43 |
| FR1 MR SBFD BS | -51.656dBm | N/A | -38 |
| FR1 LA SBFD BS | -45.24dBm | N/A | -35dBm |

Proposal 3: for the feasibility of in-channel blocking requirement, analog filter could be assumed for FR1 WA SBFD and FR1 MR BS and no analog filter could be assumed for FR1 LA SBFD and FR2 SBFD BS.

Conclusions

In this contribution, we want to share some further views on RF requirements for SBFD BS and proposals/observations are made as following:

- Proposal 1: for FR1 local SBFD BS, no relaxation compared with absolute ACLR limit (DU configuration) or absolute CACLR limit (DUD) are needed;

- Proposal 2: for FR1 WA and MR SBFD BS, up to 7.4dB relaxation could be allowed.

Observation 1: BS co-channel coordination .e.g. beam nulling or beam coordination could alleviate the in-band blocking level.

Observation 2: in-channel blocking requirement for FR1 SBFD BS are summarize as following:

|  | Without CLI | With CLI coordination | In-band blocking |
| --- | --- | --- | --- |
| FR1 WA SBFD BS | -51.28dBm | -63.8dBm | -43 |
| FR1 MR SBFD BS | -51.656dBm | N/A | -38 |
| FR1 LA SBFD BS | -45.24dBm | N/A | -35dBm |

Proposal 3: for the feasibility of in-channel blocking requirement, analog filter could be assumed for FR1 WA SBFD and FR1 MR BS and no analog filter could be assumed for FR1 LA SBFD and FR2 SBFD BS.

References

- RP-234035,New WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei, Samsung, Approved.

- TR 38.858

- R4-2321091, TP to TR 38.858 Impact on BS RF requirements, ZTE, Agreed

- R4-2413514, Way Forward for [112][307] NR_duplex_evo_General, Samsung

- R4-2416584,Way forward for [112bis][306] NR_duplex_evo_General, Samsung

- R4-2502310	Way Forward for [114][306] NR_duplex_evo_General, approved.

- R4-2502380	Way Forward for [114][307] NR_duplex_evo_BSRF, approved.

Annex

Figure 1. FR1 WA SFBD BS with 40+20+40MHz configuration [-7.76dBm/MHz]

Figure 2. FR1 MR SFBD BS with 40+20+40MHz configuration [-18.54dBm/MHz]

Figure 3. FR1 LA SFBD BS with 40+20+40MHz configurationm [-32.5dBm/MHz]

Figure 3. FR1 WA SFBD BS with 80+20MHz configuration [-13.5dBm/MHz]

Figure 3. FR1 MR SFBD BS with 80+20MHz configuration [-22.2dBm/MHz]

Figure 3. FR1 LA SFBD BS with 80+20MHz configuration [-36.2dBm/MHz]

TP

12.2.9	In-channel adjacent subband leakage for SBFD

12.2.9.1 General

In-channel adjacent subband leakage (ICASL) is the filtered mean power centred on the declared frequency region of UL sub-band configuration within one NR carrier.

The requirement shall apply during the transmitter ON period.

12.2.9.2 Limits and Basic limits

For SBFD operation in the unpaired spectrum, the ICASL shall be less than or equal to the value specified in table 12.2.9.2-1.

Table 12.2.9.2-1: SBFD Base station ICASL absolute basic limit

| SBFD BS category / BS class | ICASL absolute basic limit |
| --- | --- |
| Category A Wide Area BS | TBD dBm/MHz |
| Category B Wide Area BS | TBD dBm/MHz |
| Medium Range BS | TBD dBm/MHz |
| Local Area BS | TBD dBm/MHz |

Table 12.2.9.2-2: Filter parameters for the UL sub-band

| RAT | Filter on the assigned UL sub-band and corresponding filter bandwidth |
| --- | --- |
| NR | NR of the closet BW with SCS that provides largest transmission bandwidth configuration compared with UL sub-band |

12.2.9.3 Minimum requirement for SBFD BS type 1-H

The ICASL requirements for SBFD BS type 1-H are that for each TAB connector TX min cell group and each applicable basic limit in clause 12.2.9.2, the power summation emissions at the TAB connectors of the TAB connector TX min cell group shall not exceed a BS limit specified as the basic limit + X, where X = 10log10(NTXU,countedpercell).

NOTE:	Conformance to the SBFD BS type 1-H spurious emission requirement can be demonstrated by meeting at least one of the following criteria as determined by the manufacturer:

1)	The sum of the in-channel adjacent sub-band leakage power measured on each TAB connector in the TAB connector TX min cell group shall be less than or equal to the limit as defined in this clause for the respective frequency span.

Or

2)	The  in-channel adjacent sub-band leakage power at each TAB connector shall be less than or equal to the SBFD BS type 1-H limit as defined in this clause for the respective frequency span, scaled by -10log10(n), where n is the number of TAB connectors in the TAB connector TX min cell group.

12.5.10	OTA In-channel adjacent subband leakage for SBFD

12.5.10.1 General

OTA In-channel adjacent subband leakage (ICASL) is the filtered mean power centred on the declared frequency region of UL sub-band configuration within one NR carrier. The measured power is TRP.

The requirement shall be applied per RIB during the transmitter ON period.

12.5.10.2 Minimum requirement for SBFD BS type 1-O

For SBFD BS type 1-O, the ICASL shall less than or equal to a basic limit defined in table 12.2.9.2-1 + X (where X = 9 dB).

12.5.10.3 Minimum requirement for SBFD BS type 2-O

For SBFD BS type 2-O, the ICASL shall be less than or equal to the value specified in table 12.5.10.3-1.

Table 12.5.10.3-1: SBFD Base station ICASL absolute basic limit

| SBFD BS category / BS class | ICASL absolute basic limit |
| --- | --- |
| Wide Area BS | TBD dBm/MHz |
| Medium Range BS | TBD dBm/MHz |
| Local Area BS | TBD dBm/MHz |

Table 12.5.10.3-2: Filter parameters for the UL sub-band

| RAT | Filter on the assigned UL sub-band and corresponding filter bandwidth |
| --- | --- |
| NR | NR of the closet BW with SCS that provides largest transmission bandwidth configuration compared with UL sub-band |