3GPP TSG-RAN WG4 Meeting#115                                R4-2507520

Malta, MT, 19th – 23th May, 2025

Agenda item:	7.25.3.2

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on modification of existing Tx requirements for FR1 and FR2-1 for SBFD BS

Document for:	Approval

Introduction

In the RAN#102 meeting, new WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD) has been approved. It’s expected to start the RAN4 meeting from April meeting. Since we have already reached good progress on the SBFD BS RF impacts in SI phase and outcome was captured in TR 38.858. In this contribution, we would like to share some further views on these remaining open issues.

Discussion

## 2.1. Tx requirements

### 2.1.1. Additional OBUE requirement

Issue 1-1: Deployment scenarios for FR1 SBFD macro BS - Way forward - Encourage companies to provide more inputs on whether SBFD-SBFD co-ex case should be in Rel-19 scope. - If it is in the scope of Rel-19, companies are also encouraged to provide input on whether/how to derive the additional requirements. FFS aspects that may need to be taken into account include: - Expected emissions and blocking levels, which could be estimated by means of considering, e.g.: - Option 1: Potential expected coupling loss and total receiver desensitization (considering self-interference, co-channel interference and adjacent operator interference) considering relevant site deployment scenarios - Option 2: Co-existence simulation and potentially analysis of co-located scenarios - Expected spectrum allocation and sub-band sizes - Expected feasible filtering performances considering the above two aspects Issue 1-2: Joint measurement for normal DL symbols/slots and SBFD symbols/slots - Way forward - Further discuss on the following aspects in the performance WI - Whether the following two slots configured with flexible symbols could be used for EVM measurement. - Flexible symbols per slot is configured with both SBFD symbols and guard period symbols or - Flexible symbols per slot is configured with SBFD symbols, guard period and also uplink symbols. - Separately measure the averaged EVM for SBFD and non-SBFD, and the averaged EVM for SBFD slots shall be calculated only based on the data samples where DLs are allocated within the SBFD slots. - Use the following text proposal as starting point.

Similar as Co-located ACLR requirement, during the last RAN4 meeting, there were some discussions on how to define OBUE requirement for SBFD capable BS in order to ensure the coexistence performance for SBFD BSs operating in adjacent channel shown as following. During the Rel-18 SBFD SI phase, the following coexistence scenario was not evaluated, however the CLI interference is quite similar as Case 3 where NR DL interfering SBFD UL. Especially for Scenario 1, Case 3, the severe performance degradation could be observed for SBFD UL due to the strong CLI from NR DL in the adjacent channel. Even with ACIR requirement tightening 8dB, the 5% throughput loss is still up to 43.5% and the mean throughput loss could be minimized down to 4.67%. However as we know that, for AAS based SBFD BS, due to the higher operating frequency and larger channel bandwidth and small transceiver’s PA linearity, there are no much ACLR/OBUE requirement to be further improved. In other words, even though RAN4 is capable to agree to define more tightening requirement e.g. with 10dB tightening requirements than the legacy requirements , it is still not feasible to be achieved for SBFD capable BS by improving the front-end PA linearity.

As mentioned before, the RF filter for wide area SBFD BS and coupling loss between aggressor and victim system is important to verify its co-location coexistence performance. In addition, for band n41, single operator is holding the whole 160MHz spectrum. For n78, two operators are holding the contiguous 200MHz spectrum and do the spectrum sharing. For n79, two operators are holding the contiguous 160MHz spectrum and some spectrum sharing might be also feasible. For U6GHz in China, there are about 700MHz spectrum available to be allocated, it might be possible operating band to have multiple operator coexistence scenario in China.

In short, for SBFD vs SBFD coexistence scenario among different operators, we propose to focus on the band n104 or treat it as 1st priority if that agreeable.

Proposal 1: for SBFD vs SBFD coexistence scenario among different operators, propose to treat the band n104 as 1st priority, for other interested bands, it depends on the operator’s demand.

In addition, as we discussed in the previous RAN4 meeting, how to guarantee the co-location coexistence performance between two different SBFD carries is highly dependent on the transmitter analog performance and also its coupling loss between the aggressor and victim system.

If the analog filter attenuation at the certain freq offset away from the RF bandwidth edge could be achieved as 34dBc together with MCL as 67dB which will results in -116dBm/MHz. This leakage interference is supposed to be 7dB lower than victim’s noise figure which is aligned with the legacy principle for co-location spurious requirement. However we might need to more time to verify this coupling loss based on the U6GHz BS in the OTA chamber. Meanwhile more analysis might be needed to get the baseline/default value for the required freq offset to achieve 34dBc attenuation, otherwise it will be very ambiguous to define this requirement with the declared freq offset to meet the requirement which will cause lots of product fragmentation.

Some initial analysis for analog filtering design at U6GHz is provided as following for the information, more data would be provided later on. Regarding the coupling loss between different U6GHz BS could be potentially provided after the May meeting or during the May meeting.

|  | Freq offset to the operators’ spectrum edge |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| BS RF bandwidth | 10MHz | 15MHz | 20MHz | 40MHz | F1 | F2 |
| 100MHz | 2.2 | 3 | 10 | 20 | 34? | TBD |
| 200MHz | TBD | TBD | TBD | TBD | TBD | TBD |
| 300MHz | TBD | TBD | TBD | TBD | TBD | TBD |
| 400MHz | TBD | TBD | TBD | TBD | TBD | TBD |

Proposal 2a: if it is agreeable to define the co-location OBUE requirement, propose to get the baseline/default value for the required freq offset to achieve the required attenuation, otherwise it will be very ambiguous to define this requirement with the declared freq offset to meet the requirement which will cause lots of product fragmentation.

Proposal 2b: both the co-location requirement and also coexistence requirement between two SBFD operators should be guaranteed if the coexistence scenario is agreeable.

Conclusions

In this contribution, we want to share some initial views on RF requirements for SBFD BS and proposals/observations are made as following:

Proposal 1: for SBFD vs SBFD coexistence scenario among different operators, propose to treat the band n104 as 1st priority, for other interested bands, it depends on the operator’s demand.

Proposal 2a: if it is agreeable to define the co-location OBUE requirement, propose to get the baseline/default value for the required freq offset to achieve the required attenuation, otherwise it will be very ambiguous to define this requirement with the declared freq offset to meet the requirement which will cause lots of product fragmentation.

Proposal 2b: both the co-location requirement and also coexistence requirement between two SBFD operators should be guaranteed.

References

- RP-234035,New WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei, Samsung, Approved.

- TR 38.858

- R4-2321091, TP to TR 38.858 Impact on BS RF requirements, ZTE, Agreed

- R4-2409958, Way forward for [111][313] NR_duplex_evo, Approved.

- R4-2413283, On modification of existing TX RF requirements for SBFD, Huawei

- R4-2413515, Way Forward for [112][308] NR_duplex_evo_BSRF, Approved

- R4-2411640,On the potentially new requirements for SBFD operation for FR1 and FR2-1, Samsung

- R4-2416585, Way Forward for [112bis][307] NR_duplex_evo_BSRF, Approved.

- R4-2419907	Way Forward for [113][306] NR_duplex_evo_General, Approved

- R4-2419908	Way Forward for [113][307] NR_duplex_evo_BSRF, Approved.

- R4-2502310	Way Forward for [114][306] NR_duplex_evo_General, approved.

- R4-2502380	Way Forward for [114][307] NR_duplex_evo_BSRF, approved.

- R4-2504751	Way Forward for [114bis][306] NR_duplex_evo_General

- R4-2504752	Way Forward for [114bis][307] NR_duplex_evo_BSRF

TP EVM

B.7	Averaged EVM

EVM is averaged over all allocated downlink resource blocks with the considered modulation scheme in the frequency domain, and a minimum of Ndl slots where Ndl is the number of slots in a 10 ms measurement interval.

For FDD the averaging in the time domain equals the Ndl slot duration of the 10 ms measurement interval from the equalizer estimation step.

-	Where Ni is the number of resource blocks with the considered modulation scheme in slot i.

-	The EVM requirements shall be tested against the maximum of the RMS average at the window W extremities of the EVM measurements:

-	Thus   is calculated using in the expressions above and  is calculated using  in the  calculation.

-	Thus we get:

EVM=maxEVMframe,l,EVMframe,h

For TDD, let NdlTDD be the number of slots with downlink symbols within a 10 ms measurement interval, the averaging in the time domain can be calculated from  NdlTDD slots of different 10 ms measurement intervals and should have a minimum of Ndl slots averaging length where Ndl is the number of slots in a 10 ms measurement interval.

-	EVMframe is derived by: Square the EVM results in each 10 ms measurement interval. Sum the squares, divide the sum by the number of EVM relevant locations, square-root the quotient (RMS).

EVMframe=1i=1NdlTDDNii=1NdlTDDj=1NiEVMi,j2

-	Where Ni is the number of resource blocks with the considered modulation scheme in slot i.

-	The EVMframe is calculated, using the maximum of EVMframe at the window W extremities. Thus EVMframe,l is calculated using t=∆tl and EVMframe,h is calculated using t=∆tℎ (l and h, low and high; where low is the timing ∆c−W/2 and and high is the timing ∆c+W/2).

EVMframe=maxEVMframe,l,EVMframe,h

-	In order to unite at least Ndl slots, consider the minimum integer number of 10 ms measurement intervals, where Nframe is determined by.

Nframe=10×NslotNdlTDD

and Nslot=1 for 15 kHz SCS, Nslot=2 for 30 kHz SCS and Nslot=4 for 60 kHz SCS normal CP.

-	Unite by RMS.

EVM=1Nframek=1NframeEVMframe,k2

B.7a	Averaged EVM for FR1 SBFD BS

EVM is averaged over all allocated downlink resource blocks with the considered modulation scheme in the frequency domain, and a minimum of Ndl slots where Ndl is the number of slots in a 10 ms measurement interval.

For SBFD, let NdlTDD be the number of slots with downlink non-SBFD symbols or SBFD symbols within a 10 ms measurement interval, the averaging in the time domain can be calculated from  NdlTDD slots of different 10 ms measurement intervals and should have a minimum of Ndl slots averaging length where Ndl is the number of slots in a 10 ms measurement interval.

-	EVMframe is derived by: Square the EVM results in each 10 ms measurement interval. Sum the squares, divide the sum by the number of EVM relevant locations, square-root the quotient (RMS).

EVMframe=1i=1NdlTDDNii=1NdlTDDj=1NiEVMi,j2

-	Where Ni is the number of resource blocks with the considered modulation scheme in slot i.

-	The EVMframe is calculated, using the maximum of EVMframe at the window W extremities. Thus EVMframe,l is calculated using t=∆tl and EVMframe,h is calculated using t=∆tℎ (l and h, low and high; where low is the timing ∆c−W/2 and and high is the timing ∆c+W/2).

EVMframe=maxEVMframe,l,EVMframe,h

-	In order to unite at least Ndl slots, consider the minimum integer number of 10 ms measurement intervals, where Nframe is determined by.

Nframe=10×NslotNdlTDD

and Nslot=1 for 15 kHz SCS, Nslot=2 for 30 kHz SCS and Nslot=4 for 60 kHz SCS normal CP.

-	Unite by RMS.

EVM=1Nframek=1NframeEVMframe,k2

C.7	Averaged EVM

EVM is averaged over all allocated downlink resource blocks with the considered modulation scheme in the frequency domain, and a minimum of  Ndl slots where Ndl is the number of slots in a 10 ms measurement interval.

For TDD, let NdlTDD be the number of slots with downlink symbols within a 10 ms measurement interval, the averaging in the time domain can be calculated from NdlTDD slots of different 10 ms measurement intervals and should have a minimum of Ndl slots averaging length where Ndl is the number of slots in a 10 ms measurement interval.

-	EVMframe is derived by: Square the EVM results in each 10 ms measurement intervals. Sum the squares, divide the sum by the number of EVM relevant locations, square-root the quotient (RMS).

EVMframe=1i=1NdlTDDNii=1NdlTDDj=1NiEVMi,j2

-	Where Ni is the number of resource blocks with the considered modulation scheme in slot i.

-	The EVMframe is calculated, using the maximum of EVMframe at the window W extremities. Thus EVMframe,l is calculated using t=∆tl and EVMframe,h is calculated using t=∆tℎ (l and h, low and high; where low is the timing ∆c−W/2 and and high is the timing ∆c+W/2).

EVMframe=maxEVMframe,l,EVMframe,h

-	In order to unite at least Ndl slots, consider the minimum integer number of 10 ms measurement intervals, where Nframe is determined by.

Nframe=10×NslotNdlTDD

and Nslot=4 for 60 kHz SCS and Nslot=8 for 120 kHz SCS.

-	Unite by RMS.

EVM=1Nframek=1NframeEVMframe,k2

C.7a	Averaged EVM for FR2 SBFD BS

EVM is averaged over all allocated downlink resource blocks with the considered modulation scheme in the frequency domain, and a minimum of  Ndl slots where Ndl is the number of slots in a 10 ms measurement interval.

For SBFD, let NdlTDD be the number of slots with downlink non-SBFD symbols or SBFD symbols within a 10 ms measurement interval, the averaging in the time domain can be calculated from NdlTDD slots of different 10 ms measurement intervals and should have a minimum of Ndl slots averaging length where Ndl is the number of slots in a 10 ms measurement interval.

-	EVMframe is derived by: Square the EVM results in each 10 ms measurement intervals. Sum the squares, divide the sum by the number of EVM relevant locations, square-root the quotient (RMS).

EVMframe=1i=1NdlTDDNii=1NdlTDDj=1NiEVMi,j2

-	Where Ni is the number of resource blocks with the considered modulation scheme in slot i.

-	The EVMframe is calculated, using the maximum of EVMframe at the window W extremities. Thus EVMframe,l is calculated using t=∆tl and EVMframe,h is calculated using t=∆tℎ (l and h, low and high; where low is the timing ∆c−W/2 and and high is the timing ∆c+W/2).

EVMframe=maxEVMframe,l,EVMframe,h

-	In order to unite at least Ndl slots, consider the minimum integer number of 10 ms measurement intervals, where Nframe is determined by.

Nframe=10×NslotNdlTDD

and Nslot=4 for 60 kHz SCS and Nslot=8 for 120 kHz SCS.

-	Unite by RMS.

EVM=1Nframek=1NframeEVMframe,k2