3GPP TSG-RAN WG4 Meeting #115                                                          R4-2507523

Malta, MT, 19th – 23th May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.176-1 | CR | 0075 | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (TEI19) CR to TS 38.176-1: spec structure simplification for co-locationco-existence requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to the RAN task, the spec structure simplification for transmitter spurious emission in subclause 6.6.5.5.2 and subclause 6.6.5.5.3 |  |  |  |  |  |  |  |  |
| Summary of change: |  | The table is simplified to include the general requirement level |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If not approved, the specification cannot be improved as requested by RAN task. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.5.5.2 and 6.6.5.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

6.6.5.5.2	Additional spurious emissions requirements

These requirements may be applied for the protection of system operating in other frequency ranges. The limits may apply as an optional protection of such systems that are deployed in the same geographical area as the IAB-node, or they may be set by local or regional regulation as a mandatory requirement for an NR operating band. It is in some cases not stated in the present document whether a requirement is mandatory or under what exact circumstances that a limit applies, since this is set by local or regional regulation. An overview of regional requirements in the present document is given in clause 4.5.

Some requirements may apply for the protection of specific equipment (UE, MS and/or BS) or equipment operating in specific systems (GSM, CDMA, UTRA, E-UTRA, NR, etc.) as listed below.

The spurious emission basic limits are provided in table 6.6.5.5.2-1 where requirements for co-existence with the system listed in the first column apply for IAB-MT and IAB-DU. For a multi-band connector, the exclusions and conditions in the Note column of table 6.6.5.5.2-1 apply for each supported operating band.

Table 6.6.5.5.2-1: IAB-DU and IAB-MT spurious emissions basic limits for co-existence with systems operating in other frequency bands

| Frequency range for co-existence requirement | Operating band to co-exist with | Basic limits (dBm/MHz) | Measurement bandwidth | Notes |
| --- | --- | --- | --- | --- |
| Frequency range of downlink operating band | GSM850, CDMA850 or GSM900 | -57 | 100 kHz |  |
|  | DCS1800 or PCS1900 | -47 | 100 kHz |  |
|  | Other | -52 | 1 MHz |  |
| Frequency range of uplink operating band | GSM850, CDMA850, GSM900, DCS1800 or PCS1900 | -61 | 100 kHz |  |
|  | Other | -49 | 1 MHz |  |
| NOTE 1: When DL spectrum of operating band is overlapping with or partially overlapping with or adjacent to DL spectrum of UTRA bands or E-UTRA bands or NR bands, this is not applicable anymore. NOTE 2: When UL spectrum of operating band is overlapping with or partially overlapping with UTRA bands or E-UTRA bands or NR bands, the protection of the IAB receiver of own or different IAB in clause 6.6.5.2.2 is applicable for UL spectrum . NOTE 3: When operating band is coexisting with UTRA, E-UTRA and NR TDD bands, more stringent requirement between DL and UL protection limit should apply. |  |  |  |  |

NOTE 1:	As defined in the scope for spurious emissions in this clause the co-existence requirements in table 6.6.5.2.2-1 do not apply for the ΔfOBUE frequency range immediately outside the downlink operating band (see table 5.2-1). Emission limits for this excluded frequency range may be covered by local or regional requirements.

NOTE 2:	Table 6.6.5.2.2-1 assumes that two operating bands, where the frequency ranges in table 5.2-1 would be overlapping, are not deployed in the same geographical area. For such a case of operation with overlapping frequency arrangements in the same geographical area, special co-existence requirements may apply that are not covered by the 3GPP specifications.

6.6.5.5.3	Co-location with base stations and IAB-nodes

These requirements may be applied for the protection of other BS, IAB-DU or IAB-MT receivers when GSM900, DCS1800, PCS1900, GSM850, CDMA850, UTRA FDD, UTRA TDD, E-UTRA, NR BS, IAB-DU or IAB-MT are co-located with IAB-MT and/or IAB-DU.

The requirements assume a 30 dB coupling loss between transmitter and receiver and are based on co-location with same class.

The basic limits are in table 6.6.5.2.3-1 for an IAB-DU and IAB-MT. Requirements for co-location with a system listed in the first column apply, depending on the declared IAB-DU and IAB-MT class. For a multi-band connector, the exclusions and conditions in the Note column of table 6.6.5.2.3-1 shall apply for each supported operating band.

Table 6.6.5.5.3-1: IAB-DU and IAB-MT spurious emissions basic limits for co-location with BS or IAB-node

| Frequency range for | Operating band to be co-located with | Basic limits [dBm] |  |  | Measurement | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| co-location requirement |  | WA IAB-DU and WA IAB-MT | MR IAB-DU | LA IAB-DU and LA IAB-MT | bandwidth |  |
| Frequency range of co-located uplink operating band | Co-located with GSM850, CDMA850 or GSM900 | -98 | -91 | -70 | 100 kHz | Note 1 |
|  | Co-located with DCS1800 or PCS1900 | -98 | -91 | -80 | 100 kHz |  |
|  | n51, n91, n93 | N/A | N/A | -88 | 100 kHz |  |
|  | n53 | N/A | -91 | -88 | 100 kHz |  |
|  | n100, n101 | -96 | N/A | N/A | 100 kHz |  |
|  | n96, n102 | N/A | -90 | -87 | 100 kHz |  |
|  | n104 | -95 | -90 | -87 | 100 kHz |  |
|  | Other | -96 | -91 | -88 | 100 kHz |  |
| NOTE 1: As defined in the scope for spurious emissions in this clause, the co-location requirements in table 6.5.4.5.3-1 do not apply for the frequency range extending ΔfOBUE immediately outside the BS transmit frequency range of a downlink operating band (see table 5.2-1). NOTE 2: When DL spectrum of operating band is overlapping with or partially overlapping with or adjacent to UL spectrum of UTRA bands or E-UTRA bands or NR bands, this is not applicable anymore. |  |  |  |  |  |  |

NOTE 1:	As defined in the scope for spurious emissions in this clause, the co-location requirements in table 6.6.5.2.3-1 do not apply for the frequency range extending ΔfOBUE immediately outside the transmit frequency range of a IAB-MT and IAB-DU. The current state-of-the-art technology does not allow a single generic solution for co-location with other system on adjacent frequencies for 30dB antenna to antenna minimum coupling loss. However, there are certain site-engineering solutions that can be used. These techniques are addressed in TR 25.942 [8].

NOTE 2:	Table 6.6.5.2.3-1 assumes that two operating bands, where the corresponding transmit and receive frequency ranges in table 5.2-1 would be overlapping, are not deployed in the same geographical area. For such a case of operation with overlapping frequency arrangements in the same geographical area, special co-location requirements may apply that are not covered by the 3GPP specifications.