3GPP TSG-RAN WG4 Meeting # 115															R4-2507531

Malta, MT, 19th – 23rd May, 2025

Agenda item:			7.24.3

Source:	Samsung

Title:	Views and progress on UE RF discussion for NR MIMO Phase 5

Document for:	Discussion

1	Introduction

RAN4 has discussed UE RF aspects of Rel-19 NR MIMO Phase 5 focusing on the objectives of 3-antenna-port transmissions in FR1 and beam correspondence requirements for UL mTRP in FR2 in the WID [1]. Thanks to the companies who have contributed and compromised until the last meeting, RAN4 has achieved considerable progress and come to a close for the technical discussion. In this contribution, we would like to summarize what we have been done for this WI based on previous WFs [2-6].

2	Discussion

2.1	FR1 (3Tx)

Power class

For applicable power classes for 3Tx, legacy power classes such as PC3/2/1.5 were discussed, and RAN4 first agreed to have PC3 and PC2 [3]. However, given the limited assumption in WID, e.g., UL full power transmission mode 1 and 2 are not supported, 3Tx PC1.5 took many meetings to find out its technical feasibility for handheld UEs along with Tx diversity consideration. RAN4 concluded that there is no consensus for PC1.5 3Tx in Rel-19 at the last meeting [6].

Observation 1:	RAN4 first agreed to have PC3 and PC2 for 3Tx.

Observation 2:	RAN4 concluded that there is no consensus for PC1.5 3Tx in Rel-19.

MPR

Regarding the MPR requirements for each power class for 3Tx, in the beginning of the WI, it was first discussed how to derive MPR requirement for 3Tx power classes given the small gap between 1Tx and 2Tx MPRs for PC2. RAN4 made an agreement that 3Tx MPR can reuse the legacy MPR tables as much as possible [4]. Based on the principle, RAN4 has reached a consensus to reuse the single carrier PC3 MPR for 3Tx PC3 [4] and 2Tx PC2 MPR for 3Tx PC2 [6] as copied in Table 1 and Table 2, respectively.

Observation 3:	RAN4 made an agreement that 3Tx MPR can reuse the legacy MPR tables as much as possible.

Observation 4:	RAN4 has reached a consensus to reuse the single carrier PC3 MPR for 3Tx PC3 and 2Tx PC2 MPR for 3Tx PC2.

Table 1: Maximum power reduction (MPR) for power class 3

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.51 | ≤ 1.21 | ≤ 0.21 |
|  |  | ≤ 0.52,3 | ≤ 0.52 | 02,4 |
|  | Pi/2 BPSK w Pi/2 BPSK DMRS | ≤ 0.52,3 | 02 | 02,4 |
|  | QPSK | ≤ 1 |  | 05 |
|  | 16 QAM | ≤ 2 |  | ≤ 1 |
|  | 64 QAM | ≤ 2.5 |  |  |
|  | 256 QAM | ≤ 4.5 |  |  |
| CP-OFDM | QPSK | ≤ 3 |  | ≤ 1.5 |
|  | 16 QAM | ≤ 3 |  | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |
| NOTE 1: Applicable for UE operating in TDD mode with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and if the IE powerBoostPi2BPSK is set to 1 and 40 % or less slots in radio frame are used for UL transmission for bands n40, n41, n77, n78 and n79. The reference power of 0 dB MPR is 26 dBm. NOTE 2: Applicable for conditions where note 1 does not apply. NOTE 3: For 3 MHz channel bandwidth the Pi/2 BPSK edge allocation MPR is 1 dB NOTE 4: For a UE indicating support for UE capability [powerBoostRel18] or [powerBoostTSRel18] and if the IE [powerBoostPi2BPSKRel18] is set to 1, the reference power is increased by [ΔPPowerBoost - ΔPPowerClass] NOTE 5: For a UE indicating support for UE capability [powerBoostRel18] or [powerBoostTSRel18] and if the IE [powerBoostQPSKRel18] is set to 1, the reference power is increased by [ΔPPowerBoost - ΔPPowerClass] |  |  |  |  |

Table 2: Maximum power reduction (MPR) for power class 2 with dual Tx

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.5 | ≤ 1 | 01 |
|  | QPSK | ≤ 3.5 | ≤ 2 | 0.52 |
|  | 16 QAM | ≤ 3.5 | ≤ 2.5 | ≤ 1.5 |
|  | 64 QAM | ≤ 3.5 | ≤ 3 |  |
|  | 256 QAM | ≤ 5.5 |  |  |
| CP-OFDM | QPSK | ≤ 4.0 | ≤ 3.5 | ≤ 2 |
|  | 16 QAM | ≤ 4.0 | ≤ 3.5 | ≤ 2.5 |
|  | 64 QAM | ≤ 4.5 |  |  |
|  | 256 QAM | ≤ 8.0 |  |  |
| NOTE 1: For a UE indicating support for UE capability [powerBoostRel18] or [powerBoostRel18TS] and if the IE [powerBoostPi2BPSKRel18] is set to 1, the reference power is increased by ΔPPowerBoost NOTE 2: For a UE indicating support for UE capability [powerBoostRel18] or [powerBoostRel18TS] and if the IE [powerBoostQPSKRel18] is set to 1, the reference power is increased by ΔPPowerBoost |  |  |  |  |

MIMO layers

The 3Tx UL MIMO configuration for verification was simply agreed with TPMI index 0 for 3-layer UL MIMO codebook-based transmission [2]. For the MIMO layer discussion for full power mode 0, RAN4 also had a discussion about which MIMO layer can be supported with 3Tx for ULFPTx. Although the supported MIMO layer for mode 0 is independent with the supportive power class of 3Tx, it was finally agreed that only 1-layer is used as the performance verification purpose for 3Tx [4]

Observation 5:	For 3Tx full power mode 0, only 1-layer is agreed for the performance verification purpose.

2.2	FR2 (UL mTRP)

For the beam correspondence requirement considering UL mTRP under the objective of asymmetric DL sTRP/UL mTRP, RAN4 has discussed whether it has impact on the UE RF requirement in Rel-19, and got a confirmation that it can be also considered to transmit SSB from the UL only TRP in this WI [7].

However, since there would be no motivation to introduce PL offset to identify pathloss between the UL TRP and the UE if the UL TRP can receive SSBs, and it would be the same scenario as previous general mTRP from UE’s perspectives as Figure 1 below, we still believe that it is meaningless to introduce additional beam correspondence requirement for Scenario 4 in Figure 1 below.

Figure 1: Beam correspondence under multi-TRP operations

In short, as shown in Figure 1, the UE still can support Scenario 2 and 4 based on the legacy beam correspondence requirement of Rel-15. We still believe that RAN4 should refrain from considering asymmetric DL sTRP/UL mTRP in UE RF discussions unless they are going to have a suffix/clause for every single feature/capability. If some companies still want it as same as Rel-15 requirement without a technical reason, it should be applicable to all UL mTRP scenarios enabled from the previous release as noted in our latest WF, which is when the IE unifiedTCI-StateType is configured as ‘separate’ [6].

Observation 6:	UE still can support Scenario 2 and 4 based on the legacy beam correspondence requirement of Rel-15.

Proposal 1:	If it is still required for some reasons, the new beam correspondence requirement should be applicable to all UL mTRP scenarios enabled from the previous release as noted in our latest WF, which is when the IE unifiedTCI-StateType is configured as ‘separate’.

3	Conclusion

This contribution provides a summary of WI discussions and some takeaways along the UE RF part based on the previous WFs. Following observations and a proposal were derived for consideration in this meeting.

Observation 1:	RAN4 first agreed to have PC3 and PC2 for 3Tx.

Observation 2:	RAN4 concluded that there is no consensus for PC1.5 3Tx in Rel-19.

Observation 3:	RAN4 made an agreement that 3Tx MPR can reuse the legacy MPR tables as much as possible.

Observation 4:	RAN4 has reached a consensus to reuse the single carrier PC3 MPR for 3Tx PC3 and 2Tx PC2 MPR for 3Tx PC2.

Observation 5:	For 3Tx full power mode 0, only 1-layer is agreed for the performance verification purpose.

Observation 6:	UE still can support Scenario 2 and 4 based on the legacy beam correspondence requirement of Rel-15.

Proposal 1:	If it is still required for some reasons, the new beam correspondence requirement should be applicable to all UL mTRP scenarios enabled from the previous release as noted in our latest WF, which is when the IE unifiedTCI-StateType is configured as ‘separate’.

Reference

[1] 	RP-242394,	WID revision: NR MIMO Phase 5, RAN#105

[2] 	R4-2414310,	WF on UE RF requirements for Rel-19 MIMO enhancement, RAN4#112

[3] R4-2417116,	WF on UE RF requirements for MIMO enhancement, RAN4#112-bis

[4]	R4-2420400,	WF on UE RF requirements for MIMO enhancement, RAN4#113

[5] R4-2502951,	WF on UE RF requirements for MIMO enhancement, RAN4#114

[6]	R4-2505111,	WF on UE RF requirements for MIMO enhancement phase 5, RAN4#114-bis

[7] R4-2500005,	Reply to RAN4 LS on UE RF issues for Rel-19 MIMO Enhancement, RAN4#114