3GPP TSG-RAN WG4 Meeting #115	R4-2507532

# Malta, MT, 19th – 23rd May 2025

Source:	Nokia

Title:	CSI reporting requirement and testing framework for CSI prediction

Agenda item:	7.21.3

Document for:	Discussion

Introduction

This paper discusses the CSI reporting requirement framework for CSI prediction. In this paper, we address the challenges related to testability and LCM requirements for AI/ML-based CSI prediction use cases.

Discussion

LCM requirements

The testing goals were discussed and agreed upon in RAN4#112, including the use of LCM to verify the minimum performance of AI/ML functionality/feature.

Agreement: - The testing goal is to verify whether the minimum performance of AI/ML functionality/feature can be achieved - FFS on whether and how many different test configurations/parameters are used for tests. - LCM would be tested

The scope of the LCM discussion was addressed in RAN4#113.

Agreement: - The discussion should consider use-cases specific aspects Agreement: - Whether transition from AI to non-AI would be UE initiated or network initiated - This will be a decision for other groups - RAN4 to assess what RRM requirements are needed for each case.

Also, RAN2 has made agreements related to LCM procedures.

Agreements on procedures from #127: - Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities. - Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side. - “Step 3”: Following configurations are provided from NW to UE: - 1) UE is allowed to do UAI reporting via OtherConfig. - 2) Network may provide NW-side additional condition. FFS on the RRC signalling and whether it is mandatory or optional. - 3) FFS on configuration (e.g. inference configuration) of supported functionalities. FFS on the content of configuration. - UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE (e.g. inference configuration). FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3. - “Step 4”: UE reports applicable functionality in the following scenarios: - 1) Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI - 2) As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration), FFS via UAI or RRCReconfigurationComplete, etc - Step 5: - 1) Network configures inference configuration to UE after applicable functionality reporting, if inference configuration based on supported functionality is not provided in Step 3 (i.e. inference configuration is provided in Step 5). - 2) If inference configuration based on supported functionality is provided in Step 3, it is up to network implementation whether to provide an updated configuration or not. - The applicable functionality may be activated by receiving its inference configuration when it is provided in Step 5. FFS the initial activation state. FFS on initial state of applicable functionality if inference configuration of supported functionality is provided in Step 3. FFS on additional L1/L2 signaling for activation/deactivation. FFS if multiple applicable functionalities can be activated at the same time. FFS what is the granularity of functionality

Additionally, the agreement related LCM procedures in RAN4#111 are as follows:

Agreement: RAN 4 to consider defining RRM requirements for LCM procedures based on the specific use cases (beam management and positioning) and candidate are: - Identification - Selection - Activation - Deactivation - Switching - Fallback to non-AI operation - Performance monitoring - Others are not precluded

RAN4 needs to discuss how to manage dynamic and non-static conditions for CSI prediction. Initially, RAN4 can assume the LCM procedures defined in RAN2. For instance, a delay requirement may be needed in the procedure due to the potential processing time of the UE to determine failure or performance degradation. Therefore, RAN4 should also investigate how the RAN2 approach impacts RAN4 requirements.

The LCM operations including activation, deactivation, and fallback can be different behaviors depending on CSI report configurations. Currently, AIML beam managements (BM) take into account the different CSI report configurations such as periodic, semi-periodic, or aperiodic CSI reporting. The outcome and principle of the LCM and performance monitoring in the AIML BM will be similarly applicable to the AIML CSI prediction. Therefore, similar to the BM cases, RAN4 should consider that the delay components of LCM operations can vary depending on the specific CSI report configurations.

RAN4 to discuss whether LCM-related requirements are needed for AIML CSI prediction.

RAN4 to study the LCM requirements focusing on activation and deactivation aspects.

In RAN1#120bis meeting there has been a progress related to performance monitoring with following agreements.

Agreement For CSI prediction using UE-side model, for performance monitoring, support UE assisted performance monitoring subject to an additional UE capability, and UE assisted performance monitoring is based on Type 3 performance monitoring Agreement For CSI prediction using UE-side model, for performance monitoring type 3, support SGCS as a performance metric.

Also, the following agreement of RAN1#120 meeting indicates the FFS defining the details for the Type 3 monitoring reporting.

Agreement For CSI prediction using UE-side model, for reporting contents of UE assisted performance monitoring, down-select one alternative by RAN1#121. - Alt 1: one SGCS is calculated based on predicted CSI for one inference reporting, ground truth CSI - Alt 2: one SGCS is calculated based on predicted CSI for one inference reporting, and ground truth CSI, another SGCS is based on ground truth CSI and CSI (non-predicted) corresponding to the latest CSI-RS transmission occasion not later than CSI reference resource of the inference reporting instance. - Alt 3: statistic of reporting contents (e.g., mean, x% CDF of SGCS values) of inference reporting instances within configured monitoring window - Monitoring window can be configured by NW - FFS on signalling details - FFS on whether to report per prediction instance, selected prediction instance(s), averaged over prediction instances - FFS on report frequency granularity (e.g., per wideband or per subband or averaged over subband or selected subband) - FFS on whether to report per layer, the first layer, averaged over layer - FFS on how to quantize and report mechanism

While RAN1 is discussing three alternatives 1, 2, and 3, RAN4 should discuss whether a requirement needs to be specified for performance monitoring where SGCS is used as a performance metric.

RAN4 to discuss whether and how to specify requirements on performance monitoring for CSI predictions.

Conclusion

In this contribution, we have discussed the RRM impact of AI/ML-based CSI prediction. The proposals are presented as follows.

- RAN4 to discuss whether LCM-related requirements are needed for AIML CSI prediction.

- RAN4 to study the LCM requirements focusing on activation and deactivation aspects.

- RAN4 to discuss whether and how to specify requirements on performance monitoring for CSI predictions.

- References

- R4-2420334, “WF on AI/ML air interface part 1,” RAN4#113, Orlando, USA.

- R4-2502856, “WF on the requirements for AI/ML air interface,” RAN4#114, Athens, Greece.