3GPP TSG-RAN WG4 Meeting #115	R4-2507533

# Malta, MT, 19th – 23rd May, 2025

Source:	Nokia

Title:	RRM core requirements and testing framework for AI/ML positioning accuracy enhancement

Agenda item:	7.21.5

Document for:	Discussion

Introduction

In RAN#102, the Release-18 study on artificial intelligence (AI)/machine learning (ML) for NR air interface (FS_NR_AIML_Air) [1] was completed [2] and the findings (with agreements and open issues) were documented in the outcome technical report 3GPP TR 38.843 V18.0.0 [3]. Furthermore, a new Release-19 work item on artificial intelligence (AI)/machine learning (ML) for NR air interface (NR_AIML_Air) [4] was also approved for the normative work of the general AI/ML framework for air interface and to enable the recommended use cases in the preceding study.

During the SI, as mentioned in the extract from 3GPP TR 38.843 V18.0.0 [3] below, following sub use cases for positioning accuracy enhancement are considered:

The following are selected as representative sub-use cases: - Direct AI/ML positioning: - AI/ML model output: UE location - e.g., fingerprinting based on channel observation as the input of AI/ML model - AI/ML assisted positioning: - AI/ML model output: new measurement and/or enhancement of existing measurement - e.g., LOS/NLOS identification, timing and/or angle of measurement, likelihood of measurement More specifically, the following Cases are considered for the study: - Case 1: UE-based positioning with UE-side model, direct AI/ML or AI/ML assisted positioning - Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning One-sided model whose inference is performed entirely at the UE or at the network is prioritized in Rel-18 SI.

During SI, RAN4 studied different test metrics for performance requirements for AI/ML enabled positioning use case.  Here is an extract from 3GPP TR 38.843 V18.0.0 [3] covering these aspects:

Both direct AI/ML positioning and AI/ML assisted positioning are considered. For metrics for positioning requirements/tests, the candidate options include - Option 1: positioning accuracy: Ground truth vs. reported - only option available for direct positioning - Option 2: CIR/PDP, channel estimation accuracy - Option 3: ToA, RSTD and RSRP, and RSRPP - Option 4: others (e.g., intermediate KPIs, LoS/NLoS)/combinations of the above The feasibility and testability of different options should be further justified in WI.

The issues on AI/ML based positioning require further discussion on selected sub use cases. In this paper, the performance requirements for UE-based positioning (i.e., case 1) and NG-RAN node assisted positioning (i.e., cases 3a and 3b) are discussed in the aspects of performance test metric, measurement accuracy requirements, testability, and LCM-related latency requirements.

Discussion

The issues related to Cases 1 and 3 are discussed in this section. Case 2 is not focused on this paper according to the revised WID [4].

Requirements for Case1

This section discusses the requirements in the use cases with a UE-side model.

Measurement accuracy requirements

Agreement Path-based measurement refers to the measurement in the existing specifications (up to Rel-18) including measurement reporting, with potential enhancements on the number of reported paths (if needed).

In RAN1#117, it is confirmed that the path-based measurement in the context of AIML-based positioning is the existing specifications’ measurements including their measurement reporting. When the measurements in AIML positioning are the same as the measurements defined in the legacy positioning, the various target positioning requirement types for NR positioning in Rel-18 summarized in TR38.859 can be considered as the candidate measurements in AIML positioning. In particular, the functionality input may be one of the existing measurements such as PRS-RSRP, PRS-RSRPP, etc. Also, if any new measurements such as CIR, DP, and/or PDP are agreed in RAN1 as functionality input, there are impacts in RAN4 measurement accuracy requirements for these new measurements.

In our view, the input to the AI/ML functionality should have and satisfy any measurement accuracy requirements to ensure the AI/ML functionality performs as expected. Hence, RAN4 should define the requirements on the measurement types supported in the legacy positioning (e.g., RSTD, PRS-RSRP, and PRS-RSRPP). Moreover, RAN4 should study how to define the requirements on new measurement is supported (e.g., CIR, DP, and PDP).

For AIML positioning in Case 1, RAN4 to specify the requirements on the measurements supported in legacy positioning (e.g., RSTD, PRS-RSRP, and PRS-RSRPP) and define the requirements on the new measurements (e.g., CIR, DP, and PDP) based on RAN1 progress.

Additionally, the accuracy requirements can be applicable to the inputs and outputs of AIML model. While the types of input and output are depending on the use cases, defining the requirements for the input or output will have an impact on the positioning accuracy. Therefore, RAN4 can discuss whether/when the measurement accuracy should be defined.

RAN4 to study the necessity of any requirements for the input of the AIML model in Case 1.

Requirements for consistency between training and inference

In NR positioning, DL-PRS configuration is provided in a hierarchy. There can be at most 4 positioning frequency layers (PFL) and each positioning frequency layer has at most 64 TRPs. Each TRP per positioning frequency layer can have 2 DL-PRS Resource sets, thus resulting in a total of 8 DL-PRS resource sets per TRP and each DL-PRS resource set can have up to 64 DL-PRS resources. Each DL-PRS resource corresponds to a beam. Having 2 different DL-PRS resource sets per positioning frequency layer per TRP allows gNB to configure one DL-PRS resource set of wide beams and another DL-PRS resource set of narrow beams for each positioning frequency layer.

Figure 1 DL-PRS configuration hierarchy considering PFL, DL-PRS resource sets and DL-PRS resources.

Existing measurement requirements for NR positioning in TS 38.133 for RSTD, RTOA, UE/gNB Rx-Tx time difference and for PRS-RSRPP / SRS-RSRPP are based on paths. However, accuracy requirements are only defined for first path RSTD, RTOA, UE Rx-Tx, gNB Rx-Tx and PRS-RSRPP / SRS-RSRPP, although additional paths can be reported but for them, no accuracy requirements exist.

Single PFL and dual PFL requirements are specified in TS 38.133 for RSTD, PRS-RSRP and UE Rx-Tx time difference measurements, but accuracy requirements are distinguished only for RSTD. For dual PFL, accuracy for RSTD is decreased from Y=32 Tc to Y=256 Tc, see clause 10.1.23.2 and furthermore the RF margin is increased as defined in Table 10.1.23.2-5a for FR1 and Table 10.1.23.2-6a for FR2. This accuracy degradation and additional uncertainty for dual PFL will result in more difficult (and longer) training and inherence procedures for AI/ML positioning. Thus, single PFL should be the starting point for AI/ML positioning.

RAN4 requirements (e.g., in Case 1) should be first defined with respect to single PFL.

When collecting data for training UE-side model cases, the measurements (such as channel samples in the form of CIR, PDP, DP) serve as the training input. Consequently, the data collection process that creates the dataset for training models is dependent on the positioning configuration used to obtain these measurements. When a trained model is used in the inference stage, if the current positioning configuration (e.g., PFL) is different from the positioning configuration used in the training stage, it is not effective to use such a trained model for inference.

Therefore, in both training and inference stages, it is essential to remain in the same positioning configuration (i.e., to maintain consistent configuration). To maintain the consistent positioning configuration in training and inference stages, the consistency needs to be checked to verify the same positioning configuration. In particular, the UE and network (e.g., LMF) must ensure the positioning configuration during inference phase matches with the training phase’s positioning configuration. Specifically, such a consistent positioning configuration can be achieved by validating the PF) across training and inference. Hence, the requirements on the measurements can be defined to ensure that same PFL is configured in training and inference.

The consistency between training and inference should be achieved by having a requirement on consistent configuration (e.g., same PFL) when UE-sided models are used.

LCM related aspects: latency requirements for LCM actions

WID [4] also discusses RAN4-related aspects to be considered during WI phase for Beam Management and Positioning use cases. The WID covers core requirements aspects as follows.

- Core requirements for the above three use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above three use cases. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring.

The WID also covers the aspects related to performance part.

- For Beam Management, Positioning Accuracy enhancement and CSI prediction use cases, specify performance requirements and test cases for AI/ML LCM procedures (including performance monitoring) and UE features enabled by UE-sided models - Specify necessary performance requirements and tests (including metrics) for the above-mentioned use cases - Specify necessary test cases and performance requirements for LCM procedure, including performance monitoring.

Also, the agreement related LCM procedures in RAN4#111 are as follows:

Agreement: RAN 4 to consider defining RRM requirements for LCM procedures based on the specific use cases (beam management and positioning) and candidate are: - Identification - Selection - Activation - Deactivation - Switching - Fallback to non-AI operation - Performance monitoring - Others are not precluded

In Case 1, UE can be capable of performing different positioning techniques, e.g., capabilities of the different AIML models/functionalities and legacy positioning techniques while initially having different requirements for each model/functionality and technique/algorithm. If a performance monitoring process detects a performance degradation to a certain point, it is possible to switch the currently running model / functionality with another model / functionality. In the same case, UE can be also enabled to perform a fallback operation to a legacy/default positioning technique and algorithm. For instance, a fallback operation is needed when the measurements can be more suitable for a legacy/default positioning technique than the current model/functionality. It is also possible to utilize AIML feature to determine a measurement requirement feature. If UE’s measurement on the upcoming RS indicates a potential satisfactory of positioning measurements, the UE may be required to process positioning measurements. By avoiding unnecessary processing of positioning measurements with a detected performance degradation, UE can reduce the power consumption. Therefore, when the AI/ML functionality is degrading the system performance lower than any required level associated with the current model/functionality, the continuous operation of such a functionality with the detected performance degradation may have a catastrophic impact on the overall system performance.

Therefore, it is crucial to detect the performance degradation and stop this model/functionality, either by falling back to legacy method or by switching to another model/functionality, within a specified time. The specified time allowed to switch/disable the model/functionality should guarantee that the system performance remains within the acceptable levels.

If an LCM action is required and it is not taken in a timely manner, the performance degradation for AI/ML enabled Positioning use case may be degraded to undesirable level.

The time latency on UE’s LCM actions has an impact on the performance.

In RAN4#111 meeting, various candidates for were considered for defining RRM requirements for LCM procedures based on the specific use cases (beam management and positioning). Since RRM requirements may not be necessary for all listed candidates, RAN4 should prioritize the following candidates with a high impact on RRM requirements:

- Activation

- Deactivation

- Switching

- Fallback to non-AI operation, and

- Performance monitoring.

RAN4 to define RRM requirements for LCM focusing on Activation, Deactivation, Switching, Fallback to non-AI operation, and Performance monitoring.

Model performance monitoring

AIML positioning discussion in RAN1 has been made the agreements on model performance monitoring.

RAN1 #116-bis Agreement For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility, benefits, and potential specification impact of the following options with regard to how to generate information on ground truth label: - Option A. The target UE side performs monitoring metric calculation. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE. - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. - Option A-3. Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE. - Option A-4. PRU measurement (and the corresponding PRU location if not already known at the UE-side) are sent from PRU to the target UE side (e.g., target UE, OTT server). - Note: Option A-4 can be realized by implementation in a manner transparent to specification if the PRU sends information to the target UE side in a proprietary method. - Option B. The LMF performs monitoring metric calculation. - Option B-1. at least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF. - Option B-2. PRU’s channel measurement is sent via LMF to the target UE, and the inference result (i.e., the model output corresponding to PRU’s channel measurement) is sent by the target UE to LMF. Note: exact method to perform the monitoring metric calculation is up to implementation. Note: Other options are not precluded. RAN1 #119 Agreement For model performance monitoring of AI/ML positioning Case 1, support at least: - Option A. The target UE side performs monitoring metric calculation. - The target UE may signal the monitoring outcome to the LMF. - FFS: content of monitoring outcome - FFS: Option B RAN1 #120bis Agreement For model performance monitoring of AI/ML positioning Case 1, “FFS: content of monitoring outcome” in RAN1#119 agreement is resolved by: - the content of monitoring outcome includes at least an indication that the target UE cannot perform the Case 1 positioning method.

In Case 1, the model performance monitoring is expected to run on the UE, and a signal may be sent from the UE to LMF. In Option A-3, the Rel-18 assistance data is sent from LMF to the target UE for PRU measurement. PRU may be a reference UE strategically positioned at a fixed, known location to the LMF. For instance, PRU can be seen a device with a known location that assists in determining the location of a UE. Since the precise location of the PRU is assumed to be known, the PRU measurements may require a high precision. To enhance the positioning accuracy, the PRU measurement may need to have a separate accuracy requirement that is stricter than the requirement applied to a regular UE.

Also, in a recent RAN1#120bis meeting, the content of monitoring outcome includes at least an indication that the target UE cannot perform the Case 1 positioning method. In this case, a requirement can be considered to determine whether target UE can or cannot perform the Case 1 positioning. Thus, RAN4 can discuss whether to study the necessity of the requirements on the performance monitoring in AIML positioning.

RAN4 to discuss the requirements for performance monitoring when target UE indicates that it ‘can’ perform the Case 1 positioning method.

Testing Framework for UE-based Positioning

This section discusses testability aspects and performance test metric in the use cases with a UE-side model and refers to Case 1 (UE based positioning). In view of testability aspects for UE-based positioning, it is a prerequisite that reporting of the estimated location coordinates is enabled. This is considered in the following section.

Reporting of estimated positioning coordinates by UE

In fact, reporting of estimated location coordinates by UE is supported in LPP spec TS 37.055. The spec refers to different location coordinate types based on geographic shapes as defined in TS 23.032:

#### LocationCoordinateTypes The IE LocationCoordinateTypes defines a list of possible geographic shapes as defined in TS 23.032 [15]. -- ASN1START LocationCoordinateTypes ::= SEQUENCE { ellipsoidPoint BOOLEAN, ellipsoidPointWithUncertaintyCircle BOOLEAN, ellipsoidPointWithUncertaintyEllipse BOOLEAN, polygon BOOLEAN, ellipsoidPointWithAltitude BOOLEAN, ellipsoidPointWithAltitudeAndUncertaintyEllipsoid BOOLEAN, ellipsoidArc BOOLEAN, ..., [[ highAccuracyEllipsoidPointWithUncertaintyEllipse-r15 BOOLEAN OPTIONAL, -- Need ON highAccuracyEllipsoidPointWithAltitudeAndUncertaintyEllipsoid-r15 BOOLEAN OPTIONAL -- Need ON ]], [[ ha-EllipsoidPointWithScalableUncertaintyEllipse-r16 BOOLEAN OPTIONAL, -- Need ON ha-EllipsoidPointWithAltitudeAndScalableUncertaintyEllipsoid-r16 BOOLEAN OPTIONAL -- Need ON ]], [[ local2dPointWithUncertaintyEllipse-r18 BOOLEAN OPTIONAL, -- Need ON local3dPointWithUncertaintyEllipsoid-r18 BOOLEAN OPTIONAL -- Need ON ]] } -- ASN1STOP - NOTE 3: In this version of the specification, the GAD shapes local2dPointWithUncertaintyEllipse and local3dPointWithUncertaintyEllipsoid are supported for DL-TDOA and DL-AoD only.

This also aligns with the previous RAN1 #120 agreement replicated below.

Agreement For AI/ML based positioning Case 1, from RAN1 perspective, when the label data of location is generated by LMF and transferred from LMF to UE, label and quality indicator of label can be provided by reusing existing IEs. From RAN1 perspective, the existing IE can use one of the geographic shapes defined in TS 23.032. The location estimate uncertainty and confidence (if included with the geographic shapes) can serve as quality indicator of the label.

That means aside the location coordinate types defined in Rel-15 and Rel-16, Rel-18 allows to configure / report 2D and 3D location coordinates, respectively, with a certain uncertainty, both for DL-TDOA and DL-AoD positioning methods. Thereagainst, for the multi-RTT positioning method, signaling of such location coordinates is not supported.

The estimated location coordinates are included in the IEs NR-DL-TDOA-LocationInformation, as shown below for DL-TDOA positioning method, and NR-DL-AoD-SignalMeasurementInformation, respectively, and are conveyed in the ProvideLocationInformation message to the LMF, if requested by the LMF in the RequestLocationInformation message.

#### NR-DL-TDOA-LocationInformation The IE NR-DL-TDOA-LocationInformation is included by the target device when location information derived using NR DL-TDOA is provided to the location server. -- ASN1START NR-DL-TDOA-LocationInformation-r16 ::= SEQUENCE { measurementReferenceTime-r16 CHOICE { systemFrameNumber-r16 NR-TimeStamp-r16, utc-time-r16 UTCTime, ... } OPTIONAL, ..., [[ locationCoordinates-r17 LocationCoordinates OPTIONAL, -- Cond batch1 locationSource-r17 LocationSource-r13 OPTIONAL -- Cond batch2 ]] } -- ASN1STOP - Conditional presence; - Explanation. batch1; The field is mandatory present if the field nr-DL-TDOA-LocationInformationInstances is present in IE NR-DL-TDOA-ProvideLocationInformation; otherwise it is not present.. batch2; The field is optionally present, need ON, if the field nr-DL-TDOA-LocationInformationInstances is present in IE NR-DL-TDOA-ProvideLocationInformation; otherwise it is not present.. - NR-DL-TDOA-LocationInformation field descriptions. measurementReferenceTime This field specifies the time for which the location estimate is valid.. locationCoordinates This field provides a location estimate using one of the geographic shapes defined in TS 23.032 [15]. NOTE 1.. locationSource This field provides the source positioning technology for the location estimate. NOTE 1.. NOTE 1: In the case of locationCoordinates for multiple NR positioning methods are provided, the locationCoordinates and locationSource shall be present in only one of NR-DL-TDOA-ProvideLocationInformation or NR-DL-AoD-ProvideLocationInformation.

Most of the location coordinate types are based on an “ellipsoid point with uncertainty circle or ellipse”, as seen in Figure 2 below.

Figure 2. Description of an uncertainty Ellipse [TS 23.032]

Coding both of the point and of the uncertainty is also specified in clause 6 of TS 23.032. For both parameters, higher and lower accuracy definitions exist. For instance, for the uncertainty, for the case of high accuracy, the range 0m to 46.5m is defined in TS 23.023 with a non-equal granularity of 6mm or higher, as depicted in the table of TS 23.023 below. Other resolutions for the uncertainty are lower.

Table 6.2a-1: Example values for the high accuracy uncertainty function

| Value of K | Value of uncertainty |
| --- | --- |
| 0 | 0 m |
| 1 | 0.006 m |
| 2 | 0.01212 m |
| - | - |
| 20 | 0.14578 m |
| - | - |
| 40 | 0.36241 m |
| - | - |
| 60 | 0.68430 m |
| - | - |
| 80 | 1.16263 m |
| - | - |
| 100 | 1.87339 m |
| - | - |
| 120 | 2.92954 m |
| - | - |
| 127 | 3.40973 m |
| - | - |
| 255 | 46.49129 m |

In conclusion, reporting of location estimates is supported in the current specifications. Reuse of this reporting facility for Case1 is hence possible, given that a certain location coordinate type from the existing ones is selected. The UE then is enabled to report its local 3D coordinates with origin in a known reference location together with its estimated location uncertainty, both inferred through its AI/ML model.

Reporting of location estimates is supported in the current specifications.

Reuse of the reporting facility for Case 1 is possible, given that a certain location coordinate type from the existing ones is selected.

The UE is enabled to report its local 3D coordinates with origin in a known reference location together with its estimated location uncertainty, both inferred through its AI/ML model.

RAN4 to use existing location coordinate reporting to identify positioning accuracy requirements for Case 1.

Testability aspects in Case 1

In RAN4#110, we discussed the testability of Case 1 where the test metric in Case 1 can be the positioning accuracy.  Therefore, in this paper, we further discuss the testability aspects of positioning accuracy test metric. To define the feasible requirements for Case 1, it is useful to consider a possible test framework with the four essential steps of identification, setting, running, and evaluation as shown in Figure 3.

Figure 3. Test framework in Case 1

When AI/ML inference results consist of positioning coordinates, the AI/ML functionality can be verified based on the ground truth. The ground truth may consist of the location point(s) with known positioning co-ordinates, for example, in form of a test area/grid as shown in Figure 4. To know the co-ordinates of each location point, a PRU (i.e., Positioning Reference Unit location is known to gNB/LMF) is placed, or the position co-ordinates of each location point form a known reference location derived by, e.g., using GNSS based positioning. In that case, while UE moves from one location point to another location point, the AI/ML inference results at each location corresponding to the location point is reported to the test equipment and compared to the ground-truth information for verification of positioning accuracy.

Figure 4. Grid map providing a ground-truth information for inference evaluation

The test setup can be implemented for a field test. If testing is performed in a limited indoor space, the same test setup can be scaled down to be fitted into the laboratory space. For instance, the test map granularity may be further reduced to a certain level depending on the achievable precision of an AI/ML-based positioning technique, and configured bandwidth size, and channel conditions.

Positioning accuracy can be verified based on the ground truth which may consist of the location points with known positioning co-ordinates (e.g., PRU or GNSS based).

RAN4 should specify (i) the test mechanisms for positioning accuracy and (ii) the metric for positioning accuracy performance in Case 1.

Regarding test mechanisms for positioning accuracy, a grid map in a limited indoor space should be defined to derive the ground truth. The UE is placed on one of these location points and receives PRS resources from a set of TRPs with known location, the set size may be e.g. four. The PRS resources are on the same PFL. The measurement period is derived from the set size of TRPs for single PFL according to TS 38.133. The UE reports its position co-ordinates to the test system (emulating LMF), which compares the reported co-ordinates to ground-truth.

RAN4 to specify a test grid map with known location points in indoor space and an appropriate set of TRPs for the positioning measurement in Case 1.

Performance test metric in Case 1

Assuming the reuse of the existing location coordinate types, as described above, we observe the following issue depicted in Figure 5 related to how to calculate the positioning error.

Figure 5. Example of different calculation methods for the 2D positioning error depending on whether to account for UE location uncertainties Ux and Uy.

The above shown grid map provides the ground-truth information for inference evaluation. The UE reports both its estimated location point and the uncertainty ellipse in case of 2D reporting consisting of semi-axis Ux and Uy. Different calculation methods are possible.

- Error 1 calculation method takes into account the reported location uncertainty and calculates the distance of the closest point of the uncertainty shape to the actual UE location. This refers to a best-case scenario.

- Error 2 calculation method does not take into account the reported location uncertainty and calculates the distance of the estimated UE location (center of uncertainty shape) without location uncertainties to the actual UE location. This refers to an average scenario.

- Error 3 calculation method takes into account the reported location uncertainty and calculates the distance of the furthest point of the uncertainty shape to the actual UE location. This refers to a worst-case scenario.

On one hand, performance may be preserved if the actual UE location is covered by the uncertainty shape, on the other side, a UE may ensure this by simply increasing the location uncertainties. The objective, however, is to reduce the uncertainty shape as much as possible. Thus, error calculation methods 2 or 3 are preferred to be used for determining the performance metric for Case 1.

RAN4 to select either error calculation method 2 (without location uncertainty) or 3 (applying worst case uncertainty error) for deriving the performance metric for Case 1.

Based on the above and more generally, the metric for positioning accuracy for Case 1 identifies the three-dimensional error e of the absolute reported positioning co-ordinates relative to the evaluated ground truth, i.e. Δx, Δy and Δz, using a Local Coordinate System (LCS) or a Global Coordinate System (GCS) reference and

- the estimated location reported by UE without location uncertainties or

- the estimated location reported by UE offset by the 3D location uncertainties, such that the 3D error distance is maximized.

It can be derived according to Eq.1:

e= Δx2+Δy2+Δz2	 (Eq.1)

RAN4 to select the three-dimensional error e of the absolute reported positioning co-ordinates relative to the evaluated ground truth as performance metric for the positioning accuracy in Case 1.

- 2.2 Requirements for Case 3a/3b

- 2.2.1 Testing Framework for gNB-assisted Positioning

This section discusses testability aspects and performance test metric in the use cases with a gNB-side model (use cases 3a/3b).

Testability aspects and Performance test metric in Case 3a

For Case 3a, RAN1 has defined path-based measurements (Part A) and the label (Part B), which may include timing information and quality indicator for timing information or LOS/NLOS indicator according to RAN1#120bis agreements on data collection replicated below.

Agreement For training data collection of Part B in AI/ML based positioning Case 3a, for the case when Part B label includes timing information, support the following for providing label and quality indicator of label, - The corresponding label is provided by LMF to gNB in the format of timing information. “Timing Measurement Quality” in 38.455 can serve as quality indicator of the label. Note: It is assumed that user data privacy of non-PRU UE is preserved. Conclusion For training data collection of Part B in AI/ML based positioning Case 3a, for the case when Part B label includes the LOS/NLOS indicator, - The corresponding label is provided from LMF to the gNB by using the existing IE LOS-NLOS-Indicator-r17. There is no need of a separate field for quality indicator of LOS/NLOS indicator.

From RAN4 perspective, neither a performance test metric nor a test framework needs to be defined when path-based measurements and Part B label and quality indicator of label are reported by gNB in Case 3a. Hence, RAN4 does not have to specify a test framework for path-based measurements and label-based information.

RAN4 to not specify a test framework for path-based measurements and label-based information in Case 3a.

- Testability aspects and Performance test metric in Case 3b

For Case 3b, RAN1#117 agreed to introduce, aside the existing legacy Rel-18 path-based measurements, sample-based measurements of the estimated channel response in time domain. RAN1#119 further agreed the starting point and the length Nt of continuous samples as well as reporting of Nt’ samples with timing information reported relative to a reference time. The values for Nt and Nt’ are still under discussion in RAN1.

Agreement​ For the definition of sample-based measurement, for gNB/TRP measurement of an estimated channel response between a pair of UE and TRP, the starting time of the list of Nt consecutive samples is determined as follows.​ - starting time = first detected path rounded down with timing granularity T.​ Note: UE-side measurement is a separate discussion​

Agreement​ For Rel-19 AI/ML based positioning, for Case 3b, in addition to path-based measurement that is referring to the measurement in the existing specifications (up to Rel-18), additionally support the following enhancement to the measurement, ​ - The measurement is composed of Nt' values of the estimated channel response in time domain. The Nt’ values are selected from a list of Nt consecutive channel response values, which have timing granularity T. ​ - The timing information for the Nt' values are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR. ​ - The associated measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' values.​ - The timing information is defined relative to a reference time, same as the path-based measurement. ​ - The Nt’ selected time domain channel measurement values are expected to be those with the highest power.​ - The starting time of the list of Nt consecutive values is determined as: starting time = first detected path rounded down with timing granularity T.​ - LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. Candidate set values:​ - Nt'<=24. FFS: Nt' values. ​ - Nt = {32, 64, 128}​ - FFS: k ​ - The gNB/TRP may use different Nt', Nt and/or k values other than the signalled parameter for measurement reporting. In this case, it’s up to LMF implementation to process the reported measurement​ - FFS: whether transmit offset from gNB to LMF​ Note: measurement by UE is a separate discussion.

At RAN1#120bis, the following conclusions/agreements were achieved.

Conclusion - The offset (used in the agreement made in RAN1#119) refers to the duration (in samples) between the reference time and the starting time of the list of Nt consecutive values. Agreement For Rel-19 AI/ML based positioning, for Case 3b, “FFS: whether transmit offset from gNB to LMF” in RAN1#119 agreement is resolved by: - No offset is transmitted from gNB to LMF Conclusion For channel measurement type (A) (i.e., path-based measurement), it is not necessary to introduce enhancements on the number of reported paths in Rel-19.

No enhancements to path-based measurements are introduced. Note, the discussion is ongoing in RAN1, whether sample-based measurements for PDP and DP should also be introduced.

For Case 3b, RAN1 has defined path-based and sample-based measurements. From RAN4 perspective, neither a performance test metric nor a test framework needs to be defined when path-based or sample-based measurements are reported by gNB in Case 3b. Hence, RAN4 does not have to specify a test framework for path-based or sample-based measurements.

RAN4 to not specify a test framework for path-based and sample-based measurements in Case 3b.

Conclusion

In this paper we share our views on potential RAN4 impacts from issues related to AI/ML based positioning. In the paper, the following observations and proposals were made:

Case 1

- For AIML positioning in Case 1, RAN4 to specify the requirements on the measurements supported in legacy positioning (e.g., RSTD, PRS-RSRP, and PRS-RSRPP) and define the requirements on the new measurements (e.g., CIR, DP, and PDP) based on RAN1 progress.

- RAN4 to study the necessity of any requirements for the input of the AIML model in Case 1.

- RAN4 requirements (e.g., in Case 1) should be first defined with respect to single PFL.

- The consistency between training and inference should be achieved by having a requirement on consistent configuration (e.g., same PFL) when UE-sided models are used.

- If an LCM action is required and it is not taken in a timely manner, the performance degradation for AI/ML enabled Positioning use case may be degraded to undesirable level.

The time latency on UE’s LCM actions has an impact on the performance.

RAN4 to define RRM requirements for LCM focusing on Activation, Deactivation, Switching, Fallback to non-AI operation, and Performance monitoring.

RAN4 to discuss the requirements for performance monitoring when target UE indicates that it ‘can’ perform the Case 1 positioning method.

Reporting of location estimates is supported in the current specifications.

Reuse of the reporting facility for Case 1 is possible, given that a certain location coordinate type from the existing ones is selected.

The UE is enabled to report its local 3D coordinates with origin in a known reference location together with its estimated location uncertainty, both inferred through its AI/ML model.

RAN4 to use existing location coordinate reporting to identify positioning accuracy requirements for Case 1.

Positioning accuracy can be verified based on the ground truth which may consist of the location points with known positioning co-ordinates (e.g., PRU or GNSS based).

RAN4 should specify (i) the test mechanisms for positioning accuracy and (ii) the metric for positioning accuracy performance in Case 1.

RAN4 to specify a test grid map with known location points in indoor space and an appropriate set of TRPs for the positioning measurement in Case 1.

RAN4 to select either error calculation method 2 (without location uncertainty) or 3 (applying worst case uncertainty error) for deriving the performance metric for Case 1.

RAN4 to select the three-dimensional error e of the absolute reported positioning co-ordinates relative to the evaluated ground truth as performance metric for the positioning accuracy in Case 1.

- Case 3a/3b

RAN4 to not specify a test framework for path-based measurements and label-based information in Case 3a.

RAN4 to not specify a test framework for path-based and sample-based measurements in Case 3b.

- References

- RP-221348, Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, RAN#96, Budapest, Hungary, June 06 – June 09, 2022.

- RP-233132, Status report for SI on AI/ML for NR air interface, Qualcomm, RAN#102, Edinburgh, Scotland, December 11 – December 15, 2023.

- 3GPP TR 38.843 V18.0.0, Technical Report (TR): Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface.

- RP-250792, Revised WID on AI/ML for NR air interface, RAN#107, Incheon, S. Korea, March 12-14, 2025.