3GPP TSG-RAN4 Meeting #115	R4-2507566

# Malta, Malta, 19th May 2025 - 23rd May 2025

Agenda item:			7.8.1

Source:	Moderator (Ericsson)

Title:	Topic summary for [115][127] NR_FR1_7MHz_BW

Document for:	Information

# Introduction

The scope of this summary is the introduction of 7MHz Channel Bandwidth for n26 and n5 WI, summarizing contributions submitted under AI 7.8.2-4

Based on the agenda, this summary is divided in 4 topics: the first one handles some general aspect (e.g. Work Plan), the second one handles the system parameters and UE RF aspects, the third one is related to BS RF impacts while the last one is handling draft CRs.

The proposal for this meeting would be to reach agreement on the following remaining issues:

- Issue 1-1 (initial BWP size)

- Sub-topics 2-1 and 2-2 (A-MPR for NS_12, NS_13, NS_14 and NS_15). We should at least agree on the regions for the different NS for PC3 and possibly PC2.

- Issue 3-1-1 (NS IM offset)

- Issue 3-1-2 (ICS interferer)

- Sub-topic 4-1 (notes in CR/draftCR)

# Topic #1: General

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2506149 | Ericsson | Observation: 7 MHz could only be the maximum channel bandwidth in a band, not the minimum. Proposal: Reply to RAN2 LS that 7 MHz could be the minimum or the maximum channel bandwidth supported by a band in a bands’ combination and the required signalling should be introduced in Rel-19 (or earlier release). 7 MHz could only be a maximum channel bandwidth in a band, not the minimum. |
| R4-2506185 | T-Mobile USA | Reply LS to RAN2 on Signalling for 7 MHz Channel Bandwidth |

## Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 1-1

Sub-topic description: Reply to RAN2 LS

Issue 1-1: Reply to RAN2 LS

- Proposals: Agree with Reply LS to RAN2 R4-2506185 which mentions that 7MHz channel BW could be the maximum but not the minimum channel BW for a band, and it could be the maximum and the minimum channel BW for CA:

  - Agree

  - Disagree

- Recommended WF

  - The Reply LS R4-2506185 should be agreeable, this is aligned with the previous LS R4-2503017 that RAN4 sent to RAN2 in RAN#114 meeting.

# Topic #2: System parameters and UE RF

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2505428 | Apple | A-MPR requirements proposals |
| R4-2505719 | Huawei Technologies France | A-MPR requirements proposals |
| R4-2506062 | CATT | A-MPR requirements proposals |
| R4-2506149 | Ericsson | A-MPR requirements proposals |
| R4-2506318 | ZTE Corporation, Sanechips | A-MPR requirements proposals |
| R4-2506518 | Nokia | A-MPR requirements proposals |
| R4-2507651 | Qualcomm Incorporated | A-MPR requirements proposals Note that Qualcomm sent an update on the reflector which has been taken into account in this summary |
| R4-2507653 | Qualcomm Incorporated | PC2 UE : Band n26 A-MPR Analysis for 7 MHz Channel Bandwidth |

## Open issues summary

### Sub-topic 2-1

Sub-topic description: PC3 A-MPR.

It’s suggested to first agree on the A-MPR regions for each NS and then discuss the corresponding A-MPR values for each region.

Issue 2-1-2: NS_12: A-MPR regions

- Proposals: NS_12 A-MPR definition for 7 MHz channel BW:

  - Option 1 (Apple)

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz | 817.5 ≤ Fc < 820.5 | ≤1.8 | ≤0.72 | A3 |
|  |  | ≤1.8 | >0.72 | A1 |
|  |  | >1.8 | >1.44 | A1 |
|  | 820.5 ≤ Fc | ≤0.72 | >0 | A3 |
|  |  | >0.72 | >2.52 | A3 |

Table 2: A-MPR for NS_12 (PC3)

| Modulation/Waveform | A1 | A2 |
| --- | --- | --- |
|  | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 5 | ≤ 4 |
| DFT-s-OFDM QPSK | ≤ 5 | ≤ 4.5 |
| DFT-s-OFDM 16 QAM | ≤ 5.5 | ≤ 4.5 |
| DFT-s-OFDM 64 QAM | ≤ 5.5 | ≤ 5 |
| DFT-s-OFDM 256 QAM | ≤ 9.5 | ≤ 5 |
| CP-OFDM QPSK | ≤ 7 | ≤ 6 |
| CP-OFDM 16 QAM | ≤ 7 | ≤ 6 |
| CP-OFDM 64 QAM | ≤ 7 | ≤ 6 |
| CP-OFDM 256 QAM | ≤ 9.5 |  |

  - Option 2 (Huawei)

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz | 817.5 ≤ Fc ≤ 820.5 | ≤2.16 | >0 | A3 |

Table 2: A-MPR for NS_12

| Modulation/Waveform | A3 |
| --- | --- |
|  | Outer/Inner |
| DFT-s-OFDM QPSK | ≤ 4 |
| DFT-s-OFDM 16 QAM | ≤ 5 |
| DFT-s-OFDM 64 QAM | ≤ 5 |
| DFT-s-OFDM 256 QAM |  |
| CP-OFDM QPSK | ≤ 6.5 |
| CP-OFDM 16 QAM | ≤ 6.5 |
| CP-OFDM 64 QAM | ≤ 6.5 |
| CP-OFDM 256 QAM |  |
| NOTE: for 256QAM MPR would suffice and no A-MPR is needed. |  |

  - Option 3 (CATT)

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 3MHz | 815.5 ≤ Fc < 817.5 | ≤0.9 | >0 | A2 |
| 5MHz |  | ≤1.8 | >0 | A1 |
| 7MHz |  | ≤2.52 | >0 | A1 |
| 10MHz |  | ≤3.6 | >0 | A1 |

  - Option 4 (Ericsson)

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz | 817.5 ≤ Fc ≤ 820.5 | ≤2.7 | >0 | A1 |

  - Option 5 (ZTE)

  - Option 6 (Nokia)

Table 6.2.3.21-1: A-MPR regions for NS_12

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 3MHz | 815.5 ≤ Fc < 817.5 | ≤0.9 | >0 | A2 |
| 5MHz |  | ≤1.8 | >0 | A1 |
| 7MHz |  | ≤2.16 | >0 | A3 |
| 10MHz |  | ≤3.6 | >0 | A1 |

Table 6.2.3.21-2: A-MPR for NS_12

| Modulation/Waveform | A1 | A2 | A3 |
| --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 5 | ≤ 4 | 3 |
| DFT-s-OFDM QPSK | ≤ 5 | ≤ 4.5 | 4 |
| DFT-s-OFDM 16 QAM | ≤ 5.5 | ≤ 4.5 | 4 |
| DFT-s-OFDM 64 QAM | ≤ 5.5 | ≤ 5 | 4 |
| DFT-s-OFDM 256 QAM | ≤ 9.5 | ≤ 5 |  |
| CP-OFDM QPSK | ≤ 7 | ≤ 6 | 5 |
| CP-OFDM 16 QAM | ≤ 7 | ≤ 6 | 5 |
| CP-OFDM 64 QAM | ≤ 7 | ≤ 6 | 5 |
| CP-OFDM 256 QAM | ≤ 9.5 |  |  |

  - Option 7 (Qualcomm – updated version)

Table 1: A-MPR regions for NS_12

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 3MHz | 815.5 ≤ Fc < 817.5 | ≤0.9 | >0 | A2 |
| 5MHz |  | ≤1.8 | >0 | A1 |
| 7 MHz | Fc ≤ 817.5 | ≤1.88 | > 0 | A3 |
|  |  | > 1.88 | > 1.44 | A4 |
| 10MHz |  | ≤3.6 | >0 | A1 |

Table 2: A-MPR for NS_12

| Modulation/Waveform | A1 | A2 | A3 | A4 |
| --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 5 | ≤ 4 | ≤ 5 | ≤ 4 |
| DFT-s-OFDM QPSK | ≤ 5 | ≤ 4.5 | ≤ 5 | ≤ 4 |
| DFT-s-OFDM 16 QAM | ≤ 5.5 | ≤ 4.5 | ≤ 5.5 | ≤ 4 |
| DFT-s-OFDM 64 QAM | ≤ 5.5 | ≤ 5 | ≤ 5.5 | ≤ 4.5 |
| DFT-s-OFDM 256 QAM | ≤ 9.5 | ≤ 5 | ≤ 5.5 | ≤ 4.5 |
| CP-OFDM QPSK | ≤ 7 | ≤ 6 | ≤ 7 | ≤ 5.5 |
| CP-OFDM 16 QAM | ≤ 7 | ≤ 6 | ≤ 7 | ≤ 5.5 |
| CP-OFDM 64 QAM | ≤ 7 | ≤ 6 | ≤ 7 | ≤ 5.5 |
| CP-OFDM 256 QAM | ≤ 9.5 |  | ≤ 7 | ≤ 5.5 |

- Recommended WF

  - Discuss offline to merge A-MPR regions and then discuss corresponding values.

Issue 2-1-2: NS_13: A-MPR regions

- Proposals: NS_13 A-MPR definition for 7 MHz channel BW:

  - Option 1 (Apple)

Table 4: A-MPR regions for NS_13 (Power Class 3)

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz |  | ≤1.8 | <1.44 | A1 |
|  | 820.5 ≤ Fc | ≤1.8 | ≥1.44 | A2 |
|  |  | >1.8 | ≥1.8 | A1 |

Table 5: A-MPR for NS_13 (Power Class 3)

| Modulation/Waveform | A1 | A2 | A3 | A4 |
| --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 3.5 | ≤ 4.5 | ≤ 3 | ≤ 3 |
| DFT-s-OFDM QPSK | ≤ 3.5 | ≤ 4.5 | ≤ 3 | ≤ 4 |
| DFT-s-OFDM 16 QAM | ≤ 3.5 | ≤ 5 | ≤ 3 | ≤ 4 |
| DFT-s-OFDM 64 QAM | ≤ 4.5 | ≤ 5 | ≤ 3 | ≤ 4 |
| DFT-s-OFDM 256 QAM | ≤ 8 | ≤ 6 |  |  |
| CP-OFDM QPSK | ≤ 5 | ≤ 6.5 | ≤ 4.5 | ≤ 5.5 |
| CP-OFDM 16 QAM | ≤ 5 | ≤ 6.5 | ≤ 4.5 | ≤ 5.5 |
| CP-OFDM 64 QAM | ≤ 6 | ≤ 6.5 | ≤ 4.5 | ≤ 5.5 |
| CP-OFDM 256 QAM | ≤ 8 | ≤ 8 |  |  |

  - Option 2 (Huawei)

Table 3: A-MPR regions for NS_13

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz | Fc = 820.5 MHz | ≤1.98 | >0 | A4 |

Table 4: A-MPR for NS_13

| Modulation/Waveform | A4 |
| --- | --- |
| DFT-s-OFDM QPSK | ≤ 4.5 |
| DFT-s-OFDM 16 QAM | ≤ 5 |
| DFT-s-OFDM 64 QAM | ≤ 5 |
| DFT-s-OFDM 256 QAM |  |
| CP-OFDM QPSK | ≤ 6.5 |
| CP-OFDM 16 QAM | ≤ 6.5 |
| CP-OFDM 64 QAM | ≤ 6.5 |
| CP-OFDM 256 QAM |  |
| NOTE: for 256QAM MPR would suffice and no A-MPR is needed. |  |

  - Option 3 (Ericsson)

The following A-MPR area and corresponding A-MPR value could be considered for NS_13 (QPSK):

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz | 820.5 | ≤2.16 | <1.62 | A4 |
|  |  | ≤2.16 | ≥1.62 | A3 |

  - Option 4 (ZTE)

Table 6.2.3.22-1  A-MPR regions for NS_13

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 3MHz | 818.5 ≤ Fc < 820 | ≤0.54 | >0 | A4 |
| 5MHz | 819.5 ≤ Fc < 821.5 | ≤1.44 | <1.08 | A1 |
|  |  | ≤1.44 | ≥1.08 | A2 |
| 5MHz | Fc ≥ 821.5 | ≤0.54 | <1.08 | A1 |
|  |  |  | ≥3.24 | A3 |
| 7MHz | Fc ≥ 820.5 | ≤2.70 | <1.44 | A1 |
|  |  | ≤2.70 | ≥1.44 | A2 |

Table 6.2.3.22-2: A-MPR for NS_13

| Modulation/Waveform | A1 | A2 | A3 | A4 |
| --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 3.5 | ≤ 4.5 | ≤ 3 | ≤ 3 |
| DFT-s-OFDM QPSK | ≤ 3.5 | ≤ 4.5 | ≤ 3 | ≤ 4 |
| DFT-s-OFDM 16 QAM | ≤ 3.5 | ≤ 5 | ≤ 3 | ≤ 4 |
| DFT-s-OFDM 64 QAM | ≤ 4.5 | ≤ 5 | ≤ 3 | ≤ 4 |
| DFT-s-OFDM 256 QAM | ≤ 8 | ≤ 6 |  |  |
| CP-OFDM QPSK | ≤ 5 | ≤ 6.5 | ≤ 4.5 | ≤ 5.5 |
| CP-OFDM 16 QAM | ≤ 5 | ≤ 6.5 | ≤ 4.5 | ≤ 5.5 |
| CP-OFDM 64 QAM | ≤ 6 | ≤ 6.5 | ≤ 4.5 | ≤ 5.5 |
| CP-OFDM 256 QAM | ≤ 8 | ≤ 8 |  |  |

  - Option 5 (Nokia)

Table 6.2.3.22-1: A-MPR regions for NS_13

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 3MHz | 818.5 ≤ Fc < 820 | ≤0.54 | >0 | A4 |
| 5MHz | 819.5 ≤ Fc < 821.5 | ≤1.44 | <1.08 | A1 |
|  |  | ≤1.44 | ≥1.08 | A2 |
| 5MHz | Fc ≥ 821.5 | ≤0.54 | <1.08 | A1 |
|  |  |  | ≥3.24 | A3 |
| 7MHz | Fc = 820.5 | ≤1.62 | <1.44 | A5 |
|  |  | ≤1.98 | ≥1.44 | A6 |

Table 6.2.3.22-2: A-MPR for NS_13

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 | A6 |
| --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 3.5 | ≤ 4.5 | ≤ 3 | ≤ 3 | 2 | 2.5 |
| DFT-s-OFDM QPSK | ≤ 3.5 | ≤ 4.5 | ≤ 3 | ≤ 4 | 2 | 3.5 |
| DFT-s-OFDM 16 QAM | ≤ 3.5 | ≤ 5 | ≤ 3 | ≤ 4 | 2.5 | 3.5 |
| DFT-s-OFDM 64 QAM | ≤ 4.5 | ≤ 5 | ≤ 3 | ≤ 4 |  | 3.5 |
| DFT-s-OFDM 256 QAM | ≤ 8 | ≤ 6 |  |  |  |  |
| CP-OFDM QPSK | ≤ 5 | ≤ 6.5 | ≤ 4.5 | ≤ 5.5 | 3.5 | 4.5 |
| CP-OFDM 16 QAM | ≤ 5 | ≤ 6.5 | ≤ 4.5 | ≤ 5.5 | 3.5 | 4.5 |
| CP-OFDM 64 QAM | ≤ 6 | ≤ 6.5 | ≤ 4.5 | ≤ 5.5 | 3.5 | 4.5 |
| CP-OFDM 256 QAM | ≤ 8 | ≤ 8 |  |  |  |  |

  - Option 6 (Qualcomm updated)

Table 3: A-MPR regions for NS_13

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 3MHz | 818.5 ≤ Fc < 820 | ≤0.54 | >0 | A4 |
| 5MHz | 819.5 ≤ Fc < 821.5 | ≤1.44 | <1.08 | A1 |
|  |  | ≤1.44 | ≥1.08 | A2 |
| 5MHz | Fc ≥ 821.5 | ≤0.54 | <1.08 | A1 |
| 7 MHz | Fc = 820.5 | < 1.8 | ≤ 0.36, ≥ 1.44 | A5 |
|  |  | < 1.8 | > 0.36, < 1.44 | A6 |
|  |  | ≥ 1.8 | ≥ 1.8 | A7 |

Table 4: A-MPR for NS_13

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 | A6 | A7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 3.5 | ≤ 4.5 | ≤ 3 | ≤ 3 | ≤ 4.5 | ≤ 3 | ≤ 3.5 |
| DFT-s-OFDM QPSK | ≤ 3.5 | ≤ 4.5 | ≤ 3 | ≤ 4 | ≤ 4.5 | ≤ 3 | ≤ 3.5 |
| DFT-s-OFDM 16 QAM | ≤ 3.5 | ≤ 5 | ≤ 3 | ≤ 4 | ≤ 4.5 | ≤ 3 | ≤ 3.5 |
| DFT-s-OFDM 64 QAM | ≤ 4.5 | ≤ 5 | ≤ 3 | ≤ 4 | ≤ 4.5 | ≤ 3 | ≤ 3.5 |
| DFT-s-OFDM 256 QAM | ≤ 8 | ≤ 6 |  |  | ≤ 4.5 | ≤ 3.5 | ≤ 3.5 |
| CP-OFDM QPSK | ≤ 5 | ≤ 6.5 | ≤ 4.5 | ≤ 5.5 | ≤ 6 | ≤ 4.5 | ≤ 4.5 |
| CP-OFDM 16 QAM | ≤ 5 | ≤ 6.5 | ≤ 4.5 | ≤ 5.5 | ≤ 6 | ≤ 4.5 | ≤ 4.5 |
| CP-OFDM 64 QAM | ≤ 6 | ≤ 6.5 | ≤ 4.5 | ≤ 5.5 | ≤ 6 | ≤ 4.5 | ≤ 4.5 |
| CP-OFDM 256 QAM | ≤ 8 | ≤ 8 |  |  | ≤ 6 | ≤ 4.5 | ≤ 4.5 |

- Recommended WF

Discuss offline to merge A-MPR regions and then discuss corresponding values.

Issue 2-1-2: NS_14: A-MPR regions

- Proposals: NS_14 A-MPR definition for 7 MHz channel BW:

  - Option 1 (Apple): no A-MPR is needed for 7MHz CBW with PC3 and PC2

  - Option 2 (Huawei): No A-MPR for NS_14 with 7MHz CBW.

  - Option 3 (Ericsson): no A-MPR needed

  - Option 4 (ZTE): Propose not to consider NS_14 for 7MHz CBW in band n26.

  - Option 5 (Nokia): no A-MPR for 7 MHz CBW with n26 PC3 NS_14.

  - Option 6 (Qualcomm): no impact on A-MPR due to NS_14

- Recommended WF

  - No A-MPR is needed for NS_14 and 7 MHz channel bandwidth.

Issue 2-1-2: NS_15: A-MPR regions

- Proposals: NS_15 A-MPR definition for 7 MHz channel BW:

  - Option 1 (Apple)

Table 7: A-MPR regions for NS_15 (Power Class 3)

| Channel BW | Carrier Frequency, Fc, MHz | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz | 841.5 < Fc ≤ 845.5 | ≥4.68 | >0 | A1 |
|  |  | <4.68, ≥2.52 | ≥2.16 | A2 |
|  |  | <1.8 | ≤0.36 | A3 |
|  | 837 < Fc ≤ 841.5 | ≥0 | ≥2.7 | A2 |
|  |  | ≥5.94 | <2.7 | A3 |

Table 8: A-MPR for NS_15 (Power Class 3)

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 | A6 |
| --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Edge/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 | ≤ 5 | 1 |
| DFT-s-OFDM QPSK | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 | ≤ 5.5 | 1.5 |
| DFT-s-OFDM 16 QAM | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 | ≤ 5.5 | 2 |
| DFT-s-OFDM 64 QAM | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 | ≤ 5.5 |  |
| DFT-s-OFDM 256 QAM | ≤ 9 | ≤ 5 | ≤ 9 | ≤ 13.5 | ≤ 6 |  |
| CP-OFDM QPSK | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 10.5 | ≤ 7 | 3 |
| CP-OFDM 16 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 10.5 | ≤ 7 | 3 |
| CP-OFDM 64 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 10.5 | ≤ 7 |  |
| CP-OFDM 256 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 9 | ≤ 13.5 | ≤ 7.5 |  |

  - Option 2 (Huawei)

Table 6: A-MPR regions for NS_15

| Channel BW | Carrier Frequency, Fc, MHz | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz | 845 < Fc ≤ 845.5 |  | ≥3.6 (Zone1) | A7 |
|  |  | ≥3.24, <6.3(Zone2)1 | ≥1.98, <3.6(Zone2) | A2 |
|  |  | ≥4.14, <6.3(Zone3) 2 | ≥1.44, <3.6(Zone3) | A1 |
|  |  | ≥4.5, <6.3(Zone4)3 | ≥0.9(Zone4) | A2 |
|  |  | ≥4.5 | <0.9(Zone5) | A2 |
|  | 838 < Fc ≤ 841.5 |  | ≥4.5 (Zone1) | A8 |
| NOTE 1: 1.26 ≤ (RBend- LCRB) *12*SCS (MHz) < 2.7 NOTE 2: 2.7 ≤ (RBend- LCRB) *12*SCS (MHz) < 3.6 NOTE 3: 3.6 ≤ (RBend- LCRB) *12*SCS (MHz) |  |  |  |  |

Table 7: A-MPR for NS_15

| Modulation/Waveform | A1 | A2 | A3 | A7 | A8 |
| --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 9 | ≤ 5 | ≤ 4 |  |  |
| DFT-s-OFDM QPSK | ≤ 9 | ≤ 5 | ≤ 4 | 11.5 | 2 |
| DFT-s-OFDM 16 QAM | ≤ 9 | ≤ 5 | ≤ 4 | 12.5 | 3.5 |
| DFT-s-OFDM 64 QAM | ≤ 9 | ≤ 5 | ≤ 4 | 12.5 | 3.5 |
| DFT-s-OFDM 256 QAM | ≤ 9 | ≤ 5 | ≤ 9 | 12.5 | 4 |
| CP-OFDM QPSK | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 12 | 5 |
| CP-OFDM 16 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 12.5 | 5 |
| CP-OFDM 64 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 13 | 5 |

  - Option 5 (Nokia)

Table 6.2.3.24-1: A-MPR regions for NS_15

| Channel BW | Carrier Frequency, Fc, MHz | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz | Fc = 845.5 | ≥ 0 | ≥ -1.7*12*SCS*RBend + 8.2 | A7 |
|  |  | ≤ 1.26 | ≥ 12*SCS*RBend – 0.15 | A8 |
|  | 838.1 < Fc ≤ 841.5 | ≥ 4.68 | ≥ -0.7*12*SCS*RBend + 7.8 | A9 |

Table 6.2.3.24-2: A-MPR for NS_15

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 | A6 | A7 | A8 | A9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Edge/Inner | Edge/Inner | Edge/Inner | Edge/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 | ≤ 5 | 1 | 10.5 | 1 | 1 |
| DFT-s-OFDM QPSK | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 | ≤ 5.5 | 1.5 | 9 |  | 2 |
| DFT-s-OFDM 16 QAM | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 | ≤ 5.5 | 2 | 9 |  | 3.5 |
| DFT-s-OFDM 64 QAM | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 | ≤ 5.5 |  | 9 |  | 3.5 |
| DFT-s-OFDM 256 QAM | ≤ 9 | ≤ 5 | ≤ 9 | ≤ 13.5 | ≤ 6 |  | 9 |  |  |
| CP-OFDM QPSK | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 10.5 | ≤ 7 | 3 | 9.5 |  | 4.5 |
| CP-OFDM 16 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 10.5 | ≤ 7 | 3 | 9.5 |  | 4.5 |
| CP-OFDM 64 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 10.5 | ≤ 7 |  | 9.5 |  | 4.5 |
| CP-OFDM 256 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 9 | ≤ 13.5 | ≤ 7.5 |  | 9.5 |  |  |

  - Option 6 (Qualcomm)

Table 5: A-MPR regions for NS_15

| Channel BW | Carrier Frequency, Fc, MHz | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 3MHz | 844.5 < Fc ≤ 847.5 | ≥1.44 | >0 | A5 |
|  |  | ≤ 0.18 | ≤ 0.36 | A6 |
| 5MHz | 840.5 < Fc ≤ 846.5 | ≥3.24 | >0 | A1 |
|  |  | <3.24, ≥2.52 | ≥1.44 | A2 |
|  |  | <0.9 | ≤0.36 | A3 |
| 7 MHz | 841.5 < Fc ≤ 845.5 | ≥ 4.5 | > 0 | A7 |
|  |  | < 4.5, ≥ 2.16 | ≥ 1.8 | A2 |
|  |  | ≤ 1.44 | ≤ 0.36 | A3 |
|  | 838 < Fc ≤ 841.5 | > 0 | > 2.7 | A2 |
|  |  | ≥ 5.94 | ≤ 2.7 | A8 |
| 10MHz | 840 < Fc ≤ 844 | ≥5.76 | >1.08 | A1 |
|  |  | ≥5.76 | ≤1.08 | A4 |
|  |  | <5.76, ≥4.14 | ≥2.7 | A2 |
|  |  | <2.52 | ≤0.36 | A3 |
|  | 835 < Fc ≤ 840 | ≥7.2 | >0 | A1 |
|  |  | <7.2, ≥5.22 | ≥4.32 | A2 |
|  |  | <1.08 | ≤0.36 | A3 |

Table 6: A-MPR for NS_15

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 | A6 | A7 | A8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Edge/Inner | Outer/Inner | Edge/Outer |
| DFT-s-OFDM PI/2 BPSK | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 | ≤ 5 | 1 | ≤ 7 | ≤ 2 |
| DFT-s-OFDM QPSK | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 | ≤ 5.5 | 1.5 | ≤ 7 | ≤ 2 |
| DFT-s-OFDM 16 QAM | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 | ≤ 5.5 | 2 | ≤ 7 | ≤ 2 |
| DFT-s-OFDM 64 QAM | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 | ≤ 5.5 |  | ≤ 7 | ≤ 2 |
| DFT-s-OFDM 256 QAM | ≤ 9 | ≤ 5 | ≤ 9 | ≤ 13.5 | ≤ 6 |  | ≤ 7 | ≤ 2.5 |
| CP-OFDM QPSK | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 10.5 | ≤ 7 | 3 | ≤ 8.5 | ≤ 3 |
| CP-OFDM 16 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 10.5 | ≤ 7 | 3 | ≤ 8.5 | ≤ 3 |
| CP-OFDM 64 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 10.5 | ≤ 7 |  | ≤ 8.5 | ≤ 3 |
| CP-OFDM 256 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 9 | ≤ 13.5 | ≤ 7.5 |  | ≤ 8.5 | ≤ 3 |

- Recommended WF

  - Discuss offline to merge A-MPR regions and then discuss corresponding values.

### Sub-topic 2-2

Sub-topic description: PC2 A-MPR.

It’s suggested to first agree on the A-MPR regions for each NS and then discuss the corresponding A-MPR values for each region.

Issue 2-1-2: NS_12: A-MPR regions for PC2

- Proposals: NS_12 A-MPR definition for 7 MHz channel BW and PC2:

  - Option 1 (Apple)

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz | 817.5 ≤ Fc < 820.5 | ≤1.8 | ≤0.72 | A3 |
|  |  | ≤1.8 | >0.72 | A1 |
|  |  | >1.8 | >1.44 | A1 |
|  | 820.5 ≤ Fc | ≤0.72 | >0 | A3 |
|  |  | >0.72 | >2.52 | A3 |

Table 3: A-MPR for NS_12 (PC2), from [3]

| Modulation/Waveform | A1 | A2 | A3 |
| --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 7.5 | 3 | ≤ 5.5 |
| DFT-s-OFDM QPSK | ≤ 7.5 | 3 | ≤ 6 |
| DFT-s-OFDM 16 QAM | ≤ 7.5 | 3.5 | ≤ 6 |
| DFT-s-OFDM 64 QAM | ≤ 7.5 | 4 | ≤ 6.5 |
| DFT-s-OFDM 256 QAM | ≤ 9.5 | 5.0 | ≤ 6.5 |
| CP-OFDM QPSK | ≤ 8.5 | 4.5 | ≤ 7.5 |
| CP-OFDM 16 QAM | ≤ 8.5 | 4.5 | ≤ 7.5 |
| CP-OFDM 64 QAM | ≤ 8.5 | 4.5 | ≤ 7.5 |
| CP-OFDM 256 QAM | ≤ 9.5 | 6.5 | ≤ 7.5 |

  - Option 2 (Qualcomm)

Table 1: A-MPR regions for NS_12

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 3MHz | 815.5 ≤ Fc < 817.5 | ≤0.9 | >0 | A2 |
| 5MHz |  | ≤1.8 | >0 | A1 |
| 7 MHz | Fc ≤ 817.5 | TBD | TBD | TBD |
| 10MHz |  | ≤3.6 | >0 | A1 |

- Recommended WF

TBA

Issue 2-1-2: NS_13: A-MPR regions for PC2

- Proposals: NS_13 A-MPR definition for 7 MHz channel BW and PC2:

  - Option 1 (Apple)

Table 4: A-MPR regions for NS_13 (Power Class 3)

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz |  | ≤1.8 | <1.44 | A1 |
|  | 820.5 ≤ Fc | ≤1.8 | ≥1.44 | A2 |
|  |  | >1.8 | ≥1.8 | A1 |

Table 6: A-MPR for NS_13 (Power Class 2), from [3]

| Modulation/Waveform | A1 | A2 | A3 | A4 |
| --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer | Outer |
| DFT-s-OFDM PI/2 BPSK | ≤ 5 | ≤ 6.5 | ≤ 4 | ≤ 3 |
| DFT-s-OFDM QPSK | ≤ 5 | ≤ 6.5 | ≤ 4 | ≤ 3 |
| DFT-s-OFDM 16 QAM | ≤ 5 | ≤ 6.5 | ≤ 4 | ≤ 3 |
| DFT-s-OFDM 64 QAM | ≤ 5 | ≤ 6.5 | ≤ 4 | ≤ 3 |
| DFT-s-OFDM 256 QAM | ≤ 5 | ≤ 7 |  |  |
| CP-OFDM QPSK | ≤ 7 | ≤ 7.5 | ≤ 5.5 | ≤ 4.5 |
| CP-OFDM 16 QAM | ≤ 7 | ≤ 7.5 | ≤ 5.5 | ≤ 4.5 |
| CP-OFDM 64 QAM | ≤ 7 | ≤ 7.5 | ≤ 5.5 | ≤ 4.5 |
| CP-OFDM 256 QAM | ≤ 8 | ≤ 8 |  |  |

- Recommended WF

TBA

Issue 2-1-2: NS_14: A-MPR regions for PC2

- Proposals: NS_14 A-MPR definition for 7 MHz channel BW and PC2:

  - Option 1 (Apple): no A-MPR is needed for 7MHz CBW with PC3 and PC2

- Recommended WF

  - No A-MPR is needed for NS_14 and 7 MHz channel bandwidth.

Issue 2-1-2: NS_15: A-MPR regions for PC2

- Proposals: NS_15 A-MPR definition for 7 MHz channel BW and PC2:

  - Option 1 (Apple)

- Table 9: A-MPR regions for NS_15 (Power Class 2)

| Channel BW | Carrier Frequency, Fc, MHz | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz | 841.5 < Fc ≤ 845.5 | ≥4.68 | >0 | A1 |
|  |  | <4.68, ≥2.52 | ≥2.16 | A2 |
|  |  | <4.68, ≥2.52 | <2.16, ≥1.44 | A2 |
|  |  | <1.8 | ≤0.36 | A3 |
|  | 837 < Fc ≤ 841.5 | ≥0 | ≥2.7 | A2 |
|  |  | ≥5.94 | <2.7 | A3 |

- Table 10: A-MPR for NS_15 (Power Class 2), from [3]

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 | A6 |
| --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 12 | ≤ 6.5 | ≤ 7 | ≤ 10.5 | ≤ 3 | ≤ 4 |
| DFT-s-OFDM QPSK | ≤ 12 | ≤ 6.5 | ≤ 7 | ≤ 10.5 | ≤ 3 | ≤ 4 |
| DFT-s-OFDM 16 QAM | ≤ 12 | ≤ 6.5 | ≤ 7 | ≤ 10.5 | ≤ 3 | ≤ 4.5 |
| DFT-s-OFDM 64 QAM | ≤ 12 | ≤ 6.5 | ≤ 7 | ≤ 10.5 | ≤ 3 | ≤ 5 |
| DFT-s-OFDM 256 QAM | ≤ 12 | ≤ 6.5 | ≤ 9 | ≤ 13.5 | ≤ 4.5 | ≤ 5 |
| CP-OFDM QPSK | ≤ 13.5 | ≤ 8 | ≤ 7 | ≤ 12 | ≤ 4.5 | ≤ 6 |
| CP-OFDM 16 QAM | ≤ 13.5 | ≤ 8 | ≤ 7 | ≤ 12 | ≤ 4.5 | ≤ 6 |
| CP-OFDM 64 QAM | ≤ 13.5 | ≤ 8 | ≤ 7 | ≤ 12 | ≤ 4.5 | ≤ 6 |
| CP-OFDM 256 QAM | ≤ 13.5 | ≤ 8 | ≤ 9 | ≤ 13.5 | ≤ 6.5 | ≤ 7 |

- Recommended WF

  - TBA

# Topic #3: BS

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2505803 | Nokia | WA ICS requirement 7; 15; G-FR1-A1-7; -100.6; -79.9; DFT-s-OFDM NR signal, 15 kHz SCS, 14 RBs. Interfering CW signal offset (±355kHz) for the narrowband Rx IM requirement: ±400kHz offset instead of the previous agreement of ±355kHz |
| R4-2506063 | CATT | Observation 1: For ICS requirements for 3MHz or 5MHz channel bandwidth, wanted signal and interfering signal occupy the whole transmission bandwidth configuration. Observation 2: For ICS requirements for a larger channel bandwidth such as 10, 15, 20, 25,30 and 35MHz, wanted signal and interfering signal have the same number of PRBs and placed symmetrically around the channel centre, leaving un-allocated RBs in the channel bandwidth. Observation 3: It is not a consistent way if reusing 5MHz ICS for 7MHz channel bandwidth having different PRB numbers for wanted and interfering signal and un-allocated RBs. Proposal 1: RAN4 to define ICS requirements for 7MHz channel bandwidth by reusing RMC for 10/15/20/25/30/35MHz channel bandwidth and interfering signal for 5MHz channel bandwidth. Proposal 2: RAN4 to launch simulation campaign under such configuration to specify ICS requirements for 7MHz channel bandwidth. |
| R4-2506150 | Ericsson | Proposal1: Specify the In-channel selectivity requirements for 7 MHz channel BW based on the requirements for 5 MHz channel BW: BS channel bandwidth (MHz); Subcarrier spacing (kHz); Reference measurement channel; Wanted signal mean power (dBm); Interfering signal mean power (dBm); Type of interfering signal. 5, 7; 15; G-FR1-A1-7; -100.6; -81.4; DFT-s-OFDM NR signal, 15 kHz SCS, 10 RBs. |
| R4-2506323 | ZTE Corporation, Sanechips | Proposal 1: To consider the parameters in Table 1, Table 2 and Table 3 for BS in-channel selectivity RF requirement for 7 MHz channel bandwidths. |
| R4-2507437 | Huawei, HiSilicon | Proposal 1: Do not consider 14RB case for ICS interferer signal, as it is not allowed as bandwidth of the PUSCH in terms of resource blocks. Proposal 2: For 7 MHz wanted channel bandwidth, increase ICS interferer signal bandwidth of the PUSCH (as compared to 5MHz wanted channel reference), and select as 12RB, or 15 RB. Proposal 3: Define ICS interferer bandwidth of the PUSCH in terms of resource blocks as [15RB]. |

## Open issues summary

### Sub-topic 3-1

Sub-topic description: BS RF Rx requirements.

Issue 3-1-1: Narrowband blocking – interferer offset

- Proposals: Specify narrowband blocking interferer offset for 7MHz channel BW with:

  - Option1: 400 kHz (Nokia).

  - Option2: 355 kHz (previous agreement).

- Recommended WF

  - The previous agreement was to specify the interferer offset with 355 kHz but, as Nokia pointed out in their contribution, only 15 kHz SCS is specified for 7 MHz channel BW. Following TR 38.817-02 (clause 7.7 and ∆fCW formula with 15kHz SCS), it seems 400kHz should then be specified instead.

Issue 3-1-2: ICS interferer

Background: RAN4 already agreed (R4-2505098) to reuse the existing NR FRCs. For ICS, there is also a common view to reuse G-FR1-A-7 FRC. The only remaining issue is on the interferer number of RBs, the interferer mean power would then be determined by scaling.

- Proposals: Specify the size of the interferer for ICS and 7MHz channel BW with:

  - Option1: 14 RBs (Nokia).

  - Option2: 15 RBs (ZTE, Huawei)

  - Option 3: 12 RBs (Huawei, second option).

  - Option 3: 10 RBs, same as for ICS and 5 MHz channel BW (Ericsson)

  - Option 4: Initiate simulation campaign (CATT)

- Recommended WF

  - From the analysis of 4 companies, simulations are not needed to determine the interferer for ICS requirement. It should also be noted that this WI is supposed to be finalized by next RAN4#116, it would be very challenging to conclude on simulation in one meeting cycle.

As mentioned by 2 companies, the 14 RBs option doesn’t fulfil the  2α2 3α3 5α5 rule.

Further discussion would be needed on the 2 remaining options, 10 or 15 RBs.

# Topic #4: Draft CR

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| UE |  |  |
| R4-2507652 | Qualcomm Incorporated | 7 MHz Channel Bandwidth for n5 and n26 Draft CR to TS 38.101-1 – Rx requirements and annexes |
| BS |  |  |
| R4-2506061 | CATT | draftCR to TS 38.115-1 on 7MHz channel bandwidth |
| R4-2505802 | Nokia | draftCR to 38.141-1 on introduction of RF requirements for 7MHz channel bandwidth |
| R4-2506151 | Ericsson | Running CR to TS 38.104 - Introduction of 7 MHz channel BW (for email approval/endorsement) |
| R4-2506152 | Ericsson | CR to TS 38106 - 7 MHz channel BW clarification |
| R4-2506153 | Ericsson | CR to TS 37113 - Introduction of 7 MHz channel BW |
| R4-2506325 | ZTE Corporation, Sanechips | Draft CR to TS38.113: Introduction of 7 MHz NR FR1 channel bandwidth |
| R4-2506473 | ZTE Corporation, Sanechips | Draft CR to TS37.141: Introduction of 7 MHz NR FR1 channel bandwidth |
| R4-2507438 | Huawei, HiSilicon | Draft CR to TS 37.105: 7MHz channel bandwidth introduction |
| R4-2507439 | Huawei, HiSilicon | Draft CR to TS 37.145-1: 7MHz channel bandwidth introduction |
| R4-2507440 | Huawei, HiSilicon | Draft CR to TS 37.145-2: 7MHz channel bandwidth introduction |
| R4-2507441 | Huawei, HiSilicon | Draft CR to TS 38.141-2: 7MHz channel bandwidth introduction |
| R4-2507442 | Huawei, HiSilicon | Draft CR to TS 38.176-2: restriction of 7MHz channel bandwidth introduction |
| R4-2507236 | Nokia | CR to 38.174 with clarification for channel bandwidths below 10 MHz Note: This CR was submitted for AI 4.3 as maintenance for IAB work item, as it covers note that IAB doesn’t support channel bandwidth below 10 MHz, which automatically exclude 7 MHz CBW as well agreed in this WI. |
| R4-2507240 | Nokia | CR to 38.176-1 with clarification for channel bandwidths below 10 MHz Note: This CR was submitted for AI 4.3 as maintenance for IAB work item, as it covers note that IAB doesn’t support channel bandwidth below 10 MHz, which automatically exclude 7 MHz CBW as well agreed in this WI. |

## Open issues summary

### Sub-topic 4-1

Sub-topic description: draft CRs

Issue 4-1-1: Note to indicate 7 MHz channel BW is not in the TS scope

- Proposals: Where should we add the note indicating that 7 MHz channel BW is not in the scope of IAB and NCR specifications:

  - Option1: In the scope and in the “Operating bands and channel arrangement” subclause (Huawei and Ericsson)

  - Option 2: In the “Operating bands and channel arrangement” only subclause (CATT)

  - Option 3: For IAB specifications to add note that channel bandwidths below 10 MHz are not supported (Nokia)

- Recommended WF

  - To be discussed between option 1 and option 2.

Option 3 should be discussed in AI 4.3 but, if the 2 CRs for IAB are agreed, we should confirm that this note (to indicate 7 MHz is out of the TS scope) is not needed in TS 38.174 and TS 38.176-1.

Issue 4-1-2: Wording for this note

- Proposals: Which note’s wording should be used:

  - Option1: For 7MHz channel bandwidth, it is not supported in this release. (CATT)

  - Option 2: 7 MHz channel bandwidth is not supported in this version of the specification (Ericsson)

  - Option 3: 7 MHz channel bandwidth is not covered by the present release of this specification (Huawei)

- Recommended WF

To be discussed