# 3GPP TSG-RAN WG4 Meeting #115	R4-2507570

St Julian’s, Malta, May 19th – 23rd, 2025

Agenda item:			7.21.1

Source:	Moderator (Qualcomm)

Title:	Topic summary for [115][131] NR_AIML_air_part1

Document for:	Information

# Introduction

This is the summary thread for issues related to the NR AI/ML WI. The topics handled in this thread are the following:

- CSI reporting prediction

  - CSI reporting requirement and testing framework for CSI prediction (AI 7.21.3)

  - Demodulation and/or CSI reporting requirements for CSI prediction (AI 7.21.7)

- Beam prediction

  - RRM core requirement and testing framework for beam management (AI 7.21.4)

  - RRM performance requirements for beam management (AI 7.21.6)

- Positioning

  - RRM core requirement and testing framework for Positioning accuracy enhancement (AI 7.21.5)

  - RRM performance requirements for positioning accuracy (AI 7.21.6)

Issues related to the general part (agenda item 17.19.1) are treated in the AI/ML part 2 thread ([114bis][126])

A WF summarizing the agreements from RAN4#114 was agreed in RP-2502856. The discussion will take into account these previous agreements.

# Topic #1: CSI reporting requirement and testing framework for CSI prediction

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2505508 | Qualcomm Incorporated | Observation 1: According to last meeting’s contributions, majority of companies observed poor SGCS with 100Hz Doppler. Observation 2: RAN4 has agreed to introduce a test with Doppler basis, N4 = 1, and is currently investigating 5ms CSI-RS periodicity. Observation 3: In FDD tests, RAN4 is currently considering 4 ms time separation between I and II, and II to III. - Slot containing the last CSI-RS occasion of the observation window - PUSCH conveying predicted PMI - Slot where predicted PMI is applied in scheduled PDSCH Observation 4: Combination of observation 2 and 3 mean that the predicted PMI may get applied in scheduled PDSCH 3-7 ms after the time location of predicted PMI. This 3-7 ms delay may adversely impact the PDSCH throughput since channel and ideal PMI might have changed significantly during 7ms. Observation 5: In Rel-18, spec required UE to generate PUSCH containing predicted PMI, designed based on non-AI-ML prediction, in Doppler domain basis N4 = 1 scenario if time separation between last CSI-RS of observation window and PUSCH is greater than Z2’. Value of Z2’ is less than 3ms. Observation 6: RAN4 should revisit the time separation between CSI-RS and PUSCH in FDD test setup after RAN1 finalizes the minimum time separation between these two channels for computing AI-ML based predicted PMI. Proposal 1: RAN4 deprioritizes the use of 100Hz for evaluating the next step (throughput) of AI-ML based CSI prediction. Proposal 2: RAN4 considers reducing the time separations between PMI report and scheduled PDSCH with predicted PMI in Rel-19 AI-ML CSI prediction tests compared to the values assumed in Rel-18 non-AI-ML CSI prediction tests. - For example, RAN4 considers assuming the following in FDD tests: “This reported PMI cannot be applied at the gNB downlink before slot#(n+3)” |
| R4-2505596 | MediaTek inc. | Observation #1: FDD 16TX and FDD 32TX results with the same Doppler perform quite similarly. Observation #2: In lower Doppler accuracy is better than in higher Doppler. Observation #3: Rel-18 codebook quantization performance is close to raw channel prediction performance. Observation #4: In simulation assumptions there exists sufficient test setup for FDD with Periodic CSI-RS configuration. Proposal #1: We propose focusing to proposed test setup for FDD with Periodic CSI-RS configuration based on R.PDSCH.1-24.1 FDD as priority in Step-3. Proposal #2: We propose to collectively define SNR range of training dataset for Step-3 experiments. |
| R4-2505945 | Apple | In conclusion the following observations and proposals were discussed in this contribution: Observation 1: AI/ML-based prediction shows notable performance improvements over the Sample-and-Hold method for Doppler spreads up to 50 Hz. However, these gains tend to diminish as the Doppler spread increases, with little to no improvement observed around 100 Hz. Proposal 1: RAN4 to perform simulation results alignment among companies. If simulation results don’t match, companies should be encouraged to share CSI prediction datasets and/or agree on AI/ML model architectures Proposal 2: Simulation results indicate a tradeoff between training and testing losses, highlighting potential overfitting when models are not well matched to the Doppler conditions. RAN4 is invited to discuss the need for selecting or designing AI/ML models optimized for specific Doppler spreads, or alternatively, exploring model fine-tuning or generalization techniques that enable a single model to effectively support a range of Doppler conditions without requiring multiple models. |
| R4-2505974 | Samsung | In this contribution, the view on CSI reporting requirements framework was provided. Meanwhile, the initial simulation results for AI/ML CSI prediction were provided to facilitate the calibration and comparison. Proposal 1: RAN4 should also compare the ratio of throughput obtained based on following PMI with Rel-18 eType II and random PMI with Rel-15 type I single panel codebook, when defining the requirement of AI/ML based CSI prediction with Rel-18 eType II doppler codebook. Proposal 2: RAN4 may need to discuss whether the baseline assumption for practical channel estimation should be specified, or it is up to companies’ implementation Proposal 3: RAN4 shall discuss how to choose the proper SNR range for AI/ML based CSI prediction model training and test when applying the practical channel estimation. The following method can be considered as starting point: - Use the SNR value with 90% of throughput achieved based on following PMI as starting point, based on this SNR, considering ±X dB range for data collection for AI/ML training, where the SNR value is derived based on existing Rel-18 eType II Doppler codebook under the same configuration RAN4 should also discuss whether the AI model is training based on dataset with specific SNR condition or mixed dataset with the selected SNR range, when applying the practical channel estimation Observation 1: AI/ML based CSI prediction show better performance compared with nearest historical CSI scheme (benchmark#1) for cases with doppler as 20Hz and 50Hz. Generalization problem needs further study when defining requirement |
| R4-2506466 | vivo | Observation 1: Initial evaluation results for CSI prediction AI/ML model is shown in Table 2-1, for FDD and 32gNB Tx. Table 2-1. Initial evaluation results for CSI prediction AI/ML model, for FDD and 32gNB Tx. FDD, 32 gNB Tx; Test on 20Hz Doppler spread; Test on 50Hz Doppler spread; Test on 100Hz Doppler spread. Model trained by 20Hz Doppler spread; 0.993; /; /. Model trained by 50Hz Doppler spread; /; 0.902; /. Model trained by 100Hz Doppler spread; /; /; 0.298. Model trained by 20Hz, 50Hz and 100Hz Doppler spread; 0.994; 0.907; 0.296. Proposal 1: Reference AI model for CSI prediction would be needed for test metric, if it is difficult to align the simulation results. Proposal 2: AI prediction and non-AI prediction could follow the similar test procedure. Proposal 3: RAN4 to discuss Type 3 performance monitoring for UE assisted performance monitoring of CSI prediction. Other LCM requirements could wait for RAN1 further progress. If there is any spec impact, RAN4 to discuss whether to capture it in the RRM spec. |
| R4-2506523 | OPPO | In this contribution, we have discussed the CSI reporting requirement and testing framework for CSI prediction, and got the following proposals: Proposal 1: Reuse the legacy PMI requirement (γ value) as a baseline test, and let other options/proposals with higher performance requirements be further studied. Proposal 2: γAI/ML could be obtained within a CSI prediction window. If a CSI prediction window contains more than one CSI prediction instance, Option1: γAI/ML is calculated for each predicted CSI and then do the average within the CSI prediction window Option2: tue_AI/ML and trnd are calculated within the CSI prediction window first and then get the averaged γAI/ML value. Proposal 3: For CSI prediction, following test steps could be considered: - Step1: TE sends CSI-RS to DUT, within a CSI observation window (signal generating and transmitting) - Step2: DUT conducts the CSI measurement within the CSI observation window (CSI-RS receiving) - Step3: DUT conducts the CSI prediction within a CSI prediction window (Inference) - Step4: DUT reports the predicted CSI (CSI reporting) - Step5: TE calculates the γAI/ML for performance [and calculates the measurement/inference latency for LCM] |
| R4-2506604 | CMCC | This contribution provides discussion on CSI prediction. The proposals are: Proposal 1: For channel model for CSI prediction tests, it is proposed to focus on TDL channel models in Rel-19. Using CDL channel model for CSI prediction test can be further dicsussed in later release based on the progress on SCM SI. Proposal 2: for CSI prediction, for performance monitoring type 3, it is proposed to use intermediate KPI, e.g. SGCS, as performance metric for LCM. Proposal 3: it is proposed to define monitoring delay for CSI prediction performance monitoring. |
| R4-2507048 | Huawei, HiSilicon | According to the discussion, following proposals and observations are provided: Proposal 1: CDL channel model is not used in RAN4 for AI CSI prediction. Proposal 2: For generalization testing, there is no need to use non-static condition. Proposal 3: RAN4 will start to discuss whether/how to define requirements for UE-side monitoring if other WGs achieve sufficient progress. |
| R4-2507105 | Ericsson | Proposal 1: RAN4 discusses the testability of the performance monitoring accuracy requirement for CSI prediction. Proposal 2: RAN4 should consider more than one condition to guarantee the generality of the AI-based CSI prediction model. At least RAN4 should consider more than one channel model (e.g., TDLA30 and TDLC300) and antenna configuration (e.g., 16 CSI-RS ports and 32 CSI-RS ports). Proposal 3: For CSI prediction performance requirements, RAN4 discusses the feasibility of combing more than one static condition test to ensure the generality of the prediction model. Proposal 4: RAN4 should apply CDL channel model to the PMI reporting requirements with the AI/ML-based CSI prediction when a suitable CDL model is available. |
| R4-2507532 | Nokia | Proposal 1: RAN4 to discuss whether LCM-related requirements are needed for AIML CSI prediction. Proposal 2: RAN4 to study the LCM requirements focusing on activation and deactivation aspects. Proposal 3: RAN4 to discuss whether and how to specify requirements on performance monitoring for CSI predictions. |
| R4-2505513 | Qualcomm Incorporated | Observation 1: RAN4 should confirm whether to report SGCS2 or SGCS3 of above figure as step 2 evaluation metric, i.e., as the SGCS after CSI feedback. Observation 2: For 25Hz and 50Hz Doppler frequencies, the SGCS obtained with AI-ML based CSI prediction in both step 1 and step 2 are significantly higher than those obtained with “sample and hold” approach. Observation 3: The SGCS obtained by AI-ML based predicted PMI decreases significantly at 100Hz Dopler but remains higher than the one obtained by “sample and hold”. Observation 4: The SGCS performances of AI-ML model, trained based on the mixed dataset of 20, 50 and 100Hz, are almost identical to those of AI-ML model, trained based on individual datasets. Observation 5: The number of parameters of our AI-ML CSI prediction model, that got used to generate the results of this contribution, is in the order of several millions. Observation 6: The SGCS results submitted to future meetings may vary from the ones shown in Table 2 considering the complexity of the AI-ML model. |
| R4-2505946 | Apple | Proposal 1: The γ value comparing predicted PMI to random PMI can be used in the test for CSI prediction. To infer the gains of AI/ML-based CSI prediction, the γ values can be compared between AI/ML and non-AI/ML cases. Proposal 2: RAN4 should assess the performance of AI/ML-based CSI prediction using the same test configurations and conditions as those defined for non-predicting eTypeII and/or eTypeII Doppler-r18. Proposal 3: To better evaluate the generalization capability of such models, it is proposed that CSI-related testing also include CDL or other representative channel models that better reflect practical conditions. Proposal 4: RAN4 to further explore and define appropriate test setups for evaluating AI/ML-based CSI prediction utilizing the Rel-18 Type II-Doppler codebook. As an initial step, a focus on periodic CSI-RS configurations under FDD operation can serve as a practical baseline. Proposal 5: To enhance the robustness and generalization of AI/ML-based CSI prediction models, RAN4 should consider a structured approach to training data collection that combines mixed channel conditions and practical channel estimation scenarios. To build models resilient to real-world variability, data should be collected across a range of SNR values (e.g., ±3–5 dB from the reference point) and include a mix of practical channel models (e.g., CDL, UMa) and realistic estimation impairments. This mixed dataset strategy ensures the trained model can adapt to varying deployment conditions while maintaining stable prediction performance |
| R4-2505973 | Samsung | Proposal 1: Only one Tx antenna configuration will be selected for specifying the PMI requirement based on AI/ML prediction. The following Tx antenna configuration and associated CSI-RS configuration can be considered For 32Tx NZP CSI-RS for CSI acquisition; Number of CSI-RS ports (X); 32. CDM Type; CDM4 (FD2, TD2). Density (ρ); 1. First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3); Row 17,(2, 4, 6, 8). First OFDM symbol in the PRB used for CSI-RS (l0, l1); Row 17,(5, 12). For 16Tx Number of CSI-RS ports (X); 16. CDM Type; CDM4 (FD2, TD2). Density (ρ); 1. First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3); Row 12, (2, 4, 6, 8). First OFDM symbol in the PRB used for CSI-RS (l0); (5). Proposal 2: Consider rank 2 for specifying the PMI requirement based on AI/ML prediction Proposal 3: Prioritize to specify PMI requirement with periodic configuration. Proposal 4: RAN4 consider PC7 for specifying PMI requirement with AI/ML based CSI prediction Parameter; Value. Channel bandwidth and subcarrier spacing; For FDD, 10MHz/15kHz For TDD, 40MHz/30kHz. TDD DL-UL configuration; FR1.30-1 as specified in 38.101-4 Annex A.. Number of UE receiver antennas; 2 and 4. R (numberOfPMI-SubbandsPerCQI-Subband-Doppler-r18); 1. Other Test parameters not mentioned above; For FDD 2Rx, Table 6.3.2.1.6-1 in 38.101-4 For TDD 2Rx, Table 6.3.2.2.6-1 in 38.101-4 For FDD 4Rx, Table 6.3.3.1.6-1 in 38.101-4 For TDD 4Rx, Table 6.3.3.2.6-1 in 38.101-4. Proposal 5: Reused the parameters in existing requirement for Rel-18 eTpye II Doppler Codebook can be reused as baseline for specifying PMI requirement with AI/ML CSI prediction. |
| R4-2506470 | vivo | Proposal 1: RAN4 to discuss further simulation assumptions for Tput, e.g., CSI-RS configuration, codebook configuration of Rel-18 eTypeII-Doppler, and SNR. Proposal 2: Reference AI model for CSI prediction would be needed for performance requirement definition, if it is difficult to align the simulation results. Proposal 3: AI-prediction would have similar or higher gamma value compared to non-AI prediction, since same metric as non-AI prediction is used. |
| R4-2506527 | OPPO | Proposal 1: Regarding the RAN4 Simulation for CSI prediction and the Reference model for CSI prediction, RAN4 should first assess the CSI prediction evaluation results across different companies before determining whether a reference model is necessary. Proposal 2: Regarding the RAN4 Simulation for CSI prediction and the Reference model for CSI prediction, - If RAN4 achieves aligned CSI prediction evaluation results in step-2 (as described in the agreed simulation assumptions), further evaluation (step-3 comparisons) can proceed (after RAN4#115). - If alignment in step-2 proves difficult, RAN4 should identify the key factors causing discrepancies and may need to initiate a second-round evaluation to reassess Step-2 alignment (after RAN4#115). - If alignment remains unachievable after step-3, RAN4 may then trigger discussions and evaluations on a reference model for CSI prediction (during/after RAN4#116). - If alignment achieves after step-3, RAN4 will trigger evaluations on the CSI reporting requirements for CSI prediction(during/after RAN4#116). Observation 1: Regarding the updated simulation for AI/ML-based CSI prediction, the updated simulation results are shown in Table 2 and Table3. Table 2. Simulation results(step-1) for AIML based CSI prediction TX; Train; 20Hz; 50Hz; 100Hz. Rank1; Rank2-ave L1&2; Rank2-L1; Rank2-L2; Non-AI (Rank1); Rank1; Rank2-ave L1&2; Rank2-L1; Rank2-L2; Non-AI (Rank1); Rank1; Rank2-ave L1&2; Rank2-L1; Rank2-L2; Non-AI (Rank1). SGCS1; 32; Non-mix; 0.9994; 0.9992; 0.9994; 0.9990; 0.7186; 0.9032; 0.8632; 0.9032; 0.8231; 0.3698; 0.2846; 0.2289; 0.2846; 0.1731; 0.2850. Mix; 0.9989; 0.9983; 0.9989; 0.9977; 0.9017; 0.8617; 0.9017; 0.8216; 0.2774; 0.2216; 0.2774; 0.1659. Note: SGCS1 is calculated by comparing the SVD of model output with SVD of the ground truth Raw channel Table 3. Simulation results(step-2) for AIML based CSI prediction TX; Train; 20Hz; 50Hz; 100Hz. Rank1; Rank2-ave L1&2; Rank2-L1; Rank2-L2; Non-AI (Rank2-L1); Non-AI (Rank2-L2); Rank1; Rank2-ave L1&2; Rank2-L1; Rank2-L2; Non-AI (Rank2-L1); Non-AI (Rank2-L2); Rank1; Rank2-ave L1&2; Rank2-L1; Rank2-L2; Non-AI (Rank2-L1); Non-AI (Rank2-L2). SGCS2-1; 32; Non-mix; -; 0.5820; 0.6673; 0.4967; 0.5819; 0.3832; -; 0.5519; 0.6470; 0.4568; 0.3894; 0.2267; 0.2696; 0.3291; 0.2101; 0.3257; 0.2100. Mix; -; 0.5817; 0.6668; 0.4966; -; 0.5511; 0.6463; 0.4558; 0.2677; 0.3271; 0.2083. SGCS2-2; 32; Non-mix; 0.9735; 0.9809; 0.9660; 0.7272; 0.5533; 0.8105; 0.8614; 0.7595; 0.4706; 0.3147; 0.3424; 0.3982; 0.2865; 0.3945; 0.2875. Mix; 0.9660; 0.9744; 0.9576; 0.8077; 0.8597; 0.7556; 0.3398; 0.3959; 0.2837. Note: SGCS2-1 is calculated by comparing the feedbacked SVD of model output with the SVD of the ground truth Raw channel SGCS2-2 is calculated by comparing the feedbacked SVD of model output with the feedbacked SVD of the ground truth Raw channel Observation 2: For TDL channels, the SGCS calculated by comparing the feedbacked SVD of the model output with the SVD of the ground truth raw channel deteriorates significantly. When the ground truth eigenvectors also incorporate the quantization errors from CSI feedback, the SGCS calculated by comparing the feedbacked SVD of the model output with the feedbacked SVD of the ground truth raw channel shows much less degradation. Proposal 3: Consistency between training data and test data should be ensured - FFS: Whether the test emulated channel will be similar to the one used in RAN4 simulations - FFS: Whether anything special should be done during the test |
| R4-2506605 | CMCC | Proposal 1: for CSI prediction, it is proposed that AI/ML based performance requirement should not be worse than the legacy requirements for eType II Doppler codebook. Proposal 2: for CSI prediction test with static condition, it is proposed to define test cases with different configuration/parameters, e.g. antenna configuration, observation window length, prediction window length, and UE speed . Proposal 3: For channel model for CSI prediction tests, it is proposed to focus on TDL channel models in Rel-19. Using CDL channel model for CSI prediction test can be further dicsussed in later release based on the progress on SCM. Proposal 4: for CSI prediction, the evaluation results for step 1 are shown as Table 2. Table 2 SGCS for CSI prediction SGCS. ConvLSTM; 0.9249. |
| R4-2506960 | ZTE Corporation, Sanechips | The conclusions are: Observation 1. From our simulation results, it can be observed that a little performance degradation when using mixed dataset training and testing on single dataset. |
| R4-2507052 | Huawei, HiSilicon | Proposal 1: For step-1, SGCS performance results of AI CSI prediction are provided in Table 1. Table 1. SGCS performance results of AI CSI prediction (step-1) 20Hz; 50Hz; 100Hz. Rank 1; 0.9985413; 0.8673333; 0.301114. Rank 2; 0.99800384; 0.8166052; 0.233405. Note: gNB TX antennas is Option 2. Observation 1: Significant degradation in SGCS performance can be observed with 100Hz Doppler spread. Proposal 2: RAN4 will not define performance requirement for AI CSI prediction with 100Hz Doppler spread. |
| R4-2507106 | Ericsson | Observations: Observation 1: The average SGCS does not change regardless of the trained dataset for TDLA30 with 20/50/100Hz. Observation 2: AI/ML-based PMI prediction works for TDLA30-20 and TDLA30-50 regarding to the average of SGCS. The average SGCS from AI/ML-based CSI prediction model is close to that from ‘sample and hold’ for TDLA30-100. Observation 3: RAN1 is still discussing the CSI processing time for Rel-19 CSI prediction. Proposals: Proposal 1: RAN4 does not use TDLA30-100 for Rel-19 AI/ML-based CSI prediction performance requirements. Proposal 2: RAN4 waits for the RAN1 conclusion on CSI processing time for AI/ML-based CSI prediction to discuss the detailed simulation assumption for the follow PMI performance evaluation. |
| R4-2507624 | Nokia | Observation 1: In the WID of Rel-19 AI/ML for NR Air Interface, TS 38.101-4 is not listed as impacted specification for the Performance part. Proposal 1: The WID to be revised to include TS 38.101-4 for Demodulation and CSI report requirements and to be approved in RAN Plenary. Observation 2: Agreements made in RAN4#114bis is to define only PMI requirements using the same metric as for the current PMI reporting requirements in 38.101-4. Observation 3: The requirements to be introduced are using Rel-18 eType II Doppler codebook with Dopler domain basis N4=1. Observation 4: The agreement on the link-level simulation is to use TDLA channel. Proposal 2: RAN4 to specify PMI reporting requirements for CSI prediction for Rel-18 eType II Doppler codebook with Dopler domain basis N4=1 by using the same metric as in 38.101-4. Proposal 3: TDLA should be prioritized, but other propagation models should not be precluded. Proposal 4: RAN4 to not specify PDSCH demodulation requirements for Rel-19 AI/ML CSI prediction use case. |

## Open issues summary

The agreements from the previous meeting on this topic are listed below for reference (R4-2505105):

Issue 1-1: Codebook to be used in the tests

Agreement:

- In Rel-19 only PMI requirements based on AI/ML CSI prediction will be defined

- use Rel-18 eType II Doppler codebook for AI/ML based CSI predictions

Issue 1-2: Requirement baseline

Agreement:

  - Requirement to be introduced for Dopler domain basis N4=1

      - N4>1 can be further discussed in the future if needed

Issue 1-4: Test metric

Requirements to be defined based on the ratio (same metric as for the current PMI reporting requirements in 38.101-4)

      - tue_AI/ML is 90 % of the maximum throughput obtained at SNRue_AI/ML  [a given SNR] using the precoders configured according to the UE reports,

      - trnd is the throughput measured at SNRue_AI/ML  [the same SNR] with random precoding.

        - trnd is obtained with Rel-15 type I single panel codebook

      - Note: SNRue_AI/ML is calculated in the same way as SNRue and SNRfollow1,follow2 in section 6.3 of 38.101-4

Also, simulation assumptions were agreed in R4-2502970.

The open issues were grouped in the following sub-topics for further discussion:

- Requirement considerations

- Time separation between PMI report and scheduled PDSCH

- Handling of 100Hz Doppler

- SNR handling in RAN4 studies

- SNR levels for training and generalization

- Simulation results and next steps

- Generalization

- LCM

### Sub-topic 1-1

Requirements baseline

One company proposed to also consider Rel-18 eType II follow PMI performance relative to Rel-15 single Tx codebook when defining the requirements

Issue 1-1: Requirement considerations

- Proposals

  - Option 1: consider Rel-18 eType II follow PMI performance relative to Rel-15 single Tx codebook

    - Performance delate should be higher?

  - Option 2: no need to consider performance of other schemes when defining the requirements

- Recommended WF

Option 1

### Sub-topic 1-2

PMI report and scheduled PDSCH separation

The time distance between the report with the predicted PMI and the actual scheduled PDSCH should be discussed

Issue 1-2: PMI Report and Scheduled PDSCH separation

- Proposals

  - Option 1: Reduce the time separation between PMI report and scheduled PDSCH relative to the values assumed in Rel-18 prediction tests

    - RAN4 considers assuming the following in FDD tests: “This reported PMI cannot be applied at the gNB downlink before slot#(n+3)”

  - Option 2: Reuse the same values as in Rel-18 prediction tests

  - Option 3: others

- Recommended WF

  - To be discussed

### Sub-topic 1-3

100Hz Doppler handling

Several companies are proposing to de-prioritize or eliminate 100Hz Doppler from the set of considered values

Issue 1-3: Doppler handling

- Proposals

  - Option 1: De-prioritize 100Hz Doppler

    - Consider 100Hz Doppler in the future if needed

  - Option 2: Eliminate 100Hz Doppler from the set of considered values

  - Option 3: Keep 100Hz Doppler, further study the performance and consider defining a test

- Recommended WF

  - Option 1

### Sub-topic 1-4

SNR handling

Some companies raised the issue of what SNR to be used

Issue 1-4: SNR handling

- Proposals

  - Option 1: RAN4 should discuss how to choose the SNR range for AI/ML based prediction. Following is the suggestion from R4-2505974:

    - Use the SNR value with 90% of throughput achieved based on following PMI as starting point, based on this SNR, considering ±X dB range for data collection for AI/ML training, where the SNR value is derived based on existing Rel-18 eType II Doppler codebook under the same configuration

  - Option 2: other proposals

  - Option 3: there is no need to fix the SNR

- Recommended WF

  - TBD

### Sub-topic 1-5

SNR levels for training data and generalization

The issue of whether the SNR for the training data should be discussed agreed was brought up. This issue could also be related with generalization if different there would be different tests with different SNR levels.

Issue 1-5: SNR levels for training data and generalization

- Proposals

  - Option 1: Agree in RAN4 different SNR levels for training data

  - Option 2: Training data generation should be left to implementation

  - Option 3: No need to agree anything explicit in RAN4 about training data, UE performance at different SNR levels should be tested

  - Option 4: No need for any discussion on training data or such generalization tests

- Recommended WF

  - To be discussed

### Sub-topic 1-6

Simulation results and next steps

Several companies submitted simulation results and proposals for next steps (refinement of parameters, etc)

Issue 1-6: Simulation results and next stepts

- Proposals

  - Discuss the simulation results and next steps

    - Simulation results

    - Parameter refining if needed

    - Next steps

- Recommended WF

  - To be discussed

Discussion on refinement of simulation parameters and next stepsto be done mainly offline

### Sub-topic 1-7

Generalization

Several companies proposed to further study generalization issues and how to ensure that the UE performance does not degrade under different conditions.

Issue 1-7: Generalization

- Proposals

  - Option 1: Mix several static conditions to endure the generality of the prediction model

  - Option 2: Introduce tests with different channel models

  - Option 3: Use different antenna parameters

  - Option 3: Test under different SNRs

  - Option 4: further investigate the granularity of Doppler spread for which a new AI/ML model would be needed

  - Option 5: others

- Recommended WF

  - Postpone discussion to next meeting

It was agreed in a previous meeting that generalization would be discussed later on when the requirements and tests details (e.g. conditions, etc) would become more clear. Companies can still consider the above proposals when discussing the definitions of the requirements and tests.

### Sub-topic 1-8

LCM

Several companies proposed to discuss LCM procedures

Issue 1-8: LCM

- Proposals

  - Option 1: Wait for further progress in RAN1/2 before continuing the discussion in RAN4

  - Option 2: Define requirements for performance monitoring

    - For type 3 monitoring use intermediate KPI (SGCS)

    - Introduce requirements for activation/deactivation

      - Is there any difference compared to RRC configuration?

  - Option 3: RAN4 to discuss the testability of monitoring reporting options, and RAN4 could inform RAN1 if RAN1 does not make decision.

  - Option 4: RAN4 should start discussing where/how to capture LCM requirements

    - Use 38.133 spec?

- Recommended WF

  - Option 1

Once the details become clear, RAN4 will discuss whether/how to define requirements for LCM. Above proposals can be considered in future discussions

# Topic #2: RRM core requirement and testing framework for beam management

This section contains the sub-topics regarding specific issues for beam management.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2505478 | Tejas Network Limited | Observation 1: Increasing the inference delay to achieve better accuracy may lead to suboptimal system performance. Observation 2: When K is large, the likelihood of the Top-1 strongest beam becoming part of the Top-K predicted beams increases. Observation 3: An unknown TCI state change can result in increased delay during the state transition. Proposal 1: Absolute RSRP accuracy requirement should be applied to all the top K predicted beams. Proposal 2: The predicted RSRP accuracy requirement should be close to the legacy requirement to reduce the performance degradation due to AIML based BM. Proposal 3: No need to define separate requirement for inference delay, it should be considered as a component of the overall reporting delay. Proposal 4: RAN4 to define separate reporting delay requirement for BM case 1 and BM case 2. Proposal 5: The value of K should be kept small to ensure more meaningful validation and reduce the likelihood of the Top-1 strongest beam easily fitting into the Top-K predicted beams. Proposal 6: The value of N should be at least 2. Proposal 7: RAN4 to define the value of x to be sufficiently stringent to ensure meaningful validation of beam prediction accuracy. Proposal 8: The reported L1-RSRP of the strongest beam should be predicted, not measured. Proposal 9: TCI state associated with the predicted beam should be known. |
| R4-2505510 | Qualcomm Incorporated | Observation 1: RAN4 114 agreement ensures defining both beam prediction accuracy and RSRP accuracy metrics if RSRP and beam ID are reported. Based on RAN4 114bis agreement, UE will have to meet only RSRP accuracy requirements when both beam ID and RSRP are reported. Observation 2: If UE only has to meet RSRP accuracy metrics, UE’s predicted top beam can be very far from the strongest ground truth beam. Even if UE meets L1-RSRP accuracy for its predicted top beam, the reported top-1 predicted beam might not be a suitable beam as a reference for changing TCI state and uplink spatial relationship. Observation 3: 3GPP is considering L1-RSRP accuracy as a potential prediction metric for AI-ML based beam management. UE has to know its RX beam to predict L1-RSRP accuracy of a particular gNB TX beam. Observation 4: To compare between gNB beams of setA, UE needs to have some idea of the Rx gain associated with them. UE has to know the Rx beam of the predicted gNB beam even if it only has to predict top-1 (%) or top-K (%) beams. Observation 5: Legacy spec requires UE to know the RX beam of CSI-RS if it is configured without “repetition = ON” and if it is QCLed with SSB. Observation 6: In “wide-to-narrow” beam prediction, the setA CSI-RS resources whose properties that UE needs to predict are QCLed wrt type D with the setB SSBs that UE has measured. In order to receive a CSI-RS of setA beams after TCI state switch, UE can use the wide RX beam that it needs to measure the SSB that is QCLed with the CSI-RS. Observation 7: In “narrow-to-narrow” beam prediction, if SSB resources are configured during the test, UE can use the RX beams corresponding to the SSBs as the RX beam for the setA CSI-RS resources if following conditions are satisfied: - CSI-RS resources are configured without “repetition = ON” and QCLed wrt type D with SSB that are present during the tests. Observation 8: The inference delay is dependent on the model complexity and hardware architecture. The design of model and hardware architecture may vary from UE to UE. Hence, RAN4 should not define a fixed value for inference delay of AI-ML based beam prediction. Observation 9: UE may need to use different AI-ML models between beam management case 1 (“spatial only” beam prediction) and case 2 (“temporal” beam prediction). Hence, UE’s required time to infer beam might be different between beam management case 1 and case 2 Observation 10: Complexity of the required AI-ML model depends on CSI report configuration. Proposal 1: Extend following metric/KPI for beam ID prediction metric to the scenario when both beam ID and RSRP are reported. - “The successful rate for the correct prediction, - The correct prediction is considered as maximum ground-truth RSRP among top-K predicted beams larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB, - FFS on top-N (N</=K) predicted beams have to be considered in the success rate evaluation - FFS on N values - FFS on x values - Note: if x=0 and N=1, it means Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams" Proposal 2: After TCI state switches to a setA beam, spec mandates UE to “know” the RX beam of that setA beam in, at least, the following scenarios: - In “wide-to-narrow” beam prediction - In “narrow-to-narrow” beam prediction if SSB resources are configured during the tests - Note 1: In both “wide-to-narrow” and “narrow-to-narrow” beam prediction, tests ensure the following: - setA CSI-RS resources are configured without “repetition = ON”, - setA CSI-RS resources are QCLed wrt type D with SSBs - Note 2: In “wide-to-narrow” beam prediction, UE measures SSBs and predicts properties of setA CSI-RS resources. In “narrow-to-narrow” beam prediction, UE measures a subset of CSI-RS resources (setB) and predicts properties of full set (setA) of CSI-RS resources. Proposal 3: The TCI state of a beam in setA is known in the scenarios of proposal 2 if the following conditions are met: - During the period from the last prediction-based CSI reporting for the target TCI state to the completion of active TCI state switch, where the RS resource for prediction-based CSI reporting is the RS in target TCI state or QCLed to the target TCI state - TCI state switch command is received within X ms upon the last prediction-based CSI reporting - The SSB associated with the TCI state remains detectable during the TCI switching period - SNR of the SSB associated with the TCI state ≥ Y dB - FFS: Value of X and Y Proposal 4: RAN4 waits until RAN1’s conclusion on minimum time separation between setB resources and corresponding report containing predicted properties of setA to decide inference delay. |
| R4-2505603 | CAICT | Proposal 1: suggest to consider top-N (N</=K) predicted beams in the success rate evaluation. Observation 1: For beams that are configured for measurement under BM-case 1, the reported value could be either measured or predicted RSRP, up to vendor implementation. |
| R4-2505741 | Qualcomm Incorporated | Observation 1: The key requirements for AI/ML BM test setup are listed following: - Requirement#1: Channel model in the test - The test should reflect performance in the field. Hence, CDL or simplified CDL to properly verify beam management performance. - Requirement#2: Avoid the cheating - Progress A: Model training for Set A’s is generated with channel parameters #1. Set B in UE measurement should have enough randomness compared to Set A’. - Progress B: Set A is generated to make sure to use the same parameters (initial phase, angular spread, delay spread) as the Set B in the Progress A measurement step. NW antenna/beam configurations for Set A and Set B should be deterministic. For some prediction metrics (e.g., option 2 and 3), UE can pass the test without actual measurement if TE does not randomize the transmission of its beams. - Requirement#3: Support the beam number of Set A - TE needs to support Set A for ground truth verification. Observation 2: Verifying the AI/ML beam management performance in a TDL channel with single AoA without spatial characteristics makes the test too simple and inconsistent with the actual field environment. Thus it is not feasible to general proper Set A and Set B with single AoA setup. Observation 3: For single AoA test setup with two-step approach, there are following limitations: - UE has to report the UE antenna pattern with X*Y test points that is very time consuming. - UE needs to support additional test function for RSRP reporting - If Rx beam ID is one of the input for AI/ML model, additional test mode/signalling is needed - UE’s beam management performance can’t be verified properly since for each of UE position there is enough dewll time and UE beam is locked. Observation 4: There will be plenty of additional probes if BS Tx beams point to other clusters even though some of the probes could be reused. Observation 5: The UE’s antenna gain gap between peak and 50%-tile spherical coverage is over 10dB for PC3. The very weak clusters would not influence the UE beam management even if they reach the UE within the UE’s spherical coverage. Observation 6: The AoDs/ZoDs of very weak clusters are unlikely to influence the selection of the strongest beam in any reasonably designed codebook of Set A and Set B. Observation 7: The weak clusters that have a limited impact on the beam management could be removed to reduce the number of clusters in the CDL channel model. Observation 8: The clusters that have the same/similar AoAs could be merged. And the intra-cluster angle spread, e.g., CASD, CASA, CZSD, CZSA can be reduced to simplify the test setup. In this case, a new simplified channel model is emulated by test setup. Observation 9: To generate Set A and Set B, multiple AoAs/probes test setup is necessary. Observation 10: Using Multiple AoA test setup with FR2 RRM test can reuse the FR2 existing test setup to save the test cost and a simplified CDL channel model needs to be specified. Observation 11: Pros and Cons for single AoA and Multiple AoA test setup presented in Table 2. Table 2: Analysis for single AoA and Multiple AoA test setup Single AoA test setup; Single AoA test setup with two steps approach; Multiple AoA test setup with 3D MPAC; Multiple AoA test setup with FR2 RRM test setup (eIFF). Chanel model; TDL; CDL; Simplified CDL; Simplified CDL. Number of probes; 1; 1; 6-8; 4. Complexity of testing and TE implementation; Low; High Note: Very time-consuming for Step 1. And additional test mode or signaling is needed.; Medium Note: Additional probes are needed on top of 3D-MPC; Low Note: Reuse existing FR2 RRM test setup. Realistic environment; Low; Low Note: UE beam management performance can not be verified properly.; High/Medium Note: It depends on what does simplified CDL look like; High/Medium Note: It depends on what does simplified CDL look like. Whether the test system can support spatial information like UE Rx beam, UE beam pattern, etc is used; No; No; Yes; Yes. Proposal 1: Single AoA test setup incluidng the use or non-use of the two-step method should not be considerd for AI/ML BM testing. Proposal 2: RAN4 should consider the follwoing two multiple AoAs/probes options as the test seup for AI/ML beam management testing - Option 1: Multiple AoA test setup with 3D MPAC - Consider reducing the cluster number with weak power level, merging the cluster with the same/similar AoA, and decreasing intra-cluster angle spread in CDL channel model to generte a new simplified CDL channel model - Define the paramters for the new simplified CDL channel model based on that 3D-MPAC test setup is used - Take CDL-C UMa as the starting point - Option 2: Multiple AoA test setup with FR2 RRM - Conider multiple AoA test setup with FR2 RRM test setup to emulate compareble Tx beam with simplifed CDL, e.g., generating enough Tx beam number the solutions such as combinaiton of probes transmission and power offset for each of probe can be considered. - Define the paramters for the new simplified CDL channel model based on that FR2 RRM eIFF test setup is used - Take CDL-C UMa as the starting point Propsoal 3: Companeis to evlaute whether the new simiplied CDL channel model generated from candidate test sysetms can be used to properly verify AI/ML BM perforance. Validate the proposed simplified channels from Pption 1 and Pption 2 through AIML BM system level simulations. FFS on how to validate the propsoed similied channel model from Option 1 and Option 2. |
| R4-2505764 | Xiaomi | Proposal 1: For both BM case-1 and BM case-2, N can be smaller than 8 if specific Rx beam(s) is applied. Proposal 2: For BM-case 2, introduce a scaling factor Q to adjust the measurement time for RSRP predictions, where Q is determined by UE capability, to adopt to different UE implementations. Proposal 3: For BM-case 2, RAN4 to define prediction window: - Stating point of prediction window is: o The most recent occasion of the CSI-RS/SSB resource in Set B for measurement + Reference time of the earliest time instance for the predicted results - Ending point of prediction window is: o Starting point + Ttime_gap *N, where Ttime_gap is the time gap between two consecutive future time instances configured by RRC, and N is the number of future time instance(s) configured by RRC. Proposal 4: For BM-case 2, the scheduling and measurement restriction can be relaxed in the prediction window. Observation 1: When best predicted TX beam is in set B, UE knows the best RX beam. When best predicted TX beam is in set A but not in set B, it’s still possible for UE to know the RX beam corresponding to the best TX beam. Proposal 5: RAN4 may know RX beam based on UE capability. Proposal 6: For OTA test, prefer to split the test into two parts. - Part 1: Verify AI model prediction algorithm, using single AOA model. - Part 2: Verify input data accuracy of model AI based on RX beam sweeping, using multi-AOA model. Observation 2: If Emulated signal considers both TX and RX beamforming gain, RX beam pattern is UE specific. In order to reduce test complex, prefer to design a common pre-defined RX beam pattern for both TE and UE. Observation 3: if a fixed RX beamforming gain is used for multi-path TX signals, there will be some mismatch between emulated signal for test and original signals for training. The mismatch impact needs to be evaluated. Observation 4: For simplified channel model, L1-RSRP delta between different TX beams will increase if cluser number and AOA number is further reduced. It’s more easiler for AI to distingwish the best Top-K TX beam. The AI prediction requirement will be relaxed. Proposal 7: For single AOA, RAN4 to discuss whether it’s possible to design common pre-defined RX beam pattern to generate emulated single AOA signals. Proposal 8: For simplified multi-AOA CDL model, cluster number can be reduced from spatial angle offset and power aspect: - Combine clusters if AOA offset between these clusters is smaller than a threshold - Remove cluster with low power. Proposal 9: If the test metric is beam index prediction accuracy, UE may need to report ground truth of N best beam index for set A. Observation 5: For deriving ground truth of set A, SNR level should satisfy that for at least Top-M beams, the measurement error is small. SNR didn’t need to guarantee that all beams in set A can be measured accurately. Proposal 10: For deriving ground truth of set A, SNR level should satisfy that for Top-M beams, the measurement error is small. RAN4 to discuss how to decide M and how to set SNR. Proposal 11: For BM case-1, channel doppler can set to 0 or a small value to guarantee that there is neglectable L1-RSRP variation. Proposal 12: for BM case-2, channel doppler will depend on UE speed and UE trajectory. RSRP variation impact can be considered in RSRP accuracy requirement. Observation 6: UE will report best predicted L1-RSRP/beam index at T1 and measured RSRP/beam index of best beam at T2 to TE, it’s easy for UE to pass the test by cheating. Proposal 13: RAN4 to discuss how to solve the UE cheating issue if UE report both predicted result and ground truth. |
| R4-2505942 | Apple | Observation 1: The measurement period for Set-B would depend on the accuracy needed for prediction. Observation 2: The legacy definition for known TCI state cannot be used when target TCI state is based on predicted beam and not transmitted to and measured & reported by UE. Observation 3: RAN4 needs to discuss known TCI for UE side and NW side prediction. Observation 4: The testing environment for Beam Management (BM) case 1 and BM case 2 should replicate conditions that sufficiently capture the correlations of the transmit (Tx) and receive (Rx) beam patterns across the entire spectrum of propagation conditions in both spatial (angles of arrival and departure - AoAs and AoDs) and temporal domains. Observation 5: The following conditions will introduce randomness and variations in propagation conditions across both time and spatial domains for the computation of L1-RSRP - Different AoDs with respect to the Tx antenna array - Different AoAs with respect to the Rx antenna array - Different superpositions of {AoA,AoD} pairs - Fading/Variation in time domain ( different {AoA,AoD} pairs per resolvable delay bin path) - UE movement (including rotation) Observation 6: During real-world deployment, UE will encounter random radio propagations characterized by variations in both spatial and temporal domains (fading). Testing UE under similar conditions is important to reflect the realities of deployment accurately. Observation 7: The following questions need to be answered to evaluate the feasibility of the FR2 OTA-based test procedure: - How can we generate multiple beams from the Set-B and Set-A Tx beams given the limitation of two AoAs? - What assumption is made regarding Rx beam sweeping? Does the UE utilize a fixed Rx beam, or does it sweep to find the optimal Rx beam? - How can we simultaneously emulate different AoAs (Rx beam) and AoD (Tx beams)? - How can we achieve dynamic variation in the AoD domain (Tx beam sweeping) for BM case 2 prediction? Observation 8: For RAN 4 testing we should have the following goals: (1) Provide confidence that if a DUT passes the test it will also perform well in the field (2) Ensure reliability of using synthetic channels for test data in evaluating models trained on real data. Observation 9: Rather than simulating CDL channels and [64-128] Tx beams using multiple angles of arrival (AoAs) and departure (AoDs), the spatial-selective and time-varying propagation conditions can be effectively represented by adjusting the Tx power at the test equipment (TE) for each beam at different times complexity (number of different channel realizations for power measurements, different number of parameters that need to be emulated in the fading simulator) and establishing accuracy on the test metric (top-K) beam prediction accuracy Proposal 1: Use legacy measurement period as a starting point for Set-B for evaluation if sufficient to achieve good prediction accuracy. Proposal 2: For AI-BM Case 2, the number of L1-RSRP measurement occasions on Set-B should be configurable and studied further in RAN1. Proposal 3: For UE side model known TCI state is determined based on Set B transmission and UE report of top-K. Proposal 4: For NW side model - (1) TCI state is unknown if RS of target beam is not transmitted to UE prior to TCI state switch (2) If RS of target beam is transmitted to the UE and reported, legacy definition of known TCI state is applicable Proposal 5: NW side data collection, QCL type D is signaled between set A and set B RS. Proposal 6: For NW side model, TCI state is known if QCL source is signaled in both data collection and inference (to ensure UE Rx consistency), it is known. Otherwise, it is unknown. Proposal 7: (for Alt 1): Define UE capability for Alt 1 of UE implementation. When UE does not support this capability, the TCI state is unknown. Unknown TCI state switching delay is applied. Proposal 8: (for Alt 2): When gNB signal the QCL relationship between set A and set B, in training RS configuration, inference RS configuration and performance monitoring RS configuration (example of Alt 2 NW beam implementation of set A and set B), The TCI state is known when the following is met: Target TCI state is based on RS from set A beams with QCL relationship configured to a set B beam The time between TCI state switch command and transmission of the QCLed RS from set B shall not exceed X ms X = 1280 ms (legacy) UE shall send measurement report for the QCLed RS from Set B between transmission and TCI state switch TCI state and SSB of TCI state remain detectable during TCI state switching period Otherwise, it is unknown. Proposal 9: The ability to identify the Rx beam associated with an unmeasured predicted Tx beam may rely on the UE’s supported features Proposal 10: RAN4-defined prediction accuracy requirements is proposed in the context of performance monitoring Type 1, and for Option 2 (UE-assisted performance monitoring) Proposal 11: For UE-side AI/ML beam management, LCM operations such as activation, switching, and fallback can be supported via CSI report configurations. RAN4 should evaluate the latency impact of these operations for periodic, semi-persistent, and aperiodic CSI reporting modes. The analysis should cover BM-Case1 and BM-Case2, considering processing time, measurement intervals, and model availability in the UE. Proposal 12: Multi AoA based Testing Setup for BM: Evaluate the testability requirements for simulating time-varying input power to a sparse probe layout based on a simplified channel CDL model. The key aspects to be considered are: 1. How to determine the minimum number of clusters to be emulated without introducing bias in the results. Investigate a quantitative “goodness” criterion to selecting the number of probes/clusters. 2. Consider incorporating UE rotation for the test 3. How many different CDL channels we need to support (considering generalization purposes as well) Proposal 13: To establish a criterion for assessing the feasibility of a simplified sparse probe layout (Option 5) for beam management (BM) testing, the following procedure can be considered: - Train a model using the reference CDL channel and test it with the simplified sparse layout-based CDL channel. - Train a model using the simplified CDL channel and test it with the reference CDL channel model. - Train and test a model using the reference CDL channel. A similarity metric should be defined to compare the RSRP accuracies across these test cases. This metric would indicate the effectiveness of testing with the simplified CDL model. The similarity metric can be derived through simulations. Proposal 14: For BM-Case 1 spatial prediction and for verifying RSRP accuracy we propose to use the testing setup and channel emulator functionality as described in Fig 6. (multiple static configurations) Proposal 15: For testing the beam prediction/RSRP accuracy for BM-Case 2 temporal prediction we propose the framework described in this section and visualized in Fig 7, through a non static configuration of the channel emulator. Fig. 9: Proposal for single AoA BM testing setup (flowchart) Proposal 16: RAN4 to consider the set of procedures/steps shown in the flowchart for adopting a single AoA BM testing setup Proposal 17: RAN4 to evaluate the tradeoff between testing complexity (number of different channel realizations for power measurements, different number of parameters that need to be emulated in the fading simulator) and establishing accuracy on the test metric (top-K) beam prediction accuracy Proposal 18: RAN4 to evaluate if a power measurement should be performed with a single Rx beam or the optimum Rx beam should be selected from an Rx codebook to measure the power of set B. Proposal 19: RAN4 to discuss Option2 (a Reference Complex UE Rx Beam/Antenna pattern) for emulation of Rx patterns in the fading simulator Proposal 20: Distinguish between the descriptions of power based CDL channel model and the original complex domain CDL channel model Proposal 21: RAN4 to evaluate the prediction performance difference between a reference complex UE Rx beam/antenna pattern and its own complex pattern to ensure that testing will provide confidence that UE will perform well in the field if it passes the test |
| R4-2505980 | CATT | Proposal 1: RAN4 to verify at least half of predicted beams, i.e., N= K2+1. Proposal 2: The same x value applies for each comparison between the same ranked predicted beam and genie-aided beam. Value of x is further discussed based on the simulation results. Proposal 3: RAN4 not to apply absolute RSRP accuracy requirement to other beams, i.e., predicted beams with lower RSRPs. Proposal 4: RAN4 to remove the square bracket in the definition of relative RSRP accuracy, i.e., Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - reported L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), [where the beam index n owns the largest reported value]. Proposal 5: The reported L1-RSRP in the relative RSRP accuracy definition can be measured RSRP. Proposal 6: The relative RSRP accuracy requirement can apply for BM-Case 2. Observation 1: For BM-Case 1, UE measurement behaviour on SetB beams is the same as L1-RSRP measurement. The prediction for SetA beams in spatial domain is a short UE internal procedure. The legacy L1-RSRP measurement requirement can be reused for BM-Case 1. Observation 2: For BM-Case 2, UE measurement behaviour on SetB beams is the same as L1-RSRP measurement. The prediction for SetA beams over multiple future time instances is one-shot UE internal procedure. The legacy L1-RSRP measurement requirement can also be reused for BM-Case 2. Proposal 7: RAN4 to reuse legacy L1-RSRP measurement requirement for both BM-Case 1 and BM-Case 2. |
| R4-2506228 | NTT DOCOMO, INC. | Observation 1: If N = 1, the prediction reliability is doubtful. If N = K, the requirement is too strict. Proposal 1: N = 2 is better to ensure both reliability and usability. Observation 2: In the case of N = 2, x cannot be zero. If N = 2 and x = 0, the successful case is limited to the case in which the ground truth of top 2 beams is the same value. Proposal 2: x should be less than 3. Proposal 3: For case 2, prediction delay is not needed because UE predicts Set A beam based on the measurement history of Set B beam, UE can predict the best beam in advance. |
| R4-2506281 | Anritsu Corporation | Observation 1: RAN4 needs to clarify all the necessary conditions of associated test setup for both BM-Case 1 and Case 2 along with their channel models to be applied. Observation 2: Considering the UE behavior in the actual field, there might be a need to conduct tests with mixed conditions of BM-Case 1 and Case 2 and run the tests in series. For example, we conduct a test with BM-Case 1 condition as a first step and then another test with BM-Case 2 condition follows later based on the predicted results from the first BM-Case 1 test. Additional tests with BM-Case 1 condition might also be run at some points during the BM-Case 2 test. Proposal 1: Companies are encouraged to bring views on the necessity of such mixed conditional tests. (i.e. BM-Case 1 and/or BM-Case 2) Observation 3: Since the group has not completed analyzing which type of channel model can ensure the field performance of the AI model, it is not clear at this moment whether the legacy CDL channel model is necessary or the simplified CDL model can suffice. Observation 4: Because in a real field propagation path, there can be variations in the direction of arrival of each path (direction of clusters), etc. Therefore a test result does not fully reflect its performance in the real field when an AI/ML BM-enabled UE is tested using only one channel model. Proposal 2: Define multiple channel models like the existing CDL models (e.g. CDL-1/2/3) to ensure the field performance of the AI model for beam management. FFS for the necessary number of channel models. Observation 5: There are currently three kinds of presumable assumptions from the AI model input PoV. AI-model-input assumption 1: No need to input Rx Beam information in the AI model. In this case, it is expected that the UE may use an Rx beam for its RSRP measurement with a single direction. AI-model-input assumption 2: One Rx Beam information is input to the AI model. In this case, the direction of the UE Rx beam is the same as AI-model-input assumption 1 and that Rx Beam information is input to the AI model. AI-model-input assumption 3: Multiple Rx Beam information is input to the AI model. In this case, for each Tx Beam ID in set B, UE will change its Rx beam direction to multiple directions and multiple L1-RSRP measurements are made per Tx Beam ID. Multiple Rx Beam information is input to AI model per Tx Beam ID. Observation 6: BM-Case 1 tests with AI-model-input assumption 1 and 2 (i.e. less than or equal to one Rx Beam information is input to the AI model) UE can be conducted by 1AoA setup with the following choices. - Rx beam pattern is standardized or, - Rx beam pattern is measured during the evaluation. Observation 7: The feasibility of BM-Case 1 tests with AI-model-input assumption 1 and 2 UE by multi-AoA test setup depends on the channel models to be applied. Observation 8: During the test with AI-model-input assumption 3 in the multi-AoA setup, UE Rx beam would be swept to multiple directions even for the measurement period of one Tx Beam ID, where the detailed number of Rx Beam ID to be used for the measurement would be up to UE implementation. Observation 9: To enable the test of UE with the AI-model-input assumption 3 by 1AoA test setup, there is a need to agree on the use of test modes. Observation 10: The feasibility of BM-Case 1 tests with AI-model-input assumption 3 UE by multi-AoA test setup also depends on the channel models to be applied. Observation 11: Link level simulation in RAN1 appears to apply the assumption of the channel model described in TR 38.901 clause 7.6.3.2 [4], i.e. Spatially-consistent UT/BS mobility modelling. Observation 12: When considering the test method of BM-Case 2 in RAN4, we need to study how we can create the channel environment by the candidate test setups (1AoA or multiple AoA) in a case that the UE moves from one point to another. Observation 13: There is a benefit that 1AoA test system with the fading simulator can simulate the channel model even with the case that UE moves, though there is a limitation that the system cannot change the Rx Beam ID in a short period without test mode. Observation 14: For BM-Case 2, it is expected that there are some testability issues with the multi-AoA setup. - Channel models described in TR 38.901 clause 7.6.3.2 (i.e. Spatially-consistent UT/BS mobility modelling.) might be difficult to reproduce. - Positions and directions of clusters cannot be changed since measurement probes are fixed in the OTA chamber. Observation 15: Testability analysis of 1AoA and multi-AoA test setups are summarized as follows. Table 2.4-1: Testability analysis of 1AoA test setup for BM-Case 1 and/or Case 2 Test of BM-Cases; AI-model-input assumption. Assumption 1 and 2 (Less than or equal to one Rx beam information is input); Assumption 3 (Multiple Rx beam information is input). BM-Case 1; Yes Possible with any channel model conditions by simulating all channels within a fading simulator.; It depends. Testability issue remains in a case test mode is not applicable since the system cannot change the UE Rx beam ID in a short period.. BM-Case 2. Both BM-Case 1 and 2. Note; - Rx beam profile(s) needs to be standardized or obtained by measurement to include it in the fading simulator. - Feasibility issue of Rx beam sweeping with a UE supporting AI-model-input assumption 3 remains in a case the test mode is not applicable. - The required functions of the test mode could be either a function to pause L1-RSRP measurement or a function to control the Rx beam directions (ID).. Table 2.4-2: Testability analysis of multi-AoA test setup for BM-Case 1 and/or Case 2 Test of BM-Cases; AI-model-input assumption. Assumption 1 and 2 (Less than or equal to one Rx beam information is input); Assumption 3 (Multiple Rx beam information is input). BM-Case 1; It depends. Could be possible by applying a simplified CDL. FFS on the acceptable levels of the simplification. Testability issue remains in a case multiple CDL models need to be used or if the simplified CDL model cannot be used to ensure the prediction performance of the AI model in a field.. BM-Case 2; It depends. There are some restrictions on the achievable channel environment - Channel models described in TR 38.901 clause 7.6.3.2 (i.e. Spatially-consistent UT/BS mobility modelling.) might be difficult to reproduce. - Positions and directions of clusters cannot be changed since measurement probes are fixed in the OTA chamber.. Both BM-Case 1 and 2. Note; - A study has not been completed on the applicable channel models to ensure the prediction performance of the AI model in the field. So, it is not clear yet if the simplified CDL channel model can suffice.. Observation 16: There is no comprehensive test solution to evaluate both BM-Case 1 and BM-Case 2 at this moment. Proposal 3: Use different types of test setups (i.e. 1AoA or multi-AoA setup) depending on the AI-model-input assumptions and BM-Case 1 and/or 2 with case-by-case manner. |
| R4-2506347 | Ericsson | Observation 1: If only beam ID is reported, the order of top-K beams is able to indicate the order of RSRP implicitly. Observation 2: From the network perspective, obtaining a plurality of predicted beams, each subject to specific prediction accuracy requirements, is advantageous for enhancing reliability and robustness in beam management. Observation 3: For AI/ML model supporting RSRP and beam ID reporting, if only RSRP accuracy is required, an underlining problem is that the Top-1/K predicted beam is far away from the Top-1/K genie-aided beam. Observation 4: The metric/report type of prediction results, prediction accuracy or L1-RSRP offset, for prediction and the metric for monitoring may be different since they serve different purposes. Observation 5: the measurement error in datasets for training/testing may be covered by the overall measurement error. Proposal 1: Beam ID prediction metrics and KPIs should include evaluations for both K = 1 and K = [2] to ensure a comprehensive assessment of prediction performance. Proposal 2: RAN4 to determine the test scope for the case of beam ID being reported, we prefer option 2 between the two below options. - Option 1: Only the top-K beams shall be verified. - Option 2: All Set A beams meeting the SNR side condition for BM prediction shall be verified. Proposal 3: RAN4 to investigate the SNR side condition of the test metrics for the cases of beam ID reporting. Two potential options are under consideration in the context of beam ID reporting: - Option 1: Top-K beams in the test metrics shall meet the side condition for BM prediction. - Option 2: All Set A beams shall meet the side condition for BM prediction, in which, top-K has higher SNR side condition which shall be defined. Proposal 4: The parameter x in beam prediction accuracy metrics should be set to a value that reflects the expected magnitude of measurement error and other errors. Proposal 5: We’re open to introducing N (N is [2]) to enable verifying the RSRP of each beam within the top-N predicted beams. If N is introduced, the metrics may be represented to: - The ground-truth RSRP of the 1st predicted beam larger than or equal to the ground-truth RSRP of the 1st strongest genie-aided beam(s) – x1 dB, - The ground-truth RSRP of the 2nd predicted beam larger than or equal to the ground-truth RSRP of the 2nd strongest genie-aided beam(s) – x2 dB, - … - The ground-truth RSRP of the Nth predicted beam larger than or equal to the ground-truth RSRP of the Nth strongest genie-aided beam(s) – xN dB Proposal 6: Metrics/KPIs for beam management requirements/tests are outlined by the below options: - In case of the AI/ML model with label (beam ID) output type - Beam prediction accuracy is adopted. - In case of the AI/ML model with label (beam ID) + RSRP output type - Beam prediction accuracy and RSRP accuracy, are adopted simultaneously. Proposal 7: For the case of the AI/ML model with label (beam ID) + RSRP output type, fulfilling beam prediction accuracy and RSRP accuracy may result in more stringent requirements compared with the case of the AI/ML model only with label (beam ID) output type. Proper thresholds for the dual requirements shall be evaluated. Proposal 8: The absolute RSRP accuracy requirement applies to other beams besides of the top-1 of predicted beams. Proposal 9: For the absolute RSRP, the index i shall be any beam index in all beams based on predicted L1-RSRP as per UE reporting. Proposal 10: RAN4 to determine the test metric scope for the cases of RSRP reporting, we prefer option 2 between the two below options. - Option 1: Only the top-K beams shall be verified. - Option 2: All Set A beams meeting the SNR side condition for BM prediction shall be verified. Proposal 11: RAN4 to study specifying the SNR side condition of the test metrics for top-K beams for the cases of RSRP reporting, we prefer option 2 between the two below options. - Option 1: Top-K beams in the test metrics shall meet the side condition for BM prediction. - Option 2: All Set A beams shall meet the side condition for BM prediction, in which, top-K has higher SNR side condition which shall be defined. Proposal 12: In practice, reported L1-RSRP can be predicted or measured RSRP, it’s up to NW configurations. But, for test perspective, reported L1-RSRP shall be predicted and not measured RSRP by managing proper test configurations. Proposal 13: Relative RSRP accuracy requirement is applicable for Case 2. Proposal 14: Regarding measurement period for prediction, for both BM-case1 and BM-case2, the legacy L1-RSRP measurement delay shall be applied, with the assumption that the measurement accuracy remains consistent. Proposal 15: RAN4 to study whether specific requirements are necessary to evaluate the capability of a UE in tracking changes in L1-RSRP through measurements in BM-case 2. Proposal 16: RAN4 shall define measurement delay for monitoring. Proposal 17: RAN4 shall study the new known condition of the TCI state switch, to which the RS referred is which is ‘known’ by the prediction without any measurement. Proposal 18: Knowing the Rx beams is a crucial factor in TCI state switch enhancement in terms of RS prediction. Proposal 19: RAN4 to study the two questions regarding multiple/single AoA: - What is the (maximal) difference between really tested Tx/Rx beam and the artificial Tx/Rx beam? - If the difference can impact AI/ML model? Proposal 20: RAN4 to check if the 32 beams after simplified CDL channel can be produced by 8 or 12 probes which may achieve the balance between performance and implementation complexity. Proposal 21: Measuring Tx/Rx beams using a TDM approach may be a valid method; however, the control mechanism for Rx switching must be explicitly addressed. Proposal 22: The simplified CDL channel only is considered to be applied in inference, no need to be applied in training. Proposal 23: The metric to estimate the “goodness” of a simplified CDL channel model may be represented by the below steps: - Step 1: Test (inference) with the reference CDL channel. - Step 2: Test (inference) with the simplified CDL channel. - Step 3: The prediction difference between the results of the two tests in the above steps is less than a threshold with [95] percentile. Proposal 24: RAN4 to evaluate the SNR condition for the ground truth by simulation. Proposal 25: Test procedure shall at least comprise the below three steps: - Acquire the predicted RSRP under the required corresponding SNR condition - Acquire the ideal measurement of RSRP under high SNR condition - Compare the two RSRPs acquired in the above steps Note 1: Test configurations are kept same in the steps except for different SNR conditions. Note 2: Arranging the details on the sequence and test configuration may need further investigation in performance part. Proposal 26: For BM-case1, same channel is applied for measurement and inference. Proposal 27: For BM-case2, plural channel instants with a series of changes of the channel parameters are necessary to reflect temporal correlation between channels. At least, it includes: - Generating a set of channel instants, from channel #a+1 to channel #a+m, for measurement, and - Generating a set of channel instants, from channel #b+1 to channel #b+n for inference Where, - The channels instants comprise sequent and grade updates of a set of parameters. - The numbers of m and n depend on the measurement window and prediction window of BM-case2. - The channel parameters may comprise one or more than one of - Departure angles (AOD, ZOD) - Arrival angles (AOA, ZOA) - UE position and/or rotation - Channel coefficient Proposal 28: In order to avoid overfitting UE models to the test environment only, the test (including data sets) shall be able to demonstrate the robustness of the AI/ML model and match the practical field deployment conditions. |
| R4-2506464 | vivo | Draft CR on RRM core requirements for support of AI/ML for NR Air Interface for beam management CR drafting to be discussed separately |
| R4-2506467 | vivo | Proposal 1: For spatial prediction, the measurement of set B relies on one-shot measurement Proposal 2: For L1-RSRP measurement of set B, the definition and value of the beam sweeping factor follows existing requirements. Observation 1: Due to the introduction of multiple feature enhancements in previous releases, such as measurement gap enhancement, significant improvements have been made to P. Proposal 3: When defining AI/ML-based L1-RSRP measurement, specifically for sharing factor P, RAN4 will discuss the coexistence supportability of the prediction with other previously enhanced features, the candidate Options include: - Option 1: The AI/ML-based BM can be configured simultaneously with enhanced features introduced in previous releases. The corresponding L1-RSRP requirements need to take these enhancements into account, such as MUSIM gap, NCSG, etc. - Option 2: Fom the perspective of spec simplification, the requirement definition here will be based on the R15 spec version and the enhanced features introduced in previous releases are not supported by default Proposal 4: For prediction delay requirements TL1-RSRP_Prediction_Period_CSI-RS for BM, it can be defined as Table : Prediction period TL1-RSRP_Prediction_Period_CSI-RS for FR2 Configuration; TL1-RSRP_Prediction_Period_CSI-RS (ms). non-DRX; max(TReport, ceil(M*P*N)*TCSI-RS+ Tprediction_processing). DRX cycle ≤ 320 ms; max(TReport, ceil(1.5*M*P*N)*max(TDRX,TCSI-RS)+ Tprediction_processing). DRX cycle > 320 ms; ceil(M*P*N)*TDRX + Tprediction_processing. NOTE 1: TCSI-RS is the periodicity of CSI-RS configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Tprediction_processing is time for prediction post-processing, FFS on the value. NOTE 2: the requirements are applicable provided that the CSI-RS resource configured for L1-RSRP measurement is transmitted with Density = 3.. Proposal 5: For the UE Rx beam knowledge, - at least for the case that the Top-K beams to be reported are in set B and set B is a subset of set A, if beam sweeping has been done during measurements on set B, UE Rx beam information is known by UE and no further beam sweeping is needed. - for other cases, e.g., set B is not a subset of set A, RAN4 to further discuss how to obtain Rx beam information: - introducing UE capability - Considering SSB-predicted CSI-RS, where the best Tx beam is not in set B but has a QCLed relationship with a beam in set B, it can be also regard as known Proposal 6: RAN4 to update the known TCI state conditions as The TCI state is known if the following conditions are met: - During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target TCI state or used for L1-RSRP measurement for predicted RS quantities reporting to the completion of active TCI state switch, - where the RS resource for L1-RSRP measurement is the RS in target TCI state or QCLed to the target TCI state, or the RS resource for L1-RSRP measurement for predicted RS quantities reporting is the RS in target TCI state or QCLed to the target TCI state - or where the RS resource for L1-RSRP measurement for predicted RS quantities reporting is not the RS in target TCI state or QCLed to the target TCI state, but UE supports [UE capability for fast beam sweeping], - TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement - The UE has sent at least 1 L1-RSRP report for the target TCI state before the TCI state switch command or predicted RS quantities reporting - The TCI state remains detectable during the TCI state switching period - The SSB associated with the TCI state remain detectable during the TCI switching period - SNR of the TCI state ≥ -3 dB Otherwise, the TCI state is unknown.. Proposal 7: For performance monitoring for BM, the correasponding delay is defined as Tmoitoring_period_BM = Nmonitor* TL1-RSRP_Prediction_Period_CSI-RS, Where Nmonitor is the configured number of the latest transmission occasion(s) of monitoring resources with linked inference report no later than CSI reference resource corresponding to the CSI report for monitoring Proposal 8: RAN4 to define absolute RSRP accuracy as one of metrics for beam management requirements/tests, the corresponding definition is: - The absolute RSRP accuracy = predicted L1-RSRP of beam index i – ground-truth of L1-RSRP of beam index i. The index i corresponds to the index of Top-1 beam of predicted beams Proposal 9: RAN4 to use the follow definition to define the relative RSRP accuracy requirement: - Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - Reported L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), where the beam index n owns the largest reported value - Reported L1-RSRP refers to predicted L1-RSRP, Proposal 10: For Metrics/KPIs for beam ID prediction agreed in the last meeting, RAN4 not to further consider the verification of top-N (N</=K) predicted beams in the success rate evaluation Proposal 11: For Metrics/KPIs for beam ID prediction agreed in the last meeting, it will apply to the case only beam ID is reported and the case both beam ID and L1-RSRP are reported. Proposal 12: For the X value of Metrics/KPIs for beam ID prediction agreed in the last meeting, it can be determined based on the simulation results Observation 2: The simplified CDL channel with small number clusters will degrade the test dependability. Observation 3: The number of probes can be greatly reduced by grouping different Tx beams together. Only one probe or one set of probes can emulate different Tx beams in one group. Observation 4: Different test system with reduced cost and complexity can be introduced by using the Tx beam grouping method. Proposal 13: RAN4 to consider methods to group Tx beams together to reduce the number of probes in the test system. Proposal 14: RAN4 to consider the following procedures to simplify the test system: - Step 1. Generate the BS beamforming power pattern of every Tx beam - Step 2. Add the BS beamforming power pattern into the power of different clusters in specific ZoD and AoD, and derive the new power of every cluster - Step 3. Use the new power of clusters to generate the power angular spectrum (PAS) of ZoA and AoA - Step 4. Some Tx beams with the corresponding ZoA/AoA are to be grouped together based on a certain rule (e.g., the ZoA/AoA of the strongest power derived from the PAS in step 3 are the same angle or in a pre-defined range) - Step 5. Using one or small number of probes to emulate one beam group. - Step 6. Delete the unnecessary probes and merge some adjacent probes. Proposal 15: RAN4 to consider the following two approaches for design the test system Proposal 16: Companies to provide candidate probe locations for CDL-A/B/C for merging and rotating. E.g., Proposal 17: RAN4 to consider the UE Rx beamforming resolution in angular domain to perform OTA probe allocations. |
| R4-2506506 | MediaTek Inc. | Observation 1: Both DL Tx beam prediction and beam pair prediction are studied in R18 AI/ML SI, but only DL Tx beam prediction is in the scope of R19 AI/ML WI. Proposal 1: Whether Rx beam of the predicted Tx beam which is not measured can be known or not depends on different cases. - Set A and Set B are different (Set B is NOT a subset of Set A). E.g., use known SSB beams to predict CSI-RS beams: - if the predicted Tx beam which is not measured is QCL Type-D to a known Tx beam in Set B, the corresponding Rx beam is known. - Set B is a subset of Set A. - Use part of SSB beams to predict all the SSB beams: Whether Rx beam of the predicted Tx beam which is not measured can be known can be up to UE capability. - Use part of CSI-RS beams with repetition off to predict more CSI-RS beams with repetition off: If Rx beam of the QCL-ed SSB is known, then Rx beam of the predicted CSI-RS beam is also known. Observation 2: In terms of Top K/1 prediction accuracy, the performance of AI/ML BM case 1 models for different scenarios may have much difference. Proposal 2: Further study and discuss whether it is possible to define scenario-agnostic performance requirements for AI/ML BM. If not, maybe we can seek for similar solutions as TDCP, i.e., define the test cases for limited scenarios instead of defining scenario-agnostic accuracy requirements. Proposal 3: Further discuss which scenario among the following to define requirements and test cases for: - Scenario 1: Set A and Set B are different (Set B is NOT a subset of Set A). E.g., use SSB beams to predict CSI-RS beams. - Scenario 1a: AI model inputs are the RSRP with best Rx beam for each Tx beam in set B - Scenario 1b: AI model inputs are the RSRP with specific Rx beam for each Tx beam in set B - Scenario 2: Set B is a subset of Set A - Scenario 2-1: use part of SSB beams to predict all the SSB beams. - Scenario 2a-1: AI model inputs are the RSRP with best Rx beam for each Tx beam in set B - Scenario 2b-1: AI model inputs are the RSRP with specific Rx beam for each Tx beam in set B - Scenario 2-2: Use part of CSI-RS beams with repetition off to predict more CSI-RS beams with repetition off Proposal 4: Only define the accuracy requirements and test cases for one of the following two scenarios: - Scenario 2a-1: use part of SSB beams to predict all the SSB beams - AI model inputs are the RSRP with best Rx beam for each Tx beam in set B - Scenario 2-2: Use part of CSI-RS beams with repetition off to predict more CSI-RS beams with repetition off Proposal 5: Both BB error and RF error should be considered when define the requirements and test case. Observation 3: There is little impact on either Top K prediction accuracy or absolute RSRP accuracy training with measurement error or not. Observation 4: Comparing to the genie-aided ground-truth or ground-truth with measurement error, Top K prediction accuracy is a bit different and absolute RSRP accuracy of the 1st and 3rd predicted beams are similar. Proposal 6: Prefer that the metric to be used in RAN4 is also used for monitoring, i.e., x shall be 0. Proposal 7: From the point of necessity, the absolute RSRP accuracy requirements should be defined for none Top-1 beams too. Proposal 8: RSRP accuracy requirements of Top-1 of predicted beams can be defined by filtering out the dataset with SNR of the Top-1 beam in set A no less than -3dB. Further discuss how to deal with the issue that the distribution of SNR of the Top-1 beam in set A vary among different companies’ dataset as well as the test dataset generated by TE vendors. Observation 5: For different beams, e.g., predicted 1st strongest and 5th strongest beam, the predicted absolute RSRP accuracy varies. It may be the SINR difference that results in the difference between the predicted absolute RSRP accuracy of top-1 beam and other beams. Proposal 9: Further discuss whether it is workable to define a single absolute accuracy requirement which is applicable to all/some of the predicted beams with some side conditions. Proposal 10: For AI/ML BM, further discuss whether the performance under synthetic channels can reflect the performance of AI/ML models in the real field. Proposal 11: For AI/ML BM, discuss what the testing goal is, to verify the model can work well in real field or just to test UE can train a model that works well under a specific scenario. Proposal 12: If the testing goal is to verify the model can work well in real field, and the performance under a specific channel/scenario cannot reflect the performance in the real field, it is necessary to design a test system that is possible to use field data in the test. Proposal 13: If the testing goal is to test UE can train a model that works well under a specific scenario, prefer to choose a test system that is easy to implement and evaluate. Proposal 14: In AI/ML BM test, emulate the spatial-selective and time-varying in propagation conditions through adjusting Tx power at TE on each beam at different time. Observation 6: Single AoA test system can be applicable to all kinds of synthetic channels as well as using real field data. It can be easily extended to different scenarios and settings. Observation 7: With single AoA, TE can know the expected RSRP of Tx beams at UE baseband. Proposal 15: Use existing IFF or enhance IFF test systems with single AoA to test R19 DL Tx beam prediction models. Proposal 16: Standardized UE radiation pattern in 38.901 can be assumed in R19 AI/ML BM test with single AoA test system instead of using test UE’s radiation pattern. Proposal 17: The procedures of AI/ML BM test with single AoA: Beam lock at test UE during the test. - Step 1: Overall coupling loss calibration - TE transmit the signal with a fixed power, UE receive signal and report RSRP to TE. TE can get the overall coupling loss through the relationship between TE’s transmit power and the reported RSRP (i.e., including the antenna gain of the probe, OTA pathloss and UE’s Rx beamforming gain). TE can ask UE to report multiple times to reduce the impact of noise in the coupling loss calculation. - Step 2: Main test - TE gets the test dataset. The dataset can be early prepared through computer-like simulation using standardized UE radiation pattern in 38.901 and is the same for all the test UEs. - TE transmit the calibrated power on each Tx beam in set B from the test dataset by the overall coupling loss. UE measures set B and predicts Top K or RSRP of set A. In such a way, the uncertainty of beamforming gain can be removed when deciding pass/fail for each predicted RSRP. Observation 8: Simulation workload will increase linearly with the number of candidate methods to simply the channels. Proposal 18: Make a down-selection on the candidate methods of how to simply CDL channels. Proposal 19: Discuss how to emulate UE movement in multiple AoA test systems. Proposal 20: How to avoid UE cheating in the test, such as reports fake values according to the previous predicted results, shall be further discussed to use the reported RSRP measurement as ground truth. Proposal 21: Discuss how to get training dataset if to use multiple AoA test systems. Observation 9: The upper bound of SNR with rough beam are much lower than that with fine beam. Proposal 22: Fine beam should be used in AI/ML test. Observation 10: Among all the test methods, only using peak fine beam direction can make sure the lowest SNR above -6dB with 30dB dynamic power range. Observation 11: Multiple AoA test systems cannot use peak fine beam direction only. Proposal 23: Using peak fine beam direction in AI/ML BM test with single AoA test system. Observation 12: SNR dynamic range of multiple AoA test system would be at least 12dB narrower than single AoA test system. |
| R4-2506524 | OPPO | Proposal 1: For BM testability, RAN4 need to: - Clarify the limitations regarding the FR2 beams or probes that TE vendors could support - To determine the test setup for BM, a potential approach could be: - Assuming that TE supports X probes in a chamber - By utilizing X probes, X different angles of arrival (AOA) can be achieved and can be mapped to a TX beam transmission with X paths(clusters) - Different TX beams in BM set A or set B can be represented by setting different pathloss for the different AOA paths, to mimic the beam sweeping procedure - FFS the number of probes that could be supported by TE vendors - FFS how to map the X AoAs and corresponding X pathloss to different beams in BM set A/B Proposal 2: Further consider how to ensure that the BM model constructed on the DUT side can match(or approximate match) and be utilized in the testing environment on the TE side. Proposal 3: The TCI state associated with a predicted Tx beam should be known to DUT. |
| R4-2506602 | CMCC | Proposal 1: for the value of x and N in the definition of metrics/KPIs for beam ID prediction, with the understanding that N refer to the the N of Top K/N(%), it is proposed that - Either x =0 and N>1, or x of non-zero value and N=1 are OK; - It is not necessary that N>1 and x of non-zero value are considered together Proposal 2: for the case that both RSRP and beam ID are reported, it is proposed that both RSRP accuracy requirements and successful rate for the correct prediction are applied. Proposal 3: it is proposed that absolute RSRP accuracy requirement only applies to Top-1 of predicted beams. And relative accuracy requirements applies to other reported predicted beams. Proposal 4: for relative RSRP accuracy, it is proposed that the reported L1-RSRP cannot be measured RSRP, since RAN1 agreed that the RSRP of predicted beam(s)in the report of inference results is the predicted RSRP which is based on AI/ML output. Proposal 5: for relative RSRP accuracy, it is proposed that beam index n owns the largest reported value. Proposal 6: for BM case 1, it is proposed that - The relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - predicted L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), [where the beam index n owns the largest reported value] Proposal 7: for BM case 2, it is proposed that - The relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i for time instance m - predicted L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i for time instance m - ground truth of L1-RSRP of beam index n), where the beam index n owns the largest reported value among all the predicted beams. 1<=m<=M where M is the number of time instance Proposal 8: for BM case 2, it is proposed to take legacy L1-RSRP measurement period as baseline. Proposal 9: for beam prediction testing, it is proposed to consider CDL or simplified CDL. Proposal 10: to verify beam management performance, it is proposed to use multiple AoAs test systems. |
| R4-2506981 | Rohde & Schwarz | Observation 1: (Real Data) If the real data for training the model and testing AI/ML BM is not standardized, the focus moves from prediction performance, to training performance. Such requirements would also not refer to a deterministic testing environment, which is key assumption for conformance testing. Observation 2: (Real Data) For a typical conformance testing of prediction performance, standardized real data for training and testing is needed, which shifts the challenge to agree on the data and the respective scenario. Observation 3: (Real Data) Training and testing with real data can be replicated only in a single-AoA system (using the wireless cable method with the UE complex antenna pattern), since a multi-AoA system with fixed probe layout cannot mimic the variety of the real data and scenarios. Observation 4: (Real Data) Training and testing with real data requires a standardized data format and interface to the test system, as well as a description how this data is deployed by the test system. Proposal 1: (Real Data) Support of real data training and testing is recommended only in the form of 1AoA testing and for R&D purposes, as a complement to the conformance requirement defined with synthetic channels. Observation 5: (Single/Multi-AoA) Several aspects related to the UE Rx Antenna assumptions, the input to the AI/ML model and the real data usage for training and testing, have a major impact on the discussion about single- or multi-AoA testing. Proposal 2: (Single/Multi-AoA) RAN4 to discuss and conclude on the aspects in Table 1 as part of the single-/multi-AoA discussion. Table 1 – Single vs. multi AoA testing for AI/ML BM with CDL based channel Parameter; Single AoA; Multi AoA; Notes. CDL Channel Support; Only if UE Rx antenna performance is emulated as part of the channel (WCM*); Direct, clusters are emulated on different probes / AoAs; * Wireless Cable Method. CDL Channel Complexity; Complex Channel (with WCM); Simplified Channel. UE Rx Antenna Pattern Knowledge; Required for WCM (reference / declared / measured); Not required. UE Rx Antenna Performance; Static pattern during testing simulating the dynamic pattern as part of the WCM; Dynamic pattern. UE Rx Beam Sweeping Timing; Not considered; Considered. UE Rx Beam Index as input for AI/ML Model; Supported with test mode*; Supported; * Depends on the complexity of the UE Rx Beam/Antenna pattern (Codebook) considered for single AoA testing. Training / testing with real data; Supported with WCM; Unfeasible (fixed probe layout). TE Complexity; Low HW Complexity Longer test time WCM & Test mode required; High HW Complexity Shorter test time. Proposal 3: (Multi-AoA Candidate Setup) Define the implementation of eIFF test setup with 4x active 2x2 MIMO probes with probes (CATRs) placed in a single plane and separated by 30º, 60º and 60º respectively, as feIFF (further enhanced IFF) test setup. The concrete probe layout and the alignment with the coordinate system to be used for testing is FFS. Figure 1 – Example of a commercial implementation of eIFF system with 4 IFF probes / CATRs Observation 6: (Multi-AoA Candidate Setup) The feIFF test setup supports 4 Active 2x2 MIMO probes with large distinctive angular separation of up to 150°, overall big quiet zone of at least 30 cm, lowest MUs and ensures preservation of (RF-RRM) side conditions. Observation 7: (Multi-AoA Candidate Setup) The feIFF test setup is a feasible upgrade (superset) of the existing legacy eIFF setup. As such, if feIFF is concluded as capable for AI/ML BM testing, a single system covers the the whole conformance testing, ensuring continuity in the cost sensitive FR2 testing ecosystem. Proposal 4: (Multi-AoA Candidate Setup) Consider the feIFF test setup as candidate for the AI/ML BM baseline system and analyze its suitability. Observation 8: (Multi-AoA Simplified Channel) The required total AoA spread of 150° of the simplified channel in Table 2 can be emulated with the feIFF probe layout. Observation 9: (Multi-AoA Simplified Channel) The simplified channel model parameters for UMi CDL-C can be implemented in the candidate test system feIFF, with 3 possible options (Figure 3): Option A: with flattened elevation (Table 3) and strongest cluster aligned to one probe; Option B: with flattened elevation (Table 3) and mid-point of the total angular spread of the clusters is aligned to the mid-point of the spread of the probes; Option C: with flattened elevation, AoAs aligned to probe layout and no intra-cluster angle spread (Table 4). Proposal 5: (Multi-AoA Simplified Channel) Consider the channel models described Table 3 (Option A/B) and Table 4 (Option C) as candidates for CDL-based simplified channel models for multi-AoA testing of AI/ML BM. Table 3 – Channel model parameters for UMi CDL-C at 28 GHz with flat ZoA Cluster #; Absolute Delay [ns]; Power in [dB]; AOD in [°]; AOA in [°]; ZOD in [°]; ZOA in [°]. 1; 0; -7.4318; -30.4353; -134.4434; 98.9242; 74.51134. 2; 12.594; -1.2500; -20.9269; 129.1633; 99.1915; 74.51134. 5; 13.056; -5.5318; -28.0782; -152.8206; 99.5732; 74.51134. 6; 38.196; 0.0000; -11.6982; 164.1145; 99.306; 74.51134. 13; 73.71; -8.1318; -33.911; 93.1719; 100.165; 74.51134. 14; 78.498; -9.8318; -37.5066; -112.0441; 100.2604; 74.51134. Per-Cluster Parameters. Parameter; CASD in [°]; CASA in [°]; CZSD in [°]; CZSA in [°]; XPR in [dB]. Value; 0.799; 10.4021; 0.5726; 0; 7. Table 4 – Channel model parameters for UMi CDL-C at 28 GHz with AoA aligned to probe layout and no intra-cluster angle spread Cluster #; Absolute Delay [ns]; Power in [dB]; AOD in [°]; AOA in [°]; ZOD in [°]; ZOA in [°]. 1; 0; -7.4318; -30.4353; -114.436; 98.9242; 74.51134. 2; 12.594; -1.2500; -20.9269; 125.5639; 99.1915; 74.51134. 5; 13.056; -5.5318; -28.0782; -174.436; 99.5732; 74.51134. 6; 38.196; 0.0000; -11.6982; -174.436; 99.306; 74.51134. 13; 73.71; -8.1318; -33.911; 95.5639; 100.165; 74.51134. 14; 78.498; -9.8318; -37.5066; -114.436; 100.2604; 74.51134. Per-Cluster Parameters. Parameter; CASD in [°]; CASA in [°]; CZSD in [°]; CZSA in [°]; XPR in [dB]. Value; 0.799; 0; 0.5726; 0; 7. Figure 3 – Simplified UMi CDL-C at 28 GHz alignment to the candidate test system feIFF Observation 10: (Multi-AoA Simplified Channel) The evaluation of Options A and B (Table 3) becomes more complex, if the effect of the test setup / probe layout has to be included. Observation 11: (Multi-AoA Simplified Channel) The emulation of the relative ray phases with the feIFF is highly accurate and should satisfy the requirement for the accuracy for the cluster power fluctuations over time. Proposal 6: (Multi-AoA Simplified Channel) Further discuss the metric(s) to evaluate channel model implementation with the feIFF candidate test setup. Observation 12: (Multi-AoA Simplified Channel) For Option C (Table 4), there is no need to optimize the probe weights, and channel model parameters in Table 4 contain all the required information. This option is thus preferred from a TE point of view. Observation 14: (Multi-AoA Channel Validation) Based on preliminary simulations with 8 Set B & 32 Set A beams, there is only a negligible effect (deviation of max ~1%) on the prediction accuracy for Top 3 & Top 1 beams between the cases with full CDL channel model (Reference - 24 clusters) and simplified CDL channel model suitable for feIFF (Options A/B/C - 6 clusters). Proposal 8: (Multi-AoA Channel Validation) Consider preliminarily the CDL-C simplified channel models for feIFF with 6 Clusters in Table 3 and Table 4 as suitable for AI/ML BM testing. Proposal 9: (Multi-AoA Channel Validation) Start the validation of CDL-C simplified channel models for feIFF with 6 Clusters in Table 3 and Table 4 through AI/ML BM system level simulations. Proposal 10: (Multi-AoA Channel Validation) For the scope of validation, compare the prediction accuracy between the full channel and the simplified channel. Proposal 11: (Multi-AoA Channel Validation) For the scope of validation, compare the prediction accuracy, when the model is trained with the full channel before the test and with the simplified channel during the test. |
| R4-2507022 | NTU | Proposal 1: Remove the FFS part related to N in the previous meeting agreement: The successful rate for the correct prediction, - The correct prediction is considered as maximum ground-truth RSRP among top-K predicted beams larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB, - FFS on top-N (N</=K) predicted beams have to be considered in the success rate evaluation - FFS on N values - FFS on x values |
| R4-2507049 | Huawei, HiSilicon | Proposal 1: RAN4 to define the requirement of prediction delay in AI BM, where the measurement delay and inference delay are not treated as separate requirements. Proposal 2: For data collection in NW-sided model, take the existing core requirement for beam related information reporting as the starting point. Proposal 3: If measurement reporting is used for the calculation of performance metric at NW during model monitoring, reuse RAN4 legacy requirement for measurement reporting. Proposal 4: If metric reporting is introduced by RAN1 for model monitoring, RAN4 will study the related requirement. |
| R4-2507184 | ZTECorporation,Sanechips | Observation 1: The top-K beams are reported or predicted beams or top-K beams are groundtruth beams. Observation 2: For the top-K beams are groundtruth beams but not the predicted beams, may be some UE reported beams could not be verified or why should we require the RSRP accuracy of the beam that the UE doesn’t even report. Observation 3: What is the standard to verify the performance of AI system based on the absolute RSRP accuracy shall be considered or how to quantify the the small or large L1-RSRP difference shall be studied. Observation 4: One question is that the proposed margin shall be maintained or smaller than legacy value. There are two options to consider the change of proposed margin: - Option 1: Maintain the legacy measurement accuracy. In legacy, UE shall do the full beam sweeping in order to confirm the best Rx beam to receive the downlink RS and get the better communication with network. The AI system has already improve the overhead and reduce the complexity than legacy. Thus, maintain the legacy measurement accuracy could be understood. - Option 2: Smaller than legacy measurement accuracy. This is a more direct understanding since the AI performance shall have the better performance than legacy and the measurement accuracy shall be tightened. In this way, the performance of AI method could be displayed. Proposal 1: For the absolute RSRP accuracy, the index i shall be the top-K beams based on the predicted beams or UE reported beams instead of the groundtruth beams. Proposal 2: One margin for the absolute RSRP accuracy shall be studied in order to verify the performance of AI system. The proposed margin could be equal to and smaller than the legacy measurement accuracy requirements. Observation 5: The whole principle for defining relative RSRP accuracy is that the predicted L1-RSRP minus ground truth beams which is similar to legacy definition. Observation 6: If the reported L1-RSRP is reported measured RSRP, this components will disappear based on current formulation. Proposal 3: The reported L1-RSRP shall be reported predicted L1-RSRP not the measured RSRP. Observation 7: Top-K/1 is the relaxation of Top-1/1, UE vendors has extended margin on this performance metric and the prediction accuracy can be up to 97% even the measurement error has been added. Observation 8: If the margin is set too large, the threshold for performance validation becomes low enough that any UE can pass the test, leading to the realization that AI’s predictive capabilities are actually not as good as legacy methods. Proposal 4: At least x=0 shall be considered in performance metrics. Proposal 5: The concrete value of x when x>0 shall be defined based on simulation results. Proposal 6: The value of x shall not be set too large in order to guarantee the AI performance test and prevent UE easily pass the test. Proposal 7: The potential components in prediction delay contains three parts: - Measurement delay: The time for measurement of SetB which is the input for inference. - Inference delay: The time for inference from starting performing inference to generate the prediction results. - Reporting delay: The time between the report triggering and the point when the UE starts to transmit the outputs over the air. Proposal 8: The legacy L1-RSRP measurement period shall be as baseline or starting point. Proposal 9: Postpone discussion on reporting requirements until other WGs reach the conclusion on reporting mechanism. Observation 9: Different TCI state conditions mean the different TCI state switching delay. Observation 10: When the UE is aware of the current RX beam it is utilizing, TCI state activation delay is reduced. Otherwise, TCI state activation delay is not reduced. Observation 11: There are two situation for considering the TCI state activation: - Case 1: The best predicted Tx beam ID/RSRP belongs to Set B. - Case 2: The best predicted Tx beam ID/RSRP doesn’t not belong to Set B. Proposal 10: UE knows the Rx beam or the RS is in the Set B and the RS is also the L1-RSRP report by UE within 1280ms before TCI state activation, the TCI state is known. Otherwise, the TCI state is unknown. Proposal 11: The test procedure shall be clarified and unified firstly: Training phase; 1.TE configure resource sets for measurement of Set B and beam report of Set A. 2.UE measures Set B as the model input.. Inference phase; 3.Generates the test setup environment for Set B.. 4.UE predicts best Tx beam/ L1-RSRP in Set A.. 5.UE reports predicted best Tx beam/ L1-RSRP to TE in Set A.. Groundtruth phase (measurement of RSRP with high SNR); 6.TE configures resource sets for measurement of Set A.. 7.Generates the test setup environment for Set A (including high SNR).. 8.UE measures Set A under high SNR condition as ground truth.. 9.UE reports groundtruth (measurement on Set A) to TE.. TE evaluation phase; 10.Evaluates the predicted results and groundtruth and judge the performance degradation. Observation 13: The performance degradation will be occurred due to added measurement error. Observation 14: For Top-1, the prediction accuracy is below 86% even if there is no error on test data. Observation 15: The prediction is getting larger and larger when K is larger and larger whatever the measurement error is added or not. Proposal 12: RAN4 shall further study the measurement error impact on different performance metrics if new metrics are introduced. Proposal 13: Except measurement error impact, RF error influence shall also be considered in RAN4. |
| R4-2507430 | Nokia | Proposal 1: RAN4 to further discuss using the legacy L1-RSRP measurements with the same values of M, N, and P for measurement delay requirements for BM-Case1. Proposal 2: RAN4 to further discuss whether the measurement period for L1-RSRP measurement will need to be enhanced for BM-Case2, where the parameter M needs to be redefined. Proposal 3: RAN4 should consider the impact in RAN4 requirements for the Evaluation period TEvaluate_CBD_CSI-RS due to the presence of predicted beams. Proposal 4: For UE-side model in beam management use case, the LCM operations are based on CSI report configuration, in our view, the delay components of LCM operations should be considered for different CSI-report configurations (i) periodic CSI report (ii) semi-persistent CSI report and (iii) aperiodic CSI report for both BM-Case1 and BM-Case2. Proposal 5: RAN4 should consider Table below as a baseline for further discussion on LCM operations related RRM core requirements. Table: Delay components related LCM operations for different CSI-report configurations CSI-Report Configuration; Activation; Deactivation. Periodic CSI-reporting; Delay from RRC Reconfiguration Delay from UE processing related to capability report Delay due to CSI-RS measurement period; Delay from re-configuration (RRC Reconfiguration) Delay from UE processing related to capability report Delay due to CSI-RS measurement period. Semi-persistent CSI-reporting; Delay for activating semi-persistent CSI resource set Delay for selecting of semi-persistent CSI resource set Delay from UE processing related to capability report Delay due to CSI-RS measurement period; Delay for activating semi-persistent CSI resource set Delay from re-configuration (RRC Reconfiguration). Aperiodic CSI reporting; Delay related activating aperiodic CSI resource set, Delay from UE processing related to capability report Delay due to CSI-RS measurement period; Delay related activating aperiodic CSI resource set Delay from UE processing related to capability report Delay due to CSI-RS measurement period. Proposal 6: RAN4 should consider the impact in RAN4 requirements for performance monitoring when UE reports performance metric to NW while considering valid number of performance monitoring instances. Proposal 7: According to RAN2, when an active beam prediction configuration becomes inapplicable, RAN4 should further study (i) delay conditions related inapplicability reporting and (ii) beam failure evaluation period due to inapplicable functionality. Proposal 8: RAN4 to further discuss the delay requirement for AI/ML BM functionality recovery when AI/ML BM functionality failure occurs, and UE is enabled for recovery mode. Proposal 9: RAN4 to further discuss the delay requirement on AI/ML BM functionality when performance evaluation/monitoring is performed before or during activation. |
| R4-2507479 | Samsung | RAN4 Core Requirements for Supporting UE-sided Model Data collection for UE-sided model (for model training) Proposal 1: RAN4 shall confirm that no RAN4 impact is expected from data collection for UE-sided model training. RRM core requirement for model Inference Proposal 2: For absolute RSRP accuracy requirement, the requirement shall be defined as: - For core requirement and conformance testing, the absolute RSRP accuracy requirement shall only be applied to the strongest beam in the configured top-K beam RSRP report. - The existing measurement accuracy requirement is used as the baseline for prediction accuracy. Proposal 3: For relative RSRP accuracy requirement, if defined, the requirement shall be defined as: - Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - reported L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), where the beam index n owns the largest reported value given by absolute RSRP reporting. - Reported L1-RSRP can be predicted, - Reported L1-RSRP shall be predicted RSRP - Relative RSRP accuracy requirements apply for Case 1, and also for Case 2 Proposal 4: To guarantee the beam ID predication accuracy is not only applied to top-1 beam, top-N (where at least N =2) beam ID predication accuracy can be considered. Proposal 5: To make this beam ID prediction accuracy requirement meaningful to NW, this requirement should only be defined based on the assumption that TE/NW can identify which beams are the top-N predicted beams among top-K predicted beams. Proposal 6: For AI-BM Case-1, the legacy L1 measurement period (the same M, N, P) is used as the starting point for Set-B measurement to evaluate the achievable prediction accuracy. RRM core requirement for performance monitoring Proposal 7: At least for performance monitoring Type 1, Option 2 (UE-assisted performance monitoring), RAN4 prediction accuracy requirement shall be applicable. RRM core requirement for LCM Observation 1: For UE-sided model configuration, after CSI-ReportConfig for inference configuration in RRCReconfiguration signalling in Step 5: • aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after RRCReconfigurationComplete. • periodic CSI Report is considered as activated after RRCReconfigurationComplete. Observation 2: For UE-sided model activation/deactivation, L1 and MAC signalling can be used for aperiodic CSI Report and semi-persistent CSI report. Proposal 8: For UE-sided model “Network decision, network-initiated” LCM, RAN4 shall study the necessity of |
| R4-2505514 | Qualcomm Incorporated | Observation 1: Baseband error will reduce at high SNR. Observation 2: RF error may not decrease at high SNR because UE’s Rx AGC settings might be calibrated for a particular SNR value. Observation 3: Ground truth will be based on UE’s report and UE’s reported RSRP will include RF error that may not decrease with SNR. Observation 4: Baseband error at different SNRs can be modelled with Gaussian distribution. For example, at -3 dB SNR, the distribution of absolute baseband measurement error can be fitted with following Gaussian distributions: - AWGN: Gaussian with zero mean and sigma = 0.6 dB - TDL-C: Gaussian with mean = -0.12 dB and sigma = 0.85 dB Observation 5: Results of AI-ML based “spatial only” predictions are displayed in Table 3. The definitions of different metrics used to generate these results can be found in Table 2. Table 2: Definitions of Metrics for AI-ML based Beam Prediction Metric; Definition. L1-RSRP absolute accuracy; 90%-tile L1-RSRP difference between the predicted L1-RSRP of the Top-1 predicted beam and the ground truth L1-RSRP of the same beam.. Worst case L1-RSRP absolute accuracy among top-K predicted beams; 90%-ile of the worst case L1-RSRP difference between the predicted L1-RSRP of the top-K predicted beams and the ground truth L1-RSRP of the same beams. This gets calculated in following steps: - In one sample of inference reporting, assume I = arg max_{k \in K} (predicted RSRP of beam k – ground truth of beam k) - Store (predicted RSRP of beam I – ground truth of beam I) for this sample of inference - Report 90%-ile distribution of above metric. Relative L1-RSRP of beam owning the 2nd largest predicted reported value; 90%-ile of following metric: (predicted L1-RSRP of beam index i - predicted L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), [where the beam index n owns the largest reported value].. Top K/1 without margin; The percentage of the time when the Top-1 strongest beam (ground truth based) is one of the Top-K predicted beams. Top 1/K without margin; The percentage of the time when the Top-1 predicted beam is one of the Top-K strongest beams (ground truth based). Top K/N (m) without margin; The percentage of the time when the Top-K predicted beams contain m of the top-N strongest beams (ground truth based). Top 1/1 with margin of X dB; The successful rate for the correct prediction which is considered as maximum ideal RSRP among top-1 predicted beams is larger than the ideal RSRP of the strongest genie-aided beam – x dB. Table 3: Results of AI-ML based “spatial only” beam prediction Metrics; Training and testing without measurement error; Training and testing dataset, along with ground truth for training and testing, with measurement error. L1-RSRP absolute accuracy (dB); +- 4.2; +- 5.7. Worst case L1-RSRP absolute accuracy among top-K predicted beams (dB); K = 2; +- 5.52; +- 8. K = 3; +- 6.52; +- 9.2. L1-RSRP relative accuracy of beam containing the 2nd largest predicted value; +- 4.8; +- 7. Top K/N (m = 2) without margin (%); 2/2; 72.1731.; 62. 2/4; 92.4743.; 88.54. 3/2; 87.2771.; 80.97. 3/4; 96.7201.; 95.2. 4/4; 98.2979.; 97.37. 5/4; 98.9768; 98.4. Top K/1 without margin (%); K = 1; 79.4358.; 71.5. K = 3; 95.1805.; 93.2. K = 5; 97.4372; 96.5. Top 1/1 with margin of X dB (%); X = 1; 85.58; 77.83. X = 3; 92.39; 86.6. X = 5; 95.43; 91.6. Proposal 1: - RAN4 investigates performance based on case 3 of last meeting where measurement error will be considered in training dataset, model input during inference and ground-truth during training and testing. |
| R4-2505765 | Xiaomi | Observation 1: The similar metric Top-K/M is agreed in RAN1 performance monitoring, where at least one of the Top M beam(s) of the resource set(s) for monitoring is among Top-K predicted beam(s). Proposal 1: In order to align with RAN1, Beam index prediction definition should include both original option 2 and option 3: - For X value: keep both X = 0 and X=1dB. - For N value: - When X = 1 dB, N = 1. - When X = 0dB, N = 1 or 2 depending on K value. Observation 2: In legacy, absolute RSRP is only defined for the largest RSRP and relative accuracy is defined for other differential RSRP in the report. Proposal 2: Only define Absolute RSRP accuracy requirement for Top-1 of predicted beam. Proposal 3: For relative predicted RSRP accuracy, define unified definition for both BM-case 1 and BM-case 2: Relative RSRP Accuracyi,t=Predicted RSRPi,t-Predicted RSRPmax-True RSRPi,t-True RSRPmax Where Predicted RSRPmax is the largest RSRP across all time instance and beams in the report。Predicted RSRPi,t is RSRP of beam index i at time t。True RSRPmax is the true RSRP corresponding to the beam/time of Predicted RSRPmax。Where Predicted RSRPi,t-Predicted RSRPmax is predicted differential RSRP in the report. Observation 3: Significant Performance Degradation in Low SNR Groups. The performance degradation is more significant in UE groups with lower maximum SNR. Observation 4: Higher Stability in High SNR Groups. UE groups with higher maximum SNR (e.g., UE group 1 and UE group 2) show better performance stability under different measurement errors. Observation 5: The SINR distribution for all UE groups spans a large range, and this distribution can significantly impact the final accuracy performance. Proposal 4: To ensure consistent and reliable simulation results, we recommend that RAN4 align the following factors: - 1. Method of Adding Measurement Error for Different TX Beams： - Generate error distributions separately for each individual Tx-Rx beam pair based on different SNRs and apply different Gaussian standard deviations to the corresponding Tx-Rx beam pair measurements in the SLS. - 2. Gaussian Standard Deviation for Different SNRs - Establish reference standard deviation values for critical SNR operating points. - 3. Largest SNR Distribution Among All Selected UEs - Divide the largest SNR range into several groups and define reference percentages for each group. - 4. SNR Distribution for TX Beams in Set B - Align the SNR distribution for all TX beams in set B to ensure consistency across different companies. |
| R4-2505941 | Apple | Observation 1 : From the network operator's perspective, a number of optimal beams need to be known in order to optimize load balancing and make trade-offs in performance and complexity Observation 2 : Ensuring that RSRP accuracy implies beam prediction accuracy is challenging and depends on absolute RSRP accuracy and the RSRP difference between the best beams Observation 3: When using neural networks for regression, we rely on the Normalized Mean Squared Error (NMSE) criterion for training. It's crucial to ensure that the accuracy of weaker beams is not compromised by the accuracy of the strongest beams Observation 4: The measurement period for Set-B would depend on the accuracy needed for prediction. Observation 5: There is a probability that the Top-K predicted beams may not align with the Top-K beams determined by ground-truth RSRP measurements Observation 6 : For RSRP absolute accuracy, if the index i can represent any beam within the Top-K beams based on ground-truth L1-RSRP, the RSRP accuracy for beams that are part of the Top-K predicted beams but not included in the Top-K beams based on ground-truth RSRP cannot be guaranteed. Observation 7 : The legacy definition for known TCI state cannot be used when target TCI state is based on predicted beam and not transmitted to and measured & reported by UE. Observation 8 : RAN4 needs to discuss known TCI for UE side and NW side prediction. Observation 9 : RAN4's goal of aligning test conditions with real-world deployment scenarios is closely related to ensuring generalization performance. Since field deployment conditions can vary significantly across different environments, the test conditions should account for this variability Observation 10: Assuming the ML model is Lipschitz continuous with constant L, then the total error between the predicted X and the genie X is bounded by: Xgenie-Xpred≤ϵpred+L⋅ϵinput. This represents the total inference-time error between Xpred and Xgenie accounting for both: the model's prediction error when trained on clean inputs ϵpred and the additional error due to noisy inputs at inference L ϵinput For Case 2b when we compare the difference between the predicted values and the genie values containing measurements errors, we provide the following analysis: We want to bound the difference Xpred-Xmeasured. We can write: Xpred-Xmeasured≤Xgenie-Xmeasured+Xpred-Xgenie. That is, Xpred-Xmeasured≤ϵpred+L⋅ϵinput+∆. Where Δ=Xgenie-Xmeasured is the full measurement error across all dimensions of X, and ϵinput=Xgeniesubset-Xmeasuredsubset measurement error in just the subset of X used as input to the model Observation 11: Assuming the ML model is Lipschitz continuous with constant L, then the total error between the predicted X and the measured X is bounded by: Xpred-Xmeasured≤ϵpred+L⋅ϵinput+∆. Where Δ=Xgenie-Xmeasured is the full measurement error across all dimensions of X, and ϵinput=Xtruesubset-Xmeasuredsubset measurement error in just the subset of X used as input to the model Observation 12: To avoid performance degradation, the absolute RSRP accuracy of AI/ML-based beam management (BM) predictions should meet the existing legacy requirements Observation 13 : To guarantee that the UE operates within acceptable margins, it's essential to subject it to various radio conditions and additional conditions for testing and generalization validation in RAN4 Observation 14 : For BM use case the identified scenarios and configurations can be initially understood as those reported by UE through capability signaling as part of functionality identification. Observation 15 : The additional conditions for the AI/ML model training (which do not constitute part of UE capability) for the AI/ML-enabled feature/FG can serve as the different scenarios/configuration for defining generalization Observation 16: Achieving consistency between training and inference by model monitoring could result in delays and increased complexity in model management for BM use case Observation 17: If multiple models with varying generalization capabilities and requirements for network-side additional conditions are trained by different UE vendors, it would necessitate substantial standardization efforts for BM use case Observation 18: Current proposals on assistance information for additional conditions and Model Identification only serve the purpose of selecting the appropriate AI/ML model. However, this approach may not be scalable due to considerations of UE implementation complexities and granualtity of conditions/additional conditions. Complexity can increase substantially, especially if condition granularity is fine. Proposal 1: RAN4 should investigate the specification of reference AI/ML models for BM, the associated training procedures, and the training datasets to ensure alignment of simulation results. This will help pave the way for defining performance requirements for the BM use case, with careful consideration of constraints on model complexity. To initiate the discussion, we have provided a reference table as a starting point. Proposal 2: We propose that RSRP accuracy be defined as the difference between the predicted RSRP and the measured (genie) RSRP associated with the same Tx beam. Accuracy requirements should apply to all predicted beams that satisfy the predefined side conditions, including SNR. Proposal 3: Use legacy measurement period as a starting point for Set-B for evaluation if sufficient to achieve good prediction accuracy. Proposal 4: The absolute RSRP accuracy for AI/ML-based beam prediction is defined as the difference between the predicted L1-RSRP of beam index i and the ground-truth L1-RSRP of beam index i. Here, the index i can represent any beam within the Top-K beams based on predicted L1-RSRP. Proposal 5: Employ Modified Option 3 for beam prediction accuracy for top-N beams predicted beams (prediction order is not penalized) Proposal 6: Employ the composite beam ranking score (CBRS) for beam prediction accuracy for top-N beams predicted beams (prediction order is penalized) Proposal 7: For UE side model known TCI state is determined based on Set B transmission and UE report of top-K. Proposal 8: For NW side model - (1) TCI state is unknown if RS of target beam is not transmitted to UE prior to TCI state switch (2) If RS of target beam is transmitted to the UE and reported, legacy definition of known TCI state is applicable Proposal 9: NW side data collection, QCL type D is signaled between set A and set B RS. Proposal 10: For NW side model, TCI state is known if QCL source is signaled in both data collection and inference (to ensure UE Rx consistency), it is known. Otherwise, it is unknown. Proposal 11: (for Alt 1): Define UE capability for Alt 1 of UE implementation. When UE does not support this capability, the TCI state is unknown. Unknown TCI state switching delay is applied. Proposal 12: (for Alt 2): When gNB signal the QCL relationship between set A and set B, in training RS configuration, inference RS configuration and performance monitoring RS configuration (example of Alt 2 NW beam implementation of set A and set B), The TCI state is known when the following is met: Target TCI state is based on RS from set A beams with QCL relationship configured to a set B beam The time between TCI state switch command and transmission of the QCLed RS from set B shall not exceed X ms X = 1280 ms (legacy) UE shall send measurement report for the QCLed RS from Set B between transmission and TCI state switch TCI state and SSB of TCI state remain detectable during TCI state switching period Otherwise, it is unknown. Proposal 13: RAN4 should agree that the training dataset used in testing is strictly for testing purposes. In real-world applications, it will be the responsibility of UE implementations to determine the appropriate training dataset. This could involve using a vendor-specific dataset or creating a mixed dataset sourced from multiple vendors for training their models. Proposal 14: To define the testing data for BM and provide sufficient test environments/conditions for vendors to generate the necessary training data, RAN4 should select a set of standardized test conditions. UEs will be tested across these identified conditions, and it will be up to the UE implementation to either switch between models or employ a supermodel capable of accommodating all these conditions during. Example of test conditions/configurations are:  Details of the channel fading characteristics (CDL, LOS/NLOS, etc)  Different paterns of set B/ set A  Tx codebooks,Tx antenna architecture layout, antenna spacing  SNR conditions  Carrier Frequency  Doppler conditions, measurement window configurations and window of predicted time instances (BM case 2) Proposal 15: For all potential non-tested conditions that are part of possible deployment scenarios, including additional conditions not explicitly addressed during RAN4 testing, we propose to employ Post-Deployment Procedures. These procedures will augment RAN4 conformance testing to effectively manage and assess performance across all possible deployment conditions Proposal 16: RAN4 should consider adopting a realistic approach to model RF errors, as the prediction accuracy difference of the AI/ML model varies significantly between the worst-case scenario (independent RF errors) and the best-case scenario (same RF error for all Tx beams). To study the impact of measurement errors, the method of RF error modeling should be included in RAN4 discussions. Potential approaches include: Using a truncated uniform distribution; Using a truncated Gaussian distribution. Proposal 17: To ensure that AI/ML-predicted RSRP values meet legacy performance expectations, one of the following should be pursued: - Improve the accuracy of the measured RSRP values used as input to the AI/ML model, so that the resulting predictions satisfy the absolute RSRP accuracy requirement. - Instead of relying on AI/ML to directly predict RSRP values, the AI/ML model is used solely as a top-beam classifier that predicts the optimal beam indices. The network then transmits the corresponding beams, and the UE performs actual RSRP measurements on these beams. In this approach, only the beam index classification accuracy needs to be defined as a performance requirement - Alternatively, tighten the acceptable prediction accuracy requirement for AI/ML-based RSRP prediction so that it aligns more closely with the legacy RSRP margin. - Noise aware training: training the model with noisy inputs and genie outputs (or close to genie, high SNR). Clean-input-trained models suffer from distribution shift at inference since the model never saw noisy data and may amplify input errors. Noisy-input-trained models internalize the mapping from corrupted inputs to correct outputs. Proposal 18: To avoid the extra complexity of managing associated IDs across infra-vendors, develop a framework where the NW can equalize the differences between its own proprietary codebook and a RAN4 specified codebook for training the model. The network could employ a proprietary RSRP Equalizer to align UE-based predictions with those that account for the network's specific codebook (CB) implementations. Proposal 19: In the RAN4 core requirement, it is mandated that the consistency or association between Set B beams and Set A beams during both training and inference must be guaranteed. This serves as additional assistance information from the network side for testing UE-sided models during inference and monitoring Proposal 20: RAN4 should define identified scenarios/configurations associated with the UE capability report of an AI/ML-enabled Feature FG. For defining generalization tests, the additional conditions can serve as the other identified scenarios/configurations for the BM use case Proposal 21: RAN4 should investigate the feasibility of providing assistance information for the additional conditions to aid generalization and consistency across training and testing when defining requirements. Other additional conditions that are not part of UE capability can be used to define generalization tests Proposal 22: For additional conditions that cannot be shared due to proprietary concerns, RAN4 can explore the feasibility of using a virtual ID to indicate the specific conditions under which a model was trained. This approach would assist in the proper selection of UE models to support generalization. Additionally, RAN4 should identify which additional conditions should be exclusively reserved for generalization tests. Proposal 23: RAN4 should analyze how changes in test conditions, such as propagation conditions, affect the performance of AI/ML-enabled features. This analysis will help define the appropriate number of requirements and tests needed for each AI/ML-enabled feature and its corresponding ID combination. Proposal 24: RAN4 to consider option 1 as a baseline for achieving consistency between training and inference for both set A and set B for beam spatial and temporal prediction Proposal 25: Consistency should be considered in RAN4 only for static conditions and not-static conditions should test the UE ability to adapt and employ diverse AI/ML models to match the conditions (generalization) Proposal 26: In order to ease the burden for testing models with different NW additional conditions, it would beneficial to train the UE-side model with mixed dataset from various gNB settings, thus reducing the number of AI/ML models (selected by NW-side additional conditions) required to guarantee generalization and maintain the system performance for BM use case Proposal 27: For achieving consistency between training and inference, if the model drifts due to misalignment of network-side additional conditions, the alignment could be achieved through an ID assigned by the network during training data collection. This ID indicates the association of the training data with the additional conditions implied to generate those data Proposal 28: For achieving consistency between training and inference, if the UE lacks the ID but the network possesses the model ID, then the network can transfer the model to the UE, and the UE can update its list of models Proposal 29: For achieving consistency between training and inference, if the UE has a model that partially supports the network-sided additional conditions, then the network can trigger data collection for fine-tuning the UE-sided model. Subsequently UE updates its list of model IDs. Proposal 30: The UE updates its model list with a new model derived from either fine-tuning, model transfer from the NW, or monitoring to ensure the consistency of additional conditions. Then, the UE assigns an ID to the new model that supports the NW's additional conditions and shares this information with the NW. If some of the new conditions/configurations are standardized, the UE updates its capability report accordingly. Proposal 31: If there is no ID available to associate training data with additional conditions, and monitoring procedure fails to guarantee the consistency of the model with the additional conditions then UE should fallback to legacy mode. Proposal 32: Investigate the feasibility of enhancing the generalizability of the AI/ML model and reducing the number of AI/ML models and the testing burden for the beam management case by supplementing the core AI/ML input signals with both network (NW) and UE auxiliary information signals integral to its inference engine Proposal 33: Investigate the feasibility of training the models with a mixed dataset associated with both network (NW) and UE auxiliary information signals to further enhance the generalizability of the AI/ML model for the beam management case and reduce the number of generalization tests. |
| R4-2505982 | CATT | Observation 1: Different performance requirements need to be separately defined for beam ID prediction and RSRP prediction. Proposal 1: For AI/ML based beam management with both predicted RSRP and beam ID reported, two options can be considered for defining performance requirements: - Option 1: The legacy L1-RSRP performance requirements are reused, regardless of overhead reduction ratio. - Option 2: The legacy L1-RSRP performance requirements are relaxed with margins, and larger margin values are used for higher overhead reduction ratios. Proposal 2: For AI/ML based beam management with predicted beam ID reported only, RAN4 can define a lower bound for the successful rate as performance requirement regardless of the values of K, N, x and SINR. |
| R4-2506469 | vivo | Proposal 1: For RRM performance requirements for beam management, RAN4 to consider the following requirements； - Absolute Predicted RSRP accuracy - Relative Predicted RSRP accuracy - Monitoring accuracy for performance accuracy (to verify whether the UE correctly reports the number of accurate predictions out of N inference attempts.) Proposal 2: For test cases for AI/ML based beam management, RAN4 to define RRM test cases for the following requirements: - Prediction delay requirement - Performance monitoring reporting delay requirements - Functionality activation delay requirement |
| R4-2506526 | OPPO | Proposal 1: The definition of top-N needs further clarification to determine whether additional top-N evaluation is required. Proposal 2: If both RSRP and beam ID are reported, without including RSRP values for all beams in BM-set A, both Beam ID prediction accuracy and RSRP accuracy of the reported beams should be considered in RAN4 tests. Proposal 3: If both RSRP and beam ID are reported, with RSRP values for all beams in BM-set A, RAN4 could only evaluate the RSRP accuracy of all beams. Proposal 4: Relative RSRP accuracy requirement could be applied to other predicted beams with differential RSRP reporting in RAN4 tests. Observation 1: Regarding the impact of measurement error on BM performance evaluation, the following aspects should be further considered - Whether the impact of measurement error obtained from the SLS UMA channel (measuring error added from a CDL channel evaluation) can be directly applied to the OTA testing environment - Differences in implementation (e.g., BS antenna configurations, BS antenna element radiation pattern, Beamforming characteristic of the BS pattern) among companies may result in inconsistent evaluation results |
| R4-2506603 | CMCC | Proposal 1: For RSRP accuracy for AI/ML based beam management, it is proposed that AI/ML based performance requirement should not be worse than the legacy measurement accuracy requirements. Proposal 2: for the report mapping of inferenced L1-RSRP, it is proposed that legacy L1 report mapping, i.e. L1 part of Table 10.1.6.1-1 and Table 10.1.6.1-2 in TS38.133 are reused. - The absolute L1-RSRP is is quantized to a 7-bit value in the range [-140, -44] dBm with 1dB step size The differential L1-RSRP is quantized to a 4-bit value with 2 dB step size |
| R4-2507051 | Huawei, HiSilicon | Proposal 1: The simulation results for different prediction accuracy regarding to KPI 1 is shown in Table 1. Table 1. Measurement error impact on prediction accuracy regarding to KPI 1 KPI-1; Model input w/o measurement error; Model input w measurement error. maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB; x = 1dB; 83.02%; 60.75%. x = 2dB; 92.49%; 72.87%. x = 3dB; 96.29%; 81.84%. Observation 1: The more stringent the KPIs are, the greater the impact of the measurement error on the prediction accuracy will be. Proposal 2: The simulation results for different prediction accuracy regarding to KPI 2 is shown in Table 2. Table 2. Measurement error impact on prediction accuracy regarding to KPI 2 KPI-2; Model input w/o measurement error (case 1); Model input w measurement error (case 2b). 90%-tile L1-RSRP difference between the maximum RSRP of the Top-1/ Top-3/ Top-5 predicted beam(s) and the RSRP of the genie aided strongest beam; Avg.; CDF_strong_1; 2.547dB; 5.871dB. CDF_strong_3; 2.345dB; 5.392dB. CDF_strong_5; 2.209dB; 4.893dB. w/o avg.; CDF_strong_Top1; 2.547dB; 5.871dB. CDF_strong_Top2; 2.380dB; 5.492dB. CDF_strong_Top3; 2.139dB; 4.781dB. CDF_strong_Top4; 2.049dB; 4.445dB. CDF_strong_Top5; 1.967dB; 4.044dB. 90%-tile L1-RSRP difference between the predicted L1-RSRP of the Top-1/ Top-3/ Top-5 predicted beam(s) and the ideal L1-RSRP of the same beams; Avg.; CDF_same_Top1; 2.151dB; 7.250dB. CDF_same_Top1&2&3; 2.236dB; 7.406dB. CDF_same_Top1&2&3&4&5; 2.419dB; 7.444dB. w/o avg.; CDF_same_Top1; 2.151dB; 7.250dB. CDF_same_Top2; 2.200dB; 7.314dB. CDF_same_Top3; 2.435dB; 7.702dB. CDF_same_Top4; 2.674dB; 7.730dB. CDF_same_Top5; 2.689dB; 7.287dB. Observation 2: Regarding RSRP prediction accuracy, measurement errors have a larger impact on the Top 1 predicted beam than on the Top 5th predicted beam. Proposal 3: RAN4 define requirements with measurement error as model input. Proposal 4: RAN4 will not select KPI under which acceptable performance cannot be achieved, including - the KPI under which the prediction accuracy is less than 80%, e.g, Top K/1 with K<4 without RSRP margin. - the KPI under which the 90%-tile L1-RSRP difference is larger than 5dB, e.g, maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB with x<3dB. |
| R4-2507431 | Nokia | The following Proposals were made for AI/ML BM use case: Proposal 1: The success rate of correct prediction can be computed as a weighted average (S_w) of the strongest N beams among the top-K predicted beams, where 2≤N≤K Weighted success rate, Sw=1Nt n=1Nti=1Nwi,n lRSRPbi,n≥RSRPbi,n-X Proposal 2: RAN4 to discuss adaptation of Option 3 with the following changes: - At least N strongest beams among the top-K predicted beams must be considered in the success rate evaluation, where N≥2 - RSRP of each of the strongest N predicted beams must be compared with the RSRP of its equal ranked genie-aided beam - X Proposal 3: Value(s) of X to be determined from simulations of the ongoing study Proposal 4: RAN4 to discuss and explore the feasibility of generating Set A and Set B beams, during the conformance testing of the AI/ML-BM use case, by selecting the number of probes and controlling the transmit power of each probe. How to decide or select the required Tx power of the selected probes is FFS. Proposal 5: RAN4 to discuss and decide the contents of pre-alignment check which needs to be performed between the TE and DUT before the exact start of the conformance test. Proposal 6: RAN4 to discuss and explore the feasibility of performing beam sweeping using UE rotation in multiple iterations during the conformance testing of AI/ML based BM use case. Proposal 7: For the verification/testing of generalization related aspects in RAN4 for AI/ML BM, RAN4 should define the generalization test which consists of multiple subtests each associated with the testing of different static scenarios based on parameters listed in Table below. Table: Parameters for Generalization Scenarios Parameters; Description. UE Speed; Slow / Medium / Fast. SINR (Deployment Scenario); Good / Bad Radio conditions. Propagation Model; TDL/CDL. Proposal 8: RAN4 to consider designing a new metric to verify the generalization performance of the AI/ML BM functionality in different (static) scenarios. |

## Open issues summary

Previous agreements on this topics are listed below for reference (R4-2505105):

Issue 2-1: Metrics/KPIs for beam ID prediction

Agreement:

- Metrics/KPIs for beam ID prediction, at least for the case if only beam ID is reported:

The successful rate for the correct prediction,

  - The correct prediction is considered as maximum ground-truth RSRP among top-K predicted beams larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB,

  - FFS on top-N (N</=K) predicted beams have to be considered in the success rate evaluation

    - FFS on N values

  - FFS on x values

Note: if x=0 and N=1, it means Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams"

Issue 2-2: Absolute RSRP accuracy

Agreement:

- Absolute RSRP accuracy requirement applies to Top-1 of predicted beams at least

  - FFS whether to apply to other beams

Issue 2-3: Relative RSRP accuracy

Agreement:

- RAN4 to use the follow as baseline for further discussions, if the relative RSRP accuracy requirement is defined

  - Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - reported L1-RSRP of beam index n) -  (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), [where the beam index n owns the largest reported value].

    - Reported L1-RSRP can be predicted,

    - FFS whether reported L1-RSRP can be measured RSRP

  - Relative RSRP accuracy requirements apply for Case 1, and FFS on whether it can apply for Case 2

Issue 2-4: Measurement period for inference

Agreement:

- For measurement delay component of the prediction delay:

  - Use “legacy” L1-RSRP measurement as the baseline for Case 1

    - FFS whether same values for M,N,P should be reused

  - FFS for case 2

Issue 2-6: Test system setup

Companies are invited to bring further analysis on the test system setup.

At least the following aspects to be considered in the analysis:

    - Whether the test can/should reflect performance in the field and whether field data should be usable in the test

    - What channel model should be used

    - UE prediction model input (e.g. whether any spatial information like UE Rx beam, UE beam pattern, etc is used)

    - Pros and cons of proposed single AoA and multi AoA setups

For the multi AoA test setups, companies are invited to bring further analysis on the following aspects:

  - Define candidate test setups which are already feasible (e.g. based on eIFF, MPAC etc.)

  - Propose simplified channels suitable for the defined candidate test setup(s)

  - Validate the proposed simplified channels through AIML BM system level simulations, especially by comparing the performance with the non-simplified (theoretical) channels.

  - Assess the implementation of the simplified channels on the candidate systems, after the probe layout and the channel model have been clarified.

The open issues were grouped in the following sub-topics for further discussion:

- Beam ID prediction KPI parameters

- Measurement period for inference

- Rx beam knowledge and TCI state handling

- Relative RSRP accuracy definition

- Simulation results and next steps

- Impact to other requirements

- LCM

- Test system basic setup

- Test system channel model

- Absolute RSRP accuracy applicability

- Draft CR for 38.133

### Sub-topic 2-1

Beam ID prediction KPIs

The KPI was agreed in the last meeting, however, some of the parameters N, x are still FFS.

- Metrics/KPIs for beam ID prediction, at least for the case if only beam ID is reported:

The successful rate for the correct prediction,

  - The correct prediction is considered as maximum ground-truth RSRP among top-K predicted beams larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB,

  - FFS on top-N (N</=K) predicted beams have to be considered in the success rate evaluation

    - FFS on N values

  - FFS on x values

Note: if x=0 and N=1, it means Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams"

Issue 2-1: Metrics/KPIs for beam ID prediction

Proposals

  - Option 1: adopt N

    - N=2

    - Further study the actual value

    - N>2

    - N= K2+1

  - Option 2: Do not use N, remove the FFS on top-N

  - Option 3: x<3dB

  - Option 4: further study x based on simulations

  - Option 5: Others (e.g. different distributions, etc)

- Recommended WF

  - Option 1/2 and 3/4 to be discussed separately

### Sub-topic 2-2

Measurement period for inference

Some agreements were made in the previous meeting on the measurement period, however, some details are still to be agreed.

Agreement:

- For measurement delay component of the prediction delay:

  - Use “legacy” L1-RSRP measurement as the baseline for Case 1

    - FFS whether same values for M,N,P should be reused

  - FFS for case 2

Issue 2-2: Measurement period for inference

- Proposals

  - Option 1: use the same values for M,N,P for Case 1

    - Option 1a:

- For M and N, reuse the same values defined in existing L1-RSRP measurement period requirements

- For P, RAN4 to discuss which following alternatives to adopted:

  - Alt 1: The definition of P will be based on the R15 spec version and the enhanced features introduced in previous releases are not supported by default

  - Alt 2: The definition of P needs to take these enhancements introduced after R15 into account, such as MUSIM gap, NCSG, etc

  - Option 2: use the legacy L1-RSRP also for case 2

    - Further discuss what value to use for M

  - Option 3: Others

- Recommended WF

  - Option 1 and Option 2

### Sub-topic 2-3

Rx beam knowledge and TCI state handling

The issue of UE Rx beam knowledge for a predicted Tx beam and how this relates to whether a TCI state is known or unknown has been constantly brought up but hasn’t been discussed in the previous meetings. This influence the handling of the TCI state switching and would be part of the core requirements.

Issue 2-3: Rx beam knowledge and TCI handling

- Proposals

  - Option 1: UE always knows the Rx beam to receive a predicted Tx beam. TCI state known/unknown should follow the legacy requirements (predicted beam should be handled as a measured/reported beam)

  - Option 2: UE knows the Rx beam for the case when predicted RSRP is reported. TCI state known/unknown should follow the legacy requirements (predicted beam should be handled as a measured/reported beam)

  - Option 3: UE Rx beam knowledge is optional, introduce a capability. TCI state known/unknown is based on this capability

  - Option 4: Split into different cases:

- Set A and Set B are different (Set B is NOT a subset of Set A). E.g., use known SSB beams to predict CSI-RS beams:

  - if the predicted Tx beam which is not measured is QCL Type-D to a known Tx beam in Set B, the corresponding Rx beam is known.

- Set B is a subset of Set A.

  - Use part of SSB beams to predict all the SSB beams: Whether Rx beam of the predicted Tx beam which is not measured can be known can be up to UE capability.

  - Use part of CSI-RS beams with repetition off to predict more CSI-RS beams with repetition off: If Rx beam of the QCL-ed SSB is known, then Rx beam of the predicted CSI-RS beam is also known.

  - Option 5: The TCI state is known when the following is met:

    - Target TCI state is based on RS from set A beams with QCL relationship configured to a set B beam

    - The time between TCI state switch command and transmission of the QCLed RS from set B shall not exceed X ms

      - X = 1280 ms (legacy)

    - UE shall send measurement report for the QCLed RS from Set B between transmission and TCI state switch

    - TCI state and SSB of TCI state remain detectable during TCI state switching period

    - Otherwise, it is unknown.

  - Option 6: The TCI state is known when the following is met:

    - The TCI state switch command is received within a delay X*1280 ms upon the last transmission of the RS resource for beam reporting or measurement or prediction.

    - FFS: Further analysis on defining X values

  - Option 7: other

- Recommended WF

  - To be discussed

### Sub-topic 2-4

Relative RSRP accuracy

A tentative definition for the relative RSRP accuracy was agreed in the last meeting, however, some details are still to be discussed/agreed

Agreement:

- RAN4 to use the follow as baseline for further discussions, if the relative RSRP accuracy requirement is defined

  - Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - reported L1-RSRP of beam index n) -  (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), [where the beam index n owns the largest reported value].

    - Reported L1-RSRP can be predicted,

    - FFS whether reported L1-RSRP can be measured RSRP

  - Relative RSRP accuracy requirements apply for Case 1, and FFS on whether it can apply for Case 2

Issue 2-4: Relative RSRP accuracy

- Proposals

  - Option 1: Relative RSRP accuracy requirement could be applied to other predicted beams with differential RSRP reporting in RAN4 tests.

    - Should apply to both Case 1 and Case 2

  - Option 2: Reported L1-RSRP is the predicted L1-RSRP

  - Option 3: clarify that the prediction is for time instance m for Case 2

    - The relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i for time instance m - predicted L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i for time instance m - ground truth of L1-RSRP of beam index n), where the beam index n owns the largest reported value among all the predicted beams. 1<=m<=M where M is the number of time instance

  - Option 4: For BM case 2, Relative accuracy can happen in two dimensions:

    - Intra-time beam comparison (same time, different beams)

    - Inter-time beam comparison (same beam, different times)

- Recommended WF

  - Option 1 and Option 2

### Sub-topic 2-5

Simulation results

Issue 2-5:	Simulation results

- Proposals

  - Option 1: Discuss the simulation results based on summary – to be prepared by vivo

    - Simulation results, any insights on how to define the requirements?

    - Refinement of simulation assumptions (e.g. error modelling, etc)

    - Next steps for simulations

- Recommended WF

  - To be discussed

### Sub-topic 2-6

Impact to other requirements

Use of beam prediction could impact other RRM requirements which rely on measurement reports. Possible impact to CBD was brought up in one of the contributions. RAN4 could discuss how to handle such cases

Issue 2-6:	Simulation results

- Proposals

  - Option 1: RAN4 to further analyze which requirements could be impacted. Companies to bring analysis until next meeting on impacted requirements and potential changes.

  - Option 2: RAN4 to only discuss the impact to CBD evaluation period:

    - RAN4 should consider the impact in RAN4 requirements for the Evaluation period TEvaluate_CBD_CSI-RS due to the presence of predicted beams

  - Option 3: RAN4 does need to discuss such impact, the changes would implicitly apply by assume a predicted beam as measured/reported

  - Option 4: There is no impact, this is not in scope

  - Option 5: others

- Recommended WF

  - To be discussed

### Sub-topic 2-7

LCM

Issue 2-7:	LCM Requirements

- Proposals

  - Option 1: the delay components of LCM operations should be considered for different CSI-report configurations (i) periodic CSI report (ii) semi-persistent CSI report and (iii) aperiodic CSI report for both BM-Case1 and BM-Case2.

  - Option 2: For performance monitoring for BM, the corresponding delay is defined as

    - Tmonitoring_period_BM = Nmonitor* TL1-RSRP_Prediction_Period_CSI-RS,

      - Where Nmonitor is the configured number of the latest transmission occasion(s) of monitoring resources with linked inference report no later than CSI reference resource corresponding to the CSI report for monitoring

  - Option 3:postpone discussion until RAN1/2 monitoring framework is clear

- Recommended WF

  - To be discussed

### Sub-topic 2-8

Test system basic setup

The basic test setup has been discussed for several meetings without much progress.

Issue 2-8: Test system setup

- Proposals

  - Option 1: Single AoA with UE antenna pattern knowledge

  - Option 2: Single AoA without any UE antenna pattern knowledge

  - Option 3: Multi AoA based on MPAC

  - Option 4: Multi AoA based on RRM testing setup

- Recommended WF

  - To be discussed

### Sub-topic 2-9

Test system channel model

In the previous meeting it was agreed that a CDL-based channel model will be taken as starting point.  Further details need to be discussed

Issue 2-9: Test system setup

- Proposals

  - Option 1: “full” CDL model to be emulated in the test

  - Option 2: simplified CDL should be used

  - Option 3: others

- Recommended WF

  - To be discussed

If Option 2 is chosen, simplification methods should be discussed.

### Sub-topic 2-10

Absolute RSRP accuracy applicability

In the previous meeting it was agreed that absolute RSRP would apply to the top predicted beam but applicability to the other predicted beams is still FFS

Agreement:

- Absolute RSRP accuracy requirement applies to Top-1 of predicted beams at least

  - FFS whether to apply to other beams

Issue 2-10: Absolute RSRP accuracy applicability

- Proposals

  - Option 1: applicable only to top predicted beam

  - Option 2: should apply to all K predicted beams

  - Option 3: postpone this discussion to the performance part as it relates to accuracy

  - Option 4: others

- Recommended WF

  - Option 3

### Sub-topic 2-11

Draft CR

Draft of a CR to introduce requirements in 38.133 was presented in R4-2506464. Some high level structure of the specs should be discussed.

Issue 2-11: Beam prediction CR

- To be discussed

  - High level structure: in which clauses should the beam prediction specs go

    - LCM

    - Prediction(inference) requirements

    - Performance requirements (to be finalized later but initial agreement on structure would be desirable)

  - Use “legacy” formatting or try new “pseudocode” based option?

  - Other suggestions

# Topic #3: RRM core requirement and testing framework for Positioning accuracy enhancement

This section contains the sub-topics regarding specific issues for positioning

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2505947 | Apple | Proposal 1: RAN4 to further discuss the feasibility and how to define requirements for positioning accuracy for case 1. Defining requirements is also dependent on RAN1/2 conclusion regarding whether UE will report positioning Proposal 2: For Assisted AIML Positioning, the KPI test metric (e.g., LOS/NLOS) needs to be considered for validating the positioning accuracy. RAN4 should inevstiagte how to establish requirements for LOS/NLOS indicators, considering their potential impact on performance and accuracy in positioning systems. Proposal 3: For AIML positioning, RAN4 should take as baseline the requirements on the measurements supported in legacy positioning (RSTD, PRS-RSRP, and PRS-RSRPP) and define the requirements on the new measurements (e.g., CIR, DP, and PDP) based on RAN1 progress. Proposal 4: RAN4 to study defining performance accuracy requirements for any reported UE measurements defined by other groups. Proposal 5: RAN4 should define measurement requirements for both path-based and sample-based reporting in relation to use case 2b. Proposal 6: Before RAN4 investigates testing Case 2b in AI/ML for positioning, it is important to first examine the relationship between the measurement accuracy reported by the UE and the resulting positioning accuracy at the LMF. Proposal 7: RAN4 to define performance accuracy requirements for use case 3a for the measurements reported by gNB. The specifics and scope of the core and performance requirements for use case 3a are contingent on the outcomes of discussions within RAN1 and RAN2 Proposal 8: RAN4 to define performance accuracy requirements for measurements performed at gNB for use case 3b. The specifics and scope of the core and performance requirements for use case 3a are contingent on the outcomes of discussions within RAN1 and RAN2 |
| R4-2505981 | CATT | Proposal 1: RAN4 not to define performance requirements for case 1. Proposal 2: RAN4 to reuse the UL RTOA reporting requirement for sample-based timing measurement. Proposal 3: RAN4 to wait for RAN1 progress before discussing how to define the reporting requirement for sample-based timing measurement. |
| R4-2506468 | vivo | Proposal 1: RAN4 to define delay requirement for case 1 at least considering the following procedures: PRS measuring, AI processing. Legacy requirement (if exists) can be reused. Proposal 2: RAN4 to postpone the discuss of requirement for positioning case 2b. Proposal 3: RAN4 to define delay requirement for case 2a at least considering the following procedures: PRS measuring, AI processing, UE reporting intermediate features to LMF. Legacy requirement (if exists) can be reused. Proposal 4: RAN4 not to define requirements for positioning case 3. Proposal 5: RAN4 to wait for more progress from other WGs to consider define delay requirements for monitoring for positioning case 1. Observation 1: PRUs can be deployed in the test scenario to provide measurements together with the location information. Proposal 6: RAN4 to consider deploy PRUs in conformance or the pre-deployment test so as to derive the ground truth value of UE position and intermediate features. Proposal 7: RAN4 to discuss the key factors that affect the performance and consider introduce reference scenario/model/dataset for AI/ML based positioning test for generalization. |
| R4-2506525 | OPPO | Proposal 1: For case1, RAN4 will not define positioning accuracy requirements in R19 WI. Observation 1: For case1(AI/ML assisted positioning), the feasibility of using intermediate results(output of AI/ML models with non-linear processing) for RAN4 tests is not clear. - how to get the label data for intermediate results(e.g. RSTD, identification of LoS/NLoS and other metrics that agreed in RAN1) and test these intermediate results is not clear. |
| R4-2506607 | CMCC | Proposal 1: for case 1, it is proposed to define positioning accuracy requirement, which is the difference between ground truth (real position) and reported position. - At least accuracy requirements for indoor factory (InF) scenario can be defined Proposal 2: for case 1, it is proposed to define delay related requirements, e.g. prediction delay, LCM related delay. |
| R4-2507050 | Huawei, HiSilicon | Proposal 1: For Case 3b, reuse the legacy requirement for reporting of timing information or timing and power information from gNB to LMF. Proposal 2: For Case 3a, RAN4 to discuss whether to reuse the legacy requirement for reporting of timing information from gNB to LMF, if there is no explicit indicator introduced to distinguish whether the timing information is obtained by legacy method or by Rel-19 AI/ML. Proposal 3: RAN4 to discuss the requirement for Rel-19 AI/ML based timing information reporting if introduced by other WGs. Proposal 4: RAN4 to discuss requirements for LCM procedure especially performance monitoring for AI/ML positioning, based on RAN1 conclusion on performance monitoring schemes and also RAN4 conclusion on the requirements for inference. |
| R4-2507185 | ZTECorporation,Sanechips | Observation 1: RAN plenary has already precluded case2a and case2b. Proposal 1：RAN4 shall not define any RRM requirements on case 2a and case 2b. Observation 2: In legacy, there are two kinds of location request: UE-triggered location request and NW-triggered location request. The latest agreement contains the implicit location request which is the NW triggers the location request, otherwise no reporting scheme is needed. Proposal 2: If RAN1 has the agreement on the reporting scheme can use the legacy location request, there are two options here: Option 1: If it is NW-triggered location request, UE shall report its location and the report requirements shall be confirmed. Option 2: If it is UE-triggered location request, it is no need to report the location to NW which is up to UE implementation. Observation 3: If the ground truth is known to test equipment, the accuracy can be quantified compared to defined ground truth, such as AI/ML positioning error. Then the model performance can be evaluated and the subsequent operation of model can be clarified. Observation 4: The UE location is the coordinate and the PRU will be set on anywhere, if we set PRU randomly, we do not think the accuracy can be accurately evaluated. Observation 5: If UEs have the GNSS capability, they also can be the reference UEs. However, if we choose the reference GNSS UEs, the GNSS error shall be considered when evaluating the AI/ML positioning accuracy error. Proposal 3: The ground truth can be the PRU and the test map shall be considered. Reference GNSS UEs shall be considered. |
| R4-2507533 | Nokia | Case 1 For AIML positioning in Case 1, RAN4 to specify the requirements on the measurements supported in legacy positioning (e.g., RSTD, PRS-RSRP, and PRS-RSRPP) and define the requirements on the new measurements (e.g., CIR, DP, and PDP) based on RAN1 progress. RAN4 to study the necessity of any requirements for the input of the AIML model in Case 1. RAN4 requirements (e.g., in Case 1) should be first defined with respect to single PFL. The consistency between training and inference should be achieved by having a requirement on consistent configuration (e.g., same PFL) when UE-sided models are used. - If an LCM action is required and it is not taken in a timely manner, the performance degradation for AI/ML enabled Positioning use case may be degraded to undesirable level. The time latency on UE’s LCM actions has an impact on the performance. RAN4 to define RRM requirements for LCM focusing on Activation, Deactivation, Switching, Fallback to non-AI operation, and Performance monitoring. RAN4 to discuss the requirements for performance monitoring when target UE indicates that it ‘can’ perform the Case 1 positioning method. Reporting of location estimates is supported in the current specifications. Reuse of the reporting facility for Case 1 is possible, given that a certain location coordinate type from the existing ones is selected. The UE is enabled to report its local 3D coordinates with origin in a known reference location together with its estimated location uncertainty, both inferred through its AI/ML model. RAN4 to use existing location coordinate reporting to identify positioning accuracy requirements for Case 1. Positioning accuracy can be verified based on the ground truth which may consist of the location points with known positioning co-ordinates (e.g., PRU or GNSS based). RAN4 should specify (i) the test mechanisms for positioning accuracy and (ii) the metric for positioning accuracy performance in Case 1. RAN4 to specify a test grid map with known location points in indoor space and an appropriate set of TRPs for the positioning measurement in Case 1. RAN4 to select either error calculation method 2 (without location uncertainty) or 3 (applying worst case uncertainty error) for deriving the performance metric for Case 1. RAN4 to select the three-dimensional error e of the absolute reported positioning co-ordinates relative to the evaluated ground truth as performance metric for the positioning accuracy in Case 1. Case 3a/3b RAN4 to not specify a test framework for path-based measurements and label-based information in Case 3a. RAN4 to not specify a test framework for path-based and sample-based measurements in Case 3b. |
| R4-2507431 | Nokia | The following Proposals were made for AI/ML Positioning use case: Proposal 9: RAN4 to use positioning accuracy as the performance metric in Case 1. Proposal 10: RAN4 to assume an AWGN channel in performance requirements if any applicable scenario exists in Cases 1 and 3. Proposal 11: RAN4 to reuse the report mapping for UL SRS-RSRP to Case 3b. Proposal 12: RAN4 to introduce gNB Rx-Tx time difference reporting accuracy requirements (i.e., path-based measurements) by reusing the legacy accuracy requirements in Case 3b. Proposal 13: RAN4 to study feasibility of reusing the legacy performance requirements for the sample-based measurement, and how to specify separate performance requirements for sample-based measurement if reusing is not feasible in Case 3b. Proposal 14: RAN4 to not specify a requirement on training data collection of Part B in Case 3a/3b. |
| R4-2507663 | Ericsson | Proposal 1: Existing measurement period requirement for positioning measurements is used as a baseline to define the measurement delay requirement for the UE capable of supporting positioning use case 1. Proposal 2: Requirement for RRC_CONNECTED mode where the measurements for positioning inference are performed by the UE inside the measurement gap is prioritized. Proposal 3: No impact from TEG is considered on the measurement period requirement for measurements performed by UE in use case 1 for positioning inference. Proposal 4: Impact of CSSF is taken into account for the measurement period requirement for use case 1. Observation 1: Number of Rx beams performed by the UE to receive DL PRS resources is transparent in UE based solutions. Observation 2: Issue on the feasibility of defining the accuracy requirement for use case 1 is yet to resolved in RAN4 and therefore the impact of number of samples on the measurement delay requirement for use case 1 cannot be evaluated at this point of time. |
| R4-2507664 | Ericsson | In this contribution, FFS items from previous meeting are addressed and report mapping tables for use case 3b are proposed. The observations and proposals below summarize the discussions presented in the paper. Observation 1: Performance requirement in terms of the accuracy of the location estimated by the UE is not specified for non-AI/ML UE based positioning supported by the NR specifications. Observation 2: Achievable positioning accuracy is scenario specific, and the same level of positioning accuracy cannot be achieved in different scenarios. Observation 3: Producing a test setup that mimics for example InF or IOO or UMi or UMa with lot more cells than in the legacy setup is hard to be achieved in a lab. Proposal 1: RAN4 to conclude that defining requirement in terms of the accuracy of estimated UE position for case 1 to be met in any scenario is not feasible. Observation 4: Depending on the request from LMF, gNB can report path-based measurements to the LMF. For such measurement reporting, no enhancement to Rel. 18 reporting has been introduced. Observation 5: Depending on the request from LMF, gNB can report sample-based measurements to the LMF. For such measurement reporting, report mapping table needs to be specified in RAN4 specification. Observation 6: For sample-based reporting, gNB may either report timing measurement or timing and power measurements associated to Nt´ samples of the estimated channel response. Observation 7: Reporting granularities for sample-based timing measurement reporting is same as the reporting granularities supported for legacy measurements UL-RToA and gNB Rx-Tx time difference measurement. Proposal 2: Reporting range for sample-based timing measurements is from -985024Tc to 985024Tc. Proposal 3: Table 1 is used to define measurement report mapping table for sample-based timing measurement reporting for reporting granularity corresponding to k = 0. Table 1. Measurement report mapping for k=0. Reported Value; Measured Quantity Value; Unit. SAMPLE_TIME_0000000; -985024 > SAMPLE_TIME; Tc. SAMPLE_TIME_0000001; -985024  SAMPLE_TIME < -985023; Tc. SAMPLE_TIME_0000002; -985023  SAMPLE_TIME < -985022; Tc. ; ; …. SAMPLE_TIME_0985023; -2  SAMPLE_TIME < -1; Tc. SAMPLE_TIME_0985024; -1  SAMPLE_TIME  0; Tc. SAMPLE_TIME_0985025; 0 < SAMPLE_TIME  1; Tc. SAMPLE_TIME_0985026; 1 < SAMPLE_TIME  2; Tc. SAMPLE_TIME_0985027; 2 < SAMPLE_TIME  3; Tc. …; …; …. SAMPLE_TIME_1970048; 985023 < SAMPLE_TIME  985024; Tc. SAMPLE_TIME_1970049; 985024 < SAMPLE_TIME; Tc. Proposal 4: Table 2 is used to define measurement report mapping for sample-based timing measurement reporting for reporting granularity corresponding to k = 1. Table 2. Measurement report mapping for k=1. Reported Value; Measured Quantity Value; Unit. SAMPLE_TIME_000000; -985024 > SAMPLE_TIME; Tc. SAMPLE_TIME_000001; -985024  SAMPLE_TIME < -985022; Tc. SAMPLE_TIME_000002; -985022  SAMPLE_TIME < -985020; Tc. ; ; …. SAMPLE_TIME_492511; -4  SAMPLE_TIME < -2; Tc. SAMPLE_TIME_492512; -2  SAMPLE_TIME  0; Tc. SAMPLE_TIME_492513; 0 < SAMPLE_TIME  2; Tc. SAMPLE_TIME_492514; 2 < SAMPLE_TIME  4; Tc. SAMPLE_TIME_492515; 4 < SAMPLE_TIME  6; Tc. …; …; …. SAMPLE_TIME_985024; 985022 < SAMPLE_TIME  985024; Tc. SAMPLE_TIME_985025; 985024 < SAMPLE_TIME; Tc. Proposal 5: Table 3 is used to define measurement report mapping for sample-based timing measurement reporting for reporting granularity corresponding to k = 2. Table 3. Measurement report mapping for k=2. Reported Value; Measured Quantity Value; Unit. SAMPLE_TIME_000000; -985024 > SAMPLE_TIME; Tc. SAMPLE_TIME_000001; -985024  SAMPLE_TIME < -985020; Tc. SAMPLE_TIME_000002; -985020  SAMPLE_TIME < -985018; Tc. ; ; …. SAMPLE_TIME_246255; -8  SAMPLE_TIME < -4; Tc. SAMPLE_TIME_246256; -4  SAMPLE_TIME  0; Tc. SAMPLE_TIME_246257; 0 < SAMPLE_TIME  4; Tc. SAMPLE_TIME_246258; 4 < SAMPLE_TIME  8; Tc. SAMPLE_TIME_246259; 8 < SAMPLE_TIME  12; Tc. …; …; …. SAMPLE_TIME_492512; 985020 < SAMPLE_TIME  985024; Tc. SAMPLE_TIME_492513; 985024 < SAMPLE_TIME; Tc. Proposal 6: Table 4 is used to define measurement report mapping for sample-based timing measurement reporting for reporting granularity corresponding to k = 3. Table 4. Measurement report mapping for k=3. Reported Value; Measured Quantity Value; Unit. SAMPLE_TIME_000000; -985024 > SAMPLE_TIME; Tc. SAMPLE_TIME_000001; -985024  SAMPLE_TIME < -985016; Tc. SAMPLE_TIME_000002; -985016  SAMPLE_TIME < -985008; Tc. ; ; …. SAMPLE_TIME_123127; -16  SAMPLE_TIME < -8; Tc. SAMPLE_TIME_123128; -8  SAMPLE_TIME  0; Tc. SAMPLE_TIME_123129; 0 < SAMPLE_TIME  8; Tc. SAMPLE_TIME_123130; 8 < SAMPLE_TIME  16; Tc. SAMPLE_TIME_123131; 16 < SAMPLE_TIME  24; Tc. …; …; …. SAMPLE_TIME_246256; 985016 < SAMPLE_TIME  985024; Tc. SAMPLE_TIME_246257; 985024 < SAMPLE_TIME; Tc. Proposal 7: Table 5 is used to define measurement report mapping for sample-based timing measurement reporting for reporting granularity corresponding to k = 4. Table 5. Measurement report mapping for k=4. Reported Value; Measured Quantity Value; Unit. SAMPLE_TIME_000000; -985024 > SAMPLE_TIME; Tc. SAMPLE_TIME_000001; -985024  SAMPLE_TIME < -985008; Tc. SAMPLE_TIME_000002; -985008  SAMPLE_TIME < -984992; Tc. ; ; …. SAMPLE_TIME_061563; -32  SAMPLE_TIME < -16; Tc. SAMPLE_TIME_061564; -16  SAMPLE_TIME  0; Tc. SAMPLE_TIME_061565; 0 < SAMPLE_TIME  16; Tc. SAMPLE_TIME_061566; 16 < SAMPLE_TIME  32; Tc. SAMPLE_TIME_061567; 32 < SAMPLE_TIME  48; Tc. …; …; …. SAMPLE_TIME_123128; 985008 < SAMPLE_TIME  985024; Tc. SAMPLE_TIME_123129; 985024 < SAMPLE_TIME; Tc. Proposal 8: Table 6 is used to define measurement report mapping for sample-based timing measurement reporting for reporting granularity corresponding to k = 5. Table 6. Measurement report mapping for k=5. Reported Value; Measured Quantity Value; Unit. SAMPLE_TIME_00000; -985024 > SAMPLE_TIME; Tc. SAMPLE_TIME_00001; -985024  SAMPLE_TIME < -984992; Tc. SAMPLE_TIME_00002; -984992  SAMPLE_TIME < -984960; Tc. ; ; …. SAMPLE_TIME_30781; -64  SAMPLE_TIME < -32; Tc. SAMPLE_TIME_30782; -32  SAMPLE_TIME  0; Tc. SAMPLE_TIME_30783; 0 < SAMPLE_TIME  32; Tc. SAMPLE_TIME_30784; 32 < SAMPLE_TIME  64; Tc. SAMPLE_TIME_30785; 64 < SAMPLE_TIME  96; Tc. …; …; …. SAMPLE_TIME_61564; 984992 < SAMPLE_TIME  985024; Tc. SAMPLE_TIME_61565; 985024 < SAMPLE_TIME; Tc. |

## Open issues summary

The agreements from last meeting are listed below for convenience:

Issue 3-2: LOS/NLOS indicator

Agreement:

the LOS/NLOS indicator is out of scope of WID. Stop discussions on it.

Issue 3-3: Rx-Tx time difference for case 3a

Agreement:

- Introduce gNB Rx-Tx time difference reporting accuracy requirements

    - Reuse legacy accuracy requirements

Issue 3-4: Report mapping for gNB Rx-Tx time difference

Agreement:

- Reuse the existing report mapping for gNB Rx-Tx time difference measurements

The open issues were grouped in the following sub-topics for further discussion:

            - Case 1 requirements

            - Report mapping for sample based timing measurement reporting

            - Case 3b requirements

### Sub-topic 3-1

Requirements for case 1

It has been discussed for several meetings whether to define requirements for case 1 without any progress

Issue 3-1: Requirements for case 1

Proposals

  - Option 1: RAN4 will define new positioning accuracy requirements

    - Accuracy should be the difference between ground truth and reported position

    - See R4-2507533 for possible testing solution

  - Option 2: existing requirements for legacy positioning techniques(RSTD, PRS-RSRP, etc) should be reused for Case 1

  - Option 3: RAN4 to define only reporting delay(including inference delay) requirements

  - Option 4: RAN4 does not define any requirements for case 1

- Recommended WF

  - Option 2

Defining new requirements(tighter than existing ones will be a very long and complicated process)

### Sub-topic 3-2

Report mapping for sample based timing measurement reporting

A proposal for the reporting range and report mapping is provided in R4-2507664.

Issue 3-2: Report mapping for sample based timing measurement reporting

Proposals

  - Option 1: Agree Proposals 2, 3, 4,5,6,7,8

  - Option 2: Others

- Recommended WF

  - Option 1

### Sub-topic 3-3

Case 3b requirements

Issue 3-3: Requirements for case 3b

- Proposals

  - Option 1:

    - RAN4 to introduce gNB Rx-Tx time difference reporting accuracy requirements (i.e., path-based measurements) by reusing the legacy accuracy requirements in Case 3b.

    - RAN4 to study feasibility of reusing the legacy performance requirements for the sample-based measurement, and how to specify separate performance requirements for sample-based measurement if reusing is not feasible in Case 3b.

  - Option 2: Others

- Recommended WF

  - Option 1