3GPP TSG-RAN WG4 Meeting #115 	R4-2507579

Malta, MT, May 19th – 23rd, 2025

Agenda Item: 7.25.4.1

Source: MediaTek inc.

Title: Discussion on SBFD UE-to-UE CLI

Document for: Discussion

# Introduction

The work item (WI) for Evolution of NR duplex operation: Sub-band full duplex (SBFD) has the following objectives [1]:

The objectives are as follows: - For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier: - Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4] - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4] - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Note: Without dedicated optimization for dynamic/flexible TDD. - Specify BS RF requirements for SBFD operation at gNB [RAN4] - Specify applicable RRM core requirements for CLI handling mechanisms [RAN4] - Specify other RRM core requirements for SBFD operation, if identified [RAN4]

In the last meeting, RRM group discussed several issues on the impacted RRM specification and whether new RRM requirements are needed to handle the new SBFD operation and CLI handling mechanism, where the issues and agreements are captured in the way forward (WF) [2] and collection of companies proposals in Topic summary [3]. In this contribution, we provide further discussion on the issues captured in the WF.

# Discussion on the RRM impact for CLI handling mechanisms

In legacy 5G NR TDD, the RRM specification cover the CLI measurements delay and accuracy for SRS-RSRP and CLI-RSSI. The UE reports these measurements to the gNB to help the dynamic allocation of uplink/downlink in time. Nevertheless, these measurements are based on L3 measurements, i.e. it requires L3 filtering, which has longer measurement delay compared to L1 measurements. Therefore, in SBFD Rel-19 WID objective, new CLI measurements and reporting shall be introduced for L1 based UE-to-UE CLI with new measurement quantities, such as L1-SRS-RSRP, L1-CLI-RSSI.

# Discussion on the RRM impact for L1-SRS-RSRP

In the previous meeting, RAN4 RRM reached agreement with related further study point, which are captured in the WF.

Issue 1-2-2: Scheduling restriction:

Agreements (from Thu AH session) meeting #114 - For L1-SRS-RSRP, - For UL, scheduling restrictions always apply, for both FR1 and FR2. - For DL, scheduling restriction does not apply to the symbols with SRS if UE is able to support FDM-ed DL reception and SRS measurement if RAN1 will define optional UE capability for FDMed DL reception and SRS measurement; otherwise (if UE does not support the capability or RAN1 does not define the capability), scheduling restriction applies. - The restricted symbols at least include symbols for measurement resource and Y symbols before symbols for measurement resource and Y is re-using same number as R16 requirements, - Note: this time has accounted for the UL/DL switching time. - FFS whether there is restriction on symbols after the symbols for measurement resource considering UL/DL transition. - Agreement (online session on Monday with update) - For UL, - Scheduling restrictions always apply, for both FR1 and FR2. - The restricted symbols include symbols for SRS resource and Y symbols before, where Y is re-using same number as R16 requirements. - For the symbols after the SRS resource, not define scheduling restriction on symbols after symbols for SRS resource in TS 38.133, with the understanding that the UL/DL switching time is covered by RAN1 specification TS 38.211. - For DL, - if UE is able to support FDM-ed DL reception and SRS measurement, scheduling restriction does not apply except when SRS resources are not TypeD QCL-ed with PDCCH/PDSCH in FR2, otherwise (if UE does not support the capability), scheduling restriction applies. - The restricted symbols include symbols for SRS resource and Y symbols before symbols for SRS resource, where Y is re-using same number as R16 requirements. - FFS whether restricted symbol(s) is needed for the symbol(s) after SRS resource to consider the switching between UL subband and DL subband.

For the measurements of L1-CLI-RSRP, given that the UE is configured to perform measurements (i.e. DL behaviour) within UL subband. In addition, it was agreed that the reference timing for L1 SRS-RSRP measurement, UE will apply a constant offset relative to the downlink reference timing in the serving cell, and the constant offset value is derived by UE implementation and shall be at least Tc*NTA_offset. In addition, in the previous meeting, RAN4 agreed to define restricted symbol before the reference signal and FFS the symbols after the reference signal resources. Yet, it should be pointed out that the UL/DL switching is about 13us, which is larger than a CP, hence, the number of restricted symbols should be rounded up to one symbol.

- the number of restricted symbols after the symbols for measurement resource should be an integer number and consider UL/DL transition, hence, the number of symbols should equal to one symbol.

Issue 1-1-6: Measurement restriction:

Agreements - Measurement requirements do not apply for L1-SRS-RSRP when the measurement resources are partially or fully overlapping with SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement or measurement gaps. - When measurement resources for L1-SRS-RSRP and L1-CLI-RSSI are partially or fully overlapping, UE is not required to measure both L1-SRS-RSRP and L1-CLI-RSSI, FFS the following options - Option 1: Prioritize L1-SRS-RSRP measurement - Option 2: Prioritize L1-CLI-RSSI measurement - Option 3: Leave it to UE implementation whether to measure L1-SRS-RSRP or L1-CLI-RSSI

For collision between different CLI measurements, it is acceptable to capture that in RAN4 spec that UE is not expected to perform measurement using Method #1 and Methods#3 in the same OFDM symbol. Hence, leave it to UE implementation whether to measure L1-SRS-RSRP or L1-CLI-RSSI.

- When measurement resources for L1-SRS-RSRP and L1-CLI-RSSI are partially or fully overlapping, UE is not required to measure both L1-SRS-RSRP and L1-CLI-RSSI, leave it to UE implementation whether to measure L1-SRS-RSRP or L1-CLI-RSSI.

# Discussion on the RRM impact for L1-CLI-RSSI

In the previous meeting, RAN4 RRM reached agreement with related further study point, which are captured in the WF.

Issue 1-3-2: Scheduling restriction:

Agreements (adhoc session on Tuesday with addition) - Method #1: - For UL, - Scheduling restrictions always apply, for both FR1 and FR2. - The restricted symbols include symbols for RSSI resource. - Do not define scheduling restriction on symbols after symbols for RSSI resource, with the understanding that the UL/DL switching time is covered by RAN1 specification TS 38.211 clause 4.3.2 - For DL, - Scheduling restriction does not apply except when RSSI resources are not TypeD QCL-ed with PDCCH/PDSCH in FR2, the restricted symbols include symbols for RSSI resource - Method #3: - For UL, - Scheduling restrictions always apply, for both FR1 and FR2. - The restricted symbols include symbols for RSSI resource. - Do not define scheduling restriction on symbols after symbols for RSSI resource, with the understanding that the UL/DL switching time is covered by RAN1 specification TS 38.211 clause 4.3.2 - For DL, - Scheduling restriction does not apply except when RSSI resources are not TypeD QCL-ed with PDCCH/PDSCH in FR2, the restricted symbols include symbols for RSSI resource. - Note: the common understanding in RAN4 if the RSSI in UL suband is too strong compare to the wanted signals in DL suband(s), UE may fail to decode the PDCCH or PDSCH. - FFS whether restricted symbol(s) is needed for the symbol(s) after RSSI resource to consider the switching between UL subband and DL subband. - Generic assumption for defining RRM requirements, not intend to add it in the core requirement. - For all Method #1, Method #2 and Method #3, - Note: The RF BW is wide enough to cover DL active BWP, including PRBs from both DL subband(s) and UL subband that fall within the DL active BWP.

For DL case of L1-CLI-RSSI Method #3, given that Method#3 is based on performing measurement within UL subband then RAN4 should follow the requirements defined for Method#2 as baseline.

- L1-CLI-RSSI Method #3, RAN4 to follow the requirements defined for Method#2 as baseline for defining scheduling restriction for Method #3.

# Discussion on remaining issues

### Sub-topic 1-1: General issues #### Issue 1-1-5: Requirement applicability ### Sub-topic 1-2: L1-SRS-RSRP #### Issue 1-2-4: Side condition #### Issue 1-2-5: Measurement accuracy ### Sub-topic 1-3: L1-CLI-RSSI #### Issue 1-3-4: Side conditions #### Issue 1-3-5: Measurement accuracy #### Issue 1-3-6: SCS for performing measurement

Issue 1-1-5: Requirement applicability:

In the previous meeting, RAN4 discussed applicability of L1-SRS-RSRP and L1-CLI-RSSI measurement requirement in terms of frequency location of measurement resources. In our view, RAN4 should follow RAN1 agreement in terms of measurements resources, hence, L1 CLI measurement resources must be within the DL active BWP of the UE, where L1-SRS-RSRP confined within UL subband, while L1-CLI-RSSI based method #1 and #3 to be confined within DL and UL subband, respectively.

Agreement (from RAN1#118) For L1 UE-to-UE CLI measurement and reporting, CLI measurements is performed within the active DL BWP and the following are supported: - Method#1: UE measures RSSI within DL subband - Method#2: UE measures RSRP of aggressor UE within UL subband - FFS: Method#3: UE measures RSSI within UL subband

- L1 CLI measurement resources must be within the DL active BWP of the UE.

Issue 1-2-4/5: Side condition and accuracy requirements of L1-SRS-RSRP:

For SRS BW, following the mandatory simulation assumptions, the BW should follow the same BW used in the simulation assumption. Hence, the SRS BW shall be 48PRB.

- The SRS BW of L1-SRS-RSRP is equal to 48 PRBs.

For SRS Es/Iot, our preference to discuss this issue during the Performance part, unless impacts to core requirements are identified.

- The SRS Es/Iot of L1-SRS-RSRP is to be discussed during the performance part.

For accuracy requirement, our preference to discuss this issue during the Performance part, unless impacts to core requirements are identified.

- The accuracy requirements of L1-SRS-RSRP is to be discussed during the performance part.

Issue 1-3-4/5: Side condition and accuracy requirements of L1-CLI-RSSI:

For side condition and accuracy requirement, our preference to discuss this issue during the Performance part, unless impacts to core requirements are identified.

- The side conditions and accuracy requirements of L1-CLI-RSSI is to be discussed during the performance part.

Issue 1-3-6: SCS for performing measurement:

Given that method#1 and method#3 have no specific reference signals, thus, UE shall perform L1-CLI-RSSI measurement with the SCS of the DL active BWP.

- UE shall perform L1-CLI-RSSI measurement with the SCS of the DL active BWP, for both method #1 and method #3.

# Summary

In this contribution, we have the following proposals:

Proposal 1: the number of restricted symbols after the symbols for measurement resource should be an integer number and consider UL/DL transition, hence, the number of symbols should equal to one symbol.

Proposal 2: When measurement resources for L1-SRS-RSRP and L1-CLI-RSSI are partially or fully overlapping, UE is not required to measure both L1-SRS-RSRP and L1-CLI-RSSI, leave it to UE implementation whether to measure L1-SRS-RSRP or L1-CLI-RSSI.

Proposal 3: L1-CLI-RSSI Method #3, RAN4 to follow the requirements defined for Method#2 as baseline for defining scheduling restriction for Method #3.

Proposal 4: L1 CLI measurement resources must be within the DL active BWP of the UE.

Proposal 5: The SRS BW of L1-SRS-RSRP is equal to 48 PRBs.

Proposal 6: The SRS Es/Iot of L1-SRS-RSRP is to be discussed during the performance part.

Proposal 7: The accuracy requirements of L1-SRS-RSRP is to be discussed during the performance part.

Proposal 8: The side conditions and accuracy requirements of L1-CLI-RSSI is to be discussed during the performance part.

Proposal 9: UE shall perform L1-CLI-RSSI measurement with the SCS of the DL active BWP, for both method #1 and method #3.

# References

- RP-241614, “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, June 2024.

- R4-2504904, “WF on RRM requirements for NR_duplex_evo”, Huawei, HiSilicon, Wuhan, China, April 2025.

- R4-2503623, “Topic summary for [114bis][213] NR_duplex_evo”, Huawei, HiSilicon, Wuhan, China, April 2025.

- R1-2501620, “LS on L1 based UE-to-UE CLI measurement and reporting”, RAN1, Athens, Greece, Feb 2025.