3GPP TSG-RAN WG4 Meeting #115 	R4-2507582

Malta, MT, May 19th – 23rd, 2025

Agenda Item: 7.30.3

Source: MediaTek inc.

Title: Discussion on RRM requirements for XR

Document for: Discussion

# Introduction

The work item (WI) for XR (eXtended Reality) for NR Phase 3 has the following objectives [1]:

The Rel-19 XR ph3 objectives are as follows: - Study and if justified, specify aspects related to multi-modality (intra-UE) (with coordination with SA2/SA4 as needed by LS request). Aim to facilitate efficient and effective support for XR application with Multiple QoS flows with multi-modal inter-dependencies, meeting multi-modal QoS requirements, e.g. synchronization and/or coordination. Efficiency enhancements are expected to be visible in terms of capacity or power consumption. [RAN2]. - Note: Check in RAN#105 (check also other WG involvement if needed). - Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4] - Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4] - Specify Enhancements for Scheduling, as follows: - For the UL, Study and if justified, Specify enhancements using delay/deadline information, for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs. [RAN2]. - Note: LCP implementation complexity need to be taken into account when evaluating solutions. - Note: Check in RAN#105 - Specify the following user plane enhancements [RAN2] - RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget. - If justified, define a mechanism for transmitter to inform the receiver of SN gap (or missing SNs) in PDCP. - Specify Core requirements related to the above objectives as necessary [RAN4]

In this contribution, the potential directions that leads to enhance the gap cancellation mechanisms are discussed then other RRM impact issues are raised, which requires some studies.

# Discussion

RAN4 RRM has discussed and identified the impact on RRM requirements as below.

# Discussion on XR skipping requirements impact

In the previous RRM meeting, issues related to the scenarios are captured in the way forward [2].

## Issue 3-2: Formulas for measurement delay extension <Way forward>: - Option 1: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - Max(200 ms × CSSFinter, 8 × Max(MGRP , SMTC period) × CSSFinter + Nskip × Max(MGRP , SMTC period)), - where Nskip is the number of skipped MG occasions in the measurement period. - Use this formula as example and follow similar approach for the other cases. - Option 2: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - Max(200 ms, ( 8 + Nskip) Max(MGRP , SMTC period)) CSSFinter - where Nskip is the number of skipped MG occasions in the measurement period. - Use this formula as example and follow similar approach for the other cases. - Option 3: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - Max(200 ms, ( 8 + Nskip) Max(MGRP , SMTC period)) CSSFinter - where Nskip is the number of skipped MG occasions in the measurement period. - Use this formula as example and follow similar approach for the other cases. - FFS: CSSFinter ## Issue 3-3: Formulas for measurement delay extension with DRX <Way forward>: - Option 1: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - DRX cycle ≤ 320 ms - Max(200 ms CSSFinter, Ceil(8 1.5) Max(MGRP, SMTC period, DRX cycle) CSSFinter + Nskip × Max(MGRP, SMTC period, DRX cycle) ) - DRX cycle > 320 ms - 8 DRX cycle CSSFinter + Nskip DRX cycle - Option 2: Inter-frequency cell detection period requirements with MG cancellation due to XR can be as in the table below for FR1 and FR2-1, respectively: - DRX cycle ≤ 320 ms - Max(200 ms, Ceil((8+XPSS/SSS,gaps) 1.5) Max(MGRP, SMTC period, DRX cycle)) CSSFinter - DRX cycle > 320 ms - Ceil(8+XPSS/SSS,gaps) DRX cycle CSSFinter ## Issue 3-4: Conditions for measurement skipping <Way forward>: - Option 1: No conditions are defined - Option 2: Define the maximum number of gaps skipped during the measurement period. - Option 3: Define a maximum separation between two non-skipped gaps for measurement

Issue 3-2: Formulas for measurement delay extension

Furthermore, RAN4 discussed the impact on the measurements delay requirements due to MG cancellation. The number of samples for a single layer measurement was decided based on simulation evaluation for different channel environment. Given that the XR is meant to operate with NR devices then the focus shall be on the impact of measurement gap cancellation and how to compensate the dropped measurement samples. In previous meetings, RAN4 already agreed to extend the delay requirements to compensate the skipped measurement gap occasions, also, RAN4 agreed to follow NR-U like approach to define the requirements. In addition, the way forward captured two options for information and based on the NR-U like approach, Option 2 should be supported, i.e. ‘Max(200 ms, Ceil(( 8 + Nskip ) * Kgap) × Max(MGRP , SMTC period)) × CSSFinter’, where Nskip is the number of  skipped MG occasions in the measurement period.

- RAN4 shall keep the number of measurement samples for a frequency layer for L3 measurements unimpacted due to XR traffic.

- RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as: ‘Max(200 ms, Ceil(( 8 + Nskip ) * Kgap) × Max(MGRP , SMTC period)) × CSSFinter’, where Nskip is the number of  skipped MG occasions in the measurement period.

Issue 3-3: Formulas for measurement delay extension with DRX

For the scenario of DRX, same concept shall be applied in here, i.e.,

- RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

  - DRX cycle ≤ 320 ms

    - Max(200 ms, Ceil((8 + Nskip) 1.5  Kgap)  Max(MGRP, SMTC period, DRX cycle)  CSSFinter)

  - DRX cycle > 320 ms

    - Ceil ((8 + Nskip)  Kgap)  DRX cycle  CSSFinter

- RAN4 to define measurement delay extension with DRX cycle <= 320ms due to measurement skipping for inter-frequency measurements with gap as: ‘Max(200 ms, Ceil(( 8 + Nskip ) * Kgap) × Max(MGRP , SMTC period, DRX cycle)) × CSSFinter’, where Nskip is the number of  skipped MG occasions in the measurement period.

- RAN4 to define measurement delay extension with DRX cycle > 320ms due to measurement skipping for inter-frequency measurements with gap as: ‘Ceil ((8 + Nskip)  Kgap)  DRX cycle  CSSFinter’, where Nskip is the number of  skipped MG occasions in the measurement period.

Issue 3-4: Conditions for measurement skipping

Furthermore, cancelling multiple measurement gaps occasions might result in mobility performance degradation. Therefore, RAN4 should discuss how to limit such impact. One option is to limit the max number of cancelled gap occasions within a specific time window.

- RAN4 shall discuss and define the max number of occasions cancelled during a measurement period.

# Summary

In this contribution, we have the following proposals:

Proposal 1: RAN4 shall keep the number of measurement samples for a frequency layer for L3 measurements unimpacted due to XR traffic.

Proposal 2: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as: ‘Max(200 ms, Ceil(( 8 + Nskip ) * Kgap) × Max(MGRP , SMTC period)) × CSSFinter’, where Nskip is the number of  skipped MG occasions in the measurement period.

Proposal 3: RAN4 to define measurement delay extension with DRX cycle <= 320ms due to measurement skipping for inter-frequency measurements with gap as: ‘Max(200 ms, Ceil(( 8 + Nskip ) * Kgap) × Max(MGRP , SMTC period, DRX cycle)) × CSSFinter’, where Nskip is the number of  skipped MG occasions in the measurement period.

Proposal 4: RAN4 to define measurement delay extension with DRX cycle > 320ms due to measurement skipping for inter-frequency measurements with gap as: ‘Ceil ((8 + Nskip)  Kgap)  DRX cycle  CSSFinter’, where Nskip is the number of  skipped MG occasions in the measurement period.

Proposal 5: RAN4 shall discuss and define the max number of occasions cancelled during a measurement period.

# References

- RP-234080, “New WID: XR (eXtended Reality) for NR Phase 3”, MediaTek (Moderator), Dec 2023.

- R4-2504911, “WF on RRM requirements for XR_Ph3”, Nokia, Wuhan, China, April, 2025.