3GPP TSG-RAN WG4 Meeting #115	R4-2507620

Malta, MT, May 19 – 23, 2025

Source:	Nokia

Title:	NR Demodulation Performance Phase 5: BS demodulation performance requirements for MMSE-IRC

Agenda item:	7.17.4

Document for:	Discussion

Introduction

Further agreements on Rel-19 NR demodulation performance Phase 5: BS Demodulation MMSE-IRC from RAN4#114bis meeting are documented in the way forward (WF) [1]. Those which will impact the specifications are summarized below [1]:

Number of interferers for 2Rx - Online agreement - Consider 1 interferer for 2Rx for both HomNet and HetNet scenarios Propagation condition - Online agreement: - TDLA30-10 for HetNet and TDLC300-100 for HomNet - RAN4 will discuss the applicability for different BS classes and different test scenarios Manufacturer declaration on MMSE-IRC receiver - Agreement: - Introduce new manufactory declaration for BS supporting enhanced performance requirements.

This contribution provides our views on the open issues in [1] and another open issue raised by the moderator in the email discussion prior to RAN4#115 meeting.

Discussion

Referring to the WF [1], the remaining issues can be classified into 2 topics, namely, Simulation and Applicability rules. The open issue raised by the moderator in RAN4#115 pre-meeting email discussion, can be classified into the Simulation topic. In the following, our views on the remaining open issues are provided in each of the respective subsections.

Simulation

Simulation results collection

In RAN4#114bis, not all companies provided their simulation results. Furthermore, not all agreed test cases were simulated by most companies.  Hence, the full set of simulation results needs to be collected in RAN4#115 as suggested in the following [1]:

Simulation result collection - Companies are encouraged to provide simulation results for all the agreed test cases in the next meeting.

To address the issue above, in this meeting we provide our simulation results in [2].

DMRS configuration

There is one issue raised by the moderator in a pre-meeting email discussion, namely:

Issue: DMRS configuration - Ratio of PUSCH EPRE to DM-RS EPRE Option 1: Ratio of PUSCH EPRE to DM-RS EPRE - -3dB with Number of DM-RS CDM group(s) without data = 2 Other options are not precluded

Referring to [6] Table: 8.2.1.1-1 Test parameters for testing PUSCH, the Ratio of PUSCH EPRE to DM-RS EPRE = -3dB and Number of DM-RS CDM group(s) without data = 2.

In 38.104, Table 8.2.1.1-1, the following parameters are stated: Ratio of PUSCH EPRE to DM-RS EPRE = -3dB and Number of DM-RS CDM group(s) without data = 2.

RAN4 shall use Ratio of PUSCH EPRE to DMRS EPRE = -3dB and Number of DM-RS CDM group(s) without data = 2.

Applicability rules

The remaining open issues in this subtopic are below:

Applicability for different BS classes and test scenarios - Online agreement: - Follow the LTE approach – Only specifies that HomNet requirements are not applicable for Local Area BS. - FFS whether to consider additional applicability rule on how the test is performed: - Option 1: If a BS declared to support Wide Area BS, Medium Range BS and pass the tests for HomNet, the test for HetNet can be skipped - Option 2: Not to consider additional applicability rule on how the test is performed for different test scenarios or BS classes.

Referring to LTE in [5], the “Note*: Not applicable for Local Area BS and Home BS.” inside the tables 8.2.6.1-1 to 8.2.6.1-6 implies that the HomNet requirements are not applicable for Local Area BS and Home BS. As agreed in RAN4#114bis, Rel-19 NR BS MMSE-IRC will follow such an applicability rule as in LTE on Local Area BS. The remaining issue for discussion in RAN4#115 is whether to have an additional applicability rule on how the test is performed for Wide Area and Medium Range BS.

It is more probable that HomNet will use Wide Are or Medium Range BS rather than Local Area BS. Hence, Wide Area BS and Medium Range BS need to be tested with HomNet requirements. As Local BS needs only to pass HetNet requirements, so do Wide Area BS and Medium Range BS need to pass HomNet requirements.

Wide Area BS and Medium Range BS are more probable to be deployed in HomNet rather than Local BS. Hence, Wide Area BS and Medium Range BS need to be tested with HomNet requirements.

As Local BS needs only to pass HetNet requirements, so do Wide Area BS and Medium Range BS need to pass only HomNet requirements.

As in LTE and already agreed in RAN4#114bis, RAN4 to adopt the following for NR: “HomNet requirements are not applicable for Local Area BS.” Furthermore, RAN4 to add the following: “For Wide Area BS and Medium Range BS, it is sufficient to pass HomNet requirements”.

Conclusion

In this contribution, we provide our views on Rel-19 NR Demodulation Performance Phase 5 UL BS MMSE-IRC. The following Observations and Proposals were made:

Observation 1: In 38.104, Table 8.2.1.1-1, the following parameters are stated: Ratio of PUSCH EPRE to DM-RS EPRE = -3dB and Number of DM-RS CDM group(s) without data = 2.

Proposal 1: RAN4 shall use Ratio of PUSCH EPRE to DMRS EPRE = -3dB and Number of DM-RS CDM group(s) without data = 2.

Observation 2: Wide Area BS and Medium Range BS are more probable to be deployed in HomNet rather than Local BS. Hence, Wide Area BS and Medium Range BS need to be tested with HomNet requirements.

Observation 3: As Local BS needs only to pass HetNet requirements, so do Wide Area BS and Medium Range BS need to pass only HomNet requirements.

Proposal 2: As in LTE and already agreed in RAN4#114bis, RAN4 to adopt the following for NR: “HomNet requirements are not applicable for Local Area BS.” Furthermore, RAN4 to add the following: “For Wide Area BS and Medium Range BS, it is sufficient to pass HomNet requirements”.

- References

- R42504768, Way Forward for [114bis][321] NR_demod_Ph5_Part1_General_BS, 3GPP RAN4#114, China Telcom, Wuhan, China, April 7-11, 2025

- R4-2507621, Simulation Results – NR BS Demodulation Performance Requirements for MMSE-IRC, 3GPP RAN4#115, Nokia, Malta, MT, May 19-23, 2025

- R4-2502028, Simulation results summary for BS requirement with MMSE-IRC receiver, 3GPP RAN4#114, Samsung, Athens, Greece, February 17-21, 2025

- 3GPP TR 36.884, Performance requirements of MMSE-IRC receiver for LTE BS, v13.1.0, 09-2016

- 3GPP TS 36.104, Evolved Universal Terrestrial Radio Access (E-UTRA) - Base Station (BS) radio transmission and reception, v18.5.0, 03-2024

- 3GPP TS 38.104, NR - Base Station (BS) radio transmission and reception, V18.6.0, 06-2024