3GPP TSG-RAN WG4 Meeting #115	R4-2507621

Malta, MT, May 19 – 23, 2025

Source:	Nokia

Title:	Simulation Results – NR BS Demodulation Performance Requirements for MMSE-IRC

Agenda item:	7.17.4

Document for:	Information

Introduction

Further agreements on simulation parameters have been made during RAN4#114bis meeting as documented in [5]. Those agreements, together with other previous agreements in [1], [2], [3] and [4], would be used in the link-level simulations for defining requirements for Rel-19 BS MMSE-IRC.

In this contribution, we provide our simulation results based on the agreements documented in the WFs. Our views on Rel-19 BS Demodulation MMSE-IRC can be found in [6].

Receiver Algorithms, Interference Modeling and Key Parameters

Receiver Algorithms

As described in [2], the agreements on the receiver algorithms for the MMSE-IRC and MMSE are as follows:

MMSE-IRC receiver assumption - Reuse the same receiver model for MMSE-IRC in TR36.884 [7] as below: ## MMSE-IRC receiver The MMSE-IRC receiver weight matrix is usually defined as where and denote the estimated channel matrix and covariance matrix, respectively. To obtain the MMSE-IRC receiver weight matrix, the covariance matrix including the sources of inter-cell interference needs to be estimated. The covariance matrix is estimated at DM-RS REs by following equations: , , where the DM-RS symbols are used to estimate the covariance matrix, and Nsp is the number of sampling DM-RS REs. The estimation of interference covariance matrix is performed at per PRB and per TTI basis.. Baseline receiver assumption (for performance gain study only) - MMSE receiver as defined in TR36.884 [7]: ## Baseline receiver: MMSE receiver The MMSE receiver weight matrix is expressed as follow: , where and denote the estimated channel matrix and noise power, respectively. P1 is the transmission power of UE scheduled by the serving cell and is equal to ..

Interference Modeling

As stated in [3], the INR (Interference-to-Noise Ratio) is now used instead of the DIP (Dominant Interference Proportion) as in LTE. Nonetheless, the values of the INR themselves are derived from their corresponding DIP values from LTE in [7]. They are tabulated in [3] as below:

- Cases for link level evaluations based on the legacy LTE profile Scenario; (DIP1, DIP2) dB; Transferred (INR1, INR2) dB. HetNet with 1 interferer; (-0,43, N/A); (9.82, N/A). HetNet with 2 interferers; (-0,43, -13,78); (12.38, -0.97). HomNet with 1 interferer; (-1,11, N/A); (5.35, N/A). HomNet with 2 interferers; (-1,11, -10,91); (7.28, -2.52)..

Other Parameters

The following parameters have been agreed in [3], [4], [1] and [5]:

DMRS configuration - Cover DMRS configuration type 1.[3] - Not to consider DMRS configuration type 2.[1] - Use DMRS port #0 for both serving and interfering PUSCH [3] - On DMRS sequence generation, use different DMRS sequence for each gNB [3]: - For CP-OFDM: nSCID =0 for all cells and NID0=0, 1 and 2 for cell #0 (serving), cell #1 and cell#2 (interfering) respectively PUSCH mapping type [3] - Cover both PUSCH mapping Type A and Type B - PUSCH time domain allocation S=0, L=14 Test metric [3] - Use 70% of maximum throughput as metric for NR BS MMSE-IRC demodulation Antenna configuration [3] - Cover 2/4/8 Rx with 1Tx for simulation evaluation purpose Propagation condition [4][5] – For the target cell [5]: TDLA30-10 for HetNet and TDLC300-100 for HomNet – For the interference cell, use TDLC300-100.[4] Channel bandwidth and waveform [1] - For CP-ODFM: - For 15kHz SCS, cover 5MHz, 20MHz - For 30kHz SCS, cover 10MHz, 100MHz - Not to cover DFT-s-OFDM MCS [1][5] - MCS 16 for 2Rx, 4Rx and 8Rx Number of interferers for 2Rx [5] - Consider 1 interferer for 2Rx for both HomNet and HetNet scenarios Test requirement definition [1] Use the SNR based requirement definition

Simulation Results

In this section, the simulation results based on the agreed simulation parameters, cf.,  [1], [2], [3], [4] and [5] are presented.

The following parameters concerning the Interfering Cells’ (ICs) signals are fixed, namely,

- Modulation: 16QAM [3]

- Propagation condition: TDLC300-100 [4]

Furthermore, the following note is added in [4]:

- Note: Reuse the same FRC A.3, A.4 and A.5 with Additional DM-RS position = pos1 and 1 transmission layer defined in TS38.104 [8].

Considering the DMRS configuration, following Table: 8.2.1.1-1 Test parameters for testing PUSCH in [8], the simulation results presented here are using:

- Number of DM-RS CDM group(s) without data = 2, and

- Ratio of PUSCH EPRE to DM-RS EPRE = -3 dB.

Subcarrier Spacing 15 KHz

The simulation results for SCS 15 KHz are tabulated in Table 1 and Table 2 for Bandwidth (BW) sizes of 5MHz and 20 MHz, respectively.

Table 1. SCS 15 KHz, BW 5 MHz

| Number of |  | Serving Cell |  | (INR1, INR2) dB | Mapping Type | SNR dB |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rx | ICs | MCS | Propagation Condition |  |  | MMSE-IRC | MMSE | Gain |
| 2 | 1 | 16 | TDLA30-10 | (9.82, N/A) / [HetNet] | A | 10,4 | 14,8 | 4,3 |
|  |  |  |  |  | B | 10,4 | 14,7 | 4,3 |
|  |  |  | TDLC300-100 | (5.35, N/A) / [HomNet] | A | 11,4 | 13,2 | 1,8 |
|  |  |  |  |  | B | 11,4 | 13,3 | 1,8 |
| 4 | 2 |  | TDLA30-10 | (12.38, -0.97) / [HetNet] | A | 7,0 | 13,5 | 6,5 |
|  |  |  |  |  | B | 7,0 | 13,4 | 6,4 |
|  |  |  | TDLC300-100 | (7.28, -2.52) / [HomNet] | A | 7,4 | 11,0 | 3,6 |
|  |  |  |  |  | B | 7,5 | 11,0 | 3,5 |
| 8 | 2 |  | TDLA30-10 | (12.38, -0.97) / [HetNet] | A | 3,0 | 10,7 | 7,8 |
|  |  |  |  |  | B | 3,0 | 10,7 | 7,7 |
|  |  |  | TDLC300-100 | (7.28, -2.52) / [HomNet] | A | 3,2 | 7,8 | 4,6 |
|  |  |  |  |  | B | 3,1 | 7,7 | 4,6 |

Table 2. SCS 15 KHz, BW 20 MHz

| Number of |  | Serving Cell |  | (INR1, INR2) dB | Mapping Type | SNR dB |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rx | ICs | MCS | Propagation Condition |  |  | MMSE-IRC | MMSE | Gain |
| 2 | 1 | 16 | TDLA30-10 | (9.82, N/A) / [HetNet] | A | 10,8 | 15,0 | 4,2 |
|  |  |  |  |  | B | 10,7 | 15,0 | 4,3 |
|  |  |  | TDLC300-100 | (5.35, N/A) / [HomNet] | A | 11,4 | 13,1 | 1,7 |
|  |  |  |  |  | B | 11,4 | 13,1 | 1,7 |
| 4 | 2 |  | TDLA30-10 | (12.38, -0.97) / [HetNet] | A | 6,8 | 13,7 | 6,9 |
|  |  |  |  |  | B | 6,7 | 13,6 | 6,9 |
|  |  |  | TDLC300-100 | (7.28, -2.52) / [HomNet] | A | 7,6 | 11,1 | 3,5 |
|  |  |  |  |  | B | 7,6 | 11,1 | 3,5 |
| 8 | 2 |  | TDLA30-10 | (12.38, -0.97) / [HetNet] | A | 3,0 | 10,9 | 7,9 |
|  |  |  |  |  | B | 2,9 | 10,7 | 7,8 |
|  |  |  | TDLC300-100 | (7.28, -2.52) / [HomNet] | A | 3,3 | 8,2 | 4,8 |
|  |  |  |  |  | B | 3,4 | 8,0 | 4,6 |

Subcarrier Spacing 30 KHz

The simulation results for SCS 30 KHz are tabulated in Table 3 and Table 4 for BW sizes of 10 MHz and 100 MHz, respectively

Table 3 SCS 30 KHz, BW 10 MHz

| Number of |  | Serving Cell |  | (INR1, INR2) dB | Mapping Type | SNR dB |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rx | ICs | MCS | Propagation Condition |  |  | MMSE-IRC | MMSE | Gain |
| 2 | 1 | 16 | TDLA30-10 | (9.82, N/A) / [HetNet] | A | 10,6 | 14,6 | 4,0 |
|  |  |  |  |  | B | 10,6 | 14,6 | 4,0 |
|  |  |  | TDLC300-100 | (5.35, N/A) / [HomNet] | A | 11,7 | 13,4 | 1,7 |
|  |  |  |  |  | B | 11,8 | 13,3 | 1,6 |
| 4 | 2 |  | TDLA30-10 | (12.38, -0.97) / [HetNet] | A | 7,2 | 13,4 | 6,2 |
|  |  |  |  |  | B | 7,2 | 13,3 | 6,2 |
|  |  |  | TDLC300-100 | (7.28, -2.52) / [HomNet] | A | 8,1 | 11,3 | 3,2 |
|  |  |  |  |  | B | 8,1 | 11,3 | 3,2 |
| 8 | 2 |  | TDLA30-10 | (12.38, -0.97) / [HetNet] | A | 3,3 | 10,9 | 7,6 |
|  |  |  |  |  | B | 3,3 | 10,8 | 7,4 |
|  |  |  | TDLC300-100 | (7.28, -2.52) / [HomNet] | A | 3,9 | 8,3 | 4,5 |
|  |  |  |  |  | B | 3,9 | 8,2 | 4,3 |

Table 4. SCS 30 KHz, BW 100 MHz

| Number of |  | Serving Cell |  | (INR1, INR2) dB | Mapping Type | SNR dB |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rx | ICs | MCS | Propagation Condition |  |  | MMSE-IRC | MMSE | Gain |
| 2 | 1 | 16 | TDLA30-10 | (9.82, N/A) / [HetNet] | A | 11,7 | 15,4 | 3,7 |
|  |  |  |  |  | B | 11,7 | 15,4 | 3,7 |
|  |  |  | TDLC300-100 | (5.35, N/A) / [HomNet] | A | 11,7 | 13,3 | 1,6 |
|  |  |  |  |  | B | 11,7 | 13,2 | 1,6 |
| 4 | 2 |  | TDLA30-10 | (12.38, -0.97) / [HetNet] | A | 7,7 | 14,0 | 6,3 |
|  |  |  |  |  | B | 7,7 | 14,0 | 6,3 |
|  |  |  | TDLC300-100 | (7.28, -2.52) / [HomNet] | A | 8,0 | 11,3 | 3,4 |
|  |  |  |  |  | B | 8,0 | 11,3 | 3,3 |
| 8 | 2 |  | TDLA30-10 | (12.38, -0.97) / [HetNet] | A | 3,5 | 11,3 | 7,8 |
|  |  |  |  |  | B | 3,6 | 11,3 | 7,7 |
|  |  |  | TDLC300-100 | (7.28, -2.52) / [HomNet] | A | 3,8 | 8,5 | 4,7 |
|  |  |  |  |  | B | 3,8 | 8,4 | 4,6 |

Summary

In this contribution, we provide our simulation results on Rel-19 NR Demodulation Performance Phase 5: BS demodulation performance for MMSE-IRC. More on our observations and the corresponding proposals can be found in [6].

- References

- R4-2502272, Way Forward for [114][323] NR_demod_Ph5_Part1_General_BS, 3GPP RAN4#114, China Telcom, Athens, Greece, January 17-21, 2025

- R4-2413570, WF on NR demodulation performance: Phase 5, 3GPP RAN4#112, China Telcom, Maastricht, Netherlands, August 19-23, 2024

- R4-2416548, WF on [112bis][319] NR_demod_Ph5_Part1_General_BS, 3GPP RAN4#112bis, China Telcom, Hefei, China, October 14-18, 2024

- R4-2419780, Way Forward for [113] [321] NR_demod_Ph5_Part1_General_BS, 3GPP RAN4#113, China Telcom, MediaTek, Orlando, USA, November 18-22, 2024

- R4-2504768, Way Forward for [114bis][321] NR_demod_Ph5_Part1_General_BS, China Telcom, Wuhan, China, April 7-11, 2025

- R4-2507620, NR Demodulation Performance Phase 5: BS demodulation performance requirements for MMSE-IRC, 3GPP RAN4#115, Nokia, Malta, MT, May 19-23, 2025

- 3GPP TR 36.884, Performance requirements of MMSE-IRC receiver for LTE BS, v13.1.0, 09-2016

- 3GPP TS 38.104, NR - Base Station (BS) radio transmission and reception, V18.6.0, 06-2024