3GPP TSG-RAN WG4 Meeting #115	R4-2507623

Malta, MT, May 19 – 23, 2025

Source:	Nokia

Title:	On Enhancements for Air-to-Ground Network for NR

Agenda item:	7.12.7

Document for:	Discussion

Introduction

The Way Forward (WF) from the discussions on Rel-19 Enhancements for Air-to-ground network for NR on RAN4#114bis is documented in [1]. The discussions covered 3 topics, namely, DL CA, DL MIMO and UL MIMO. There were some agreements made during RAN4#114bis, and some of them are summarized below:

<Topic 1> DL CA Issue 1-1-1: CA Capability Agreement: - CA_C - CA_AX Issue 1-1-2: CA Duplex mode Agreement: - TDD 30 kHz + TDD 30 kHz - FDD 15 kHz + TDD 30 kHz FFS: - FDD 15 kHz + FDD 15 kHz Issue 1-1-3: Test setups Issue 1-1-3-1: TDD pattern Agreement: - Use legacy TDD pattern 7DS2U as the staring point FFS: - new TDD pattern 30D4S6U Issue 1-1-3-3: MCS Agreement: - 16QAM MCS 13, 64QAM MCS 22, 256QAM MCS 24 for initial simulation Issue 1-1-3-4: Propagation condition Agreement: - AWGN+220Hz for FDD 15kHz，AWGN+500Hz for TDD 30kHz Issue 1-1-3-5: Antenna configuration Agreement: - 2T2R and 2T4R Issue 1-1-3-6: Test metric Agreement: - 70% of max TP Issue 1-1-3-7: Bandwidth Agreement: - all bandwidth configuration for FDD 15kHz and TDD 30kHz - Single carrier requirements for each of the considered bandwidth sizes in the bandwidth combination sets shall then be specified. FFS: - RAN4 to clarify the bandwidth combination sets for the FR1 intra-band contiguous CA in CA_n79C and for FR1+FR1 inter-band CA in CA_n3A-n39A. Issue 1-1-3-8: Others Agreement: - For the other test parameters, the configuration in Table 5.2A-1/2/3 in TS 38.101-4 can be reused <Topic 2> DL MIMO Issue 2-1-1: Antenna configuration Agreement: - 2T2R and 2T4R FFS the antenna correlation: - Proposal 1: XP-low Issue 2-1-2: TDD pattern Agreement: - Use legacy TDD pattern 7DS2U as the staring point - FFS whether the new TDD pattern 30D4S6U will be used Issue 2-1-3: Rank Agreement: - Rank 2 Issue 2-1-4: MCS Agreement: - For initial simulation, consider 16QAM Table 1 MCS13, 64QAM Table 1 MCS 22 and 256QAM Table 2 MCS 24 Issue 2-1-5: Propagation condition Agreement: - AWGN+220Hz for FDD 15kHz，AWGN+500Hz for TDD 30kHz Issue 2-1-6: Other simulation assumptions Agreement: - For other simulation assumptions, the agreements from Rel-18 can be reused Issue 2-2-1: PMI Agreement: - No need to introduce PMI requirement

In [1], the open issues on the above-mentioned 3 topics are also documented.

In this contribution, we share our views on the open issues to be discussed in RAN4#115.

Discussion

DL CA

The issues in this topic that are going to be addressed here are:

Issue 1-1-2: CA Duplex mode Agreement: - TDD 30 kHz + TDD 30 kHz - FDD 15 kHz + TDD 30 kHz FFS: - FDD 15 kHz + FDD 15 kHz Issue 1-1-3-7: Bandwidth Agreement: - all bandwidth configuration for FDD 15kHz and TDD 30kHz - Single carrier requirements for each of the considered bandwidth sizes in the bandwidth combination sets shall then be specified. FFS: - RAN4 to clarify the bandwidth combination sets for the FR1 intra-band contiguous CA in CA_n79C and for FR1+FR1 inter-band CA in CA_n3A-n39A.

Examples of band combination for intra-band contiguous CA and FR1+FR1 inter-band CA are given in the Objective of the Core part in [2] as highlighted below:

### 4.1 Objective of SI or Core part WI or Testing part WI The core part of the work item includes: - Specify the RF and RRM core requirements for intra-band co-located and inter-band co-located DL CA [RAN4]: - FR1 intra-band contiguous CA - Example band combination: CA_n79C - FR1+FR1 inter-band CA - Example band combination: CA_n3A-n39A - Specify the RF requirements for support of UL MIMO with 2TX for single CC for UE. [RAN4] - Specify the TAE requirements for support of DL MIMO for BS. [RAN4].

The example band combinations given in the WID are CA_n79C and CA_n3A_n39A. n79 is a TDD band, n3 is an FDD band and n39 is a TDD band.

RAN4 to keep the agreements in RAN4#114bis and prioritize TDD 30 kHz + TDD 30 kHz and FDD 15 kHz + TDD 30 kHz.

For bandwidth issue, it was already agreed in RAN4#114bis that all bandwidth configuration for FDD 15kHz and TDD 30kHz will be considered.

If there are no current agreements in RAN4 on Rel-19 ATG stating specific bandwidth combination sets for CA_n79C and CA_n3A-n39A, RAN4 to keep the agreements in RAN4#114bis to consider all bandwidth configuration for FDD 15 kHz and TDD 30 kHz.

DL MIMO

The open issue to be addressed in this topic is:

Issue 2-2-2: RI - Proposal 1: - Investigate the feasibility of introducing the RI requirement. - For antenna configuration and propagation channel, reuse the configuration from DL MIMO demodulation test. For other test parameters, reuse the configuration from legacy. - Decide the SNR and γby simulation results

It is already agreed in RAN4#114bis that Rel-19 ATG will consider only Rank 2 and no PMI requirements to be specified.

RAN4 to not specify requirements for RI.

UL MIMO

The open issues to be addressed here are:

Issue 3-1-1: Whether to reuse existing requirements for ATG UL MIMO Agreement: - The legacy PUSCH test cases in Clause 8.2.1, 8.2.2 and 8.2.3 with 2Tx 2 layer will be applied for ATG Issue 3-1-2: Whether to introduce new test cases for ATG UL MIMO - Proposals - Proposal 1: - incremental PUSCH 2 layer test cases need to be introduced - Proposal 2: - Do not introduce new 2Tx PUSCH requirements for ATG scenario

In RAN4#114bis, it has already agreed that the legacy PUSCH test cases in Clause 8.2.1 with 2Tx 2 layer will be applied for ATG.

As it has already agreed in RAN4#114bis that PUSCH test cases in Clause 8.2.1 will be applied for Rel-19 ATG, RAN4 do not need to introduce new 2Tx PUSCH requirements.

Conclusion

In this contribution, we provide our views on Rel-19 Enhancements for ATG network for NR. The following Observations and Proposals were made:

Proposal 1: RAN4 to keep the agreements in RAN4#114bis and prioritize TDD 30 kHz + TDD 30 kHz and FDD 15 kHz + TDD 30 kHz.

Observation 1: For bandwidth issue, it was already agreed in RAN4#114bis that all bandwidth configuration for FDD 15kHz and TDD 30kHz will be considered.

Proposal 2: If there are no current agreements in RAN4 on Rel-19 ATG stating specific bandwidth combination sets for CA_n79C and CA_n3A-n39A, RAN4 to keep the agreements in RAN4#114bis to consider all bandwidth configuration for FDD 15 kHz and TDD 30 kHz.

Proposal 3: RAN4 to not specify requirements for RI.

Proposal 4: As it has already agreed in RAN4#114bis that PUSCH test cases in Clause 8.2.1 will be applied for Rel-19 ATG, RAN4 do not need to introduce new 2Tx PUSCH requirements.

- References

- R4-2504755, WF on NR_ATG_enh demodulation requirement, 3GPP RAN4#14bis, CMCC, Wuhan, China, April 7-11, 2025

- RP-243285, Revised WID on Enhancements for Air-to-ground network for NR, 3GPP RAN#106, CMCC, et al., Madrid, Spain, December 9-12, 2024