3GPP TSG-RAN WG4 Meeting #115	R4-2507624

Malta, MT, May 19 – 23, 2025

Source:	Nokia

Title:	On Demodulation and CSI Reporting Requirements for CSI Prediction

Agenda item:	7.21.7

Document for:	Discussion

Introduction

The Work Item Description (WID) of Rel-19 Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface is documented in [1]. The Objective of the Performance part of the Work Item (WI) is described as follows [1]:

### 4.2 Objective of Performance part WI NOTE: Leave empty if the WI proposal does not contain a RAN performance part. - For Beam Management, Positioning Accuracy enhancement and CSI prediction use cases, specify performance requirements and test cases for AI/ML LCM procedures (including performance monitoring) and UE features enabled by UE-sided models - Specify necessary performance requirements and tests (including metrics) for the above-mentioned use cases - Specify necessary test cases and performance requirements for LCM procedure, including performance monitoring. Note: the following aspects may be considered: - Relation to legacy requirements - Performance monitoring and LCM aspects considering use-case specifics - Generalization aspects - Static/non-static scenarios/conditions and propagation conditions for testing (e.g. CDL, field data, etc.) - UE processing capability and limitations - Post-deployment validation due to model change/drift - RAN5 aspects related to testability and interoperability to be addressed on a request basis

As stated above, RAN4 needs to specify necessary performance requirement for CSI prediction use case.

To address the WI’s Objective of the Performance part, in RAN4#115 meeting, one Agenda Item is dedicated for discussions on Demodulation and/or CSI reporting requirements for CSI prediction.

This contribution provide our initial views on demodulation and/or CSI reporting requirements for CSI prediction.

Discussion

General Issue

The WID of a WI should list all the impacted specifications in which the outputs of the WI will be documented. For Rel-19 AI/ML for NR Air Interface, the impacted specifications are listed in the WID [1] as follows:

# 5 Expected Output and Time scale New specifications {One line per specification. Create/delete lines as needed}. Type; TS/TR number; Title; For info at TSG#; For approval at TSG#; Remarks. Impacted existing TS/TR {One line per specification. Create/delete lines as needed}. TS/TR No.; Description of change; Target completion plenary#; Remarks. 38.300; Feature introduction.; TSG#109; Core part. 38.214; Feature introduction.; TSG#109; Core part. 38.215; Feature introduction.; TSG#109; Core part. 38.331; Feature introduction.; TSG#109; Core part. 38.305; Feature introduction.; TSG#109; Core part. 37.355; Feature introduction.; TSG#109; Core part. 38.455; Feature introduction.; TSG#109; Core part. 38.133; Feature introduction.; TSG#109; Core part. 38.133; Feature introduction.; TSG#111; Perf. Part. NOTE: If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec. If an existing spec is affected by both (Core part and Perf. part), then it has to be listed twice with appropriate approval dates.

From the list above, there is no proper specification in which the Demodulation and CSI reporting requirements for CSI prediction can be included. If the work on Demodulation and CSI reporting for CSI prediction is expected to produce the necessary requirements, the outcome would impact TS 38.101-4.

In the WID of Rel-19 AI/ML for NR Air Interface, TS 38.101-4 is not listed as impacted specification for the Performance part.

The WID to be revised to include TS 38.101-4 for Demodulation and CSI report requirements and to be approved in RAN Plenary.

Demodulation and CSI reporting requirements

In the following, we will address the Demodulation and/or CSI reporting requirements for CSI prediction by considering the discussions and agreements in RAN4 on the Core part. As the discussions in the Core part were mainly on issues related to the CSI reporting, we first address the CSI reporting requirements followed by the Demodulation requirements.

CSI reporting requirements

The discussions on AI/ML CSI prediction have started from RAN4#110. Several agreements have been made during several RAN4 meetings (RAN4#110 to RAN4#114bis). The WF of Rel-19 AI/ML air interface from RAN4#114bis meeting is documented in [2]. The following agreements from [2] should be taken into consideration in the discussions on CSI reporting requirements:

Issue 1-1: Codebook to be used in the tests Agreement: - In Rel-19 only PMI requirements based on AI/ML CSI prediction will be defined - use Rel-18 eType II Doppler codebook for AI/ML based CSI predictions Issue 1-2: Requirement baseline Agreement: - Requirement to be introduced for Dopler domain basis N4=1 - N4>1 can be further discussed in the future if needed Issue 1-4: Test metric Requirements to be defined based on the ratio (same metric as for the current PMI reporting requirements in 38.101-4) - tue_AI/ML is 90 % of the maximum throughput obtained at SNRue_AI/ML [a given SNR] using the precoders configured according to the UE reports, - trnd is the throughput measured at SNRue_AI/ML [the same SNR] with random precoding. - trnd is obtained with Rel-15 type I single panel codebook - Note: SNRue_AI/ML is calculated in the same way as SNRue and SNRfollow1,follow2 in section 6.3 of 38.101-4

Furthermore, there was a related discussion on the link-level simulation assumptions in RAN4#114bis, which outcome is documented in [3].

Agreements made in RAN4#114bis is to define only PMI requirements using the same metric as for the current PMI reporting requirements in 38.101-4.

The requirements to be introduced are using Rel-18 eType II Doppler codebook with Dopler domain basis N4=1.

The agreement on the link-level simulation is to use TDLA channel.

RAN4 to specify PMI reporting requirements for CSI prediction for Rel-18 eType II Doppler codebook with Dopler domain basis N4=1 by using the same metric as in 38.101-4.

TDLA should be prioritized, but other propagation models should not be precluded.

Demodulation requirements

It is already agreed in RAN4#114bis that RAN4 will only define PMI requirements for Rel-19 AI/ML CSI prediction use case. Hence, the CSI reporting requirements shall be prioritized over demodulation requirements. Furthermore, if the reported PMI indices using AI/ML and using legacy non-AI/ML are the same, the PDSCH demodulation performance would be the same as the gNB will use the same precoder for the downlink transmission. It is expected that there would be no PDSCH demodulation performance requirements needed.

RAN4 to not specify PDSCH demodulation requirements for Rel-19 AI/ML CSI prediction use case.

Conclusion

In this contribution, we provide our views on Rel-19 AI/ML . The following Observations and Proposals were made:

Observation 1: In the WID of Rel-19 AI/ML for NR Air Interface, TS 38.101-4 is not listed as impacted specification for the Performance part.

Proposal 1: The WID to be revised to include TS 38.101-4 for Demodulation and CSI report requirements and to be approved in RAN Plenary.

Observation 2: Agreements made in RAN4#114bis is to define only PMI requirements using the same metric as for the current PMI reporting requirements in 38.101-4.

Observation 3: The requirements to be introduced are using Rel-18 eType II Doppler codebook with Dopler domain basis N4=1.

Observation 4: The agreement on the link-level simulation is to use TDLA channel.

Proposal 2: RAN4 to specify PMI reporting requirements for CSI prediction for Rel-18 eType II Doppler codebook with Dopler domain basis N4=1 by using the same metric as in 38.101-4.

Proposal 3: TDLA should be prioritized, but other propagation models should not be precluded.

Proposal 4: RAN4 to not specify PDSCH demodulation requirements for Rel-19 AI/ML CSI prediction use case.

- References

- RP-250792, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, 3GPP TSG RAN Meeting #107, Qualcomm, Incheon, S. Korea, March 12-14, 2025

- R4-2505105, WF on requirements for AI/ML air interface normative work, 3GPP TSG RAN4#114bis, Qualcomm, Wuhan, China, April 7-11, 2025

- R4-2505229, Updated simulation assumptions for AIML CSI prediction, 3GPP RAN4#14bis, Oppo, et. al, Wuhan, China, April 7-11, 2025