3GPP TSG-RAN WG4 Meeting #115	R4-2507635

Malta, MT, 19th – 23rd May, 2025

Agenda item:		7.13.3.3

Source:	Nokia

Title:	On TX co-existence spurious emissions requirements

Document for:		Discussion

# Introduction

Way forward [1] was agreed in RAN4#114bis regarding transmitter co-existence spurious emission requirements. The WF includes evaluation methodologies together with assumptions to be used with the evaluation. This contribution further elaborates Nokia’s views on the topic.

# Discussion

## Evaluation methodology

It was agreed in the WF:

### Sub-topic 2-1: Evaluation methodology Way forward: - Consider MCL methodology, including AAS and non-AAS implementations. - TR 25.942 is used as reference for the methodology derivation. - System level evaluations are not precluded. - Aim Rel-19 conclusion.

As record in TR 25.942 [2], the existing TX spurious emission co-existence requirements were specified using deterministic analysis with MCL based on UMTS operating at 2 GHz carrier frequency. These requirements have been reused for E-UTRA and then NR. Therefore, one way to re-evaluate these limits for high frequency bands (as requested by ETSI MSG TFES in [3]) is to use deterministic analysis with MCL on NR operating at high frequency bands like n104.

As an alternative, system level evaluations (by simulation) were not precluded in the agreed WF. Some preliminary system level simulation results are provided in this document.

## Simulation assumptions

It was agreed in the WF:

### Sub-topic 2-4: Frequency bands for co-existence work Way forward: - Aggressor carrier frequencies: prioritize 2 GHz, 3.5 GHz, - Victim frequency bands: prioritize lower bound frequency of band n104, i.e, 6425 MHz. ### Sub-topic 2-7: System-level simulation assumptions Way forward: For AAS BS: follow TR 38.922 as baseline For 2 and 3.5 GHz we reuse 4.4 For non-ASS: follow 37.840 as starting point. For Propagation Model, use TR 38.901 as starting point.

As a reference, the MCL from the system level simulation using the E-UTRA simulation assumptions (including propagation model) in macro-cell environment at 2 GHz frequency range (instead of 3.5 MHz which are yet to be agreed on) in TR 37.840 [4] and TR 37.842 [5] are provided in figures 1 (non-AAS BS) and 2 (AAS BS) below. It can be observed in both simulated cases, over 99% of the coupling loss is above the current MCL assumption of 70 dB.

(a) Overall															(b) Zoom-in

Figure 1: MCL with non-AAS BS at 2 GHz with 750 m ISD

(a) Overall															(b) Zoom-in

Figure 2: MCL with AAS BS at 2 GHz with 750 m ISD

On the other hand, the AAS antenna gain pattern in the out-of-band frequency range has not been agreed in RAN4. However, it can be reasonably assumed that with a large frequency separation between the aggressor and victim frequency bands, the correlation between 2 antenna elements at the aggressor frequency band would approach zero in view from the victim frequency band, and thus the AAS antenna gain pattern from the aggressor to the victim would approach the element gain pattern of the aggressor.

The MCL from the system level simulation using the BS antenna characteristics at 7125 - 8400 MHz frequency range (instead of 6425 MHz which are yet to be agreed on) with the simulation assumptions in macro-cell environment at 14800 - 15350 MHz frequency range in TR 38.922 [6] and the propagation loss calculated at the victim carrier frequency of 6425 MHz are provided in figure 3 below. Here the correlation between 2 antenna elements at the aggressor (2 GHz or 3.5 GHz) frequency band is assumed to be zero in view from the victim (6425 MHz) frequency band.

(a) Overall															(b) Zoom-in

Figure 3: MCL using 7125 - 8400 MHz BS antenna characteristics at 6425 MHz carrier frequency with 450 m ISD

It can be observed from figure 3 that over 99% of the coupling loss is above the current MCL assumption of 70 dB.

# Conclusion

This document provides some preliminary system level simulation results which show that over 99% of the coupling loss is above the current MCL assumption of 70 dB in all simulated macro-cell environments. However, the existing (-52 dBm/MHz) TX spurious emission co-existence requirements for DL protection is derived using 53 dB MCL in micro-cell environment. Therefore, the system level simulation assumptions in micro-cell environment should be discussed and agreed on first to re-evaluate the current MCL assumption using system level evaluations.

Observation: Over 99% of the coupling loss is above the current MCL assumption of 70 dB in all simulated macro-cell environments.

Proposal: To discuss and agree on the system level simulation assumptions in micro-cell environment to re-evaluate the current MCL assumption of 53 dB using system level evaluations.

# References

- R4-2504766 “Way Forward for Tx spurious emissions co-existence limits re-evaluation”, RAN4#114bis, Wuhan, China, 7th-11th April, 2025.

- 3GPP TR 25.942 v18.0.0: “Radio Frequency (RF) system scenarios”.

- RP-242432, “Transmitter spurious emissions: co-existence requirement limits for high frequency bands”, ETSI TC MSG/TFES.

- 3GPP TR 37.840 v12.1.0: “Study of Radio Frequency (RF) and Electromagnetic Compatibility (EMC) requirements for Active Antenna Array System (AAS) base station”.

- 3GPP TR 37.842 v13.3.0: “Evolved Universal Terrestrial Radio Access (E-UTRA) and Universal Terrestrial Radio Access (UTRA; Radio Frequency (RF) requirement background for Active Antenna System (AAS) Base Station (BS)”.

- 3GPP TR 38.922 v19.0.0: “NR; Study on International Mobile Telecommunications (IMT) parameters for 4400 - 4800 MHz, 7125 - 8400 MHz and 14800 - 15350 MHz”.