3GPP TSG-RAN WG4 Meeting #115	R4-2507649

St. Julians, Malta, May 2025

Source	:	Qualcomm Incorporated

Title	:	TP to TR38.749 on 24 GHz EESS band phase 2 requirements : Carrier    Aggregation A-MPR Analysis

Agenda item	:	6.17.2

Document for	:	Approval

# 1.	Introduction

In WRC-19, resolution 750, progressively stronger emissions limits were recommended for UEs and gNBs to protect EESS services in the 23.6 – 24.0 GHz band. The next significant event pertaining to this protected band is the adoption of ‘phase 2’ limits, currently recommended for September 2027, but brought ahead by several regional regulators. In RAN#105, a new WID [17] was approved for this purpose. In this proposal, we present the A-MPR analysis for phase 2 limits in carrier aggregation mode based on prior work demonstrated on phase 1 limits.

# 2. 	TP

/* begin text proposal*/

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1] 3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] ITU-R Radio Regulations, 2020 Edition

[3] EU Decision 2019/784, Commission Implementing Decision (EU) 2019/784 of 14 May 2019 on harmonisation of the 24,25-27,5 GHz frequency band for terrestrial systems capable of providing wireless broadband electronic communications services in the Union.

[4] EU Decision 2020/590, Commission Implementing Decision (EU) 2020/590 of 24 April 2020 amending Decision (EU) 2019/784 as regards an update of relevant technical conditions applicable to the 24,25-27,5 GHz frequency band.

[5] ECC(20) 055 Annex 18, CEPT letter dated 6 March 2020 ‘CEPT response on additional input regarding the impact of the WRC-19 outcome on the harmonised technical conditions for the 26 GHz band’

[6] ECC Decision(18)06, Harmonised technical conditions for Mobile/Fixed Communications Networks (MFCN) in the band 24.25- 27.5 GHz, Approved 06 July 2019, last amended 20 November 2020.

[7] R4-2417982, Discussion on Rel-19 EESS, Huawei, Hisilicon, RAN4#113

[8] R4-2415726, 24 GHz EESS band phase 2 requirements and UE compliance, Qualcomm Incorporated, RAN4#112bis

[9] R4-2415329, Initial considerations on EESS protection requirements, Apple, RAN4#112bis

[10] R4-2416065, mmWave UE spurious emission UE RF, Nokia, RAN4#112bis

[11] 3GPP TS 38.104: “NR; Base Station (BS) radio transmission and reception”.[12] 3GPP TS 38.141-2: "NR; Base Station (BS) conformance testing; Part 2: Radiated conformance testing".

[13] 3GPP TS 38.174: “NR; Integrated Access and Backhaul (IAB) radio transmission and reception”.

[14] 3GPP TS 38.176-2: “NR; Integrated Access and Backhaul (IAB) conformance testing; Part 2: Radiated conformance testing”

[15] 3GPP TS 38.106: “NR repeater radio transmission and reception”.

[16] 3GPP TS 38.115-2: “NR; Repeater conformance testing - Part 2: Radiated conformance testing.

[17] 	RP-242371, ‘New WID on mmWave UE spurious emission’, China Unicom et al, RAN#105, Melbourne, AUS, Sept. 2024

[18] 	R4-2000216, ‘Impact of WRC19 resolutions on FR2’, Qualcomm, RAN4#94-e, Feb. 2020

/* end text proposal*/

/* begin text proposal*/

## 7.5	Carrier Aggregation: A-MPR Analysis

### 7.5.1	Overview of EESS requirements in 23.6 to 24.0 GHz

The emission limits for UEs over the protected EESS band (23.6  f  24.0, GHz) are summarized below.

| Spectrum emission limit (dBm) | Measurement BW (MHz) | Handle | Notes |
| --- | --- | --- | --- |
| +1 | 200 | NS_203 | Phase 1 |
| -5 | 200 | NS_205 | Phase 2 |
| -8 | 200 | NS_201 | Reference for back off derivation |

Table 1: 3GPP ‘additional emissions requirements’ for the EESS band corresponding to WRC19 limits

Signalling solution for the application of phase 2 EESS emission limits is represented by NS_205. As is evident, NS_203 involved a 9 dB relaxation from NS_201 limits, while NS_205 represents a 3 dB relaxation.

Note: Any time there is a new combination of region and applicability date of the phase 2 regulations, a new NS will need to be created.

## 7.5.2	Legacy procedure to derive NS_203 A-MPR

NS_203 backoff values for single CC UL and contiguous allocations in contiguous CA were derived from those of the now obsolete NS_201 by extrapolating for a 9 dB relaxation relative to the NS_201 emissions limit [18]. This procedure is reproduced below, and the reader is directed to [18] for further details. Note the factor of 1 for the rate of signal power reduction to reduction of first order products, and a factor 1/3  for rate of signal power reduction to reduction of third order products.

| PC1 | Back off requirements (dB) by spur type: |  | A-MPR assuming co-location of all types (dB) |
| --- | --- | --- | --- |
|  | First Order | Higher Order |  |
| CA_NS_203, contiguous allocations | 9-1x9=0 | < 9-1/3*9, i.e. < 6 | 6.5 (reducing to 0.0 with enough frequency offset) |

Table 1: PC1 NS_203 A-MPRs derivation

| PC2/3/4 | Back off requirements (dB) by spur type: |  | A-MPR assuming co-location of all types (dB) |
| --- | --- | --- | --- |
|  | First Order | Higher Order |  |
| CA_NS_203, contiguous allocations | 0 | <5-1/3*9, i.e. < 2 | 2.5 (reducing to 0.0 with enough frequency offset) |

Table 2: PC2/3/4/5/6/7 NS_203 A-MPRs derivation

## 7.5.3	Derivation for backoff requirements for Phase 2 (‘NS_205’)

### 7.5.3.1	Contiguous RB allocations

The procedure used in [18] is now repeated for NS_205 in tables 2.4.1-1 -2, with a 3 dB relaxation from NS_201, rather than 9 dB for NS_203.

| PC1, contiguous allocations | Back off requirements (dB) by spur type: |  | Summary back off requirement (dB) |
| --- | --- | --- | --- |
|  | First Order | Higher Order |  |
| CA_NS_205 | 9-1x3=6 | < 9-1/3*3, i.e. < 8 | 9.0 (reducing to 6.0 with enough frequency offset) |

Table 3: PC1 CA_NS_205, derivation of necessary backoff

| PC2/3/4, contiguous allocations | Back off requirements (dB) by spur type: |  | Summary back off requirement (dB) |
| --- | --- | --- | --- |
|  | First Order | Higher Order |  |
| CA_NS_205 | 0 (assume marginal) | <5-1/3*3, i.e. < 4 | 5.0 (reducing to 0.0 with enough frequency offset) |

Table 4: PC2/3/4/5/6/7 CA_NS_205, derivation of necessary backoff

Note that for PC1 with NS_205 (Table 3 above), no amount of frequency offset will restore the required back-off to 0, it stays floored at 6 dB. This is because first order spurs do not fall off with distance from the UL signal (a first order approximation). Therefore, PC1 UEs in n257/n261 will also need backoff due to first order spurs (despite the large frequency distance from the EESS protection band).

### 7.5.3.2	Non-contiguous RB allocations

NC RB allocation refers to the condition where the UL comprises multiple contiguous chunks of RBs separated in frequency. NC RB allocations primarily manifest with non-contiguous CA, but because the standard does not preclude NC RB allocations in contiguous CA, UEs need to check for this latter case as well and take appropriate back off for emissions compliance. Consequently, the standard must provide for both cases, and they are treated together here.

In general, owing to some frequency distance between band edge and the protected band, some non-contiguous RB allocations can ‘concentrate’ more emissions in the protected band than contiguous CA. Consequently, larger back-offs are foreseen for emissions compliance.

Due to wide frequency spread of PA output for the NC allocation case, we turned to measurements from commercially available hardware. Our method is outlined below:

- Employ ‘RAN4 calibration’ procedure of normalizing the UE’s TRP limit to the 0 dB MPR reference waveform, as outlined in 6.2.2 in TS38.101-2

- 2 NC RB allocation test condition: Since the measurement BW is large for NS_205, power in signal is more important than PSD. In this context, only the relative powers in the two clusters are important, the RB lengths in each allocation less so. To capture the worst-case relative power condition (to maximize low side IMD product), our tests were conducted with a 2RB + 1RB CP-OFDM UL with equal PSD.

- Correction for higher number of RB allocations: As the number of contiguous RB allocations that comprise the NC RB allocation arrangement increases, the amount of power in IMD tones inside the protected band can increase from the 2-cluster baseline. Mathematical analysis shows an additional 2 dB of back off is required to cover these conditions.

For Steps 1 and 2 (i.e. limited to 2-allocation consideration), our observations from measurements are presented below.

| UE power class | TRP limit | Back-off needed for 2-allocation UL for NS205 | Back-off needed for UL with more than 2 allocations for NS205 |
| --- | --- | --- | --- |
| PC1 | 35 dBm | 10 dB | 12 dB |
| PC2/3/4/5/6/7 | 23 dBm | 6 dB | 8 dB |

Table 5: Backoffs required for phase 2 EESS compliance

A final consideration is first-order spurs. These are not caused by non-linearities in the PA, and so, remain unchanged whether generated by single RB allocation or multiple. The back-off requirements for first-order spurs are thus identical to those derived for contiguous RB allocations.

The data and arguments are summarized as follows for non-contiguous RB allocations:

| PC1, NC allocations | Back off requirements (dB) by spur type: |  | Summary back off requirement (dB) |
| --- | --- | --- | --- |
|  | First Order (unchanged from contiguous case) | Higher Order |  |
| [NS_205] as well as CA_[NS_205] | 6 | 12 | 13.0 (reducing to 6.0 with enough frequency offset) |

Table 6: PC1 NS_205, derivation of necessary backoff

| PC2/3/4, NC allocations | Back off requirements (dB) by spur type: |  | Summary back off requirement (dB) |
| --- | --- | --- | --- |
|  | First Order (unchanged from contiguous case) | Higher Order |  |
| [NS_205] as well as CA_[NS_205] | 0 | 8 | 8.0 (reducing to 0.0 with enough frequency offset) |

Table 7: PC2/3/4/5/6/7 NS_205, derivation of necessary backoff

/* end text proposal*/